3GPP TSG RAN WG1#112bis-e                              	                R1- 2303382

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.3

Source:	Transsion Holdings

Title:	Discussion of dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and decision

Introduction

In RAN1#110bis-e meeting, the following agreements were made: [1]

Agreement To study and if necessary, specify, enhancements to assist the scheduler in determining waveform switching, such as: - Reporting power headroom related information - Other solutions are not precluded Agreement Dynamic waveform switching enhancement in R18 is applicable to PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1. - Note: The above does not imply that dynamic switching enhancement in R18 is applicable or not applicable to other cases of PUSCH (e.g. PUSCH transmission with a Type 1 or Type 2 configured grant, PUSCH scheduled by DCI format 0_0).

In RAN1#111 meeting, the following agreements were made: [2]

Agreement For DCI based solution, - For supported dynamically scheduled PUSCH, support dynamic waveform switching indication from UL scheduling DCI - Note: “Supported dynamically scheduled PUSCH” is to be confirmed in further discussion - Note: It does not imply that the waveform switching indication applies to other transmission or not - Indication from non-UL scheduling DCI is not supported. Note: the working assumption made in RAN1#110b-e for “Support at least one of the following options for the dynamic waveform indication in R18” does not need to be confirmed Working Assumption Support new 1-bit field for dynamic waveform indication from UL scheduling DCI - Note: no change of the current size alignment procedure between UL DCI and DL DCI Agreement Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching: - Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform - Target waveform can be same or different from waveform of an actual PUSCH transmission - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission - FFS determination of target waveform, target RB allocation, target modulation order - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms - PHR triggering enhancements, e.g. - Network-triggered PHR - PH becomes lower (higher) than a threshold - PHR triggered by waveform switching - Reporting of recommended waveform or request to switch waveform - Other solutions not precluded

In this contribution, we focus on the issues related to dynamic switching between DFT-S-OFDM and CP-OFDM, including the DCI size alignment and enhancements to PHR.

Discussion

DCI size alignment between DFT-S-OFDM and CP-OFDM

- In the existing specification, whether DFT-S-OFDM waveform or CP-OFDM waveform is used for PUSCH transmission depends on whether transform precoding is enabled by the network. For example, if transform precoding is enabled, the DFT-S-OFDM waveform is used, otherwise the CP-OFDM waveform is used. The size of the DCI field or the presence or absence of the DCI field varies for different waveforms. The relevant fields include:

- Precoding information and number of layers

- Antenna ports

- PTRS-DMRS association

- DMRS sequence initialization

- FDRA

- The presence or absence of fields such as ‘PTRS-DMRS association’ and ‘DMRS sequence initialization’ depends on which waveform is used. When DFT-S-OFDM waveform is used, the above fields will not appear. When CP-OFDM waveform is used, the above fields are 1 bit or 2 bits. Secondly, the presence of fields such as ‘Precoding information and number of layers’ and ‘Antenna ports’ does not depend on the waveform, but the size of the fields is related to the waveform. Thirdly, different resource allocation types in fields like ‘FDRA’ limit the selection of waveforms. Therefore, the existing DCI size determination method is based on the RRC configuration. Using different waveforms will lead to inconsistent size of DCI.

- In order to reduce the complexity of PDCCH blind decoding, it is necessary to ensure that the size of DCI indicating CP-OFDM is consistent with that indicating DFT-S-OFDM. Furthermore, the size of each DCI field should be set large enough to meet the required size of the information, regardless of the indicated waveform. Otherwise, the DCI can not provide enough bits of information for a given waveform.

- Proposal 1: For DCI format 0_1/0_2 containing dynamic waveform indication, bit width of each field is set to the maximum between the bit width of the field if transform precoding is disabled and the bit width of the field if transform precoding is enabled, if different.

- Since the size of each DCI field should be set large enough to meet the required size of the information, regardless of the indicated waveform. For a given waveform, the bit width of a field may be large than the required size N, in that case, UE shall decode the field using the N least significant bits.

- Proposal 2: For the waveform indicated in the DCI, if the bit width N of a field would be smaller than the bit width of the field set as per the above, UE decodes the field using N least significant bits.

Enhancements of PHR to report impact of change of waveform

In last meeting, the following FL proposal was proposed and discussed [3].

FL proposal 3-1: Support reporting of power headroom information applicable to a reference PUSCH using a target waveform different from waveform of actual PUSCH: - Support at least the case where reference PUSCH has same RB allocation and modulation order as actual PUSCH (FFS: RB allocation not compatible with target waveform) - FFS: Type of power headroom information (power headroom and/or Pcmax,c) - FFS: Whether it can be included in same PHR as legacy PH types - FFS: Triggering of information

The PHR needs to be enhanced to report the impact of waveform changes, otherwise the network cannot estimate the change of Pcmax under different waveforms, and cannot rely on the MPR requirements from TS 38.101, because the actual power reduction may be significantly larger than the MPR. Especially,  Currently, the UE calculates and reports power headroom and Pcmax based on the waveform used by PUSCH carrying PHR, which cannot reflect the impact of waveform changes on transmission power. One possible solution is that UE calculates and reports the power headroom and Pcmax for different waveforms based on the same underlying transmission parameters.

- Proposal 3: Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform:

- Target waveform can be different from waveform of an actual PUSCH transmission

- Target RB allocation and/or target modulation order can be same from respective properties of an actual PUSCH transmission

- Reports PCMAX,f,c and/or Type 1 power headroom for target waveform in addition to actual used waveform

- Currently, a UE triggers a PHR when the periodic timer expires, or when the prohibit timer expires and the pathloss changes exceed the threshold. However, these trigger conditions do not allow UE to trigger PHR when waveform switching is required. Because waveform switching usually occurs when the UE is close to its maximum transmission power or the UE has sufficient power headroom. One possible solution is that when the power headroom becomes lower than a threshold which means the UE is close to its maximum transmission power, the UE triggers a PHR. Or when the power headroom becomes larger than a threshold which means the UE has sufficient power headroom, the UE triggers a PHR.

- Proposal 4: PHR triggering enhancements:

- PH becomes lower (higher) than a threshold

Conclusion

In this contribution, we have discussed the issues related to dynamic switching between DFT-S-OFDM and CP-OFDM, including the DCI size alignment and enhancements to PHR. Based on the discussion in section 2, we provide the following proposals:

- Proposal 1: For DCI format 0_1/0_2 containing dynamic waveform indication, bit width of each field is set to the maximum between the bit width of the field if transform precoding is disabled and the bit width of the field if transform precoding is enabled, if different.

- Proposal 2: For the waveform indicated in the DCI, if the bit width N of a field would be smaller than the bit width of the field set as per the above, UE decodes the field using N least significant bits.

- Proposal 3: Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform

- Target waveform can be different from waveform of an actual PUSCH transmission

- Target RB allocation and/or target modulation order can be same from respective properties of an actual PUSCH transmission

- Reports PCMAX,f,c and/or Type 1 power headroom for target waveform in addition to actual used waveform

- Proposal 4: PHR triggering enhancements:

- PH becomes lower (higher) than a threshold

Reference

- Draft_Minutes_report_RAN1#110bis-e_v020, 2022-10

- Draft_Minutes_report_RAN#111_v010, 2022-11

- R1-2302222 Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM, InterDigital, 2023-03