3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303387

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | [DRAFT] | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the generation of the Type1 HARQ-ACK codebook for MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current specification, if the Type-1 HARQ-ACK codebook is configured, and if any DCI for scheduling PDSCH reception is not received, the UE does not generate the Type-1 HARQ-ACK codebook and does not transmit a HARQ-ACK PUCCH. Similarly, if the UE is configured with unicast DCI formats and multicast DCI formats for PDSCH reception and UE is provided fdmed-ReceptionMulticast, there are three cases on the DCI reception status from the UE side: - In the first case, only the unicast DCI format is received; - In the second case, only the multicast DCI format is received; - In the third case, both the unicast DCI format and the multicast DCI format are received. In the current specification, the generation of the Type-1 HARQ-ACK codebook only covers the third case. However, the generation of the Type-1 HARQ-ACK codebook is missing for the first case and the second case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the corresponding contents for the first case and the second case above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior is unclear in the first case or the second case above. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<Unchanged parts are omitted>

If a UE is provided fdmed-ReceptionMulticast and the UE is configured to monitor PDCCH for detection of unicast DCI formats and to monitor PDCCH for detection of multicast DCI formats

-	a serving cell is placed in a first set SU of NcellsDL,U serving cells if the UE is configured to monitor PDCCH for DCI formats 1_0/1_1/1_2 for scheduling on serving cell c, and

-	a serving cell is placed in a second set SM of NcellsDL,M serving cells if the UE is configured to monitor PDCCH for detection of DCI format 4_1/4_2 for scheduling on serving cell c, and

-	serving cells are placed in a set according to an ascending order of a serving cell index

the UE generates

-	a Type-1 HARQ-ACK codebook for the set SU of serving cells by setting NcellsDL=NcellsDL,U in the following pseudo-code, if the unicast DCI format is only received from the set SU of serving cells, or

-	a Type-1 HARQ-ACK codebook for the set SM of serving cells by setting NcellsDL=NcellsDL,M in the following pseudo-code, if the multicast DCI format is only received from the set  SM of serving cells, or

-	a Type-1 HARQ-ACK codebook for the set SU and the set SM of serving cells separately by setting NcellsDL=NcellsDL,U and NcellsDL=NcellsDL,M in the following pseudo-code, if both the DCI formats 1_0/1_1/1_2 and the DCI format 4_1/4_2 is received from the set SU  and the set SM of serving cells, respectively. The UE concatenates the HARQ-ACK codebook generated for the set SU followed by the HARQ-ACK codebook generated for the set SM to obtain a total number of OACK HARQ-ACK information bits.

<Unchanged parts are omitted>