3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303388

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | [DRAFT] | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on HARQ-ACK multiplexed in PUSCH for MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For PDSCH for MBS scheduled by DCI format 4_1, the HARQ-ACK feedback can be disabled by RRC signaling. If the HARQ-ACK feedback is disabled, the UE may not generate HARQ-ACK codebook or transmit PUCCH resource if the UE only decodes a downlink DCI with HARQ-ACK disabled. Only if the HARQ feedback is enabled by RRC signaling, the UE may generate HARQ-ACK codebook and transmit the PUCCH resource by following the unicast procedure. This principle is also applied to the HARQ-ACK codebook in the PUSCH. Agreement When UE does not detect any DCI for G-RNTI(s) with HARQ-ACK enabled when at least one configured G-RNTI is with HARQ-ACK enabled by RRC, UE does not need to generate the Type-1 HARQ-ACK codebook for PUCCH transmission. • Whether there is any specification impact can be further discussed.. In the current specification, the UE may generate a HARQ-ACK codebook to be multiplexed in PUSCH if it receives a PDSCH scheduled DCI format 4_1 no matter whether the HARQ feedback is enabled. It is not correct. The condition of HARQ-ACK enabled by RRC signaling should be added. |
| Summary of change: |  | Add the condition of ‘having enabled associated HARQ-ACK information report as described in clause 18’ for DCI format 4_1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour for HARQ-ACK multiplexing in PUSCH is not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

< Unchanged parts are omitted >

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field, and

-	is not provided fdmed-ReceptionMulticast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, or

-	is provided pdsch-HARQ-ACK-Codebook = 'semi-static' only for one of unicast and multicast HARQ-ACK information

the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a TCI state update, or only a PDSCH that is scheduled by a DCI format 1_0 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or scheduled by a DCI format 4_1 having enabled associated HARQ-ACK information report as described in clause 18 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the TCI state update or only for the PDSCH reception as described in clause 9.1.2.

< Unchanged parts are omitted >