3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303391

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | [DRAFT] | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on intra-UE multiplexing with NACK-only PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The physical priority can be configured for MBS. The current intra-UE multiplexing procedures are also applied to MBS. In intra-UE multiplexing, when a physical channel with higher priority overlaps with a physical channel with lower priority in the time domain, the physical channel with lower priority channel should be cancelled and only the physical channel with higher priority is transmitted. The conidition is that the UE would transmit at least the physical channel with higher priority. For NACK-only feedback for MBS, if the UE decodes PDSCH correctly, the UE does not transmit corresponding NACK-only PUCCH. This is similar as the SR transmission where the UE transmits the PUCCH only in the case of positive SR. The UE procedures for SR PUCCH overlapping with the other PUCCH or PUSCH with different priorities can be reused for overlapped channels involving NACK-only PUCCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of UE behavior for NACK-only PUCCH overlapping with the other PUCCH or PUSCH with different priorities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior for NACK-only PUCCH overlapping with the other PUCCH or PUSCH with different priorities is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

<Unchanged parts are omitted>

If a UE would multiplex second multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with first multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the second HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are 'ACK'. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'. If a UE would only transmit a first PUCCH with only positive SR and a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode, where the first and second PUCCHs would overlap in time in a slot and have same priority index, it is up to UE implementation for the UE to transmit either the first PUCCH or the second PUCCH.

When a UE determines that a first PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode that has a priority index overlaps in the time domain with other PUCCHs or PUSCHs that has a different priority indexes, the UE follows the procedure for a first PUCCH with SR overlapping with a second PUCCH or PUSCH in the time domain, as described in clause 9, with replacing a first PUCCH with SR by a first PUCCH with HARQ-ACK information.

<Unchanged parts are omitted>