3GPP TSG RAN WG1 #112bis                               R1-2303393

E-meeting, April 17th – March 21st 2023

Agenda Item:	9.1.1.1

Source:		TCL Communication Ltd.

Title:			Discussion on unified TCI framework extension for multi-TRP operation

Document for:	Discussion and Decision

1 Introduction

During RAN#94-e meeting, the Rel-18 WID of MIMO evolution for DL and UL was approved. The following objective is related to this AI.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, our opinions on unified TCI framework extension for multi-TRP operation are presented.

2 Discussion

2.1 TCI state indication

The following agreement on TCI selection field in DCI format 1_1/1_2 was made in RAN1#112 meeting:

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value

In current spec, a 2-bit SRS resource set filed in DCI format 0_1/0_2 is used to indicate that UL signals/channels are transmitted to which TRP for SDCI based MTRP operation. Similarly, a 2-bit TCI selection filed in DCI format 1_1/1_2 is agreed is used to indicate that DL signals/channels are transmitted from which TRP for SDCI based MTRP operation in the last meeting. In Table 7.3.1.1.2-36 in TS 38.212 V17.1.0, the codepoint “10” and “11” in SRS resource set filed in DCI format 0_1/0_2 have the same usage. Therefore, for the codepoint “11” in TCI selection filed in DCI format 1_1/1_2, we suggest to set the same usage as the codepoint “10” or to reserve this codepoint.

- Two options to use the codepoint “11” in TCI selection filed in DCI format 1_1/1_2.

  - Option 1: set as the same usage as the codepoint “10”.

  - Option 2: reserve this codepoint.

On unified TCI framework extension for S-DCI based MTRP, when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception. The legacy mechanism to determine the default TCI state(s) can be reused. The default TCI state(s) can be the two activated TCI state(s) with lowest ID corresponding to two TRPs.

- The default TCI state(s) can be the two activated TCI state(s) with lowest ID corresponding to two TRPs.

The following agreement on SRS resource set filed in DCI format 0_1/0_2 was made in RAN1#112 meeting:

Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

If the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from those used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission, the spatial Tx filter(s) of the PUSCH transmission should follow those of indicated SRS resource(s) prior to those of indicated joint/UL TCI state(s).

- The spatial Tx filter(s) of the PUSCH transmission should follow those of indicated SRS resource(s) prior to those of indicated joint/UL TCI state(s).

The following agreement on PUCCH transmission on unified TCI framework was made in RAN1#112 meeting:

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

In current spec on spatial relation framework, a MAC CE is used to indicate the association between a PUCCH resource and a spatial relation for MDCI based MTRP operation. According to the indicated spatial relation, a UE knows to transmit the PUCCH to which TRP by using which beam. On unified TCI framework for MDCI based MTRP, there are two indicated TCI state corresponding to different TRP and there is not a MAC CE to indicate the association. The UE does not know to transmit the PUCCH to which TRP by using which beam. Therefore, it is needed to associate the PUCCH resource with TRP.

Actually, a straightforward way is to configure the parameter coresetPoolIndex in each PUCCH resource such that a UE can know transmits the PUCCH to which TRP by using which beam. Some rules may need to set to satisfy this configuration. For example, the PUCCH resource scheduled by the DCI with coresetPoolIndex value 0 cannot be configured with coresetPoolIndex value 1.

- Support opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission.

The following agreement on PUCCH transmission on unified TCI framework was made in RAN1#112 meeting:

Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

In our opinion, Alt3 is the most flexible alternative. No matter a PDSCH transmitted from one TRP or two TRPs, if a UE uses both of two indicated joint/DL TCI states to receive the PDSCH, the UE can receive the whole PDSCH. Alt3 can be set as a default rule such that Alt1 is not necessary. Another Alt including Alt2, Alt3A and Alt4 has a chance to let the UE apply one TCI state to receive the PDSCH. If the PDSCH transmitted from two TRPs, the UE cannot receive the whole PDSCH.

- Support Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception.

2.2 Power control for simultaneous UL transmission

The following agreement on TCI state indication was made in RAN1#109-e meeting:

Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 (approved on May 24th, see below).

In current spec, only one panel in a UE can perform a UL transmission on a carrier of a serving cell at the same time in spite of a UE with one or more panels. That means there is only one UL transmission occasion at the same time. Therefore, a UE is configured a maximum power and the UE only needs to calculate a scheduling power according to the scheduling parameters such as P0, alpha, closed loop index, path loss, PRB number and so on. If the scheduling power is not larger than the maximum power, the UE will utilize the scheduling power for a UL transmission. If the scheduling power is larger than the maximum power, the UE will utilize the maximum power for a UL transmission.

For STxMP, the UE can transmit two UL channels/signals on two panels simultaneously. That means there are at most two UL transmission occasions on two panels at the same time. Therefore, a UE needs to calculate two scheduling power on two panels. The maximum power in this situation is different from that in current spec. A UE can be configured a UE-specific maximum power or two panel-specific maximum powers or both of them.

In our opinion, the best option is that a UE is configured a UE-specific maximum power and calculates two scheduling powers. This option does not introduce the concept of a panel entity such as it has little spec impact. Meanwhile, each panel has a maximum power limit. If the sum of two scheduling powers is not larger than the UE-specific maximum power, a UE allocates two scheduling powers to two panels as actual UL transmission power, respectively. If the sum of two scheduling powers is larger than the UE-specific maximum power, a UE allocates the UE-specific maximum powers to two panels according to the priority of two UL signals/channels transmitted by two panels, respectively.

- For STxMP operation, a UE is configure a UE-specific maximum power and calculates at most two scheduling powers.

When the sum of two scheduling powers is larger than the UE-specific maximum power, the sequence of power allocation for scheduling panels should be determined. After the determination of the sequence, a proper power is allocated to a panel which is at the head of the sequence, and the rest power is allocated to a panel which is at the back of the sequence. Some rules to determine the sequence of power allocation should be introduced, such as priorities, power magnitudes or transmission timings of UL channels/signals.

The priority rule is taken as an example. A UE allocates a proper power to a panel which transmits a UL signal/channel with a higher priority, and allocates the rest power to a panel which transmits a UL signal/channel with a lower priority. This guarantees the channel quality of a UL signal/channel with a higher priority.

Observation: When the sum of two scheduling powers is larger than the UE-specific maximum power, the sequence of power allocation for scheduling panels should be determined.

2.3 Switching between joint and separate TCI states

In current spec, for single TRP operation with unified TCI framework, switching between joint and the separate TCI states is not dynamic which is configured in the higher parameter unifiedtci-StateType. For multi-TRP operation with unified TCI framework, this proposal implies that the switching between the joint and separate TCI states should be TRP-specific. Therefore, it is proposed to extend the higher parameter unifiedtci-StateType to TRP-specific higher parameters, for example, unifiedtci-StateType-TRP1 and unifiedtci-StateType-TRP2.

- Recommend TRP-specific switching between joint and separate TCI states.

Considering flexible, MAC CE indication is proposed to support dynamic switching between joint and the separate TCI states. For example, the MAC CE with 2 bits where the former bit indicates that the TCI state corresponding to the first TRP is whether joint or separate and the latter bit indicates that the TCI state corresponding to the second TRP is whether joint or separate. This MAC CE can also be merged into another MAC CE, such as unified TCI states activation/deactivation MAC CE.

- Recommend dynamic switching based on MAC CE indication between joint and the separate TCI states.

3 Conclusion

In this contributions, our opinions on unified TCI framework extension for multi-TRP operation are listed as follows.

- Two options to use the codepoint “11” in TCI selection filed in DCI format 1_1/1_2.

  - Option 1: set as the same usage as the codepoint “10”.

  - Option 2: reserve this codepoint.

- The default TCI state(s) can be the two activated TCI state(s) with lowest ID corresponding to two TRPs.

- The spatial Tx filter(s) of the PUSCH transmission should follow those of indicated SRS resource(s) prior to those of indicated joint/UL TCI state(s).

- Support opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission.

- Support Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception.

- For STxMP operation, a UE is configure a UE-specific maximum power and calculates at most two scheduling powers.

Observation: When the sum of two scheduling powers is larger than the UE-specific maximum power, the sequence of power allocation for scheduling panels should be determined.

- Recommend TRP-specific switching between joint and separate TCI states.

- Recommend dynamic switching based on MAC CE indication between joint and the separate TCI states.