3GPP TSG RAN WG1 #112bis-e		                                                                 R1-2303395

e-Meeting, April 17th – April 26th, 2023

Title:	Draft reply LS to RAN2 on SL resource (re)selection

Response to:	R1-2302278 (R2-2302036)

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	[RAN1], ZTE, Sanechips

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	hu.yuzhou@zte.com.cn

Overall description

RAN1 thanks RAN2 for the LS R1-2302278 (R2-2302036) on SL resource (re)selection. In this LS, RAN2 provided a UE behavior that UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission, and FFS for the multiple consecutive slots transmission (MCSt) case and asked whether RAN1 has concern on this behavior.

In RAN1’s understanding, we have already specified similar UE behavior in mode 2, such as resource pre-emption/re-evaluation in Rel-16. In that case, when UE detects some of resources are no longer feasible to be used for transmission, PHY will report the resource to MAC layer and MAC layer triggers resource (re)selection to replace those resources. Following the same UE behavior, when an LBT failure indication is reported from PHY for a PSSCH transmission, UE triggers resource (re)selection to select a new resource. And for MCSt case, the same mechanism can be applied as well, i.e. if an LBT failure indication is reported from PHY for any one of a PSSCH transmission in MCSt, UE triggers resource (re)selection for the resource which fails the LBT, and can further apply resource (re)selection to other already-reserved resource(s) in MCSt by UE implementation.

In summary, from RAN1’s perspective, there is no concern on the UE behavior that UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission. And RAN1 also thinks the similar behavior for pre-emption and re-evaluation in Rel-16 can be applied to MCSt case.

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above information into account.

Date of Next TSG-RAN1 Meetings:

RAN1#113			22nd -26th May 2023	   			Incheon, KR

RAN1#114			21st -25th Aug 2023	   			Toulouse, France