3GPP TSG RAN WG1 #112bis-e		R1-2303397

e-Meeting, April 17th – 26th, 2023

Agenda item:	5

Source: 	ZTE, Sanechips

Title: 	About LS on LBT and SL resource (re)selection

Document for:	Discussion and decision

Introduction

In RAN2 LS R1-2302283(R2-2302303), RAN2 informed that the LBT impact on resource (re)selection for both intra-UE case and inter-UE case has been discussed in RAN2, and made the following agreements [1].

- RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

- RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

In this contribution, the views on LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case) on unlicensed band will be discussed.

Discussions

Potential LBT impact in inter-UE case

As shown in Figure 1, if UE-A selected a resource which is close/next to the slots for other UE’s transmission, e.g. UE-B or UE-C, there may be LBT impact to/from other UE’s reserved resource. The following cases can be further discussed:

Case 1: UE-A’s resource followed by the slot of other UE’s resources;

Case 2: UE-A’s resource before the slot of other UE’s resources.

UE-A

transmission

UE-B

transmission

UE-C

transmission

LBT

LBT

Figure 1: Example of LBT for reserved resource(s)

In case 1 above, if UE-A is determined to perform type 1 to access the channel, if the gap between UE-A’s resource and UE-B’s resource is too small to complete type 1 channel access, UE-A would fail to LBT and cannot perform SL transmission on that resource.

In case 2 above, if UE-A selects a resource just before UE-C’s reserved resource, and UE-C has to perform type 1 channel access, then UE-C would fail to access the channel due to less time gap between UE-A’s resource and UE-C’s resource.

Whether/how to handle the LBT impact in inter-UE case

To address the issue in case 1 and 2, one potential solution is that UE-A avoids to select a resource which is too close to any reserved resource by other UEs. However, the solution will lead to quite a few issues that need to be discussed, among which several are listed as below

Issue 1: Over-excluding candidate resources. The candidate resources in set A would cause significant interference to other UE’s transmission.

Issue 2: In case 1, other UEs (e.g. UE-B) may not transmit on the resource in the slot before UE-A’s candidate resource actually, then the time gap is large enough for UE-A to do type 1 LBT and UE-A can select a resource after the slot of UE-B’s resource. There are many cases that UE-B may not transmit, such as UE-B has completed its transmission; UE-B may not succeed its LBT for that transmission; UE-B has reselected other resource, etc.

Issue 3: In case 2, similar situation of UE-B in Issue 2 would happen to UE-C, then UE-A doesn’t have to avoid selecting a resource right before other UE’s resources.

Issue 4: In case 1, even if UE-B transmits on its resource finally, UE-A can still transmit on the resource following by the slot of UE-B’s resource, for example, the slot of UE-A is right in the COT which is shared by UE-B or other UEs and the COT sharing criteria is met by UE-A.

Issue 5: In case 2, similar situation of UE-A in Issue 4 would be happened on UE-C, then disadvantages may turn into advantages, i.e. if UE-A selects a resource right before UE-C’s resources and UE-A successfully initiated a COT and shared it to UE-C, it is actually helpful for UE-C’s transmission if we consider the situation that UE-C may not succeed the LBT by itself.

Some agreements on COT sharing are achieved as follows [2][3][4]:

Agreement

- For UE-to-UE COT sharing, continue considering the following alternatives:

  - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT.

    - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information

    - FFS any additional conditions

  - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT.

    - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information

    - FFS how to determine a SL UE is a target receiver

    - FFS: details of the channel type of the COT initiating UE’s transmission

    - FFS any additional conditions

  - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s).

    - FFS: details of the channel type of the responding UE’s transmission(s)

- gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18

- FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA

Agreement

For UE-to-UE COT sharing,

- When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE.

  - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator

- When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE

  - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information.

    - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE.

    - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE.

- FFS: UE forwarding/relaying information about a COT initiated by another UE.

Agreement

- A responding UE over a shared COT can be:

  - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator

    - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE

    - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE

  - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator

    - FFS Limitations on what additional IDs may be included and how they may be indicated

Based on above analysis, considering LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case) on unlicensed band, both advantages and disadvantages exist if we follow the solution section 2.2 and the issue is not essential either. If the LBT impact for inter-UE cases during sensing and resource selection procedure is not considered, following RAN2’s agreement as below [5], even if UE-A failed LBT for a transmission, it can reselect another resource and continue its transmission, there is no critical issue any more.

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

To balance advantages and disadvantages, also considering there are quite a few essential issues to be addressed under the remaining Rel-18 budget, we think the LBT impact for inter-UE cases should not be considered during sensing and resource selection procedure, and the following proposal is given:

The LBT impact for inter-UE cases should not be considered during sensing and resource selection procedure.

Conclusion

Based on the discussion on LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case) on unlicensed band, the following proposal is given:

Proposal 1:	The LBT impact for inter-UE cases should not be considered during sensing and resource selection procedure.

References

R1-2302283(R2-2302303) LS on LBT and SL resource (re)selection, Nokia, RAN1 #112bis-e

Chair's notes RAN1#109-e, Chairman, RAN1 #119-e

Chair's notes RAN1#111, Chairman, RAN1 #111

Chair's notes RAN1#112, Chairman, RAN1 #112

Chair's notes RAN2#121, Chairman, RAN2 #121