3GPP TSG RAN WG1 112bis-e		R1-2303400

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.4.1.1

Source: 	ZTE, Sanechips

Title: 	Discussion on channel access mechanism for SL-U

Document for:	Discussion and decision

- Introduction

In this contribution, analysis and proposals for channel access mechanism for SL-U are presented, including details on dynamic channel access mechanism, semi-static channel access mechanism, CPE	 starting position, UE-UE COT sharing, resource allocation for SL-U, etc.

- Dynamic channel access mechanism

  - Types of channel access

    - Type 1 SL channel access procedure

In RAN1 #110b-e meeting, the following conclusions were agreed for Type 1 SL channel access procedures.

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

In RAN1 #112 meeting, the following conclusions were discussed for CAPC of S-SSB/PSFCH transmission.

Agreement The CAPC level that should be used for S-SSB transmissions: - Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission Agreement The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission

For Type 1 channel access of S-SSB, it is suggested that mp=2 with p=1 in the channel access priority class (CAPC) table defined in the above agreement is used. Considering CAPC table for SL-U has been defined in the above agreement, and it is better that the mp value not included in the above agreed SL-U CAPC table is not allowed. On the other hand, considering that a transmission duration for one communication node (e.g. sidelink or WIFI node) on the unlicensed spectrum is usually significantly longer than 9us, compared with mp=2, the channel access opportunity is almost not improved by using mp=1.

- For Type 1 channel access procedure of S-SSB：

mp=2 with p=1

In NR Uu, mp=2 is used for Type 1 channel access of the NR PUCCH with p=1. Based on that, it is suggested that mp=2 can be used for Type 1 channel access procedure corresponding to PSFCH transmission.

- For PSFCH，mp=2 with p=1 is used for its Type 1 channel access procedure.

    - Type 2 channel access procedure for S-SSB/PSFCH without a shared channel occupancy

In RAN1 #111 meeting, the following conclusion was agreed for channel access procedure for a UE without a shared channel occupancy.

# Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

From the perspective of the system, if Type 2A channel access mechanism is allowed for all PSFCH occasions, the frequency of PSFCH transmission can be very high, e.g., up to 1 PSFCH occasion per slot, and it is not friendly to other systems (such as WIFI). Considering that, one way is that Type 2A is not adopted as the channel access mechanism of PSFCH transmissions without a shared channel occupancy. The other way is that Type 2A can be used as initial channel access mechanism for only part of PSFCH occasions, and Type 1 LBT channel access mechanism is adopted for remaining PSFCH occasions.

If Type 2A channel access mechanism can be used w/o shared channel occupancy, it should be guaranteed that the number of transmissions or total transmission time of S-SSB and PSFCH transmissions does not exceed the limitations of regulation.

- For PSFCH transmissions without a shared channel occupancy, one of the following is supported targeting to regulation requirement:

Type 2A channel access mechanism is not supported for PSFCH

Type 2A channel access mechanism can be used for only part of PSFCH occasions

In TS 37.213, the parameter TA is used for obtaining the maximum energy detection threshold XThresh_max, and its value is defined as following.

TA=5dB for transmissions including discovery burst(s) as described in clause 4.1.2, and TA=10dB otherwise;

Referring to the above statement in TS 37.213, without a shared channel occupancy, it is suggested that TA=5dB should be used for the Type 2A channel access procedure of S-SSB.

- Without a shared channel occupancy, TA=5dB should be used for the Type 2A channel access procedure of S-SSB.

In NR-U, the discovery burst duty cycle is at most 120 for Type 2A channel access procedures without a shared channel occupancy and no observation period is specified. Similarly, it is suggested that it is up to UE implementation for determining the observation period of Type 2A channel access procedure of S-SSB.

- It is up to UE implementation for determining the observation period of Type 2A channel access procedure of S-SSB without a shared channel occupancy.

  - Contention window adjustment

In RAN1 #110-bis meeting, the following conclusion was agreed for CW adjust in Type 1 SL channel access procedure.

Agreement - RAN1 is to study the definition of a “SL reference duration” following the NR-U principle and RAN1 is to agree on the definition before down-selection to an option for CW adjustment for SL HARQ-ACK feedback enabled/disabled and each cast type - In Type 1 SL channel access procedure, further study the following cases and options. Other options are not precluded. - CW adjustment when SL-HARQ feedback is disabled ( at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled): - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 4: If a CWp is consecutively used K times for generation of Ninit, CWp is updated for each priority class p∈1,2,3,4 to the next higher allowed value. - Option 5: If a collision indicator is received, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - FFS: the (pre-)configuration ratio values - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p; otherwise CWp is increased. - FFS whether groupcast option 1 (NACK-only) with SL-HARQ feedback enabled can be supported for SL-U. If supported, further study the following options (at least if all transmissions within the latest SL reference duration are groupcast option 1 transmissions) - Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 2: - If ‘NACK’ or a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - When neither ‘NACK’ nor a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, - Option A: CWp is reset to CWmin,p for every priority class p∈1,2,3,4. - Option B: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - Option 3: An ACK-only procedure is used instead of a NACK-only procedure. In this case, if at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p, otherwise CWp is increased - Option 4: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 5 (option 3+legacy): ACK feedback is performed when a TB is successfully decoded in addition to the legacy NACK-only procedure. In this case, if ACK only is received related to any transmissions within the latest SL reference duration then CWp=CWmin,p, otherwise CWp is increased. - CW adjustment for unicast with SL-HARQ feedback enabled (at least In case only unicast PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 2: If at least one ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class p∈1,2,3,4 CWp=CWmin,p ; otherwise CWp is increased. - FFS the case when UE is operating with different SL-HARQ feedback schemes (e.g., UE has concurrent broadcast transmission + unicast with SL-HARQ enabled, or GC option 1 + GC option 2, etc in the SL reference duration).

In RAN1 #112 meeting, the following conclusions were agreed for the definition of SL reference duration.

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

In NR-U, when a channel access priority class p has not been used for any transmissions on the channel, the channel access with CAPC=p for the first transmission uses CWp=CWmin,p, and the subsequent transmission updates corresponding CWpvalue based on the previous transmissions’ HARQ feedback, and if HARQ feedback is not available after the last update of CWp, CWp is either increased to the next higher allowed value or maintained as the last updated value.

  - CW adjustment when SL-HARQ feedback is disabled

For the CW adjustment when SL-HARQ feedback is disabled within the latest COT, it is suggested the CW adjustment mechanism without HARQ feedback in NR-U should be reused as much as possible. Therefore, it is suggested that option 1 is adopted as the CW adjustment mechanism when SL-HARQ feedback is disabled.

- Option 1 below is adopted as the CW adjustment mechanism when SL-HARQ feedback is disabled within the latest COT.

- Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p

  - CW adjustment in groupcast option 2

The CW adjustment mechanism in groupcast option 2 includes two CW adjustment options, option 1 and option 2. In Option 2, as long as one among multiple RX UEs feedback the ACK, the CWp will be reset to CWmin,p. We think it is too aggressive and not friendly to other technologies such as WIFI, and option 2 can be seen as a special case of option 1, thus we prefer option 1.

In the CW adjustment mechanism option 1 in groupcast option 2, it is FFS whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’. We think it is equivalent to use either ‘ACK’ or ‘NACK’ as the ratio and it follows similar principle as NR-U. When some of the received SL HARQ-ACK feedbacks are ‘ACK’, if the ACK ratio is greater than a threshold, the CWp will be reset to CWmin,p. Otherwise, increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. Similarly, when some of the received SL HARQ-ACK feedbacks are ‘NACK’, if the ‘NACK’ ratio is less than a threshold, the CWp will be reset to CWmin,p. Otherwise, increase CWp to the next higher allowed value.

We think it is unreasonable that CWp is adjusted based on the ratio of received ‘ACK+NACK’. For example, assuming that all RX UEs in groupcast option 2 can correctly receive SCI but feedback NACK due to PSSCH receiving failure, PSCCH/PSSCH TX UE will adjust CWp to CWmin,p if CW adjustment is based on ‘ACK+NACK’. We think that it is unreasonable to adjust CWp to CWmin,p when all HARQ-feedback is NACK, and it is unfair to channel competition of other technology in the unlicensed spectrum.

In addition, the CW adjustment mechanism for unicast with SL-HARQ feedback enabled was agreed in RAN1#110-bis-e meeting. For the CW adjustment of SL unicast with SL-HARQ feedback enabled, if only NACK is received and no ‘ACK’ is received related to any transmissions within the latest SL reference duration, CWp is increased. We believe that the logic of CW adjustment mechanism for unicast should be also applied to groupcast option 2. When only one RX UE is included in groupcast option 2, its CW adjustment should be the same as that in unicast with SL-HARQ feedback enabled. If the ratio of received ‘ACK+NACK’ is used to adjust CW, under the condition of only one RX UE is included in groupcast option 2, when only NACK is received and no 'ACK' is received related to any transmissions within the latest SL reference duration, one PSSCH TX UE will adjust CW to the minimum value. Obviously, its CW adjustment is inconsistent with CW adjustment for unicast with SL-HARQ feedback enabled. It is suggested that the agreed CW adjustment mechanism of SL unicast with SL-HARQ feedback enabled should be respected. Therefore, it is suggested that ratio of received ‘ACK+NACK’ cannot be used to adjust CW in groupcast option 2 with SL-HARQ feedback enabled.

- CW adjustment for groupcast option 2 with SL-HARQ feedback enabled, adopt the following method:

Based on a (pre-)configurable ratio of received ACK in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value

  - CW adjustment mechanism in groupcast option 1

In NR-U, there is no HARQ-ACK feedback mechanism similar to SL groupcast option 1 (NACK only) with SL-HARQ feedback enabled. If Option 1, i.e., always using the latest CWp, were supported to adjust the CW, the CW adjustment is not accurate. It is suggested that as much information as possible should be used for CW adjustment in order to improve the performance of CW adjustment.

In Option 3（ACK only）, when only a few RX UEs receive PSSCH correctly and feedback ACK, the PSCCH/PSSCH TX will not retransmit the PSCCH/PSSCH, although many other RX UEs do not receive the PSCCH/PSSCH correctly. Therefore, this will result in the degradation of SL communication performance and needs to be improved in the sense that the Rx UEs not receiving PSSCH correctly shall provide feedback as well.

In Option 4, CBR/CR includes the statistics of historical resource load. However, the difference between the historical start time of CBR/CR and the time of adjusting CW is too long. Considering that the interference change is usually significant during the CBR measurement window, CBR/CR may not reflect the instant interference in the recent period very well, so it is not very accurate to use CBR/CR to adjust CW.

Based on the above discussion, we do not support Option 1, Option 3, or Option 4 for CW adjustment of groupcast option 1 (NACK only) with SL-HARQ feedback enabled. Among the remaining Option 2 and Option 5, we think Option 5 is better. In Option 5, if PSCCH/PSSCH TX UE can receive NACK, CW is adjusted to the next higher allowable value. If PSCCH/PSSCH TX UE only receives ACK, the CW window is adjusted to CWmin,p. If the PSCCH/PSSCH TX UE does not receive ACK or NACK, it can be inferred that the RX UEs failure to send the PSFCH due to their LBT failure or neither the PSCCH nor PSSCH is received correctly, which means that the channel conditions at the sides of RX UEs are not enough good, so the PSCCH/PSSCH TX UE adjusting CW to the next higher CW value seems reasonable.

Also, the definition of reference duration in the above RAN1 #112 meeting can be reused in option 5 since option 5 is also an ACK/NACK based feedback, and the contention window can be adjusted based on the reference duration agreed in RAN1 #112 meeting.

- Option 5(common NACK+common ACK) should be adopted for CW adjustment of groupcast option 1 with SL-HARQ feedback enabled, and the definition of reference duration for ACK/NACK based feedback can be reused.

  - Multi-channel access procedures

In RAN1 #110b-e meeting, the following conclusions were agreed for dynamic channel access mode with multi-channel case in SL-U.

# Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

For S-SSB, it is not yet decided whether multiple S-SSB resources can be configured for different RB sets. If so, it is suggested that only one S-SSB is transmitted on one of multiple channels in order to ensure the power and coverage of S-SSB. With transmitting one S-SSB on one of multiple channels, a UE can independently perform channel access process on each channel, and transmit an S-SSB on the one among these channels with successful channel competition. Also, a UE can jointly perform the multiple channel access procedure, with the successful multi-channel access procedure, the UE selects one from these multiple channels in order to transmit S-SSB on the one of these channels. It is suggested that it is up to UE implementation whether/how to perform the multi-channel access process.

- It is suggested that it is up to UE implementation whether to jointly perform channel access on multiple channels or to independently perform channel access on each channel, if it is agreed that S-SSB can be (pre-)configured in more than one RB set.

For the transmission of PSFCH, a typical scenario is that a UE needs to send PSFCH on different LBT channels. Considering that the PSFCHs sent on different LBT channels are usually independent with each other, it is suggested that a UE can perform independent Type 1/2 LBT on each LBT channel. More specifically, after determining the LBT type independently for each channel, UE can perform independently Type 1/2 LBT on each LBT channel.

- For multiple PSFCH transmissions on multiple channels, independent Type 1/2 LBT is performed on each LBT channel.

In addition, it should be decided by RAN4 whether the multiple PSFCH transmissions on multiple channels is limited to contiguous RB sets.

- It should be decided by RAN4 whether the multiple PSFCH transmissions on multiple channels is limited to contiguous RB sets.

- Semi-static channel access mechanism

For semi-static channel access mechanism in NR-U, a UE can perform UL transmission in channel occupancy time only when the UE can detect a DL transmission at beginning of a gNB period of semi-static channel access or when the UE initiates a channel occupancy at beginning of a UE FFP. Based on this observation, for semi-static channel access mechanism of SL-U, gNB shared COT is not supported for SL UE, it is suggested that a UE can initiate a COT via sending a transmission at beginning of a SL UE period of semi-static channel access. In semi-static channel access mechanism, the relevant SL FFP parameters can include the fixed frame period, the offset of fixed frame period, etc, and the fixed frame period can be used to determine the idle period in the fixed frame period. In SL-U, it is suggested that the fixed frame period and its offset are configurable as in NR-U.

As shown in figure 2, a UE selects a resource in a SL UE period, it can send a SL transmission at the beginning of in the period to initiate a COT and send another SL transmission on its selected resource in the channel occupancy time.

Figure 1: FBE Period and channel occupancy time

Figure 2: SL transmissions of one UE in the FBE period

- For semi-static channel access mechanism of SL-U, it is suggested that a UE should perform a transmission at beginning of a period of semi-static channel access to guarantee its SL transmission in the channel occupancy time within the FFP.

- For SL-U, the SL fixed frame period and its offset to an even radio frame are configurable for semi-static channel access mechanism.

- CPE starting position

In RAN1 #111 meeting, the following conclusion was agreed for CPE starting positions for SL transmissions.

# Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details

For a single CPE starting position for PSFCH，in order to support FDM among PSFCH transmissions from different UEs, it is suggested that a common CPE starting position is supported for different UEs in a resource pool. Therefore，it is suggested that either (pre-)configured in each RP or pre-defined CPE starting position is adopted for PSFCH.

- Either (pre-)configured in each RP or pre-defined single CPE starting position is adopted for PSFCH.

In order to avoid blocking problem among different UEs in the sidelink system, a common default CPE starting position should be tried firstly to access the channel for multiple UEs at least in the same slot. How to select a default CPE starting position should be further discussed, for example, the first one of multiple (pre-)configured CPE starting positions is used as the common default CPE starting position.

- A common default CPE starting position should be selected for multiple UEs at least in the same slot.

For the criteria for selecting one of the multiple CPE starting positions, it is suggested that at least the improvement of channel access opportunities should be considered. For the case UEs transmit on a channel in TDM，one UE can select one CPE according to priority level. For the case UEs transmit on a channel in FDM, the selection of the actual CPE starting position can be done in a hierarchical way by trying the (pre-)configured multiple CPE starting positions one by one. In case a common default CPE starting position fails, other CPE starting position is tried to access the channel.

- When multiple CPE starting positions are (pre-)configured for PSSCH/PSCCH,

When a UE has a full RB set allocation for PSCCH/PSSCH transmission in a slot，one UE can select one CPE according to priority level

When a UE has a partial RB set allocation for PSCCH/PSSCH transmission in a slot，one UE shall try to access the channel using available CPE starting positions with a common default CPE starting position among the (pre-)configured starting positions until the channel access procedure is succeeded

In order to increase the channel access opportunities of S-SSB, it is suggested that the multiple CPE starting positions should be supported for at least S-SSB on legacy occasions. For the additional S-SSBs, whether multiple CPE starting positions are supported needs to be further studied and discussed, and the blocking problem between S-SSB and PSSCH/PSCCH needs to be considered.

- For the transmission of S-SSB，

The multiple CPE starting positions should be supported for at least S-SSB on legacy occasions

FFS：Whether to support the multiple CPE starting positions for additional S-SSBs

Note：The blocking problem between S-SSB and PSSCH/PSCCH needs to be considered for the design of additional S-SSBs

- UE-UE COT sharing

In the following discussion, we will discuss COT sharing for Load Based Equipment. With regard to adaptivity for Load Based Equipment, a device that initiates a sequence of one or more transmissions is denoted as the Initiating Device. Otherwise, the device is denoted as a Responding Device.

In RAN1 # 112 meeting, the following conclusion was agreed for UE-UE COT sharing.

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) FFS: all other details and additional restrictions

For UE-UE COT sharing information, The signaling overhead of additional IDs (s) is significant, and it is suggested that additional ID(s) is not supported. For example, for additional IDs including either a pair of source/destination IDs or three pairs of source/destination IDs, 32 bits and 96 bits are required as additional signaling overhead respectively.

For groupcast/broadcast, even if additional IDs(s) are not introduced, sufficient COT sharing can be performed within a group of UEs corresponding to the groupcast/broadcast. Therefore, additional IDs(s) may not be introduced for COT sharing of groupcast/broadcast.

For unicast, the introduction of additional ID (s) can increase the COT sharing probability only when there are multiple unicast links for the COT initiating UE and the CAPC limit at these responding UEs is met. In practical applications, we suspect that the aforementioned unicast COT sharing scenario does not often occur. It is unwise to increase the COT sharing probability of the above atypical scenario at the expense of significant signaling overhead. When additional ID (s) are included in SCI, it means that all SCIs in a resource pool contain additional ID (s), although a UE does not share its COT. This will further increase the signaling overhead caused by additional IDs (s). If the additional ID (s) is included in the MAC CE, considering the latency of the additional ID (s) obtained by the response UE, it means that there is little or even no remaining shared COT. In summary, it is not necessary to introduce additional IDs for UE-UE COT sharing.

For groupcast/broadcast, the necessity of introducing additional IDs(s) for COT sharing is insufficient.

For unicast, additional ID (s) can increase the COT sharing probability only when there are multiple unicast links for the COT initiating UE and the CAPC limit is met, and

The COT initiating UE doesn’t know the CAPC of the responding UE(s)

When additional ID (s) is included in SCI, it means that all SCIs in a resource pool contain additional ID (s) at the expense of significant signaling overhead

When additional ID (s) is included in MAC CE, considering the latency of the additional ID (s) obtained by the response UE, it means that there is little or even no remaining shared COT

As it is still not clear what these additional ID(s) are, and considering IDs are always provided by higher layer, RAN2 should be involved to support these additional ID(s).

- Before supporting additional ID (s) for COT sharing, the following two issues should be confirmed with RAN2:

For a transmission of one link from one UE, whether the source and destination IDs corresponding to other links associated with the UE are also available for this link

Whether a link can be identified by the truncated source/destination ID

Based on the above discussion, it is suggested not to support additional IDs for COT sharing. If additional ID is supported, the signaling overhead of additional ID should be reduced. In order to reduce the signaling overhead , for unicast, additional ID should only include the source ID of the initiator UE. For broadcast/groupcast, the additional ID should only include the destination ID corresponding to the initiator UE.

As shown in Figure 3, the COT sharing information, sent by initiator UE0 at time t3, includes the source ID1 as the additional ID. After UE1 and UE2 receive the source ID1 as the COT sharing information, UE1 and UE2 can infer that the destination IDs(associated with the source ID1) corresponding to their respective links are destination ID1 and destination ID2, respectively. Therefore, UE1 and UE2 can be as response UE, and use the COT shared by initiator UE0 to send unicast information to initiator UE0. For broadcasting/groupcast, the principle is similar to the above description.

- For the additional ID for COT sharing, it is suggested to adopt one of the following alternatives:

Alt1: Additional ID is not supported

Alt2: The signaling overhead of additional ID should be reduced:

For unicast, additional ID should only include the source ID of the initiating UE

For broadcast/groupcast, the additional ID should only include the destination ID

Figure 3: The indication of COT sharing information

For the indication of COT sharing parameters, it is suggested that at least the remaining COT duration should be indicated, by an Initiating Device, such that the transmissions inheriting a shared COT does not exceed the shared COT duration. In addition to remaining COT duration, it is suggested that CAPC level and the legacy L1 ID should also be indicated for UE-UE COT sharing.

- Contents of COT sharing information includes the followings:

CAPC level

Remaining COT duration

legacy L1 ID

When performing PSFCH transmission(s), the following three cases are listed about how a responding UE utilize a COT shared by a COT initiating UE.

- Case 1: A responding UE utilize a COT shared by a COT initiating UE when all the responding UE’s PSFCH transmissions are intended for the COT initiating UE.

- Case 2: A responding UE utilize a COT shared by a COT initiating UE when at least one of the responding UE’s PSFCH transmissions is intended for the COT initiating UE.

- Case 3:A responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator.

Considering the channel access opportunities of PSFCH transmission in COT sharing, it is suggested that the all above three cases are supported.

In Figure 4, UE1 and UE2 share a COT respectively, and UE1 and UE2 send PSCCH/PSSCH to UE3 respectively in their shared COTs. When UE1 sends PSCCH/PSSCH in the last slot of the COT shared by itself, the PSFCH associated with this PSCCH/PSSCH transmission falls into the COT shared by UE2. If UE-UE COT sharing is restricted by the destination ID for PSFCH transmission and only Type 1 LBT can be used for PSFCH transmission, and then the PSFCH, associated with this PSCCH/PSSCH transmission in the last slot of the COT shared by UE1, can be blocked by PSCCH/PSSCH transmission in first slot in the COT shared by UE2.

Figure 4: PSFCH opportunity falls into another COT

- When performing PSFCH transmission(s), the following cases are supported for a responding UE to utilize a COT,

- Case 1: When all the responding UE’s PSFCH transmissions are intended for the COT initiating UE

- Case 2: When at least one of the responding UE’s PSFCH transmissions is intended for the COT initiating UE

- Case 3: A responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator

If PSFCH is supported to initiate a COT sharing, its advantage is that it can improve the channel access opportunities of SL-U and enhance the competitive advantage of SL-U technology. For example, other UEs, including at least PSSCH RX UEs, perform channel contention before sending their HARQ feedback through PSFCH, and share the occupied resources with the UEs sending the PSCCH/PSSCH. Therefore, the channel access opportunities of the PSSCH TX UE can be improved. Compared with other technologies, SL-U's competitive advantage can be improved to a certain extent. However, it is not clear whether there are some potential problems for initiating a COT sharing by PSFCH transmission. Also, it is not clear for how to realize COT sharing by PSFCH transmission in aspects such as COT information indication. Therefore, it is suggested that RAN1 further discuss and study whether/how to support PSFCH transmission to initiate a COT sharing.

- RAN1 further discuss and study whether/how to support PSFCH transmission to initiate a COT sharing.

- Resource allocation

  - Resource indication

In RAN1 #110 meeting, the following conclusions are agreed for resource indication and multi-consecutive slots transmission.

Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

According to the above RAN1 #110 meeting agreements, it is suggested to reuse R16 SL TRIV as baseline in order to indicate at least a set of multi-consecutive slots transmission. In addition, it is also necessary to indicate multiple non-consecutive slots transmission as legacy resource indication, e.g. the transmission interval not less than HARQ RTT needs to be guaranteed between two adjacent transmissions of one TB. In this release, NR SL TRIV mechanism should have the flexibility to indicate both multiple non-consecutive slots and consecutive slots transmission.

- It is suggested to reuse R16 SL TRIV (with reinterpretation) to indicate either at least a set of multi-consecutive slots transmission or legacy non-consecutive slots transmission.

As shown in Figure 5 ~7, for the indication of time-domain resource for multi-consecutive slots transmission (MCSt), it is necessary to indicate the duration of at least a set of consecutive slots. When indicating multiple sets of consecutive slots, the starting time of one or more multiple sets of consecutive slots also needs to be indicated. Considering the flexibility of indicating the starting time, it is suggested that one slot is used as the granularity of indicating the start time. In addition, to save signaling overhead, it is suggested that the duration values of different sets of consecutive slots should be same as each other. With reference to Figures 5~ 7, it is suggested that at least 1~2 sets of consecutive slots can be indicated based on R16 NR SL TRIV, a retransmission slot location in R16 TRIV can be used to indicate the duration of two sets of consecutive slots, another retransmission slot in R16 TRIV can be used to indicate the first slot of the second set of consecutive slots. Whether to support the indication of more than two sets of consecutive slots need to be further discussed.

Figure 5: Indicate a set of consecutive slots

Figure 6: Indicate two sets of consecutive slots

Figure 7: Indicate three sets of consecutive slots

With enabled HARQ-ACK, the time gap between two adjacent transmissions of a TB should not be less than HARQ RTT. If the resources reserved by a SCI can only be used for the transmission of a TB, the resource allocation mechanism of indicating at least a set of consecutive slots cannot be used for transmissions with enabled HARQ-ACK. As a result, the application scenario of indicating at least a set of consecutive slots is very limited. Therefore, it is suggested that at least a set of consecutive slots reserved based on R16 NR SL TRIV can be used for transmissions of different TBs.

- At least 1~2 sets of consecutive slots can be indicated based on R16 NR SL TRIV.

- Based on R16 NR SL TRIV, both the starting time and duration of a set of consecutive slots should be indicated.

- For a set of consecutive slots indicated based on R16 NR SL TRIV, it can be used for transmissions of different TBs.

  - Mode 2 resource allocation enhancement

In RAN1 #110b-e meeting, the following conclusions are agreed for multi-consecutive slots transmission.

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

For multiple TBs from different traffic packages, their parameters, such as prioTX, remaining PDB, LsubCH and Prsvp_TX, are usually different from each other. Therefore, it is difficult for guaranteeing that selected resources of multiple TBs from different traffic packages are time-domain continuous. Therefore, we suggest not to introduce additional standardization work to guarantee that selected resources of multiple TBs from different traffic packages are time-domain continuous. It can be up to UE implementation whether/how to select time-domain continuous resource for multiple TBs from different traffic packages. For standardization work for supporting MCSt, it is suggested that the mechanism of time-domain continuous resources is designed with only considering multiple TBs from the same traffic package.

For MCSt mechanism of multiple TBs from the same traffic package, these parameters, such as prioTX, remaining PDB, LsubCH and Prsvp_TX, are usually same for different TBs, and thus the above Option 1 is sufficient for supporting MCSt.

- When L1 is triggered for reporting a subset of candidate resources for MCSt, the following option 1 is adopted for transmission of a single TB and multiple TBs.

  - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 (same as Rel-16)

In the above Option A and Option B, the interference level of candidate resources in SA in Option A is higher than that of in Option B if the RSRP level is taken as the average or a larger value over the multi slot resources. In addition, for the comparison between Option B and Option C, the resource selection of continuous slot transmission in Option B is more flexible. The MAC layer can determine the number of consecutive slot transmissions according to the actual transmission. In Option B, it is suggested that no further PHY procedure need be enhanced in order to determine candidate resources in SA.

- When L1 reports a subset of candidate resources for MCSt, the following Option B is adopted, and no further PHY procedure need be enhanced in order to determine candidate resources in SA.

  - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16

    - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots

In addition, due to the existence of PSFCH symbols, even if a UE transmits on multiple continuous slots, continuous sidelink transmission without any gap cannot be guaranteed. To solve this problem, it may be considered that occupying signals may be transmitted on a PSFCH occasion among the continuous slots, even if a UE does not transmit PSFCH in the PSFCH occasion.

- In case of a continuous sidelink transmission, in order to maintain continuous sidelink slots transmission for a UE, it is suggested that occupying signals is transmitted on PSFCH symbols on which there is no PSFCH reception or transmission.

  - Redundant resources for a TB

For mode 1 resource allocation of SL-U, considering the potential LBT failure, a base station may allocate redundant resources to a TB of a UE, comparing with the Rel-16/17 sidelink. Considering that the resources allocated by the base station for different UEs in mode 1 are usually orthogonal, if redundant resources are allocated for each UE, there will not be enough resources allocated to different UEs. One solution is to allow one resource to be allocated to multiple UE (e.g., these UEs are geographically adjacent). After a TB is successfully transmitted, the UE may no longer use the remaining allocated resources corresponding to this TB, and then another UE can use these unused allocated resources. When a resource is allocated to multiple UEs, transmission conflict is likely to occur among these UEs on the same resource. How to resolve transmission conflict among these UEs needs to be further discussed and resolved.

In SL-U mode 1 resource allocation, a base station may allocate redundant resources to a TB of a UE, comparing with the Rel-16/17 sidelink.

- Considering the effective use of redundant resources used for creasing the reliability of one TB, it is suggested that the same resource can be allocated for multiple different UEs.

FFS: How to resolve the transmission conflict from different UEs on the same resource

- Conclusion

In this contribution, the channel access mechanisms related topics are discussed. Based on the discussion, we have the following observation and proposals:

Observation 1: For groupcast/broadcast, the necessity of introducing additional IDs(s) for COT sharing is insufficient.

Observation 2: For unicast, additional ID (s) can increase the COT sharing probability only when there are multiple unicast links for the COT initiating UE and the CAPC limit is met, and

• The COT initiating UE doesn’t know the CAPC of the responding UE(s)

• When additional ID (s) is included in SCI, it means that all SCIs in a resource pool contain additional ID (s) at the expense of significant signaling overhead

• When additional ID (s) is included in MAC CE, considering the latency of the additional ID (s) obtained by the response UE, it means that there is little or even no remaining shared COT

Observation 3: In SL-U mode 1 resource allocation, a base station may allocate redundant resources to a TB of a UE, comparing with the Rel-16/17 sidelink.

Proposal 1: For Type 1 channel access procedure of S-SSB：

• mp=2 with p=1

Proposal 2: For PSFCH，mp=2 with p=1 is used for its Type 1 channel access procedure.

Proposal 3: For PSFCH transmissions without a shared channel occupancy, one of the following is supported targeting to regulation requirement:

• Type 2A channel access mechanism is not supported for PSFCH

• Type 2A channel access mechanism can be used for only part of PSFCH occasions

Proposal 4: Without a shared channel occupancy, TA=5dB should be used for the Type 2A channel access procedure of S-SSB.

Proposal 5: It is up to UE implementation for determining the observation period of Type 2A channel access procedure of S-SSB without a shared channel occupancy.

Proposal 6: Option 1 below is adopted as the CW adjustment mechanism when SL-HARQ feedback is disabled within the latest COT.

Option 1: For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p

Proposal 7: CW adjustment for groupcast option 2 with SL-HARQ feedback enabled, adopt the following method:

• Based on a (pre-)configurable ratio of received ACK in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value

Proposal 8: Option 5(common NACK+common ACK) should be adopted for CW adjustment of groupcast option 1 with SL-HARQ feedback enabled, and the definition of reference duration for ACK/NACK based feedback can be reused.

Proposal 9: It is suggested that it is up to UE implementation whether to jointly perform channel access on multiple channels or to independently perform channel access on each channel, if it is agreed that S-SSB can be (pre-)configured in more than one RB set.

Proposal 10: For multiple PSFCH transmissions on multiple channels, independent Type 1/2 LBT is performed on each LBT channel.

Proposal 11: It should be decided by RAN4 whether the multiple PSFCH transmissions on multiple channels is limited to contiguous RB sets.

Proposal 12: For semi-static channel access mechanism of SL-U, it is suggested that a UE should perform a transmission at beginning of a period of semi-static channel access to guarantee its SL transmission in the channel occupancy time within the FFP.

Proposal 13: For SL-U, the SL fixed frame period and its offset to an even radio frame are configurable for semi-static channel access mechanism.

Proposal 14: Either (pre-)configured in each RP or pre-defined single CPE starting position is adopted for PSFCH.

Proposal 15: A common default CPE starting position should be selected for multiple UEs at least in the same slot.

Proposal 16: When multiple CPE starting positions are (pre-)configured for PSSCH/PSCCH,

• When a UE has a full RB set allocation for PSCCH/PSSCH transmission in a slot，one UE can select one CPE according to priority level

• When a UE has a partial RB set allocation for PSCCH/PSSCH transmission in a slot，one UE shall try to access the channel using available CPE starting positions with a common default CPE starting position among the (pre-)configured starting positions until the channel access procedure is succeeded

Proposal 17: For the transmission of S-SSB，

• The multiple CPE starting positions should be supported for at least S-SSB on legacy occasions

• FFS：Whether to support the multiple CPE starting positions for additional S-SSBs

• Note：The blocking problem between S-SSB and PSSCH/PSCCH needs to be considered for the design of additional S-SSBs

Proposal 18: Before supporting additional ID (s) for COT sharing, the following two issues should be confirmed with RAN2:

• For a transmission of one link from one UE, whether the source and destination IDs corresponding to other links associated with the UE are also available for this link.

• Whether a link can be identified by the truncated source/destination ID

Proposal 19: For the additional ID for COT sharing, it is suggested to adopt one of the following alternatives:

• Alt1: Additional ID is not supported

• Alt2: The signaling overhead of additional ID should be reduced:

- For unicast, additional ID should only include the source ID of the initiating UE

- For broadcast/groupcast, the additional ID should only include the destination ID

Proposal 20: Contents of COT sharing information includes the followings:

• CAPC level

• Remaining COT duration

• legacy L1 ID

Proposal 21: When performing PSFCH transmission(s), the following cases are supported for a responding UE to utilize a COT,

• Case 1: When all the responding UE’s PSFCH transmissions are intended for the COT initiating UE

• Case 2: When at least one of the responding UE’s PSFCH transmissions is intended for the COT initiating UE

• Case 3: A responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator

Proposal 22: RAN1 further discuss and study whether/how to support PSFCH transmission to initiate a COT sharing.

Proposal 23: It is suggested to reuse R16 SL TRIV (with reinterpretation) to indicate either at least a set of multi-consecutive slots transmission or legacy non-consecutive slots transmission.

Proposal 24: At least 1~2 sets of consecutive slots can be indicated based on R16 NR SL TRIV.

Proposal 25: Based on R16 NR SL TRIV, both the starting time and duration of a set of consecutive slots should be indicated.

Proposal 26: For a set of consecutive slots indicated based on R16 NR SL TRIV, it can be used for transmissions of different TBs.

Proposal 27: When L1 is triggered for reporting a subset of candidate resources for MCSt, the following option 1 is adopted for transmission of a single TB and multiple TBs.

Proposal 28: When L1 reports a subset of candidate resources for MCSt, the following Option B is adopted, and no further PHY procedure need be enhanced in order to determine candidate resources in SA.

Proposal 29: In case of a continuous sidelink transmission, in order to maintain continuous sidelink slots transmission for a UE, it is suggested that occupying signals is transmitted on PSFCH symbols on which there is no PSFCH reception or transmission.

Proposal 30: Considering the effective use of redundant resources used for creasing the reliability of one TB, it is suggested that the same resource can be allocated for multiple different UEs.

• FFS: How to resolve the transmission conflict from different UEs on the same resource

- References

- RP-213678, "New WID on NR sidelink evolution", OPPO, LG Electronics, RAN #94-e, December 2021

- RP-220300, "WID revision: NR sidelink evolution", OPPO, RAN #95-e, March 2022

- Draft Report of 3GPP TSG RAN WG1 #110-e v0.2.0, 22nd – 26th August 2022

- Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.1.0, 10th – 19th October 2022

- Draft Report of 3GPP TSG RAN WG1 #111 v1.0.0, 14th – 18th November 2022

- Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0, 27th February – 3rd March 2023

- 3GPP TS 37.213 h10: "Physical layer procedures for shared spectrum channel access"

- ETSI EN 301 893 (V2.1.1) :"5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU".