3GPP TSG RAN WG1 #112bis-e		R1-2303401

e-Meeting, April 17th - April 26th, 2023

Agenda item:	9.4.1.2

Source: 	ZTE, Sanechips

Title: 	Discussion on physical layer structures and procedures for SL-U

Document for:	Discussion and decision

Introduction

In this paper, further analysis about slot structure, resource pool configuration, PSFCH and S-SSB related issues are provided based on the conclusions and agreements made in RAN1#112[1] and RAN1#111[2].

Discussions

Slot structure

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if legacy NR SL structure are reused, then PSCCH is located in the first symbol of PSSCH region. If the channel is cleared at 1st starting symbol, then UE will transmit PSCCH/PSSCH from 1st starting symbol to the symbol before the last gap symbol in the slot. And if the channel is cleared at the 2nd starting symbol, then UE will transmit PSCCH/PSSCH from 2nd starting symbol to the symbol before the last gap symbol in the slot, as shown in Figure 1.

(a)Transmission from 1st starting symbol            (b) Transmission from 2nd starting symbol

Figure 1 Legacy NR SL structure reused for slots with 2 candidate starting symbols

In our opinion, legacy NR SL structure is not proper to be reused for slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission. For one thing, the PSCCH blind decoding number in a slot will be beyond UE’s capacity in some cases. For another, if the Tx UE has performed physical layer rate matching and resource mapping according to the 1st starting symbol but fails to access the channel, there is not likely to be enough time to perform physical layer rate matching and resource mapping again according to the 2nd starting symbol in the same slot. The UE implementation complexity is indifferent to preparing two TBs, which was ruled out in previous meeting.

If legacy NR SL structure were reused for slots with 2 candidate starting symbols, two problems would need to be handled:

The PSCCH blind decoding number in a slot will be beyond UE’s capacity at Rx UE side

Physical layer rate matching and resource mapping adjustment according to 2nd starting symbol after 1st starting symbol when LBT fails at 1st starting symbol will be beyond UE’s capacity at Tx UE side

To overcome the problem described above, the PSCCH can be placed on a common position (as shown in Figure 2), and at least two advantages can be exploited:

- Advantage1: same PSCCH/PSSCH mapping for PSCCH/PSSCH transmission from 1st or 2nd starting symbol can be achieved

- Advantage2: no matter which starting symbol is finally used at the Tx UE side, only one PSCCH blind decoding occasion per slot as legacy NR SL is needed

Figure 2 Common PSCCH resource for 2 candidate starting symbols slot structure

The common symbol position of PSCCH can be (pre)defined, for example, the start symbol of common PSCCH can be defined next to the (pre-)configured 2nd PSCCH/PSSCH transmission starting symbol. When Tx UE prepares resource mapping based on 1st symbol, but obtains the channel at 2nd symbol, then a punctured PSSCH transmission can be performed. And Rx UE only needs to detect once PSCCH at the known occasion.

Common PSCCH time domain position for both 1st and 2nd starting symbol within slot is beneficial due to:

Same PSCCH mapping based on 2nd starting symbol for both 1st and 2nd starting symbol from Tx UE respective

Not increasing the PSCCH blind decoding complexity from RX UE respective

To address not increasing UE implementation complexity, for a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

Support common PSCCH time domain position starting next to 2nd PSCCH/PSSCH transmission starting symbol

Only once PSCCH/PSSCH mapping based on 2nd starting symbol for both 1st and 2nd starting symbol is performed

The AGC issue was not discussed due to the limit time budget in last meeting, the agreement in RAN1#111 is as below:

Agreement (RAN1#111) For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Since UE may start at 1st or 2nd symbol, two symbols at 1st and 2nd symbol used for AGC purpose will lead to more overhead from Tx UE respective, so option 2 is preferred that the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose at 1st symbol. However, from Rx UE respective, it is left to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot.

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from 1st starting symbol:

From Tx UE respective, PSCCH/PSSCH transmission has only 1 AGC symbol as Rel-16 NR SL UE at 1st starting symbol

From Rx UE respective, it is left to UE implementation to monitor 1 or 2 symbol(s) for AGC purpose

TBS determination

Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

After last meeting, a WA was made that the reference number of symbols is determined by (pre-)configuration for COT initiation TBS determination. In our opinion, the WA can be confirmed to cover the majority of views.

For the convenience of discussion, the reference number of symbols which is determined by (pre-)configuration can be labeled as Lref. The symbols accounted for in Lref can be the same as NR legacy SL sl-LengthSymbols which is also used to determine the TBS. In legacy NR SL value of the sl-LengthSymbols can be (pre)configured from 7 to 14, so value range of Lref can also be from 7 to 14.

Confirm the WA that if a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

At least for COT initiation, TBS is determined based on a reference number of symbols which is determined by (pre-)configuration

Value range of reference number of symbols (Lref) can be from 7 to 14

For the second FFS, it is better that the reference number of symbols is used for transmission in a shared COT is same as COT initiation. For one thing, Rx UE cannot decide whether a transmission is in COT or out of COT per agreements on indication of COT related information. With different reference number of symbols considered for transmission in a shared COT, additional indication to inform the Rx UE the COT situation is anticipated. For another, LBT may still fail in a shared COT similar to the situation as COT initiation. If a larger reference number for transmission in a share COT, legacy sl-LengthSymbols -2-NsymbPSFCH manner can still be used. So there is no need to consider different reference number of symbols used for transmission in a shared COT and for COT initiation.

Different reference number of symbols for transmission in a shared COT is not needed.

In legacy NR SL Rel-16/17, the number of symbols used to calculate TBS is sl-LengthSymbols -2-NsymbPSFCH, and value of NsymbPSFCH is determined by PSFCH (pre-)configuration and/or indicated in SCI such that the TBS of the transmissions associated with the same TB is same. In SL-U, this principle should also be observed, which is beneficial for HARQ combination at the Rx side.

TBS of the transmissions associated with the same TB should be same as legacy NR SL

The PSFCH overhead is determined by PSFCH (pre-)configuration and/or indication

Supposing the (pre-)configured value of the reference number of symbols is labeled as Lref. It was agreed that slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH. So if all the transmissions of a TB occur on the slots with PSFCH symbols, as shown in Figure 3, then legacy TBS determination method can be reused. For example, sl-LengthSymbols -2-NsymbPSFCH can be used by Tx UE to determine the TBS in this situation, value of NsymbPSFCH is indicated in SCI such that the TBS of the transmissions of the same TB is same.

Figure 3 Transmissions within resource pool with PSFCH period=1

If all the transmissions of a TB occur on the slots without PSFCH symbols, as shown in figure 4, for example, Lref -2 can be used by Tx UE to determine the TBS.

Figure 4 Transmissions within resource pool with PSFCH period=0

If some of the transmission of a TB occur on slots without PSFCH symbols and some of the transmission of a TB occur on slots with PSFCH symbols, as shown in figure 5, for example, then whether sl-LengthSymbols -2-NsymbPSFCH or Lref is used to determine the TBS needs to be identified at the Rx UE side.

Figure 5 Transmissions within resource pool with PSFCH period >1

For resource pool with PSFCH period equal to one, the number of symbols to determine TBS at Rx UE is sl-LengthSymbols -2-NsymbPSFCH.

For resource pool with PSFCH period equal to zero, the number of symbols to determine TBS at Rx UE is Lref-2.

Based on the analysis above, for the case where the PSFCH period of resource pool is larger than one, how to keep the same TBS of the transmissions associated with the same TB and recognize the number of symbols to determine TBS at Rx UE side need to be further studied.

For resource pool with PSFCH period larger than one, either Lref-2 or sl-LengthSymbols -2-NsymbPSFCH is used to calculate TBS.

Which number is used should be recognized at Rx UE side. (e.g. choose one of Lref-2 or sl-LengthSymbols -2-NsymbPSFCH according to a predefined rule or indication)

S-SSB

For S-SSB transmission, the frequency domain and time domain design have been discussed several times, the agreements in last meeting are as below:

RAN1#112 Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS. Agreement Down-select one or support both of the followings: - Option 1: Additional candidate S-SSB occasions are excluded from resource pool - Option 2: Additional candidate S-SSB occasions belong to resource pool - Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc. Agreement RAN1 further study the followings: - Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc. RAN1#111 Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details Agreement Regarding S-SSB, RAN1 further study the following: How to transmit S-SSB when a SL BWP contains multiple RB sets

Firstly, the single S-SSB frequency domain range should be determined with regard to the channelization design on unlicensed band. An RB set is configured approximately 20MHz as an LBT channel, if frequency resources of a S-SSB occupy more than one RB set, the probability of channel access failure increases a lot because UE has to successfully access every RB set that S-SSB occupies. Besides, legacy frequency resource size of S-SSB is 11 RB, while the RBs in one RB set is as many as 100 RBs for 15k SCS and 50 RBs for 30k SCS, which are enough to convey S-SSB whilst legacy S-SSB design is not changed too much. Thus the frequency range of one S-SSB is better within a RB set to avoid too much resource occupancy and LBT failure caused dropping. Besides, considering the UE S-SSB blind decode (BD) complexity, it is better that S-SSB is (pre)configured within a determined RB set. In addition, considering that a SL BWP in shared spectrum can be 20MHz, e.g., some low capability UE can only support 20MHz, and as NR SL, the designed S-SSB frequency resources should be within a SL BWP, and if an S-SSB were designed to occupy more than one RB set, then it cannot be used for 20MHz BWP case.

Frequency range of one S-SSB should be within a determined RB set from system perspective.

For the S-SSB slots, some companies suggest maintaining the COT when the COT contains multiple RB sets and includes S-SSB slot(s). In our opinion, such solution is not necessary for the following reasons:

1) Synchronization performance could degrade due to the reduced transmit power per S-SSB occasion.

2)The frequency range for a COT is UE specific, that means the SSB coverage will decrease because S-SSB will not be transmitted in SFN manner. The S-SSB detection complexity would increase at RX UE side as Rx UE may has no knowledge about the COT information before S-SSB detection

In S-SSB slot, no need to transmit S-SSB repetitions or other signals in more than one RB set to maintain a COT.

For the region where OCB is required, how to design S-SSB frequency resource needs to be discussed further. In ETSI regulation [4], OCB requirement can be temporarily exempted during a COT. But this is not suitable to S-SSB transmission. For SL-UE, S-SSB is a broadcast type signal and is usually transmitted by multiple UEs in an SFN manner. S-SSB transmission can be seen as a system level signal, while the temporary exemption can only be applied from UE perspective. Besides, small bandwidth of S-SSB will impact the S-SSB coverage due to the PSD limitation. Moreover, 11RB S-SSB only occupies 1.98MHz under 15kHz SCS which is less than minimum of 2 MHz requirement. Thus it is suggested that equipment operating temporarily with an Occupied Channel Bandwidth less than 80% of its Nominal Channel Bandwidth under a minimum of 2 MHz is not applicable for S-SSB.

Temporary exemption of OCB requirement is not applicable for S-SSB transmission due to:

It is hard to justify whether slot level SSB transmission is ‘temporary’ from system perspective

Small bandwidth of SSB will impact the SSB coverage due to the PSD limitation

11 PRBs S-SSB only occupies 1.98MHz under 15kHz SCS which is less than minimum of 2 MHz requirement

S-SSB in legacy R16 NR SL occupies continuous 11RBs in frequency domain and 14 symbols (including gap symbol for normal CP) in time domain within SL slot, the design will not meet the OCB requirement for SL-U if OCB is required in this area. To solve this problem, option1 and option3 and corresponding sub-options are to be down-selected for 15 kHz and 30 kHz SCS.

For option 1-1, interlaced RB transmission for S-SSB does not depend on other transmissions to meet OCB requirement, and S-SSB can either be configured outside resource pool or inside resource pool. If S-SSB is (pre)configured inside a resource pool, as PSSCH is interlace based RB transmission, then only interlaced RB based S-SSB design can be adopted with PSSCH FDMed in a RB set.

For option 3-1, although repetition of S-SSB in frequency domain can meet the OCB requirement, while PAPR will be a problem which needs more discussion. So option1-1 is preferred.

For option 1-1, legacy S-SSB occupies 11 RBs while there may be a case that each interlace has only 10 PRBs in a RB set, considering the RB set size are flexibly configured by high layers, two alternatives can be considered:

- Alt1 for option 1-1

S-SSB can be designed to map to 10RBs within one specific interlace no matter 10 PRBs or 11RBs are in the interlace, in this way PSSS/SSS symbols need to be punctured to 120, and PSBCH is rate matched.

- Alt2 for option 1-1

- SSB can be designed to map to 20RBs within two interlace no matter 10 PRBs or 11RBs are in the interlace, in this way, for the PSS/SSS symbols mapping, 127 REs out of 240 REs can be used to mapping the synchronization signal, the other REs can be a repetition of mapped PSS/SSS symbols, and PSBCH is rate matched

As puncturing of synchronization sequences in Alt1 may impact the S-SSB detection performance, so alt2 is preferred.

It is also noted that for the region where OCB is not required, legacy S-SSB slot structure can be reused directly. It should also be noted that interlace structure is not defined for 60k SCS. So for 60k SCS, only continuous RB based transmission where OCB is not required is supported. In all, legacy NR Rel-16 SL S-SSB definition can be reused for the region where OCB is not required.

If OCB is required, interlaced RB transmission for S-SSB (option1-1) is preferred for 15 kHz and 30 kHz SCS:

S-SSB maps to 20RBs within two adjacent interlaces for a unified design

If OCB is not required, legacy NR Rel-16/17 S-SSB slot structure is reused directly for 15 kHz and 30 kHz SCS.

60 kHz SCS S-SSB can only be used for regions where OCB is not required and legacy NR Rel-16/17 S-SSB slot structure is reused.

In NR SL operation band, the frequency location of S-SSB is (pre)configured by high layer signal sl-AbsoluteFrequencySSB, a frequency location ARFCN corresponding to the subcarrier with index 66 in the S-SS/PSBCH block  is provided by sl-AbsoluteFrequencySSB[2], as shown below:

Figure 6 legacy SL SSB frequency determination

So for SL-U SSB transmission, the frequency location of S-SSB should also be defined. For example, absolute frequency pointing to a specific RE of S-SSB as legacy NR SL can be considered, in this way UE further determines the absolute position of the whole S-SSB based on predefined rules.

Or the RB set index and interlace index can be directly signaled, then the PRBs within the RB set and IRB are used for S-SSB mapping.

Or RB set index and predefined IRB within the RB set is used so that only RB index needs to be signaled.

To determine the frequency location of S-SSB, the following information can be provided:

Alt 1: ARFCN pointing to a specific RE determining the REs of S-SSB as legacy NR Rel-16 SL

Alt 2: RB set index and IRB index within the RB set

Alt 3: RB set index and predefined IRBs within the RB set

In NR R16, S-SSB resources of each S-SSB resource set are configured with a periodicity of 160ms. Based on the (pre)configuration and synchronization priority rules, UE can complete the synchronization procedure. This mechanism can be used as the starting point of SL-U.

R16 SL synchronization procedure can be reused for SL-U.

Over unlicensed band the S-SSB signal may fail to access the channel for some of S-SSB transmissions, and this will decrease the synchronization performance. To alleviate the impact, more transmission occasions can be considered for each set of SSB resources in the synchronization period.

If additional S-SSB transmission occasions are configured, a lot of resources may be wasted if PSCCH/PSSCH cannot be transmitted on these slots. From this perspective of view, at least additional S-SSB transmission slots can be (pre)configured overlapping with the slots within a resource pool, and then S-SSB transmission and PSCCH/PSSCH transmissions which are frequency domain orthogonal in the same slot can be considered to be allowed in system level. Besides, for time domain structure in a slot for S-SSB, legacy structure (e.g. 14 symbols for normal CP) can be reused. If S-SSB transmission is on a slot in a resource pool, it means in the same slot there may be transmission of S-SSB and PSSCH from different UEs. In case S-SSB and PSSCH transmission take place at different starting point or occupies different number of symbols in the same slot, AGC problems would arise. So all symbols in the slot should be available for either S-SSB or PSSCH transmission.

Support option 2, i.e. additional candidate S-SSB transmission occasions can belong to resource pool.

To avoid AGC issue and inter-UE blocking between S-SSB and PSSCH in the same slot：

Frequency domain resource of S-SSB and PSCCH/PSSCH are orthogonal

S-SSB transmission using a subset of symbols in such slot is not allowed if multiple candidate starting symbols are not allowed.

For the additional S-SSB transmission occasions, conflict between S-SSB Tx and PSSCH Tx, S-SSB Rx and PSSCH Tx may occur. As higher layer will define the priority of S-SSB, the conflict can be handled according to the priority comparison.

For S-SSB detection, the time domain and frequency domain resource of additional S-SSB transmission occasions locate in (pre-)configured/(pre-)defined resources similar to those of legacy S-SSB, there will not be any blind detection complexity issue.

With regard to additional S-SSB occasions belong to resource pool:

Conflict between S-SSB Tx and PSCCH/PSSCH Tx, S-SSB Rx and PSCCH/PSSCH Tx can be handled according to the priority comparison

Time domain and frequency domain resource of additional S-SSB transmission occasions locate in (pre-)configured/(pre-)defined resources similar to legacy NR Rel-16/17 S-SSB transmission occasions

If S-SSB occupies all the symbols within a slot as legacy NR SL, then the S-SSB cannot be sent in current slot if UE fails to access the channel at the first symbol of the slot. In order to alleviate the impact to S-SSB transmission due to LBT failure, fewer symbols together with larger bandwidth (before resource mapping) than legacy 11PRB S-SSB structure can also be considered to carry the same S-SSB contents. The reserved S-SSB symbols in the slot can be used as the starting symbol to access the channel, as shown in Figure 7:

(a): S-SSB occupies all the symbols within a slot            (b): S-SSB occupies part of symbols within a slot

Figure 7 S-SSB slot structure comparison

Whenever the channel is sensed to be idle in part1 (any time instance within the reserved symbols) where multiple symbols used as starting position are included, UE should transmit the remaining symbols in part 1 and the S-SSB content part (part2) in the S-SSB slot. In this way, Tx UE can have more chance to transmit the S-SSB in current slot. And the signal transmitted in part 1 can be repetition of some symbols of S-SSB part (part2)

Support multiple candidate starting symbols for S-SSB transmission out of resource pool or in resource pool with two starting symbols enabled.

Regarding the number and location(s) of additional candidate S-SSB occasions, option1 is preferred. In views of NR-U, additional SSB occasions are neither contiguous nor exponential with regard to the existing SSB occasions, just try best to add more occasions in the half frame. The number of SSB occasion is extended to 10 for 15k SCS and 20 for 30k SCS.

In Rel-16 SL, S-SSB time resource can be (pre-)configured at most three sets per carrier and in each S-SSB resource set, at most 1 S-SSB transmission occasions can be (pre)configured for 15k SCS and 2 S-SSB transmission occasions can be (pre-)configured for 30K SCS. To overcome the LBT failure issue, additional candidate S-SSB occasions can be (pre-)configured in the similar way as NR-U manner with some new available values in sl-NumSSB-WithinPeriod for each SCS.

Regarding the number and location(s) of additional candidate S-SSB occasions, support adding some new available values in sl-NumSSB-WithinPeriod for each SCS if necessary.

Regarding the additional candidate S-SSB occasions in the same S-SSB period, UE which is going to transmit SSB will try all the occasions until the channel access succeeded, once UE has transmitted the SSB successfully in certain number of transmissions, the Tx UE will not need to transmit on the remaining S-SSB occasions.

In the same S-SSB period, Tx UE will try any occasions until the transmitted SSB meets certain number.

Resource pool related

Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Time domain resource indication for MCSt

Since continuous slots resource has been agreed for SL-U transmission, so for SL-U time domain resource indication, TIRV can also be reconsidered. When TRIV is used to indicate continuous slots resource. Then TRIV can be reinterpreted, or additional field can be added to indicate the duration for each time resource based on legacy TRIV.

For multi-slot consecutive transmission time domain resource location indication, two alternatives based on TRIV can be further considered:

Alt.1: TRIV is reinterpreted to indicate each set of first slot position and duration of multi-slot consecutive transmission

Alt.2: Additional filed is added to indicate the duration for each time resource in addition to legacy TRIV

Guard band PRB(s) usage

With regard to the RBs in guard band, for interlace RB based transmission, when the RBs can be used, the NR-U principle can be reused. In other words, the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets.

For continuous RB based transmission, in order to avoid inter RB set interference or resource waste, the PRB(s) in the intra-cell guard band can only be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission, no matter the PRB(s) in the intra-cell guard band are divided in part of sub-channel or not.

As discussed, the PRB(s) in the intra-cell guard band can be used for PSSCH transmission and not used for PSCCH transmission. In our opinion, these PRB(s) are not used for PSFCH/S-SSB transmission either, because PSFCH/S-SSB transmission may be transmitted independently by a UE, it is hard to guarantee the usage requirement of guard band PRB(s) is met for PSFCH/S-SSB transmission.

For usage of PRB(s) in the intra-cell guard band:

For interlace RB based transmission, PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

For continuous RB based transmission, the PRB(s) in the intra-cell guard band can only be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

The PRB(s) in the intra-cell guard band is not used for PSFCH/S-SSB transmission

IRB based transmission related

Based on the definition of subchannel, both subchannel information (start subchannel indexes and subchannel number L1) and RB set information (start RB set indexes and RB set number L2) are need for the frequency resource indication of PSSCH. If legacy SL FRIV method is used for subchannel indication and RB set indication, then the number of subchannel and RB set for reserved transmission should keep unchanged.

For interlace RB-based PSCCH/PSSCH transmission, PSSCH frequency indication is that:

Both FRIV1 for subchannel information (start subchannel indexes and subchannel number L1) and FRIV2 for RB set information (start RB set indexes and RB set number L2) are indicated

The number of RB set of the transmissions to the same TB is unchanged to reuse legacy NR SL FRIV method

The number of subchannel of the transmissions to the same TB is unchanged to reuse legacy NR SL FRIV method

As subchannel is defined within RB set, and both subchannel information and RB set information are needed to indicate the PSSCH frequency location, then the candidate resource used for sensing need to be modified to including RB set information. In resource allocation mode 2, high layer should additionally provide the number of RB sets to be used, and the candidate resource Rx,y should be changed to Rx1,x2,y to include the RB set information where x1 stands for start subchannel index and x2 stands for start RB set index.

For interlace RB-based PSCCH/PSSCH transmission, single slot candidate resource needs to be modified to capture both RB set information and subchannel definition.

When RB set and IRB based transmission defined in NR-U were reused for SL-U, there is an issue that the number of RBs in different sub-channels may be unequal based on the following two assumptions:

(1) The RBs in guard bands between RB sets in shared spectrum can be used for PSSCH transmission;

(2) The RB number in one IRB within RB set can be 10 or 11.

To simplify resource allocation and reuse legacy resource allocation/selection procedure as much as possible, the number of RBs in a sub-channel should be (pre-)configured to a fixed value assuming 10RB IRB is considered in the sub-channel. The number of RBs within a sub-channel is consequently a multiple of 10 RBs, making the size of all the sub-channels equal. From resource utilization perspective, extra RBs outside sub-channel can be used for actual SL transmissions, while how to use them can be further discussed.

For interlace RB-based PSCCH/PSSCH transmission, the number of RBs within a sub-channel is (pre-)configured to be fixed as 10*K RBs (K is the interlace(s) within a sub-channel):

Extra RB(s) outside sub-channel can be additionally used when they are available and FFS details

Contiguous RB-based transmission related

For legacy contiguous RB based resource pool, a SL UE determines the set of resource blocks assigned to a sidelink resource pool as follows:

-	The resource block pool consists of NPRB PRBs.

-	The sub-channel m for m=0,1,⋯,numSubchannel-1 consists of a set of nsubCHsize contiguous resource blocks with the physical resource block number nPRB=nsubCHRBstart+m∙nsubCHsize+j for j=0,1,⋯,nsubCHsize-1, where nsubCHRBstart and nsubCHsize are given by higher layer parameters sl-StartRB-sub-channel and sl-sub-channelSize, respectively

A UE is not expected to use the last NPRB mod nsubCHsize PRBs in the resource pool. So for SL-U contiguous RB-based transmission, remaining PRBs in resource pool is not used as legacy NR SL.

For contiguous RB-based transmission, remaining PRBs in the resource pool is not used by UE.

There is some difference between subchannel defined for contiguous RB-based transmission and subchannel defined for legacy NR SL. As shown in Figure 8, subchannel 3 and subchannel 4 is special because partial PRBs of the subchannel are within RB set while partial PRBs of the subchannel are within the guard band. As the PRBs in guard band shall be used under specific condition, so the availability of subchannel 3 or subchannel 4 should be clarified.

Figure 8: sub-channel for SL-U contiguous RB-based transmission

Specifically in Figure 8, if the number PRBs of subchannel 3 or 4 belonging to guardband is below a threshold, if UE needs a single channel resource, then the subchnanel 3 can be used as a punctured subchannel where the PRBs contained in guard band are dropped. Otherwise, subchannel 3 or 4 could not be used independently by UE.

For contiguous RB-based transmission, availability of a subchannel which contains PRB(s) within guard band needs to be justified:

Such subchannel can be independently used if the number PRBs within guard band is below a threshold

Otherwise, such subchannel could not be independently used

In mode 2 resource allocation, for candidate resource Rx,y consisting of L contiguous subchannels, as some subchannel may include PRBs within guard band, the Rx,y needs to meet the condition of guard band PRB usage. Moreover, for the region where OCB is required, resource of Rx,y also should meet the OCB requirement. Similar situation holds for mode 1 resource allocation.

For contiguous RB-based transmission, with regard to SL-U resource allocation, the following rules need to be observed:

For the region where OCB is required, the mode 1 allocated resource or mode 2 selected resource shall meet the OCB requirement

For the region where OCB is not required, legacy NR Rel 16 resource allocation mechanism is reused

The mode 1 allocated resource or mode 2 selected resource shall meet the condition of guard PRB usage

PSFCH

Time domain of PSFCH

Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

To address PSFCH transmission dropping due to LBT failure, two alternatives are discussed left in last meeting, as is known, AGC problem still exist in SL-U system. So it is better that every SL UE has unified knowledge of PSFCH resource location so that inter-UE AGC interference can be avoided. So dynamically indicating the UE specific PSFCH resource is not preferred because there is a collision risk, for example another UE who loses the PSFCH resource indication may use a PSSCH resource overlapping with the dynamically indicated the PSFCH resource. So option 1 is preferred.

Dynamically indicating the UE specific PSFCH resource will lead to PSFCH collision risk.

To address PSFCH transmission dropping due to LBT failure, option1 is supported:

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

Supporting more (pre)configured PSFCH occasions per PSCCH/PSSCH transmission is a good idea to alleviate LBT failure, but too many additional PSFCH occasions will impact the PSFCH capacity, too few additional PSFCH occasions still suffer the risk that all the PSFCH fail to access the channel.

Totally dropping PSFCH transmission when LBT failure happens is not a good choice, providing other feedback occasion is helpful to the SL-U system performance as NR-U.

Enhancement for option1 can be considered, beyond (pre)configured PSFCH, and dynamically triggered PSFCH resource which is within the (pre)configured PSFCH can be supported. If a past HARQ feedback is not received, Tx UE can trigger the Rx UE to feedback HARQ information of not only current PSSCH, but also HARQ information of other PSSCH(s) in next (pre)configured PSFCH occasion. This way can avoid PSFCH collision and PSFCH capacity problems. An illustration is shown as follow:

Figure 9 Illustration of dynamically indicated HARQ feedback occasion within the (pre)configured PSFCH

In Figure 9, PSFCH resource occasion period is 2 slots. When UE fails to clear the channel for PSFCH in slot n+1 corresponding to PSSCH1, then Rx UE can feedback the HARQ corresponding to PSSCH1 as well as HARQ corresponding to PSSCH2 in (pre)configured PSFCH occasion in slot n+3 according to the dynamic indication. The dynamically triggered additional PSFCH is more flexible than fixed additional PSFCH occasions. So dynamically indicated HARQ feedback occasion within the (pre)configured PSFCH is a preferred way to address PSFCH transmission dropping due to LBT failure.

The method to use triggered based HARQ feedback for additional PSFCH occasions on (pre)configured PSFCH can avoid PSFCH collision and PSFCH capacity problems.

Tx UE can trigger whether to feedback HARQ information of other PSSCH besides current PSSCH in (pre)configured PSFCH occasion associated with current PSSCH.

Frequency domain of PSFCH

For PSFCH frequency domain design, three main alternatives are discussed in RAN1#112[1]:

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS:the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

The alternatives of PSFCH transmission under 15 kHz and 30 kHz SCS can be analyzed in two dimensions, that is PSFCH capacity aspect and PSFCH transmission performance aspect.

- Capacity aspect discussion for PSFCH alternatives

Supposing 20M bandwidth, 15kHz SCS and NCSPSFCH= 6 pairs of cyclic shift are used for capacity analysis, so that there will be at most 100*6 candidate resource to be used for PSFCH for legacy NR SL Rel-16 UE :

- For Alt 1-1a, assuming K3 is 1, and then the PSFCH capacity would decrease 10% because 10 PRBs within the common interlace cannot be used for PSFCH transmission

- For Alt 2-2a, assuming K1 is 1, as one (pair of) cyclic shift is used as common cyclic shift out of 6 (pair of) cyclic shifts, then PSFCH capacity would decrease 16.7%

- For Alt 2-3a, one PSFCH need 10x PRBs compared with legacy NR SL PSFCH transmission, and one cyclic shift on all the PRBs within the interlace, then the PSFCH capacity would decrease 90% ([100*6-10*6]/100*6)

- For Alt 2-4a, the capacity decrease (decrease 90%) is same as Alt 2-3a, the only difference is Alt 2-4a has better PAPR performance than Alt 2-3a as UE uses different cyclic shifts on different PRBs in the interlace

- For Alt 3-2a, supposing K2 is 2, so that 2 PRBs are taken as the overhead. Then the PSFCH capacity would decrease 2%

From the PSFCH capacity analysis, it can be seen that Alt 1-1a, Alt 2-2a and Alt 3-2a are better than other alternatives under the assumption of 20M bandwidth, 15 kHz SCS and 6 pairs of cyclic shift. If changing the assumption, the capacity analysis is almost similar except for Alt 2-2a, for example if NCSPSFCH is less than 6, then Alt 2-2a may have the same capacity compared with legacy NR PSFCH transmission.

For RB-based interlace PSFCH transmission, Alt1-1a, Alt 2-2a and Alt 3-2a are better than other alternatives in capacity aspect.

- Performance aspect discussion for PSFCH alternatives

In order not to impact the PSFCH capacity significantly, the performance of Alt1-1a, Alt 2-2a and Alt 3-2a are analyzed further. Alt 1-1a and Alt 3-2a have similar performance because on a dedicated PRB resource, there is only dedicated PSFCH transmission. While for Alt 2-2a, on a dedicated PRB resource, besides a dedicated PSFCH transmission, multiple common cyclic shift signals can be transmitted by other UEs. The interference of Alt 2-2a is more serious than Alt 1-1a and Alt 3-2a, so Alt 2-2a will lead to deteriorated PSFCH receive performance due to simultaneous PSFCH transmission and common cyclic shift signal.

The interference of Alt 2-2a is more serious than that of Alt 1-1a and Alt 3-2a because of simultaneous PSFCH transmission and common cyclic shift signal transmission on the dedicated PRB(s).

For alt 1-1a and alt 3-2a, as each PSFCH transmission occupies some common PRBs and some dedicated PRB(s), the power on common PRB may be much higher than dedicated PRB, if dedicated PRB is adjacent to common PRB, IBE issue has to be considered, as shown in Figure 10 and Figure 11.

Figure 10 Example of PSFCH transmission for alt 1-1 assuming K3=1

Figure 11 Example of PSFCH transmission for alt 3-2 assuming K2=2 and K4=1

Location of common PRBs are uniformly distributed within RB set for alt 1-1a.

Location of common PRBs are centralized at two edge of RB set for alt 3-2a.

For Alt 1-1a and Alt 3-2a, as common signal and HARQ feedback signal are transmitted in FDMed manner, then the interference mainly comes from IBE. Some link level simulation results are provided to study the IBE interference in Alt 1-1a and Alt 3-2a further. Assuming the baseline SIR due to IBE from adjacent PRB is -20dB, then we simulated the PSFCH transmission performance considering the guard band with different number of PRBs.

Figure 12 Performance of 1PRB PSFCH transmission of alt 1-1a or alt 3-2a with different number of guard PRBs

In the result it can be seen that if dedicate PRB is adjacent to common PRB, and when the power on common PRB is relatively large, the performance is much worse, but if guard RB is considered to isolate interference, the performance will be improved significantly.

Dedicate PRB adjacent to common PRB will suffer IBE interference for alt 1-1a and alt 3-2a.

Guard PRBs between common PRB and dedicate PRB is beneficial for PSFCH reception performance.

Target on guard PRBs of Alt 1-1a and Alt 3-2a, as the common PRBs of Alt 3-2a locate at two edge of a RB set, it seems the guard PRBs are needed at two locations adjacent to the common PRBs, while for Alt 1-1a, the common PRBs are located distributed on 10 PRBs within an interlace, this design means PRBs need to be located at least 10 locations. So in the view of guard PRB overhead, Alt 3-2a is prioritized.

Guard PRB positions between common PRB and dedicate PRB needed for Alt 3-2a are less than Alt 1-1a.

Based on all the analysis and observations above, regarding PSFCH transmission under 15 kHz and 30 kHz SCS, Alt 3-2a is preferred. And for the PRBs convey PSFCH HARQ feedback information, at least 1PRB as legacy NR Rel-16 SL can be considered, and for the common PRBs to meet the OCB requirement, 2 common PRBs can be considered and the 2 common PRBs locate at the two edge of a RB set.

As PSD is required on unlicensed band in many regions, if dedicated PRB for PSFCH is 1PRB, then the max transmit power for the dedicated PRB is limited to PSD limitation (e.g.10 dBm/M HZ), so in order to boost the transmit power for dedicated PRB, more than one dedicated PRB where the dedicated PRBs are within a interlace can be considered.

Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, Alt 3-2a is preferred:

Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set

K4 is equal or larger than 1 (K4 PRBs are within one interlace to avoid PSD limitation if K4 is larger than 1)

Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details

Regarding PSFCH transmission under 60 kHz SCS, there is only 24 PRBs in a 20M RB set, and no interlace structure has been defined. In our opinion, one simple way is not supporting PSFCH transmission under 60 kHz SCS where OCB is required, that means only legacy NR Rel-16 one PRB method is considered for 60k Hz SCS on the region where OCB is not required.

Another way is to design the frequency structure as PSFCH transmission under 15 kHz and 30 kHz SCS, that is Each PSFCH transmission occupies 1 dedicated PRB(s) and some common PRBs, which is similar to Alt 3-2a of PSFCH transmission under 15 kHz and 30 kHz SCS.

Both ways are acceptable to us, and for a unified design of PSFCH transmission, the second way is slightly preferred.

Regarding PSFCH transmission under 60 kHz SCS, alt 1 is preferred:

Each PSFCH transmission occupies K dedicated PRB and 2 common PRBs

K4 is equal or larger than 1 (gap between each of K4 PRBs are larger than 1M Hz to avoid PSD limitation)

If Alt 1-1a or Alt 3-2a is applied, each PSFCH transmission occupies some common PRBs to meet the OCB requirement, and occupies one dedicated PRB to convey the HARQ feedback is enough consider the legacy NR SL PSFCH format. But the power needs to be split to two parts, one part for common PRB and another part for dedicate PRB, as observed above, the power on common PRB will cause IBE interference to dedicated PRB, besides guard band PRB between common PRB and dedicated PRB, decreased power can be set for common PRB. It should be pointed out that decreased power set for common PRBs is also beneficial for dedicated PSFCH performance.

To address the PSD limitation and IBE issue between common PRB and dedicated PRB of PSFCH, decreased power can be set for common PRBs.

One more thing for Alt 1-1a or Alt 3-2a is PAPR issue, cycle shift hopping like NR-U PUCCH format 0 can be considered for PSFCH transmission PRBs to alleviate the PAPR influence, with which PAPR is not a serious problem.

Power limitation for PSFCH

Agreement RAN1 further study the followings: Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

In different regions, PSD limitation is different [6], so it seems hard to update the power control formula to be applied to most situation. In our opinion, it is more proper to consider PSD limit in RAN4 rather than RAN1.

To address PSD limit over power control, PSD limitation for PSFCH is more proper to be considered in RAN4 since PSD limitation is set per region.

CSI measurement and feedback

R16 SL support wide-band CQI/RI feedback according to CSI-RS in unicast scenario, and MAC CE is used to convey the CSI feedback considering the aperiodic RS, for SL-U, transmissions cannot be guaranteed periodic, so R16 aperiodic CSI measurement and feedback mechanism can be studied as the start point for SL-U.

In shared spectrum, the channel bandwidth occupied by a UE may be as large as multiple RB sets, so sub-band CSI feedback can be considered for SL-U.

R16 CSI measurement and feedback mechanism can be reused for SL-U:

FFS sub-band (e.g., per-RB set) CSI feedback for interlace RB based transmission

Conclusion

According to the discussion above, we provide the following observations and proposals on physical layer structure for sidelink on FR1 unlicensed spectrum:

Observation 1:	If legacy NR SL structure were reused for slots with 2 candidate starting symbols, two problems would need to be handled:

•	The PSCCH blind decoding number in a slot will be beyond UE’s capacity at Rx UE side

•	Physical layer rate matching and resource mapping adjustment according to 2nd starting symbol after 1st starting symbol when LBT fails at 1st starting symbol will be beyond UE’s capacity at Tx UE side

Observation 2:	Common PSCCH time domain position for both 1st and 2nd starting symbol within slot is beneficial due to:

•	Same PSCCH mapping based on 2nd starting symbol for both 1st and 2nd starting symbol from Tx UE respective

•	Not increasing the PSCCH blind decoding complexity from RX UE respective

Observation 3:	For resource pool with PSFCH period equal to one, the number of symbols to determine TBS at Rx UE is sl-LengthSymbols -2-NsymbPSFCH.

Observation 4:	For resource pool with PSFCH period equal to zero, the number of symbols to determine TBS at Rx UE is Lref-2.

Observation 5:	Dynamically indicating the UE specific PSFCH resource will lead to PSFCH collision risk.

Observation 6:	The method to use triggered based HARQ feedback for additional PSFCH occasions on (pre)configured PSFCH can avoid PSFCH collision and PSFCH capacity problems.

Observation 7:	For RB-based interlace PSFCH transmission, Alt1-1a, Alt 2-2a and Alt 3-2a are better than other alternatives in capacity aspect.

Observation 8:	The interference of Alt 2-2a is more serious than that of Alt 1-1a and Alt 3-2a because of simultaneous PSFCH transmission and common cyclic shift signal transmission on the dedicated PRB(s).

Observation 9:	Location of common PRBs are uniformly distributed within RB set for alt 1-1a.

Observation 10:	Location of common PRBs are centralized at two edge of RB set for alt 3-2a.

Observation 11:	Dedicate PRB adjacent to common PRB will suffer IBE interference for alt 1-1a and alt 3-2a.

Observation 12:	Guard PRBs between common PRB and dedicate PRB is beneficial for PSFCH reception performance.

Observation 13:	Guard PRB positions between common PRB and dedicate PRB needed for Alt 3-2a are less than Alt 1-1a.

Proposal 1:	To address not increasing UE implementation complexity, for a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

•	Support common PSCCH time domain position starting next to 2nd PSCCH/PSSCH transmission starting symbol

•	Only once PSCCH/PSSCH mapping based on 2nd starting symbol for both 1st and 2nd starting symbol is performed

Proposal 2:	For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from 1st starting symbol:

•	From Tx UE respective, PSCCH/PSSCH transmission has only 1 AGC symbol as Rel-16 NR SL UE at 1st starting symbol

•	From Rx UE respective, it is left to UE implementation to monitor 1 or 2 symbol(s) for AGC purpose

Proposal 3:	Confirm the WA that if a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

•	At least for COT initiation, TBS is determined based on a reference number of symbols which is determined by (pre-)configuration

•	Value range of reference number of symbols (Lref) can be from 7 to 14

Proposal 4:	Different reference number of symbols for transmission in a shared COT is not needed.

Proposal 5:	TBS of the transmissions associated with the same TB should be same as legacy NR SL

•	The PSFCH overhead is determined by PSFCH (pre-)configuration and/or indication

Proposal 6:	For resource pool with PSFCH period larger than one, either Lref-2 or sl-LengthSymbols -2-NsymbPSFCH is used to calculate TBS.

•	Which number is used should be recognized at Rx UE side. (e.g. choose one of Lref-2 or sl-LengthSymbols -2-NsymbPSFCH according to a predefined rule or indication)

Proposal 7:	Frequency range of one S-SSB should be within a determined RB set from system perspective.

Proposal 8:	In S-SSB slot, no need to transmit S-SSB repetitions or other signals in more than one RB set to maintain a COT.

Proposal 9:	Temporary exemption of OCB requirement is not applicable for S-SSB transmission due to:

•	It is hard to justify whether slot level SSB transmission is ‘temporary’ from system perspective

•	Small bandwidth of SSB will impact the SSB coverage due to the PSD limitation

•	11 PRBs S-SSB only occupies 1.98MHz under 15kHz SCS which is less than minimum of 2 MHz requirement

Proposal 10:	If OCB is required, interlaced RB transmission for S-SSB (option1-1) is preferred for 15 kHz and 30 kHz SCS:

•	S-SSB maps to 20RBs within two adjacent interlaces for a unified design

Proposal 11:	If OCB is not required, legacy NR Rel-16/17 S-SSB slot structure is reused directly for 15 kHz and 30 kHz SCS.

Proposal 12:	60 kHz SCS S-SSB can only be used for regions where OCB is not required and legacy NR Rel-16/17 S-SSB slot structure is reused.

Proposal 13:	To determine the frequency location of S-SSB, the following information can be provided:

•	Alt 1: ARFCN pointing to a specific RE determining the REs of S-SSB as legacy NR Rel-16 SL

•	Alt 2: RB set index and IRB index within the RB set

•	Alt 3: RB set index and predefined IRBs within the RB set

Proposal 14:	R16 SL synchronization procedure can be reused for SL-U.

Proposal 15:	Support option 2, i.e. additional candidate S-SSB transmission occasions can belong to resource pool.

Proposal 16:	To avoid AGC issue and inter-UE blocking between S-SSB and PSSCH in the same slot：

•	Frequency domain resource of S-SSB and PSCCH/PSSCH are orthogonal

•	S-SSB transmission using a subset of symbols in such slot is not allowed if multiple candidate starting symbols are not allowed.

Proposal 17:	With regard to additional S-SSB occasions belong to resource pool:

•	Conflict between S-SSB Tx and PSCCH/PSSCH Tx, S-SSB Rx and PSCCH/PSSCH Tx can be handled according to the priority comparison

•	Time domain and frequency domain resource of additional S-SSB transmission occasions locate in (pre-)configured/(pre-)defined resources similar to legacy NR Rel-16/17 S-SSB transmission occasions

Proposal 18:	Support multiple candidate starting symbols for S-SSB transmission out of resource pool or in resource pool with two starting symbols enabled.

Proposal 19:	Regarding the number and location(s) of additional candidate S-SSB occasions, support adding some new available values in sl-NumSSB-WithinPeriod for each SCS if necessary.

Proposal 20:	In the same S-SSB period, Tx UE will try any occasions until the transmitted SSB meets certain number.

Proposal 21:	For multi-slot consecutive transmission time domain resource location indication, two alternatives based on TRIV can be further considered:

•	Alt.1: TRIV is reinterpreted to indicate each set of first slot position and duration of multi-slot consecutive transmission

•	Alt.2: Additional filed is added to indicate the duration for each time resource in addition to legacy TRIV

Proposal 22:	For usage of PRB(s) in the intra-cell guard band:

•	For interlace RB based transmission, PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

•	For continuous RB based transmission, the PRB(s) in the intra-cell guard band can only be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

•	The PRB(s) in the intra-cell guard band is not used for PSFCH/S-SSB transmission

Proposal 23:	For interlace RB-based PSCCH/PSSCH transmission, PSSCH frequency indication is that:

•	Both FRIV1 for subchannel information (start subchannel indexes and subchannel number L1) and FRIV2 for RB set information (start RB set indexes and RB set number L2) are indicated

•	The number of RB set of the transmissions to the same TB is unchanged to reuse legacy NR SL FRIV method

•	The number of subchannel of the transmissions to the same TB is unchanged to reuse legacy NR SL FRIV method

Proposal 24:	For interlace RB-based PSCCH/PSSCH transmission, single slot candidate resource needs to be modified to capture both RB set information and subchannel definition.

Proposal 25:	For interlace RB-based PSCCH/PSSCH transmission, the number of RBs within a sub-channel is (pre-)configured to be fixed as 10*K RBs (K is the interlace(s) within a sub-channel):

•	Extra RB(s) outside sub-channel can be additionally used when they are available and FFS details

Proposal 26:	For contiguous RB-based transmission, remaining PRBs in the resource pool is not used by UE.

Proposal 27:	For contiguous RB-based transmission, availability of a subchannel which contains PRB(s) within guard band needs to be justified:

•	Such subchannel can be independently used if the number PRBs within guard band is below a threshold

•	Otherwise, such subchannel could not be independently used

Proposal 28:	For contiguous RB-based transmission, with regard to SL-U resource allocation, the following rules need to be observed:

•	For the region where OCB is required, the mode 1 allocated resource or mode 2 selected resource shall meet the OCB requirement

•	For the region where OCB is not required, legacy NR Rel 16 resource allocation mechanism is reused

•	The mode 1 allocated resource or mode 2 selected resource shall meet the condition of guard PRB usage

Proposal 29:	To address PSFCH transmission dropping due to LBT failure, option1 is supported:

Proposal 30:	Tx UE can trigger whether to feedback HARQ information of other PSSCH besides current PSSCH in (pre)configured PSFCH occasion associated with current PSSCH.

Proposal 31:	Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, Alt 3-2a is preferred:

•	Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set

•	K4 is equal or larger than 1 (K4 PRBs are within one interlace to avoid PSD limitation if K4 is larger than 1)

Proposal 32:	Regarding PSFCH transmission under 60 kHz SCS, alt 1 is preferred:

•	Each PSFCH transmission occupies K dedicated PRB and 2 common PRBs

•	K4 is equal or larger than 1 (gap between each of K4 PRBs are larger than 1M Hz to avoid PSD limitation)

Proposal 33:	To address the PSD limitation and IBE issue between common PRB and dedicated PRB of PSFCH, decreased power can be set for common PRBs.

Proposal 34:	To address PSD limit over power control, PSD limitation for PSFCH is more proper to be considered in RAN4 since PSD limitation is set per region.

Proposal 35:	R16 CSI measurement and feedback mechanism can be reused for SL-U:

•	FFS sub-band (e.g., per-RB set) CSI feedback for interlace RB based transmission

References

Chair's notes RAN1#112

Chair's notes RAN1#111

TS 38.213 Physical layer procedures for control

ETSI EN 301 893, Harmonized European Standard, "Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN"

TS 38.101-1 User Equipment (UE) radio transmission and reception;Part 1: Range 1 Standalone

TS 38.889 Study on NR-based access to unlicensed spectrum

TS 38.215 Physical layer measurements

Simulation Assumption

- Table1: Link level PSFCH simulation assumption

| carrier | 6G |
| --- | --- |
| Ant number | 1Tx2Rx |
| Channel model | TDL-C |
| TTI number | 100000 |
| SCS | 15k |
| BW | 20M HZ |
| PSFCH frequency | Each PSFCH transmission occupies 1 dedicate PRB and some common PRBs |