3GPP TSG RAN WG1 #112bis-e		R1- 2303403

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.4.3

Source: 	ZTE, Sanechips

Title: 	Discussion on enhanced sidelink operation on FR2 licensed spectrum

Document for:	Discussion and decision

Introduction

In RAN #99[1], WID on sidelink enhancement for FR2 remained the same as in RAN# 98-e [2]. In RAN1#112 meeting, some agreements on enhanced sidelink operation on FR2 licensed spectrum were made as below [3].

RP-222806 Revised WID: NR sidelink evolution Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only. Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

In this contribution, we further discuss the issues on beam management for SL FR2 licensed spectrum and provide our analysis and proposals.

Discussions

Initial beam-pairing

For initial beam-pairing, one issue is the relationship between PC5 unicast link establishment and sidelink initial beam pairing, i.e. whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.

For the case that initial beam pairing procedure starts before sidelink unicast link establishment procedure

In NR Uu, every UE needs to establish Uu links with a gNB in order to obtain services and perform subsequent DL/UL traffic regardless. Unlike NR Uu, in sidelink, the link establishment among UEs are not deterministic given UE does not know which UEs it will establish a sidelink unicast link with by the time when the traffic arrives. A UE may perform unicast, groupcast or even broadcast depending on the sidelink traffic. If initial beam pairing procedure starts before sidelink unicast link establishment procedure, then: 1) Tx UE must always transmit beam training signals regardless of whether surrounding sidelink UEs want to establish a sidelink unicast link with it, which is agnostic to the Tx UE itself; 2) Rx UE must always receive/measure beam training signals transmitted by other UEs in order to obtain the initial beam pairing, even if, immediately after that, the Rx UE does not want to establish sidelink unicast link with the Tx UEs transmitting beam training signals. As a matter of fact, the Rx UE may end up performing unicast, groupcast or broadcast transmission.

Therefore, initial beam pairing procedure starting before sidelink unicast link establishment procedure may lead to unnecessary beam training and resources overhead. Given the remaining Rel-18 TU, it's not foreseeable such issues can be addressed.

Initial beam pairing procedure starting before sidelink unicast link establishment procedure may lead to unnecessary beam training and resources overhead.

Initial beam pairing is not supported before sidelink unicast link establishment procedure in Rel-18.

For the case that initial beam pairing procedure starts during sidelink unicast link establishment procedure

Initial beam paring can be performed during sidelink unicast link establishment procedure. For example, a first UE can transmit a Direct Communication Request (DCR) via multiple beams with both UE ID and beam ID, a second UE can transmit signaling related to security establishment and report the preferred beam ID(s) and relevant UE ID based on beam training using DCR. Additional beam training signal overhead is needed to support the UE specific beam training.

For the case that initial beam pairing procedure starts after sidelink unicast link establishment procedure

Initial beam paring can be performed based on PC5 RRC configured RS after sidelink unicast link establishment procedure. This does not mean that directional beam is not used during the sidelink unicast link establishment procedure. In other words, whether to use beam for sidelink unicast link establishment depends on UE implementation. While the support of directional beam training during the link establishment process itself is overhead consuming.

Initial beam pairing is supported after sidelink unicast link establishment procedure. Further assess the overhead issues that come with the initial beam pairing during sidelink unicast link establishment procedure.

Another issue for initial beam pairing is which reference signals or sidelink channels can be used for initial beam pairing.

In the last meeting, reference signals or sidelink channels that may be used for initial beam pairing were discussed, but there is no agreement on this topic. The candidate reference signals or sidelink channels were listed in the following FL Proposal.

Proposal 1-2-c: RAN1 is to study whether and how the following could be enhanced for sidelink initial beam pairing: - S-SSB - Sidelink CSI RS - PSCCH/PSSCH DMRS - PSFCH

S-SSB

In NR SL R16 and R17, S-SSB resources are shared for multiple of UEs, and S-SSB does not include UE specific ID. If S-SSB is used for initial beam training, it is necessary to enhance information carried by S-SSB, and study how to avoid conflicts of S-SSB resources among different UEs. Therefore, If S-SSB is used for initial beam training, there are at least the following issues:

1) S-SSB does not include UE specific ID. To solve this issue, it is necessary to enhance the S-PSS/S-SSS sequence generation or PSBCH content to provide the ID of UE sending S-SSB. However, this may lead to a lot of specification work for RAN1. Moreover, it is necessary to separately configure S-SSB for synchronization and for initial beam pairing, otherwise, the legacy UEs could not distinguish these two types of S-SSBs;

2) Configuring a larger number of S-SSB resources may lead to low resource utilization, or may affect sensing procedure and lead to half duplex issues. In NR SL R16/17, an S-SSB consists of one slot in the time domain and 11 RBs in the frequency domain. When there are a large number of sidelink UEs, it is necessary to configure a large number of S-SSB resources for initial beam pairing, otherwise, S-SSB resource conflicts between UEs were likely to occur. Assuming that the S-SSB resources are outside the resource pool, resource utilization will be degraded because from UE perspective the resource cannot be used to transmit data even if S-SSB is not transmitted for initial beam pairing. Assuming that the S-SSB resources are within the resource pool, sensing procedure needs to be updated to accommodate the beam training S-SSBs apart from half duplex issues;

3) Need to study new mechanisms to avoid S-SSB resource conflicts among mode 2 UEs. For mode 1 UE, gNB can avoid resource allocation conflicts among UEs through scheduling. However, for mode 2 UE, legacy resource reservation and sensing mechanisms cannot work, it is necessary to consider new mechanisms to avoid S-SSB resource conflicts among UEs.

If S-SSB is used for sidelink initial beam pairing, there are at least the following issues:

Legacy S-SSB does not include UE specific ID;

Configuring a larger number of S-SSB resources may lead to low resource utilization if the S-SSB resources are outside the resource pool, or may affect sensing procedure and lead to half duplex issues if the S-SSB resources are within the resource pool;

Need to study new mechanisms to avoid S-SSB resource conflicts among mode 2 UEs.

S-SSB is not used for sidelink initial beam pairing in Rel-18.

Sidelink-CSI-RS

In NR Uu, CSI-RS can be used for beam management. Similarly, CSI-RS can be used for initial beam pairing in sidelink. For sidelink operation on FR2 licensed spectrum, the WID [1][2] proposed that the study is limited to the support of sidelink beam management by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. In R16/R17 sidelink, CSI-RS can only be configured or transmitted after sidelink unicast link establishment procedure, therefore, CSI-RS is suitable for initial beam pairing after sidelink unicast link establishment procedure.

CSI-RS is used for initial beam pairing after sidelink unicast link establishment procedure.

PSCCH/PSSCH DMRS

As previously described, initial beam pairing can be performed during sidelink unicast link establishment procedure. In this case, PSCCH/PSSCH DMRS can be used for initial beam pairing. For example, a first UE can transmit PSCCH/PSSCH carrying DCR message and Tx beam ID by beam sweeping, a second UE can measure the PSCCH/PSSCH DMRS for beam training and respond by establishing the security with the first UE, meanwhile, the second UE can report the preferred Tx beam ID of the first UE.

As previously described, initial beam pairing can also be performed after sidelink unicast link establishment procedure. In this case, PSCCH/PSSCH DMRS can also be used for initial beam pairing. For example, a first UE can transmit PSCCH/PSSCH in which at least Tx beam ID is included by beam sweeping, a second UE can measure the PSCCH/PSSCH DMRS and report prefered Tx beam ID to the first UE by, such as, PSFCH associated with the PSSCH or MAC CE.

PSCCH/PSSCH DMRS can be used for initial beam pairing after sidelink unicast link establishment procedure. If initial beam pairing during sidelink unicast link establishment is supported in Rel-18, PSCCH/PSSCH DMRS can also be used for initial beam pairing during sidelink unicast link establishment procedure.

PSFCH

As previously described, initial beam pairing can be performed during or after sidelink unicast link establishment procedure. In this case, a first UE can transmit PSCCH/PSSCH via multiple beam directions, a second UE can measure the PSCCH/PSSCH DMRS or CSI-RS within the PSCCH/PSSCH transmission slot and feedback whether the beam of PSCCH/PSSCH meets the requirements or not to the first UE by PSFCH associated with the PSSCH.

PSFCH can be used for initial beam pairing during or after sidelink unicast link establishment procedure. If initial beam pairing during sidelink unicast link establishment is deemed worth pursing in Rel-18, PSFCH can be used for initial beam pairing also be used for initial beam pairing during sidelink unicast link establishment procedure.

Beam maintenance

Design of CSI-RS for Beam management

In section 2.1, we have discussed that the reference signals or channels that may be used for initial beam pairing. In our view, these reference signals or channels can also be used for beam maintenance. In this section we discuss the design of the reference signals or channels that may be used for initial beam pairing and beam maintenance, and we mainly study the design of CSI-RS because PSCCH/PSSCH DMRS and PSFCH can mainly adopt the legacy mechanism.

In R16/R17 sidelink, CSI-RS can only be transmitted within a unicast PSSCH transmission, which means that CSI-RS in a slot can only be transmitted in one beam direction. Otherwise, the performance of PSSCH would be affected since CSI-RS is FDMed in REs with PSSCH. AGC issue may arise as well if multiple Tx beams with significantly different directions were used in a slot while AGC training is only performed at the first symbol of the sidelink slot. Therefore, if R16/R17 CSI-RS mechanism is reused without enhancement, it may take a long time to complete a beam training.

If R16/R17 CSI-RS mechanism is reused without enhancement, it may take a long time to complete a beam training because CSI-RS can be only transmitted together with PSSCH in one beam direction within one slot.

According to the relationship between CSI-RS resources and a PSSCH transmission, the following options can be considered for CSI-RS resource configuration used for beam training:

Option 1: CSI-RS resources are TDMed with PSCCH/PSSCH in a slot, and have the same bandwidth as the PSSCH, as shown in Figure 1.

Option 2: Configure a CSI-RS resource pool, or configure dedicated CSI-RS resources in a resource pool shared with PSSCH, as shown in Figure 2 and Figure 3.

In both options, multiple CSI-RSs can be configured in a slot in order to decrease beam training time.

Figure 1: CSI-RS resources are TDMed with PSSCH in a slot, and have the same bandwidth as the PSSCH

Figure 2: CSI-RS resources in a slot of a CSI-RS resource pool

Figure 3: Dedicated CSI-RS resources TDMed with PSSCH in the same resource pool

The following options for CSI-RS resource configuration can be considered:

Option 1(non-standalone CSI-RS)：CSI-RS resources are TDMed with PSCCH/PSSCH transmission in a slot, and have the same transmission bandwidth with the PSSCH transmission;

Option 2(standalone CSI-RS)：Configure a CSI-RS resource pool, or configure dedicated CSI-RS resources which are TDMed with PSSCH in the same resource pool.

In addition, it is also necessary to consider the time-domain characteristics of CSI-RS resources for beam management. For example, considering that the resources configured for R16/R17 SL CSI-RS are aperiodic. Aperiodic CSI-RS resources for beam training should at least be supported, and whether to support periodic CSI-RS resources can be further studied.

At least support aperiodic CSI-RS resources, and FFS the support of periodic CSI-RS resources.

Tx Beam management

In NR Uu, gNB performs Tx beam sweeping, UE measures corresponding reference signals of different beam directions from gNB and reports the measurement results, e.g. CSI-RS resource indicator, to the gNB, then gNB can select Tx beam based on the measurement results reported by the UE.

Similarly, in sidelink, Tx UE can transmit reference signals by Tx beam sweeping, Rx UE can measure corresponding reference signals of different beam directions and reports the measurement results to Tx UE, then Tx UE can select a proper Tx beam based on the measurement results reported by the Rx UE.

NR DL Tx beam training procedure can be the starting point for sidelink Tx beam training procedure.

Depending on the time-domain characteristics of CSI-RS resource configuration, aperiodic beam reporting should be at least supported. Apart from aperiodic CSI-RS resources, whether to support periodic CSI-RS and periodic beam reporting needs further study.

At least support aperiodic beam reporting, and whether to support periodic beam reporting needs further study.

After configuring CSI-RS resources for beam management, CSI-RS resources of Tx UE should be indicated to Rx UE so that beam management can be performed based on the CSI-RS resources. For option 1(non-standalone CSI-RS), CSI-RS and PSSCH are transmitted in the same slot by TDM manner, and have the same bandwidth. Therefore, one way is to indicate by Tx UE whether to include CSI-RSs for beam management in current slot by sidelink control information.

For option 1(non-standalone CSI-RS), Tx UE can indicate whether to transmit CSI-RSs for beam management in current slot by sidelink control information so that beam management can be performed based on the CSI-RS resources.

For option 1(non-standalone CSI-RS), a Tx UE can transmit different CSI-RSs in different beam directions in a slot. If the angle between these different beam directions is large, the received power of the Rx UE may vary significantly. Which may cause AGC problems at the Rx UE since the Rx UE only performs AGC training at the starting symbol of the slot.

For option 1(non-standalone CSI-RS), AGC issues may occur at Rx UE if Tx UE transmits different CSI-RSs in different beam directions in a slot.

For option 2(standalone CSI-RS), one issue is how to allocate and indicate CSI-RS resources to different UEs so that beam management can be performed based on the CSI-RS resources. In our view, at least two alternatives can be considered:

Alt 1: Pre-definition: Pre-define the mapping rules between PSSCH and CSI-RS resources for beam management. Similar to the mapping method between PSSCH and associated PSFCH;

Alt 2: Dynamical indication: CSI-RS resources is dynamically indicated. Similar to aperiodic CSI-RS resources are triggered by DCI in NR Uu.

For option 2(standalone CSI-RS), the following two alternatives can be considered for the issue of how to allocate CSI-RS resources to different UEs:

Alt 1: Pre-definition: Pre-define the mapping rules between PSSCH and CSI-RS resources for beam management;

Alt 2: Dynamical indication: CSI-RS resources is dynamically indicated.

In R16/R17 sidelink, aperiodic CSI report can be triggered for Rx UE when ‘CSI request’ field is set to 1 by Tx UE. Rx UE can be configured with a CSI reporting latency boundary as indicated by the higher layer parameter sl-LatencyBoundCSI-Report. Rx UE reports CSI in MAC CE within the CSI reporting latency boundary. In our view, R16/R17 aperiodic CSI report mechanism can be considered as a starting point for aperiodic beam reporting. For example, Rx UE can send beam reporting to Tx UE within a latency boundary starting from the slot where the beam reporting request from Tx UE is received, and beam reporting can be carried by MAC CE.

R16/R17 sidelink aperiodic CSI report mechanism can be considered as a starting point for aperiodic beam reporting, e.g. introducing a latency boundary for beam reporting, beam reporting can be carried by MAC CE.

In NR Uu, a UE can be provided the configuration for beam management reporting by gNB. Report content includes CSI-RS resource indicator, L1-RSRP and L1-SINR etc. Similarly, in sidelink, the beam reporting can include preferred Tx beam ID, measured value of L1-RSRP/L1-SINR for each preferred Tx beam etc. in addition to the CSI information carried in MAC CE from Rel-16.

In MAC CE, the following report infomation for sidelink beam management can be considered: beam ID (or CRI), L1-RSRP/L1-SINR etc.

Rx Beam management

Normally a UE may have multiple unicast transmissions to receive at the same time in a slot, the multiple unicast transmissions may correspond to different preferred receiving beams if Rx beam is supported. If a UE needs to receive broadcast/groupcast transmissions in a slot, the UE should perform receiving in the slot in all directions because the UE does not know Tx beam information of Tx UE. Therefore, if UE has to receive multiple unicast transmissions or broadcast transmissions or groupcast transmissions in a slot, it is difficult for the UE to determine the preferred receiving beam. Based on the above analysis and remaining Rel-18 TU, we recommend a low priority of studying Rx beam training.

If UE has to receive multiple unicast transmissions or broadcast transmissions or groupcast transmissions in a slot, it is difficult for the UE to determine the preferred receiving beam.

Studying Rx beam training takes a low priority in R18.

Beam indication

Generally, a Tx UE should indicate its Tx beam for SL transmission to a Rx UE so that the Rx UE can determine the Rx beam for this SL transmission. If the Rx UE does not adopt directional beam reception, the Tx UE does not need to indicate a Tx beam to the Rx UE, otherwise the Tx UE should indicate its Tx beam to the Rx UE.

Whether the Tx UE indicates its Tx beam to the Rx UE depends on whether the Rx UE adopts directional beam reception.

In NR Uu, gNB can indicate the Tx beam of PDSCH by its scheduling DCI, then the Rx can decode the DCI to determine the Tx beam and corresponding Rx beam. While in NR sidelink, PSSCH and its associated PSCCH are transmitted simultaneously. Same TX beam should be applied for PSSCH and its associated PSCCH relying on legacy AGC handling frame structure. Therefore, the Tx beam of PSSCH and its associated PSCCH should be indicated in advance.

If Rx beam is supported, the Tx UE indicating the beam used for future/reserved PSCCH/PSSCH transmission to the Rx UE can be considered.

As previously described, PSCCH/PSSCH DMRS can be used for initial beam pairing after sidelink unicast link establishment procedure. Similarly, PSCCH/PSSCH DMRS can also be used for beam maintenance. Tx UE can indicate the beam ID used for current PSCCH/PSSCH DMRS transmission to the Rx UE to assist in initial beam pairing or beam maintenance.

Tx UE indicating the beam used for current PSCCH/PSSCH transmission to the Rx UE can assist in initial beam pairing or beam maintenance.

Study the Tx UE indicating the beam used for current PSCCH/PSSCH DMRS transmission to the Rx UE.

Beam failure detection/recovery

In NR SL R16/17, Sidelink RLF detection is based on a number of consecutive DTX on PSFCH reception occasions. In our view, DTX on PSFCH reception occasions can also be used for beam failure detection if ACK/NACK feedback is also used to indicate whether the beam of PSSCH associated with PSFCH meets the requirements. The reasons are as follows:

Generally, the successful decoding of the PSSCH by Rx UE means that the Tx beam of the PSSCH is preferred, then the Rx UE can transmit ACK to the Tx UE. The failure of decoding the PSSCH means that the Tx beam of the PSSCH is not preferred, or means that the Tx beam of the PSSCH is preferred but the PSSCH reception is interfered by signals from other devices. Therefore, when the Rx UE cannot successfully decode the PSSCH, it is inaccurate to determine whether the Tx beam of the PSSCH meets the requirements only based on the decoding results of the PSSCH, and it is also necessary to consider the measured value of L1-RSRP/L1-SINR of PSCCH/PSSCH DMRS or CSI-RS corresponding to the PSSCH. For example, if the Rx UE cannot successfully decode the PSSCH and the measured value of L1-RSRP/L1-SINR is greater than the threshold, Rx UE can transmit NACK to Tx UE. If the Rx UE cannot successfully decode the PSSCH and the measured value of L1-RSRP/L1-SINR is not greater than the threshold, Rx UE does not transmit PSFCH to Tx UE. In this way, as long as the PSFCH is received by Tx UE, the Tx UE can consider the Tx beam of the PSSCH is preferred.

Tx UE can determine whether the Tx beam meets the requirements based on whether it receives ACK/NACK.

Study beam failure detection based on the number of HARQ feedback DTX if ACK/NACK feedback is also used to indicate whether the beam of PSSCH associated with PSFCH meets the requirements.

The issues related to beam failure recovery can refer to those of beam maintenance as above, such as CSI-RS resource configuration for beam quality measurements, preferred Tx beam reporting etc. Therefore, the issues related to beam failure recovery can be studied after reaching some conclusions on SL beam maintenance.

The issue related to beam failure recovery is put on hold till some conclusions on SL beam maintenance are made.

Conclusion

Based on above discussion, the following observation and proposals are given:

Observation 1:	Initial beam pairing procedure starting before sidelink unicast link establishment procedure may lead to unnecessary beam training and resources overhead.

Observation 2:	If S-SSB is used for sidelink initial beam pairing, there are at least the following issues:

•	Legacy S-SSB does not include UE specific ID;

•	Configuring a larger number of S-SSB resources may lead to low resource utilization if the S-SSB resources are outside the resource pool, or may affect sensing procedure and lead to half duplex issues if the S-SSB resources are within the resource pool;

•	Need to study new mechanisms to avoid S-SSB resource conflicts among mode 2 UEs.

Observation 3:	If R16/R17 CSI-RS mechanism is reused without enhancement, it may take a long time to complete a beam training because CSI-RS can be only transmitted together with PSSCH in one beam direction within one slot.

Observation 4:	For option 1(non-standalone CSI-RS), AGC issues may occur at Rx UE if Tx UE transmits different CSI-RSs in different beam directions in a slot.

Observation 5:	If UE has to receive multiple unicast transmissions or broadcast transmissions or groupcast transmissions in a slot, it is difficult for the UE to determine the preferred receiving beam.

Observation 6:	Whether the Tx UE indicates its Tx beam to the Rx UE depends on whether the Rx UE adopts directional beam reception.

Observation 7:	Tx UE indicating the beam used for current PSCCH/PSSCH transmission to the Rx UE can assist in initial beam pairing or beam maintenance.

Proposal 1:	Initial beam pairing is not supported before sidelink unicast link establishment procedure in Rel-18.

Proposal 2:	Initial beam pairing is supported after sidelink unicast link establishment procedure. Further assess the overhead issues that come with the initial beam pairing during sidelink unicast link establishment procedure.

Proposal 3:	S-SSB is not used for sidelink initial beam pairing in Rel-18.

Proposal 4:	CSI-RS is used for initial beam pairing after sidelink unicast link establishment procedure.

Proposal 5:	PSCCH/PSSCH DMRS can be used for initial beam pairing after sidelink unicast link establishment procedure. If initial beam pairing during sidelink unicast link establishment is supported in Rel-18, PSCCH/PSSCH DMRS can also be used for initial beam pairing during sidelink unicast link establishment procedure.

Proposal 6:	PSFCH can be used for initial beam pairing during or after sidelink unicast link establishment procedure. If initial beam pairing during sidelink unicast link establishment is deemed worth pursing in Rel-18, PSFCH can be used for initial beam pairing also be used for initial beam pairing during sidelink unicast link establishment procedure.

Proposal 7:	The following options for CSI-RS resource configuration can be considered:

•	Option 1(non-standalone CSI-RS)：CSI-RS resources are TDMed with PSCCH/PSSCH transmission in a slot, and have the same transmission bandwidth with the PSSCH transmission;

•	Option 2(standalone CSI-RS)：Configure a CSI-RS resource pool, or configure dedicated CSI-RS resources which are TDMed with PSSCH in the same resource pool.

Proposal 8:	At least support aperiodic CSI-RS resources, and FFS the support of periodic CSI-RS resources.

Proposal 9:	NR DL Tx beam training procedure can be the starting point for sidelink Tx beam training procedure.

Proposal 10:	At least support aperiodic beam reporting, and whether to support periodic beam reporting needs further study.

Proposal 11:	For option 1(non-standalone CSI-RS), Tx UE can indicate whether to transmit CSI-RSs for beam management in current slot by sidelink control information so that beam management can be performed based on the CSI-RS resources.

Proposal 12:	For option 2(standalone CSI-RS), the following two alternatives can be considered for the issue of how to allocate CSI-RS resources to different UEs:

•	Alt 1: Pre-definition: Pre-define the mapping rules between PSSCH and CSI-RS resources for beam management;

•	Alt 2: Dynamical indication: CSI-RS resources is dynamically indicated.

Proposal 13:	R16/R17 sidelink aperiodic CSI report mechanism can be considered as a starting point for aperiodic beam reporting, e.g. introducing a latency boundary for beam reporting, beam reporting can be carried by MAC CE.

Proposal 14:	In MAC CE, the following report infomation for sidelink beam management can be considered: beam ID (or CRI), L1-RSRP/L1-SINR etc.

Proposal 15:	Studying Rx beam training takes a low priority in R18.

Proposal 16:	If Rx beam is supported, the Tx UE indicating the beam used for future/reserved PSCCH/PSSCH transmission to the Rx UE can be considered.

Proposal 17:	Study the Tx UE indicating the beam used for current PSCCH/PSSCH DMRS transmission to the Rx UE.

Proposal 18:	Tx UE can determine whether the Tx beam meets the requirements based on whether it receives ACK/NACK.

Proposal 19:	Study beam failure detection based on the number of HARQ feedback DTX if ACK/NACK feedback is also used to indicate whether the beam of PSSCH associated with PSFCH meets the requirements.

Proposal 20:	The issue related to beam failure recovery is put on hold till some conclusions on SL beam maintenance are made.

References

WID revision: NR sidelink evolution, OPPO, RAN #99, March 2023

RP-222806 Revised WID: NR sidelink evolution, OPPO, RAN #98-e, December 2022

Chair's notes RAN1#112, Chairman, RAN1 #112, February 2023

- R1-2301845 FL summary #3 for AI 9.4.3 Enhanced sidelink operation on FR2 licensed spectrum Moderator (Apple) 9.4.3 RAN1#112, February 2023