3GPP TSG-RAN WG1 #112bis-e	R1-2303404

e-Meeting, April 17th – April 26th, 2023

Title 	:  Draft CRs and higher layer signalling for others

Source 	:  ZTE

Agenda item	:  9.18

Document for	:  Discussion/Decision

# Introduction

After RAN1#112 meeting, the WI “multi-carrier enhancements” has been completed from RAN1 perspective. In this contribution, we provide our initial views on draft CR and RRC parameters for MC-Enh, BWP without restriction, and eDSS respectively.

# MC enhancements

## Multi-cell scheduling

### RRC signalling for multi-cell scheduling

In this section, we provide analysis of the RRC signalling for multi-cell scheduling based on the reached agreements.

For multi-cell scheduling, the multiple PDSCHs/PUSCHs scheduled by the single DCI are in the different cells.  Therefore, the scheduling related configurations, including the set of co-scheduled cells and the higher layer parameters for MC-DCI size determination may include the configurations for multiple cells. They should be in the IE PhysicalCellGroupConfig, which is used to configure the cell group specific parameters.  Considering that the up to 4 sets can be configured for the UE, the multiple sets of multi-cell scheduling should be configured by means of add and release list. The configurations for each set of cells for multi-cell scheduling are configured independently.

Proposal 1: The scheduling related configuration should be in the IE PhysicalCellGroupConfig, and an example is shown below, where maxNrofMC-Sets = 4.

PhysicalCellGroupConfig ::= SEQUENCE { MC-schedulingSetToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofMC-Sets)) OF MC-schedulingSet-r18 MC-schedulingSetToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofMC-Sets)) OF MC-schedulingSetId-r18 }

For each set, the followings should be configured.

- First, the set ID should be configured to identify a set when released.

- For each set, at most 4 cells can be configured.

- It was agreed that the DCI size and BD/CCE budget is counted only on the reference cell, which is one cell among the set of cells. It is up to gNB to configure the reference cell. Therefore, the reference cell should be configured. To reduce the RRC signalling overhead, only 4 values can be included, e.g., 0~3. The value 0 indicates the first scheduled cell in the set. The value 1 indicates the second scheduled cell in the set, and so on.

- It was also agreed that n_CI is configured by RRC for determining the CCE index of the search space with DCI format 0_X/1_X for each set of cells. Considering that the legacy value of n_CI include 0~7 and at most 4 sets can be configured for the UE, the value range of n_CI for DCI format 0_X/1_X should be 0~11.

- The same scheduling cell can be configured for multiple sets. The DCIs for scheduling the any set of cells are transmitted on the same scheduling cell. In this case, it was agreed that an indicator is included in the DCI to indicate the scheduled set of cells. Therefore, the value of the set indicator in the DCI should be configured for the set, which is similar as the CIF configuration for legacy cross carrier scheduling. The value range should be 0~3.

- For the set, the co-scheduled cell indication should be configured. Two methods are specified for co-scheduled cell indication. One is implicit indication via the FDRA field when the TDRA table for the set is not configured and the other one is explicit indication via a table of co-scheduled cell combinations when the TDRA table for the set is configured. For the explicit indication, the network should further configure the detailed configuration of the table. The separate tables are configured for downlink scheduling and uplink scheduling as agreed. The table includes at most 16 rows and each row corresponds to a scheduled cell combination. Similar as the reference cell indication, the value 0~3 is used to indicate a cell in the scheduled cell combination.

- For some DCI fields, the type is configurable between Type-1A and Type-2. Therefore, it should be set-specific configuration. These fields include antenna port for downlink or uplink, precoding information and the number of layers, SRS resource indicator.

- It was agreed that enhanced Type-3 codebook indicator is Type-1A field. Whether this field is included in DCI format 1_X or not should be configured. This is applied to PUCCH retransmission indicator and PUCCH cell indicator as well.

- It was agreed that the inclusion of PDCCH monitoring adaptation indication in DCI format 0_X/1_X is configurable. Therefore, the network can configure whether this field is included in the DCI format 0_X/1_X. Similarly, the network can configure whether SCell dormancy indication, minimum applicable scheduling offset indicator, or priority indicator is included in DCI format 0_X/1_X.

It was agreed that the size of a Type-1A field in the DCI format 0_X/1_X is determined as the maximum field size of active BWP among all cells within the set. The size of Type-2 field is determined based on the active BWP for each cell. The UE can determine the field size according the relative configurations in each serving cell in the set. There is no need to additionally configure anything for Type-1A field and Type-2 field.

For Type-1B field, it was agreed that the field size is equal to ceiling(log2(N)), where N is the number of rows in RRC-configured table with each row containing multiple indexes for all cells within the set of cells. Therefore, the table for Type-1B should be configured. The table is configured based on the legacy configuration. The legacy configuration is BWP-specific and therefore the table should also be BWP-specific. It should be configured by means of add and release list to reduce the RRC signalling overhead, where each element in the list corresponds to a BWP combination. In addition, the downlink scheduling and uplink scheduling should be configured separately. Since BWP indicator in DCI is Type-1A field, there are at most 4 active BWP combinations. The maximum number of elements in the list should also be 4. The first element is used for the configuration for the first active BWP combination, the second element in the list is used for the configuration for the second active BWP combination, and so on.

- Downlink scheduling configurations for DCI format 1_X mainly includes the configurations for Type-1B fields in DCI format 1_X

  - An index should be configured to identify a downlink scheduling set when released

  - For the time domain resource for PDSCH, a table with at most 32 rows can be configured. Each row includes the specific time domain resource for PDSCH for each scheduled cell in the set. As agreed, the specific time domain resource for a scheduled cell is indicated by an index, where the index points to the time domain resource for PDSCH according to the legacy configuration for DCI format 1_1 in the corresponding BWP. It was also agreed that multi-cell scheduling and multi-PDSCH scheduling cannot be configured simultaneously for a PUCCH group.  The value range for the index should be 0~15.

  - Similar as above, a table with at most 16 rows can be configured for rate matching pattern indicator. For each row, the value range for the index for a scheduled cell should be 0~3.

  - Similarly, a table with at most 8 rows can be configured for ZP CSI-RS trigger indication. For each row, the value range for the index for a scheduled cell should be 0~3.

  - Similarly, a table with at most 16 rows can be configured for TCI state indication. For each row, the value range for the index for a scheduled cell should be 0~8. The index points to the TCI state activated by the MAC CE for the scheduled cell in the corresponding BWP.

  - Similarly, a table with at most 16 rows can be configured for SRS request indication. For each row, the value range for the index for a scheduled cell should be 0~3.

  - Similarly, a table with at most 8 rows can be configured for SRS offset indicator. For each row, the value range for the index for a scheduled cell should be 0~3.

- Uplink scheduling configurations for DCI format 0_X mainly includes the configurations for Type-1B fields in DCI format 0_X.

  - An index should be configured to identify an uplink scheduling set when released

  - Similar as above, a table with at most 64 rows can be configured for time domain resource indication for PUSCH. For each row, the value range for the index for a scheduled cell should be 0~63 due to the fact that up to 6-bit TDRA table can be configured with PUSCH repetition.

  - A table with at most 16 rows can be configured for SRS request indication. For each row, the value range for the index for a scheduled cell should be 0~3.

  - A table with at most 8 rows can be configured for SRS offset indicator. For each row, the value range for the index for a scheduled cell should be 0~3.

Proposal 2: For each multi-cell scheduling set, the followings should be configured and an example is shown as well.

- Set index with value range of 0~3

- At most 4 scheduled cells indicated by serving cell index, respectively

- Reference cell for MC-DCI size budget and BD/CCE budget counting indicated by an index with value range 0~3

- n_CI for CCE determination for DCI format 0_X/1_X with value range 0~11

- Set indicator with value range 0~3

- Two co-scheduled cells tables for downlink and uplink, respectively

- DCI field type for antenna port for DCI format 1_X, including Type-1A, and Type-2

- DCI field type for antenna port configuration for DCI format 0_X, including Type-1A, and Type-2

- DCI field type for precoding information and number of layers configuration, including Type-1A, and Type-2

- DCI field type for SRS resource indicator, including Type-1A, and Type-2

- The presence of enhanced Type-3 codebook indicator in DCI format 1_X

- The presence of PUCCH retransmission indicator in DCI format 1_X

- The presence of PUCCH cell indicator in DCI format 1_X

- The presence of PDCCH monitoring adaption indication in DCI format 0_X/1_X

- The presence of SCell dormancy indication in DCI format 0_X/1_X

- The presence of minimum applicable scheduling offset indicator in DCI format 0_X/1_X

- The presence of priority indicator in DCI format 0_X or DCI format 1_X

- The add list and release list for DCI format 1_X for the downlink scheduling

  - Downlink scheduling set index

  - Time domain resource configuration for PDSCH

  - Rate matching pattern indicator configuration

  - ZP CSI-RS trigger state configuration

  - TCI state configuration

  - SRS request state configuration

  - SRS offset indicator configuration

- The add list and release list for DCI format 0_X for the uplink scheduling

  - Uplink scheduling set index

  - Time domain resource configuration for PUSCH

  - SRS request state configuration

  - SRS offset indicator configuration

MC-schedulingSet ::= SEQUENCE { MC-schedulingSetId-r18 INTEGER (0.. maxNrofMC-Sets-1-r18) CellinSet-r18 SEQUENCE (SIZE (1..4)) OF ServCellIndex ReferenceCellforDCI0-X/1-X-r18 INTEGER (0.. 3) nCI-r18 INTEGER (0.. 11) SetIndicatorInDCI-r18 INTEGER (0.. 3) Co-ScheduledCellIndication-r18 SEQUENCE { Downlink-r18 SEQUENCE (SIZE (1..16)) OF CellCombination Uplink-r18 SEQUENCE (SIZE (1..16)) OF CellCombination } AntennaPort-1-X-r18 ENUMERATED {Type-1A, Type-2} AntennaPort-0-X-r18 ENUMERATED {Type-1A, Type-2} PrecodingInfoandNrOflayers-0-X -r18 ENUMERATED {Type-1A, Type-2} SRSResourceIndicator-0-X ENUMERATED {Type-1A, Type-2} EnhType3DCI-Field-1-X ENUMERATED {enabled} Pucch- retxDCI-1-X ENUMERATED {enabled} pucch-sSCellDynDCI-1-X ENUMERATED {enabled} PDCCHMonitoringAdaptionPresence ENUMERATED {enabled} MinAppSchedulingOffsetPresence ENUMERATED {enabled} SCellDormancyPresence ENUMERATED {enabled} priorityIndicatorDCI-1-X ENUMERATED {enabled} priorityIndicatorDCI-0-X ENUMERATED {enabled} DownlinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF DownlinkScheduling-r18 DownlinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF DownlinkSchedulingId-r18 UplinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkScheduling-r18 DownlinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkSchedulingId-r18 } CellCombination ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) DownlinkScheduling ::= SEQUENCE { DownlinkSchedulingId-r18 INTEGER (0.. 3) pdsch-TimeDomainList SEQUENCE (SIZE (1..32)) OF pdsch-TimeDomain rateMatchIndicatorState-1-X-r18 SEQUENCE (SIZE (1..16)) OF rateMatchIndicator ZPCSI-RSTriggerState-r18 SEQUENCE (SIZE (1..8)) OF ZPCSI-RSTrigger TCIStateList-r18 SEQUENCE (SIZE (1..16)) OF TCIState SRSRequestState-r18 SEQUENCE (SIZE (1..16)) OF SRSRequest SRSOffsetIndicatorState-r18 SEQUENCE (SIZE (1..16)) OF SRSOffsetIndicator } UplinkScheduling ::= SEQUENCE { UplinkSchedulingId-r18 INTEGER (0.. 3) pusch-TimeDomainList SEQUENCE (SIZE (1..63)) OF pusch-TimeDomain SRSRequestState-r18 SEQUENCE (SIZE (1..16)) OF SRSRequest SRSOffsetIndicatorState-r18 SEQUENCE (SIZE (1..16)) OF SRSOffsetIndicator } pdsch-TimeDomain ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..15) pusch-TimeDomain ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..63) rateMatchIndicator ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) ZPCSI-RSTrigger ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) TCIState ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..7) SRSRequest ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) SRSOffsetIndicator ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3)

For Type-2 field, the field size is based on the configuration of each scheduled cell and is the largest one among the scheduled cell combinations. The configuration for the Type-2 field should be configured for each scheduled cell separately for the flexibility. For multi-cell downlink scheduling, the higher layer parameters should be included in PDSCH-Config.

- As agreed, the size of HPN field should be configured for a scheduled cell.

- As agreed, the size of RV field should be configured for a scheduled cell and one of RV0/2 and RV0/3 should be also configured in case 1-bit RV is configured since they are both supported in the current spec.

- As agreed, the RBG granularity for RA type 0 should be configured for PDSCH for a scheduled cell.

- As agreed, the RBG granularity for RA type 1 should be configured for PDSCH for a scheduled cell.

Proposal 3: The BWP-specific higher layer parameters for downlink scheduling, including the size of RV, the size of HPN, RBG size for RA type 0 and RBG size for RA type 1, should be included in the IE PDSCH-Config. An example is shown below.

PDSCH-Config ::= SEQUENCE { numberOfBitsForRV-DCI-1-X ENUMERATED {0-bit, 1-bit-rv02, 1-bit-rv03, 2-bit} harq-ProcessNumberSizeDCI-1-X INTEGER (0..4) rbg-Size-RA0-1-X ENUMERATED {config1, config2, config3}, rbg-Size-RA1-1-X ENUMERATED {n2,n4,n8,n16} }

Similarly, for multi-cells uplink scheduling, the higher layer parameters should be included in PUSCH-Config.

- As agreed, the size of HPN field should be configured for a scheduled cell.

- As agreed, the size of RV field should be configured for a scheduled cell and one of RV0/2 and RV0/3 should be also configured in case 1-bit RV is configured since they are both supported in the current spec.

- As agreed, the RBG granularity for RA type 0 should be configured for PDSCH for a scheduled cell.

- As agreed, the RBG granularity for RA type 1 should be configured for PDSCH for a scheduled cell.

Proposal 4: The BWP-specific higher layer parameters for uplink scheduling, including the size of RV, the size of HPN, RBG size for RA type 0 and RBG size for RA type 1, should be included in the IE PUSCH-Config. An example is shown below.

PDSCH-Config ::= SEQUENCE { numberOfBitsForRV-DCI-0-X ENUMERATED {0-bit, 1-bit-rv02, 1-bit-rv03, 2-bit} harq-ProcessNumberSizeDCI-0-X INTEGER (0..4) rbg-Size-RA0-0-X ENUMERATED {config1, config2, config3}, rbg-Size-RA1-0-X ENUMERATED {n2,n4,n8,n16} }

It was agreed that the search spaces are independent configured for legacy DCI formats and DCI format 0_X/1_X. In addition, the search space for DCI format 0_X and 1_X can be configured independently. Therefore, a new DCI format should be included in the search space configuration. More specifically, it should be in the higher layer parameter UE-specific as below. If the scheduling cell is not the reference cell, two linked search space should be configured in the scheduling cell and reference cell as legacy.

Proposal 5: A new DCI formats should be included in the search space configuration and an example is shown below.

ue-Specific SEQUENCE { dci-Formats ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1}, ..., [[ dci-Formats-MT-r16 ENUMERATED {formats2-5} OPTIONAL, -- Need R dci-FormatsSL-r16 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1, formats3-0-And-3-1} OPTIONAL, -- Need R dci-FormatsExt-r16 ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2} OPTIONAL -- Need R ]] [[ dci-Formats-r18 ENUMERATED {formats0-X, formats1-X , formats0-X-And-1-X} ]] }

## UL Tx switching among 3/4 bands

### Draft TP for UL Tx switching among 3/4 bands

In this section, we provide our example TPs for UL Tx switching among 3 or 4 bands for carrier aggregation.

Proposal 6: Adopt the following TP for Rel-18 UL Tx switching among 3 or 4 bands for carrier aggregation.

6.1.6 Uplink switching For uplink switching between two bands, the UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap. For uplink switching among three or four bands with restriction of up to 2 Tx simultaneous transmission, the UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4), where µUL, 1, µUL, 2, µUL, 3 and µUL, 4 correspond to the subcarrier spacing of the active UL BWP of one uplink carrier of the 1st band, 2nd band, 3rd band and 4th band, respectively. If there are two contiguous intra-band carriers in one band, then µUL, 1 = max(µUL, 1-1, µUL, 1-2), whereµUL, 1-1 and µUL, 1-2 correspond to the subcarrier spacing of the active UL BWP of the 1st uplink carrier and the 2nd uplink carrier of the band, respectively. For uplink switching among three or four bands with restriction of up to 2 Tx simultaneous transmission, if two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive slots corresponding to numerology µUL, the UE expects the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within these two consecutive slots is not less than a maximum of X us and the switching gap required for the second uplink switching, where X is indicated by UE capability [minimumSeparationTime]. 6.1.6.1 Uplink switching for EN-DC 6.1.6.2 Uplink switching for carrier aggregation 6.1.6.2.1 void Uplink switching for carrier aggregation among 3 or 4 bands For a UE indicating a capability for uplink switching with [BandCombination-UplinkTxSwitch] for a band combination, and if it is for that band combination configured with uplink carrier aggregation: If the UE is configured with uplink switching with parameter [uplinkTxSwitching-r18], when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): If the UE is to perform uplink switching between two bands, the UE shall perform the behaviour as defined in section 6.1.6.2. when the UE is to transmit a 2-port transmission on one uplink carrier on the 1st band and if the preceding uplink transmission was a 1-port transmission on a carrier on the 2nd or 3rd band and the UE is under the operation state in which 1-port transmission can be supported in the 2nd and 3rd band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers, where NTx1-Tx2 is the max of [uplinkTxSwitchingPeriod] that UE indicates for the band pair {1st band, 2nd band} and for the band pair {1st band, 3rd band}. when the UE is to transmit a 1-port transmission on one uplink carrier on the 1st band and the 2nd band, and if the preceding uplink transmission was a 2-port transmission on a carrier on the 3rd band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers, where NTx1-Tx2 is the max of [uplinkTxSwitchingPeriod] that UE indicates for the band pair {1st band, 3rd band } and for the band pair {2nd band, 3rd band}. when the UE is to transmit a 1-port transmission on one uplink carrier on the 1st band and the 2nd band, and if the preceding uplink transmission was a 1-port transmission on a carrier on the 1st band and the 3rd band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers if UE doesn’t indicate [AdvancedCapabilityDefinedbyRAN4], otherwise the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers on the 2nd band and the 3rd band, where NTx1-Tx2 is the [uplinkTxSwitchingPeriod] that UE indicates for the band pair {2nd band, 3rd band}. when the UE is to transmit a 1-port transmission on one uplink carrier on the 1st band and the 2nd band, and if the preceding uplink transmission was a 1-port transmission on a carrier on the 3rd band and the 4th band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers, where NTx1-Tx2 is the max of [uplinkTxSwitchingPeriod] that UE indicates for the band pair {1st band, 3rd band}, band pair {1st band, 4th band} , band pair {2nd band, 3rd band}and band pair {2nd band, 4th band} if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on any carrier on the same band and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 1-port transmission can be supported on one carrier on the 1st band and the 2nd band followed by no transmission on any carrier on these two bands and 1-port transmission on the other carrier on the 3rd band the UE shall consider this as if 1-port transmission was transmitted on the 3rd band and the band associated with the 3rd band as configured by [AssociatedBand], otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', if one band is not configured as dualUL for any band pair, when the UE is to transmit a 1-port transmission on one carrier on the band the UE shall consider this as if 2-port transmission took place on the transmitting carrier. The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. 6.1.6.3.1 Uplink switching for carrier aggregation with one supplementary uplink among 3 or 4 bands For a UE indicating a capability for uplink switching with [BandCombination-UplinkTxSwitch] for a band combination, and if it is for that band combination configured with uplink carrier aggregation with one supplementary uplink: - If the UE is configured with uplink switching with parameter [uplinkTxSwitching], - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of uplinks, where NTx1-Tx2 is the [uplinkTxSwitchingPeriod] that UE indicates for the band pair {band with transmission before the switching gap, band with transmission after the switching gap}.

### RRC configuration for UL Tx switching among 3/4 bands

In this section, we provide analysis for the RRC configurations for Rel-18 UL Tx switching.

Issue1: Which band pair supports concurrent transmission?

RAN1 asked RAN2 to design the RRC configuration for switchedUL and dualUL.  RAN2 has agreed to configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination).

Agreement (RAN1) Ask RAN2 to consider following alternatives and specify gNB configuration - o Alt.1: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination) - o Alt.2: configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination) - o Alt.3: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination), and configure combination(s) of serving cells (i.e., as supported serving cell pair(s) for each band pair in the band combination) for concurrent transmission Agreements (RAN2): - For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching. - Configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination) - For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band. - For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

Observation 1: Regarding the RRC configuration to indicate which band pair supports concurrent transmission, RAN2 has agreed to configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination).

Issue2: Associated band for addressing the ambiguity issue

RAN1 has agreed to introduce RRC configuration to configure an associated band for each band to address the ambiguity issue. RAN2 has agreed to introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band.

Agreement (RAN1) Following working assumption is confirmed with updates. Working Assumption At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases - Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported - if twoT is indicated, both of two Tx chains are switched to band B - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A - Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported - if twoT is indicated, both of two Tx chains are switched to band C - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter - Alt.2: based on predefined rule - Other alternative is not precluded - FFS for other potential cases Agreement (RAN1) In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives. - An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band) - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}. - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C) - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D) If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled Agreements (RAN2): - For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching. - Configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination) - For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band. - For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

Observation 2: For RRC configuration to clarify ambiguous Tx state, RAN2 has agreed to introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band.

Issue3: Priority for switching period location

RAN1 has agreed to introduce RRC configured priority for UE to determine the switching period location. RAN2 has carried out the email discussion to discuss this issue under “[Post121][045][MCE] UL TX Switching (Docomo)”. The detailed discussion can be found in section 3.2.1 of the summary for “[Post121][045][MCE] UL TX Switching (Docomo)”. In order to avoid duplicated discussion, it is suggested to keep the discussion in RAN2.

Agreement Alt.5: gNB configures priorities to each carrier/band. The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

Proposal 7: The RRC parameter design for the priority configuration for switching period location determination is up to RAN2.

Issue4: RRC configuration to differentiate 1T-2T vs 2T-2T

The main function of 1T-2T vs 2T-2T differentiation is to indicate different switching period. In Rel-17, the band pair is considered as 2T-2T if both bands are configured with 2-port transmission. 2T-2T may require longer switching period since UE needs to switch 2 Tx chains at the same time.

However, in Rel-18, one UL Tx switching may involve 3 or 4 bands, it is not clear how to determine whether it is 1T-2T switching or 2T-2T swtiching. Actually, in Rel-18, the switching period is mainly determined per Tx chain. For example, if UE switches 1st Tx from band A to band B and switches 2nd Tx chain from band C to band D, then the switching gap is determined as max{Tswitch,A-B, Tswitch,C-D } , in this case, it seems not necessary to further differentiate 1T-2T vs 2T-2T. In other words, we don’t need to further separate the case of 1T-2T and 2T-2T, the same duration of switching period will be applied regardless of the port configuration.

Observation 3:

- For Rel-17, the duration of switching period may be different depending on the number of Tx chains (i.e., 1 or 2) performing the switching.

- For Rel-18, the duration of the switching period is determined separately for each Tx first and then a combined switching gap is calculated.

Proposal 8: There is no need to further separate the case of 1T-2T and 2T-2T. The same duration of switching period will be applied regardless of the port configuration.

# BWP without restriction

In RAN#99 meeting, the following WID is approved for BWP without restriction. In this contribution, we provide our initial views for the spec support for BWP without restriction in NR, from RAN1 perspective.

### The work item includes following objectives: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

## BFD-RS configuration for BWP without restriction

Based on the agreed WID, in RAN1, we need to specify the support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (Option B-1-1 and Option B-1-2), NCD-SSB within active BWP for non-RedCap UEs (Option-C). From RAN1 spec perspective, we do not distinguish SSB for CD or NCD cases, but, in explicit manner for BFD-RS configuration, the SSB based configuration seems not to be supported in RAN1 spec (as shown in the following).

TS 38.213 (v17.5.0) Section 6 Link recovery procedures A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRS-List and candidateBeamRS-List2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1.

Based on the guidance of this WID, we need to introduce SSB into q_0 as BFD-RS configuration. Therefore, we have the following proposed text proposal (TP) as follows. The details of the related UE capability can be found in our companion contribution [1].

Proposal 9: Adopt the following TP for Clause 6 {Link recovery procedures} in TS 38.213.

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes, with subjective to UE capability, by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes, with subjective to UE capability, by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRS-List and candidateBeamRS-List2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. <Unchanged part omitted>

It should be noticed that, from RRC/MAC-CE signaling perspective, in the BFD-RS configuration/activation, any further update may not be needed, due to the fact, from signaling perspective, SSB index(es) can be explicitly configured in BFD-RS set in RRC level.

Then, for implicit manner of determining BFD-RS, considering the UE should track TRS for PDCCH demodulation based on QCL rule, further update for implicit manner may not be quite essential. But, we are open to have some further discussion, if the corresponding necessity is justified.

## RLM in BWP without restriction

In the current spec, it is restricted that the UE is required to only measure the RS in the active BWP for RLM. For Option B-1-1 and Option B-1-2, the UE can support SSB outside of the active BWP for RLM without or with interpretation, respectively. Therefore, the current restriction should be changed for such UE and it is applied to legacy UE. The following TP is proposed.

Proposal 10: Adopt the following TP for Clause 5 {Radio link monitoring} in TS 38.213.

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell, if the UE does not report [rlm-SSBoutaBWP]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS. If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell if the UE does not report [rlm-SSBoutaBWP]. <Unchanged part omitted> If a UE is configured with multiple DL BWPs for a serving cell and the UE does not report [rlm-SSBoutaBWP], the UE performs RLM using the RS(s) corresponding to resource indexes provided by RadioLinkMonitoringRS for the active DL BWP or, if RadioLinkMonitoringRS is not provided for the active DL BWP, using the RS(s) provided for the active TCI state for PDCCH receptions in CORESETs on the active DL BWP.

# eDSS

## NR PDCCH reception

### Draft TP for NR PDCCH reception

In RAN1#110 meeting, the following agreements were agreed for NR PDCCH reception for DSS [2].

Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET) Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session. Conclusion RAN1 understands above agreements applies to 15kHz SCS only.

According to the analysis in our companion contribution [3], we see no need to limit the number of LTE CRS patterns, and also no need to limit the symbols supporting PDCCH reception in symbols with LTE CRS REs. In this sense, below is a text proposal based on TS 38.213-h50.

Proposal 11: Adopt TP#1 for PDCCH reception in symbols with LTE CRS REs.

TP#1 for clause 10 of TS 38.213-h50

10 UE procedure for receiving control information <omitted text> If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate if the UE doesn’t report [PdcchReceptionOnLte-crsSymbol], or the UE monitors the PDCCH candidate otherwise. <omitted text> 10.1 UE procedure for determining physical downlink control channel assignment <omitted text> When precoderGranularity = allContiguousRBs, a UE does not expect - to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency - any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList if the UE doesn’t report [PdcchReceptionOnLte-crsSymbol], or with any RE of a SS/PBCH block. <omitted text>

### RRC configuration for NR PDCCH reception

During the discussion in RAN1#110, some companies proposed that an additional RRC signalling needs to be introduced to enable/disable the feature of PDCCH reception in symbols with LTE CRS REs.

In current spec, a UE is not required to monitor the PDCCH candidate if at least one RE of a PDCCH candidate for the UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround or of LTE-CRS-PatternList. That is, gNB will not transmit PDCCH overlapped with LTE CRS RE in practice for such cases.

In Rel-18 DSS, reception of NR PDCCH candidates that overlap with LTE CRS REs is supported, and a new UE capability is expected. gNB can transmit PDCCH overlapped with LTE CRS RE for the new UEs supporting the capability. For the legacy UE and Rel-18 UE without report supporting reception of NR PDCCH candidates that overlap with LTE CRS REs, the UE behavior is same as legacy. There would be no issue if no new Rel-18 RRC parameter is introduced. Any optimization in maintenance phase should not be pursued.

Proposal 12: No need to introduce additional RRC signalling to enable/disable the feature of PDCCH reception in symbols with LTE CRS REs.

## Two overlapping CRS rate matching patterns

### Draft TP for two overlapping CRS rate matching patterns

Based on the agreement in RAN1#110 meeting [2], and the discussion on the tdoc submitted in [4] in RAN1#110bis, the following TP#1 and TP#2 can be adopted.

Proposal 13: Adopt the following TP#1 and TP#2 for two overlapping CRS rate matching patterns.

TP#1 for sub-clause 5.1.4.2 of TS 38.214-h50

5.1.4.2 PDSCH resource mapping with RE level granularity <omitted text> A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in pdsch-ConfigMCCH or pdsch-ConfigMTCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for broadcast PDSCH. The total number of RateMatchPatternLTE-CRS for broadcast reception that a UE can be configured with is the same as for unicast in Rel-15. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1'; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig. - If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. <omitted text>

TP#2 for sub-clause 7.4.1.1.2 of TS 38.211 h40

7.4.1.1.2 Mapping to physical resources <omitted text> For PDSCH mapping type A # <omitted text> - single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12: - the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 is configured; and - the higher-layer parameter dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and - the UE has indicated it is capable of additionalDMRS-DL-Alt # For PDSCH mapping type B # <omitted text> - if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3 or lte-CRS-PatternList4 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4, then l shall be incremented by one in all slots. <omitted text>

### RRC configuration for two overlapping CRS rate matching patterns

In the RAN1#110 meeting, RAN1 agrees to introduce the following two RRC parameters, namely lte-CRS-PatternList3 and lte-CRS-PatternList4, for support of two PDSCH-overlapping LTE-CRS patterns for Rel-18 DSS. And the corresponding LS R1-2208194 is endorsed and sent to RAN2. It seems no further action in RAN1 is needed.

Observation 5: No further RAN1 action is needed due to the following RRC parameters for support of two overlapping LTE-CRS patterns have been agreed and corresponding LS R1-2208198 had been sent to RAN2.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList3 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList4 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |

# Conclusion

In this contribution, we provide our analysis draft CR and RRC parameters for for eDSS, MC-Enh and BWP without restriction, respectively, and we have the following observations and proposals.

### <<MC enhancements>>

Proposal 1: The scheduling related configuration should be in the IE PhysicalCellGroupConfig, and an example is shown below, where maxNrofMC-Sets = 4.

PhysicalCellGroupConfig ::= SEQUENCE { MC-schedulingSetToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofMC-Sets)) OF MC-schedulingSet-r18 MC-schedulingSetToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofMC-Sets)) OF MC-schedulingSetId-r18 }

Proposal 2: For each multi-cell scheduling set, the followings should be configured and an example is shown as well.

- Set index with value range of 0~3

- At most 4 scheduled cells indicated by serving cell index, respectively

- Reference cell for MC-DCI size budget and BD/CCE budget counting indicated by an index with value range 0~3

- n_CI for CCE determination for DCI format 0_X/1_X with value range 0~11

- Set indicator with value range 0~3

- Two co-scheduled cells tables for downlink and uplink, respectively

- DCI field type for antenna port for DCI format 1_X, including Type-1A, and Type-2

- DCI field type for antenna port configuration for DCI format 0_X, including Type-1A, and Type-2

- DCI field type for precoding information and number of layers configuration, including Type-1A, and Type-2

- DCI field type for SRS resource indicator, including Type-1A, and Type-2

- The presence of enhanced Type-3 codebook indicator in DCI format 1_X

- The presence of PUCCH retransmission indicator in DCI format 1_X

- The presence of PUCCH cell indicator in DCI format 1_X

- The presence of PDCCH monitoring adaption indication in DCI format 0_X/1_X

- The presence of SCell dormancy indication in DCI format 0_X/1_X

- The presence of minimum applicable scheduling offset indicator in DCI format 0_X/1_X

- The presence of priority indicator in DCI format 0_X or DCI format 1_X

- The add list and release list for DCI format 1_X for the downlink scheduling

  - Downlink scheduling set index

  - Time domain resource configuration for PDSCH

  - Rate matching pattern indicator configuration

  - ZP CSI-RS trigger state configuration

  - TCI state configuration

  - SRS request state configuration

  - SRS offset indicator configuration

- The add list and release list for DCI format 0_X for the uplink scheduling

  - Uplink scheduling set index

  - Time domain resource configuration for PUSCH

  - SRS request state configuration

  - SRS offset indicator configuration

MC-schedulingSet ::= SEQUENCE { MC-schedulingSetId-r18 INTEGER (0.. maxNrofMC-Sets-1-r18) CellinSet -r18 SEQUENCE (SIZE (1..4)) OF ServCellIndex ReferenceCellforDCI0-X/1-X-r18 INTEGER (0.. 3) nCI-r18 INTEGER (0.. 11) SetIndicatorInDCI-r18 INTEGER (0.. 3) Co-ScheduledCellIndication-r18 SEQUENCE { Downlink-r18 SEQUENCE (SIZE (1..16)) OF CellCombination Uplink-r18 SEQUENCE (SIZE (1..16)) OF CellCombination } AntennaPort-1-X-r18 ENUMERATED {Type-1A, Type-2} AntennaPort-0-X-r18 ENUMERATED {Type-1A, Type-2} PrecodingInfoandNrOflayers-0-X -r18 ENUMERATED {Type-1A, Type-2} SRSResourceIndicator-0-X ENUMERATED {Type-1A, Type-2} EnhType3DCI-Field-1-X ENUMERATED {enabled} Pucch- retxDCI-1-X ENUMERATED {enabled} pucch-sSCellDynDCI-1-X ENUMERATED {enabled} PDCCHMonitoringAdaptionPresence ENUMERATED {enabled} MinAppSchedulingOffsetPresence ENUMERATED {enabled} SCellDormancyPresence ENUMERATED {enabled} priorityIndicatorDCI-1-X ENUMERATED {enabled} priorityIndicatorDCI-0-X ENUMERATED {enabled} DownlinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF DownlinkScheduling-r18 DownlinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF DownlinkSchedulingId-r18 UplinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkScheduling-r18 DownlinkSchedulingToAddModlist-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF UplinkSchedulingId-r18 } CellCombination ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) DownlinkScheduling ::= SEQUENCE { DownlinkSchedulingId-r18 INTEGER (0.. 3) pdsch-TimeDomainList SEQUENCE (SIZE (1..32)) OF pdsch-TimeDomain rateMatchIndicatorState-1-X-r18 SEQUENCE (SIZE (1..16)) OF rateMatchIndicator ZPCSI-RSTriggerState-r18 SEQUENCE (SIZE (1..8)) OF ZPCSI-RSTrigger TCIStateList-r18 SEQUENCE (SIZE (1..16)) OF TCIState SRSRequestState-r18 SEQUENCE (SIZE (1..16)) OF SRSRequest SRSOffsetIndicatorState-r18 SEQUENCE (SIZE (1..16)) OF SRSOffsetIndicator } UplinkScheduling ::= SEQUENCE { UplinkSchedulingId-r18 INTEGER (0.. 3) pusch-TimeDomainList SEQUENCE (SIZE (1..63)) OF pusch-TimeDomain SRSRequestState-r18 SEQUENCE (SIZE (1..16)) OF SRSRequest SRSOffsetIndicatorState-r18 SEQUENCE (SIZE (1..16)) OF SRSOffsetIndicator } pdsch-TimeDomain ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..15) pusch-TimeDomain ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..63) rateMatchIndicator ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) ZPCSI-RSTrigger ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) TCIState ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..7) SRSRequest ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3) SRSOffsetIndicator ::= SEQUENCE (SIZE (1..4)) OF INTEGER (0..3)

Proposal 3: The BWP-specific higher layer parameters for downlink scheduling, including the size of RV, the size of HPN, RBG size for RA type 0 and RBG size for RA type 1, should be included in the IE PDSCH-Config. An example is shown below.

PDSCH-Config ::= SEQUENCE { numberOfBitsForRV-DCI-1-X ENUMERATED {0-bit, 1-bit-rv02, 1-bit-rv03, 2-bit} harq-ProcessNumberSizeDCI-1-X INTEGER (0..4) rbg-Size-RA0-1-X ENUMERATED {config1, config2, config3}, rbg-Size-RA1-1-X ENUMERATED {n2,n4,n8,n16} }

Proposal 4: The BWP-specific higher layer parameters for uplink scheduling, including the size of RV, the size of HPN, RBG size for RA type 0 and RBG size for RA type 1, should be included in the IE PUSCH-Config. An example is shown below.

PDSCH-Config ::= SEQUENCE { numberOfBitsForRV-DCI-0-X ENUMERATED {0-bit, 1-bit-rv02, 1-bit-rv03, 2-bit} harq-ProcessNumberSizeDCI-0-X INTEGER (0..4) rbg-Size-RA0-0-X ENUMERATED {config1, config2, config3}, rbg-Size-RA1-0-X ENUMERATED {n2,n4,n8,n16} }

Proposal 5: A new DCI formats should be included in the search space configuration and an example is shown below.

ue-Specific SEQUENCE { dci-Formats ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1}, ..., [[ dci-Formats-MT-r16 ENUMERATED {formats2-5} OPTIONAL, -- Need R dci-FormatsSL-r16 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1, formats3-0-And-3-1} OPTIONAL, -- Need R dci-FormatsExt-r16 ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2} OPTIONAL -- Need R ]] [[ dci-Formats-r18 ENUMERATED {formats0-X, formats1-X , formats0-X-And-1-X} ]] }

Proposal 6: Adopt the following TP for Rel-18 UL Tx switching among 3 or 4 bands for carrier aggregation.

6.1.6 Uplink switching For uplink switching between two bands, the UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2), where the µUL, 1 corresponds to the subcarrier spacing of the active UL BWP of one uplink carrier before the switching gap and the µUL, 2 corresponds to the subcarrier spacing of the active UL BWP of the other uplink carrier after the switching gap. For uplink switching among three or four bands with restriction of up to 2 Tx simultaneous transmission, the UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4), where µUL, 1, µUL, 2, µUL, 3 and µUL, 4 correspond to the subcarrier spacing of the active UL BWP of one uplink carrier of the 1st band, 2nd band, 3rd band and 4th band, respectively. If there are two contiguous intra-band carriers in one band, then µUL, 1 = max(µUL, 1-1, µUL, 1-2), whereµUL, 1-1 and µUL, 1-2 correspond to the subcarrier spacing of the active UL BWP of the 1st uplink carrier and the 2nd uplink carrier of the band, respectively. For uplink switching among three or four bands with restriction of up to 2 Tx simultaneous transmission, if two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive slots corresponding to numerology µUL, the UE expects the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within these two consecutive slots is not less than a maximum of X us and the switching gap required for the second uplink switching, where X is indicated by UE capability [minimumSeparationTime]. 6.1.6.1 Uplink switching for EN-DC 6.1.6.2 Uplink switching for carrier aggregation 6.1.6.2.1 void Uplink switching for carrier aggregation among 3 or 4 bands For a UE indicating a capability for uplink switching with [BandCombination-UplinkTxSwitch] for a band combination, and if it is for that band combination configured with uplink carrier aggregation: If the UE is configured with uplink switching with parameter [uplinkTxSwitching-r18], when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): If the UE is to perform uplink switching between two bands, the UE shall perform the behaviour as defined in section 6.1.6.2. when the UE is to transmit a 2-port transmission on one uplink carrier on the 1st band and if the preceding uplink transmission was a 1-port transmission on a carrier on the 2nd or 3rd band and the UE is under the operation state in which 1-port transmission can be supported in the 2nd and 3rd band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers, where NTx1-Tx2 is the max of [uplinkTxSwitchingPeriod] that UE indicates for the band pair {1st band, 2nd band} and for the band pair {1st band, 3rd band}. when the UE is to transmit a 1-port transmission on one uplink carrier on the 1st band and the 2nd band, and if the preceding uplink transmission was a 2-port transmission on a carrier on the 3rd band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers, where NTx1-Tx2 is the max of [uplinkTxSwitchingPeriod] that UE indicates for the band pair {1st band, 3rd band } and for the band pair {2nd band, 3rd band}. when the UE is to transmit a 1-port transmission on one uplink carrier on the 1st band and the 2nd band, and if the preceding uplink transmission was a 1-port transmission on a carrier on the 1st band and the 3rd band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers if UE doesn’t indicate [AdvancedCapabilityDefinedbyRAN4], otherwise the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers on the 2nd band and the 3rd band, where NTx1-Tx2 is the [uplinkTxSwitchingPeriod] that UE indicates for the band pair {2nd band, 3rd band}. when the UE is to transmit a 1-port transmission on one uplink carrier on the 1st band and the 2nd band, and if the preceding uplink transmission was a 1-port transmission on a carrier on the 3rd band and the 4th band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers, where NTx1-Tx2 is the max of [uplinkTxSwitchingPeriod] that UE indicates for the band pair {1st band, 3rd band}, band pair {1st band, 4th band} , band pair {2nd band, 3rd band}and band pair {2nd band, 4th band} if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on any carrier on the same band and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 1-port transmission can be supported on one carrier on the 1st band and the 2nd band followed by no transmission on any carrier on these two bands and 1-port transmission on the other carrier on the 3rd band the UE shall consider this as if 1-port transmission was transmitted on the 3rd band and the band associated with the 3rd band as configured by [AssociatedBand], otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', if one band is not configured as dualUL for any band pair, when the UE is to transmit a 1-port transmission on one carrier on the band the UE shall consider this as if 2-port transmission took place on the transmitting carrier. The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. 6.1.6.3.1 Uplink switching for carrier aggregation with one supplementary uplink among 3 or 4 bands For a UE indicating a capability for uplink switching with [BandCombination-UplinkTxSwitch] for a band combination, and if it is for that band combination configured with uplink carrier aggregation with one supplementary uplink by higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter [uplinkTxSwitching], - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of uplinks, where NTx1-Tx2 is the [uplinkTxSwitchingPeriod] that UE indicates for the band pair {band with transmission before the switching gap, band with transmission after the switching gap}.

Observation 1: Regarding the RRC configuration to indicate which band pair supports concurrent transmission, RAN2 has agreed to configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination).

Observation 2: For RRC configuration to clarify ambiguous Tx state, RAN2 has agreed to introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band.

Proposal 7: The RRC parameter design for the priority configuration for switching period location determination is up to RAN2.

Observation 3:

- For Rel-17, the duration of switching period may be different depending on the number of Tx chains (i.e., 1 or 2) performing the switching.

- For Rel-18, the duration of the switching period is determined separately for each Tx first and then a combined switching gap is calculated.

Proposal 8: There is no need to further separate the case of 1T-2T and 2T-2T. The same duration of switching period will be applied regardless of the port configuration.

### <<BWP without restriction>>

Proposal 9: Adopt the following TP for Clause 6 {Link recovery procedures} in TS 38.213.

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes, with subjective to UE capability, by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes, with subjective to UE capability, by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRS-List and candidateBeamRS-List2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1. <Unchanged part omitted>

Proposal 10: Adopt the following TP for Clause 5 {Radio link monitoring} in TS 38.213.

The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell, if the UE does not report [rlm-SSBoutaBWP]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS. If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell if the UE does not report [rlm-SSBoutaBWP]. <Unchanged part omitted> If a UE is configured with multiple DL BWPs for a serving cell and the UE does not report [rlm-SSBoutaBWP], the UE performs RLM using the RS(s) corresponding to resource indexes provided by RadioLinkMonitoringRS for the active DL BWP or, if RadioLinkMonitoringRS is not provided for the active DL BWP, using the RS(s) provided for the active TCI state for PDCCH receptions in CORESETs on the active DL BWP.

### <<eDSS>>

Proposal 11: Adopt TP#1 for PDCCH reception in symbols with LTE CRS REs.

TP#1 for clause 10 of TS 38.213-h50

10 UE procedure for receiving control information <omitted text> If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate if the UE doesn’t report [PdcchReceptionOnLte-crsSymbol], or the UE monitors the PDCCH candidate otherwise. <omitted text> 10.1 UE procedure for determining physical downlink control channel assignment <omitted text> When precoderGranularity = allContiguousRBs, a UE does not expect - to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency - any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround, or from LTE-CRS-PatternList if the UE doesn’t report [PdcchReceptionOnLte-crsSymbol], or with any RE of a SS/PBCH block. <omitted text>

Proposal 12: No need to introduce additional RRC signalling to enable/disable the feature of PDCCH reception in symbols with LTE CRS REs.

Proposal 13: Adopt the following TP#1 and TP#2 for two overlapping CRS rate matching patterns.

TP#1 for sub-clause 5.1.4.2 of TS 38.214-h50

5.1.4.2 PDSCH resource mapping with RE level granularity <omitted text> A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in pdsch-ConfigMCCH or pdsch-ConfigMTCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for broadcast PDSCH. The total number of RateMatchPatternLTE-CRS for broadcast reception that a UE can be configured with is the same as for unicast in Rel-15. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1'; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig. - If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. <omitted text>

TP#2 for sub-clause 7.4.1.1.2 of TS 38.211 h40

7.4.1.1.2 Mapping to physical resources <omitted text> For PDSCH mapping type A # <omitted text> - single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12: - the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 is configured; and - the higher-layer parameter dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and - the UE has indicated it is capable of additionalDMRS-DL-Alt # For PDSCH mapping type B # <omitted text> - if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3 or lte-CRS-PatternList4 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, or lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4, then l shall be incremented by one in all slots. <omitted text>

Observation 5: No further RAN1 action is needed due to the following RRC parameters for support of two overlapping LTE-CRS patterns have been agreed and corresponding LS R1-2208198 had been sent to RAN2.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList3 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList4 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |

# Reference

- RAN1#112bis-e, R1-2302765, Discussion on UE features for BWP without restriction, ZTE.

- Chair’s notes RAN1#110.

- RAN1#112bis-e, R1-2302764, Discussion on UE feature for eDSS, ZTE.

- R1-2208859, Discussion on remaining issues in Rel-18 eDSS, OPPO.