3GPP TSG RAN WG1 #112b-e	R1-2303410

e-Meeting, April 17th – April 26th, 2023

Source:	FGI

Title:	Discussion on L1 enhancements for inter-cell beam management

Agenda item:	9.10.1

Document for: 	Discussion

Introduction

In RAN1#112 meeting [1], the following agreements were made for L1 enhancements for inter-cell beam management.

Agreement

- RAN1 shares the same understanding as RAN2 on agreement:

  - The LTM mobility trigger information is conveyed in a MAC CE

- The same MAC CE is used for the LTM triggering.

Agreement

- The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following:

  - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Agreement

- For L1-RSRP measurement RS configuration

  - For SSB based L1-RSRP measurement:

    - As a starting point, at least the following information needs to be provided to a UE, e.g.

      - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst)

      - For inter-frequency: frequency domain location (e.g. center frequency), SCS

      - FFS: transmission power (for pathloss calculation)

    - Note: other parameters included in the configuration can be further discussed

    - Including above agreement into the LS

  - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration.

    - Following RAN1 understanding will be provided in the LS

      - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112:

        - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells

          - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there.

        - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells

          - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells]

        - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells

          - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations.

      - Note RAN2 has a full flexibility to design the whole RRC structure design.

      - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters.

      - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations.

Agreement

- Send an LS to RAN2,3,4 on the RAN1 agreements in this meeting

  - All agreements in AI 9.12.1 and 9.12.2 in RAN1#112 are included

  - The LS contents agreed in AI 9.12.1 (on L1 measurement configuration) and AI 9.12.2 (on RAR) are also included

Agreement

- At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s)

- FFS: beam indication for mTRP case

Agreement

Draft LS R1-2302193 is endorsed in principle by appending latest agreements.

Agreement

Final LS R1-2302194 is endorsed.

In this contribution, we provide our views on L1 enhancements for inter-cell beam management, including L1 measurement, L1 measurement reporting, and beam indication.

Discussion

L1 measurement

It was agreed that SSB based L1 RSRP is supported for intra-frequency measurement and inter-frequency measurement, but whether to support CSI-RS based L1 measurement has not been decided yet. Since CSI-RS can provide more accurate and delicate channel measurement results, it is beneficial to support CSI-RS based L1 measurement for intra-frequency and inter-frequency measurement. Furthermore, since L1-SINR reflects the ratio of the desired signal power to the interference plus noise power, it may provide more realistic measurement results for inter-cell scenario considering the interference caused by neighbouring cells. Thus, support of L1-SINR for intra-frequency and inter-frequency measurement is preferred.

For SSB-based L1-RSRP measurement, it was agreed that some information, for example, frequency domain location, or SCS, needs to be provided to a UE for inter-frequency measurement. However, the measurement gap is also a part of information which the UE might acquire, so at least for SSB based L1-RSRP measurement, the measurement gap information also needs to be provided to the UE.

Moreover, since measurement gap and some reporting requirements are different between intra-frequency and inter-frequency measurement, introducing a UE capability to report whether to support inter-frequency measurement or whether to report intra-frequency and inter-frequency in a single report may be beneficial.

Support CSI-RS based L1 measurement for both intra-frequency and inter-frequency.

Support L1-SINR for both intra-frequency and inter-frequency measurements.

For inter-frequency measurement, measurement gap information needs to be provided to a UE.

Introduce a UE capability to report whether to support inter-frequency measurement or whether to enable report for intra-frequency and inter-frequency in a single report.

L1 measurement reporting

In RAN1#111 meeting, two methods were proposed to report L1 measurement report for Rel-18 LTM. One is reporting L1 measurement on UCI, which was agreed in RAN1#111 meeting, and the other one is reporting L1 measurement on MAC-CE, which may need further study. Companies which support reporting L1 measurement report on UCI state that UCI has sufficient capacity to accommodate all L1 measurement report information. On the other hand, companies which support reporting L1 measurement report should be transmitted on MAC-CE say that reporting in UCI may cause reliability issue since the payload of L1 measurement report is too large. It is hard to make a conclusion because the L1 measurement report content is still under discussion.

- It is hard to decide whether to support L1 measurement report for Rel-18 LTM on MAC-CE before the L1 measurement report contents are determined.

In RAN1#112 meeting, L1 measurement report content corresponding to the number of candidate cells and the number of beams that the UE can report was discussed. Therefore, before deciding whether to support reporting L1 measurement for Rel-18 LTM on MAC-CE, contents of the L1 measurement report for Rel-18 LTM should be determined first. If UCI has sufficient capacity to transmit the L1 measurement report payload, then there is no need to additionally support transmitting L1 measurement report on MAC-CE.

Whether to support L1 measurement report for Rel-18 LTM on MAC-CE can be considered after the reporting contents are determined.

- Another issue raised in the L1 measurement reporting is whether to support UE/event triggering report and the details of supporting such reporting by UE. For inter-cell mobility, if considering a case that there are many candidate cells for measurement, some measurement results may not be useful to the gNB (e.g., beam quality of candidate cells is not good enough to trigger a handover), so that the UE does not need to report such measurement results to the gNB. If the UE only triggers the report while specific conditions are fulfilled (e.g., the L1 measurement result is greater than a threshold, or L1 measurement results are greater than a threshold over a specific period.) and does not transmit the unhelpful measurement results while specific conditions are not satisfied, the UE can reduce reporting overhead. Therefore, support UE/event triggering report may reduce reporting overhead.

Support UE/event triggering report for reducing reporting overhead.

Beam Indication

In the previous meeting, the scenario that beam indication is provided in the cell switch command was agreed to be supported in Rel-18 LTM. Whether to indicate the beam before or after the cell switch command was still FFS. In the case that the beam is indicated after the cell switch command, the latency of the LTM procedure cannot be reduced effectively. In addition, it seems that the legacy beam indication method can be reused in this case. Therefore, we don’t need to spend the limited TU discussing this case.

Regarding the case that the beam is indicated before the cell switch command, it is an efficient way to further reduce the latency of the LTM procedure. To ensure fast data transmission after the cell switch, it would be better to support this scenario. However, this scenario is more applicable for LTM if Rel-17 ICBM can be supported for Rel-18 LTM. From our perspective, Rel-17 ICBM can provide an efficient beam indication method for LTM. It shall be supported and enhanced for Rel-18 LTM.

Support beam indication before cell switch command to ensure fast data transmission after the cell switch.

Conclusions

In this contribution, we discussed the issues regarding L1 enhancements for inter-cell beam management which includes L1 measurement, L1 measurement reporting and beam indication.  And we have proposals as follows.

- Support CSI-RS based L1 measurement for both intra-frequency and inter-frequency.

Support L1-SINR for both intra-frequency and inter-frequency measurements.

For inter-frequency measurement, measurement gap information needs to be provided to a UE.

Introduce a UE capability to report whether to support inter-frequency measurement or whether to enable report for intra-frequency and inter-frequency in a single report.

It is hard to decide whether to support L1 measurement report for Rel-18 LTM on MAC-CE before the L1 measurement report content are determined.

Whether to support L1 measurement report for Rel-18 LTM on MAC-CE can be considered after the reporting contents are determined.

Support UE/event triggering report for reducing reporting overhead.

Support beam indication before cell switch command to ensure fast data transmission after the cell switch.

References

Chairman notes of 3GPP RAN1 #112 meeting