3GPP TSG RAN WG1 #112bis-e	R1-2303415

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Measurements and reporting for SL positioning

Document for:	Discussion & Decision

# Introduction

The work item objectives related to SL positioning/ranging in RP-223549 [1] includes:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution we discuss reporting aspects for SL positioning and measurements to support RTT type solutions.

# RTT-type solutions using SL

## Report-free RTT

In the FL summary [2], applicability of report free was discussed during RAN1 #112. Majority of sources agrees on relevance while some concerns were raised to study first the applicability before supporting this feature.

### Motivation

The positioning service requirements identified in TR 38.845 or TS 22.261 demand for latencies which can be as low as 10 ms. By enabling RTT procedures which does not rely on the Rx-Tx measurement report will enable extremely low latency ranging operation.

Furthermore, the report free operation enables determining the RTT even in scenarios where there are limitations in the transmit power.  This is because the SL-PRS itself can be decoded at very low SINR (e.g. -20dB or even lower, depending on the configuration), whereas for exchanging measurements using higher layer signaling, a higher SINR is required for transferring the messages (for example the ProvideLocationInformation). The higher SINR requirements of the physical channels used for reporting may require a SL signal with narrow bandwidth to increase SINR for the utilized bandwidth of PSSCH, for example.

In a different aspect, report-free reduces the signaling overload especially when large number of devices (responders) are participating the SL-positioning. Furthermore, with respect to the responder UE, report-free allows for completing the RTT operation over the dedicated resource pools since no measurement report is required over the communication channels.

### Operation

NR Multi-RTT method relies on the UE Rx–Tx and gNB Rx–Tx time difference measurement to determine the round trip time between the TRP and UE. A second well popular approach, is to estimate the range from a single measurement report or even without reporting, when the reply-time is known at the range determining device. Assuming for example UE1 as the range measurement device, the time of flight (tofUE1,UE2) for a signal propagation with UE2 is:

tofUE1,UE2=12( RTT-treply@UE2).

UE1 needs only to have prior information on treply UE2 to determine the time-of-flight tof from the round trip delay. The range can also be determined at a different node, such as the LMF, from a UE Rx–Tx measurement report of UE1 only. Hence, reporting-free RTT refers that at least one of the ranging devices is not required to report its measurements. Instead of reporting the Rx-Tx delay the Rx-Tx delay is either known or configured.

From the perspective of UE2, the transmit time for the SL-PRS must fulfill network synchronization requirements to avoid interference. Using a constant treply@UE2 may violate this condition, if the treply@UE2 defines the time of the first sample of the SL-PRS transmitted as response to the received SL-PRS. To avoid this a cyclic shift can be applied to the SL-PRS and treply@UE2 applies to a “virtual time of transmit”, which differs from the time of the first sample by the cyclic shift. This allows to set the transmit time related to the first sample of the SL-PRS according to the network synchronization and timing advance requirements, while the cyclic shift ensures that the receiver detects a ToA according the “virtual” time-of-transmit.

Cyclic shift is already an established concept in 3GPP (used, for example, to transmit reference signals in parallel over different antennas ports or for the PFSCH). For positioning it can utilize the implementation of a constant (configurable) delay, while maintaining the OFDM symbol timing. The principle of the adjustment of the effective time-of-transmit is depicted in Figure 1.

- A OFDM symbol (before cyclic prefix insertion) is “cyclic shifted”. This can be performed in the time-domain or equivalently in the frequency domain by applying a phase ramp to the REs

- Performing a cyclic correlation with the (not shifted) reference signal will result in a delay offset according to the cyclic shift value.

- The cyclic prefix is inserted after applying the cyclic shift to the main symbol. This ensures a correct measurement also for non-ideal symbol timing recovery by the receiving UE.

Figure 1: Adjustment of effective time-of-transmit using cyclic shift

The method can be employed to implement a report-free RTT as depicted in Figure 2:

- UE1 (“initiator”) transmits a SL-PRS to a second UE (“UE2”)

- UE2 (“responder”) detects the ToA (t2 in the figure) for the SL-PRS received from UE1 (initiator)

- The OFDM transmit framing of UE2 may be derived from the network or the signal received from the initiator and it is assumed that UE2 transmits aligned to this framing according the timing-advance (TA) setting. This defines the ToTSym, (t3 in the figure), wherein ToTSym may represent the time of the first sample of the SL-PRS

- The effective (“virtual”) time of transmit of the SL-PRS with the desired treply@UE2 can be calculated by 
 		ToTeff = ToA + treply@UE2

- The SL-PRS is modified by applying a cyclic shift to the SL-PRS, where the required cyclic shift can be calculated by 
  		tCS = ToTeff - ToTSym

Proposal 1: 	With regards to RTT methods, SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement shall be supported:

- The mechanism shall not have any impact on the OFDM symbol timing requirements

Figure 2: Example of report-free RTT operation (treply@UE2 is the difference between t5 and t2)

## Rx-Tx Measurement

Agreement For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

Proposal 2: 	Support using the actual SL-PRS transmission time for the definition of SL-PRS based Rx-Tx time difference measurement.

## Rx-Rx Timing measurements

Discussion during the study item phase did not conclude if the impact of supporting Rx-Rx timing difference cannot be already supported based on the identified SL timing measurements (i.e. SL Rx-Tx, SL RSTD and SL RTOA).

Rx-Rx Timing measurements may utilize several functionalities. Examples are:

- Clock calibration: Measure the frequency offset of a device relative to another device

- If the frequency offset is known with sufficient accuracy the measurements may be an indicator of the relative velocity

Rx-Rx timing can be defined as a timing difference measurement between two consecutive bursts from the same transmission source. Where the Rx-Rx measurement should be coherently realized so that no timing or frequency offset changes the time between a pair of measurements.

Based on the above definition, there are two approaches to support Rx-Rx measurements in the SL positioning Framework:

- Alt.1: support an additional Rx-Rx measurement which defines the UE report and processing related to a Tx-Tx transmission. The Rx-Rx measurement method may be FFS. Different methods are possible and have pros and cons depending on the required accuracy and the measurement latency and define the related requirements (e.g., no re-adjustement of the clock between two measurements)

- Alt.2: Support functionality without introducing an Rx-Rx measurements by reporting the ToA of each received signal. It can be implied from the identified SL timing measurements and potential additional UE procedure/signaling. Additional information like “clock re-adjustment in between two measurements” may be added to the ToA reports.

Proposal 2: 	For determining the relative velocity between the two UEs using Rx-Rx timing measurements within the framework of SL RTT solution, downselect between:

- Alt.1: Support an additional Rx-Rx measurement which defines the UE report and processing related to a Tx-Tx transmission

- Alt.2: Support functionality without introducing an Rx-Rx measurements (it can be implied from the identified SL timing measurements (time stamps with fine resolution, for example)

## Frequency error compensation: Single sided vs Double sided

Agreement For SL-PRS based Rx-Tx measurement for double sided RTT, consider sidelink PRS transmission without order restriction between multiple rounds of PRS transmission of involved UEs. - FFS on how to differentiate different PRS transmissions for sidelink PRS Rx-Tx measurement and report - FFS on impact of Scheme 2 resource allocation when the different orders in double sided RTT is considered and whether and how to minimize number of different orders - Aspects related to scheme 2 resource allocation are to be discussed in agenda 9.5.1.3

The double-sided RTT operation such as in Figure 3 (left), demands no specific spec impact related to the order of transmission. Double sided measurements can be considered as a subset of periodic measurements or joint use of one RS for two single sided measurements with different initiators. Beside the Tx-RX difference information on the Tx periodicity and the measured Rx periodicity (Tx-Tx delays and Rx-Rx delays or time stamps (with high resolution) for each measurement may be useful for mitigation of the frequency offset between two devices or other purposes. If time stamps are used or measurements for resources transmitted with high time offset re-initialization of the clock generator of the device may have an impact to the difference. Hence, the procedure shall ensure that a frequency offset did not change during a set of SL-PRS transmission and reception or the change can be considered as marginal. In Figure 3 (right) we depict different examples of periodic RTT measurements. First, the standard allows that the second device transmits before the signal from the first device is received (marked in green). Beside the Tx-Rx delays (e.g., t23(n) = t3(n) – t2(n) and t14(n) = t4(n) – t1(n)) other delays/periods such as the Tx-Tx delay t15(n) = t5(n) – t1(n) = t1(n+1) – t1(n) or the Rx-Rx delay t26(n) = t6(n) – t2(n) = t2(n+1) – t2(n) can be considered for frequency offset mitigation or other purpose like relative velocity. In Figure 3 (right) we depict different examples:

- The repetition rate is low (e.g., difference between two measurements in several slots or frames).
Comparing Tx-Tx measurements with Rx-Rx measurements of the other device the frequency offsets can be estimated, wherein the measured offset may be the sum of the frequency offset resulting from the clock oscillator offset and the Doppler offset according to the relative velocity.

- The repetition rate is high (two resources in a frame are configured, for example. In the figure marked in red) allows to combine the measurements to increase the reliability of the measurement in case of low SINR. Typically, the channel coherence time is higher than a slot duration. Hence, the channel characteristics changes within a slot or adjacent slots are minor.   Furthermore, additional information like change of the measured phase (e.g., phase of the correlation peak) may become useful.

Figure 3: Example on double-sided RTT (left) and sequence of single sided measurement (right).

For the evaluation of the frequency offset impact we compare following options for the SL-RTT measurements

- Single sided
RTT=t14-t232

- Double sided

  - Method 1: RTT is calculated by 
 
RTT=t14-t23+t36-t454

  - Method 2: RTT is calculated by 
  	
RTT=t14×t36-t23×t45t14+t45

- Single sided + periodic measurements
The frequency offset is estimated by comparing the Tx-Tx and Rx-Rx periodicity

For the comparison we consider three scenarios

- Both devices have a frequency error and a symmetrical reply time (t23 ≈ t45)

- Both devices have a frequency error and an unsymmetrical reply time (t23 ≄ t45)

- One device (e.g., a RSU) is synchronized to the network or use a high accuracy oscillator

For these scenarios we can conclude

- For single sided the measurement error depends on the frequency accuracy for all scenarios

- Double sided, method 1 is sensitive for unsymmetrical configuration. If the difference between t23 and t45 is high a high error may result.

- Double sided, method 2 allows also unsymmetrical configurations (t23 ≄ t45).

- Single sided along with periodic timing measurement can be combined (periodic single sided measurement) which is attractive if one device is synchronized to the network (or uses a high accuracy oscillator). In this case the frequency error of the second device (UE2) can be estimated and compensated from the two received measurements. The ratio between the periodic transmissions delay and the associated measurements defines the relative frequency offset of the devices. The measurement accuracy of the relative frequency offset depends on the periodicity. If carrier phase measurements are taken into account a high accuracy may be already achieved for a small time interval between two measurements/repetitions.

SL resource allocation in scheme 2 (UE autonomous operation) is based on SPS (semi persistent scheduling). Hence, for typical applications a semi-persistent allocation may be used anyway and several (periodic) measurements may be performed.

Observation 1:	A double sided measurement can be derived from periodic single sided measurements.

Proposal 3: 	Consider successive Rx-Tx measurements in a single report and related UE procedure to support double-sided RTT.

# Enhancements for NLOS and critical LOS scenarios

For positioning two major effects degrade the performance

- The LOS path is weak or impaired by multipath components arriving with a low delay relative to the direct (LOS) path. Examples are ground reflections or objects close to the transmitting or receiving device.

- Number of LOS links is not sufficient and measurements on NLOS links must be taken into account.

N-path reporting is partly already supported for RTOA and RSTD measurements. It may be worthwhile to support also enhanced reporting for measurements on SL PRS similar to DL-PRS and SRS measurements.

Proposal 4: 	Support additional path reporting for SL-PRS timing measurements.

# UE coordinate system and ARP

Agreement Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE).

Figure 4 - UE uses two antenna panels which are located at different positions

For high accuracy positioning it is essential to take the position offset of the ARPs into account. For the TRPs this can be implemented by simply using different anchor positions in the common coordinate system used for the position of the TRP and UE (e.g. global coordinate system (GCS) or local coordinate system (LCS)). It should however be noted that a UE may require the definition of its own UE coordinate system (UCS) with its origin being set to the UE reference point. This allows the ARPs of the UE antennas or UE antenna panels to be defined with respect to this UCS.

Proposal 5: 	For SL-PRS transmissions and measurements, consider the UE reference point and ARP location information within the UE coordinate system.

# Conclusion

We made the following observations and proposals in this contribution:

Observation 1:	A double sided measurement can be derived from periodic single sided measurements.

Proposal 1: 	With regards to RTT methods, SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement shall be supported:

- The mechanism shall not have any impact on the OFDM symbol timing requirements

Proposal 2: 	Support using the actual SL-PRS transmission time for the definition of SL-PRS based Rx-Tx time difference measurement.

Proposal 3: 	Consider successive Rx-Tx measurements in a single report and related UE procedure to support double-sided RTT.

Proposal 4: 	Support additional path reporting for SL-PRS timing measurements.

Proposal 5: 	For SL-PRS transmissions and measurements, consider the UE reference point and ARP location information within the UE coordinate system.

# References

[1] RP-223549 “New WID on Expanded and Improved NR Positioning”; Electronic Meeting, December 2022; Intel Corporation, CATT, Ericsson

[2] R1-2301952: “Summary #3 on Measurements and reporting for SL positioning” vivo