3GPP TSG RAN WG1 #112bis-e	R1-2303416

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.3

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Considerations on SL-PRS resource allocation

Document for:	Discussion & Decision

# Introduction

The work item objectives related to SL positioning/ranging in RP-223549 [1] includes:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution we discuss on SL positioning operation in scheme2 and resource allocation for dedicated and shared resource pools.

# Dedicated resource pools

- Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

In RAN1#112 it was agreed that SL-PRS are associated with an SCI. For the realization, it is still open whether a single or 2-stage SCI will be included. Although generally a unified approach for shared and dedicated resource pools should be considered, this option may result unnecessary complexity which conflicts with the motivation for defining dedicated resource pools. A new PSCCH format can be selected for the dedicated resource pool including all configuration information related to the SL-PRS

Proposal 1: 	For the dedicated resource pool for positioning a new single-stage SCI is introduced.

Within a SL-BWP one or several resource pools can be defined. Each resource pool is split in sub-channels. The bandwidth of a sub-channel and the number of symbols used per slot are configurable. The bandwidth of a sub-channels is 10 to 100 PRB = 120 to 1200 subcarrier. The resulting bandwidth per subchannel is 3.6MHz to 36MHz for SCS = 30kHz.

For the dedicated resource pool the area (number of symbols and number of PRB) required for PSCCH can be selected according to the amount of data required for the configuration. Optional the size of the PSCCH may be also configurable in the resource pool configuration.

The number of OFDM symbols used for PSCCH shall be kept to minimum in order to increase the number of OFDM symbols including SL-PRS. The resulting slot structure is depicted in Figure 1. Furthermore, an AGC symbol with the same bandwidth as the PSCCH shall be inserted.

For the sharing of the slot by several UEs the same principle as depicted in Figure 3 for the shared resource pool can be applied to the dedicated resource pool (with or without PSSCH field), characterized by

- Different sub-channels are allocated to each UE

- The SL-PRS part are FDMed (comb structure) or TDMed or a combination of it.

- Further sharing or mitigation of critical interference can be supported by using different initialization sequences of the Gold code or different cyclic shifts.

Figure 1: Dedicated slot structure

Proposal 2: 	For the dedicated resource pool the number of symbols assigned to PSCCH shall be kept to minimum in order to provide more capacity for the SL-PRS.

Proposal 3: 	For the OFDM symbols including the PSCCH are allocated in a single subchannel, whereas for the SL-PRS several subchannels can be allocated.

# Shared resource pools

##### [HIGH] Feature Lead Proposal 3.3.1-v2 In a shared resource pool, study the following options - Opt. 1: SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot - The PSSCH and SL-PRS bandwidth is the same - With regards to PSSCH and SL-PRS multiplexing - Alt. 1: Only TDMing is supported - Alt. 2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported - Opt. 2: SL-PRS and associated PSCCH are included (no PSSCH) in a slot - The SL-PRS is indicated in the 1st SCI in the PSCCH - Opt. 3: SL-PRS without PSCCH are included (no PSSCH) - SL-PRS may be reserved by a SCI siganling in the previous slot - In either option, study multiplexing option between PSCCH (if present) and SL-PRS

Rel. 16 allows to configure the number of OFDM symbols used per slot (see IE sl-BWP-Generic-r16 in TS38.331). This may allow to shorten the area used for the PSCCH and PSSCH. The remaining part can be used for SL-PRS and guard symbols. The principle is depicted in Figure 2. An example of a sharing a slot by 4 UEs is depicted in Figure 3. The structure can hence be similar as for the proposed dedicated resource pool with the following potential differences:

- The size (number of subchannels) of the PSSCH field is selected according to the TB size. The PSCCH and PSSCH may occupy several (adjacent) sub-channels

- The PSCCH and PSSCH is backward compatible.

  - The channel (or position in the slot) of the SCI related to the SL-PRS configuration.

Figure 2: Backward compatible extension of Rel-16 SL slot.

For SL-PRS the required SINR is much lower than for the PSCCH and PSSCH. According to the reference pathloss models defined by TR37.885 and the limited transmit power of a UE the feasible bandwidth for the PSCCH and PSSCH depends on the distance. Examples for a usage scenario “street canyon” as defined by TR37.885 are given in the annex. The application scenario described in the annex shows the resulting S(I)NR for different bandwidth configurations. Accordingly, the bandwidth of the PSCCH and PSSCH may be selected inline with the expected distance to the target UE. Given the low SINR detectability requirement of the SL-PRS a higher bandwidth for the SL-PRS can be selected. The SL-PRS bandwidth may be the same as the resource pool bandwidth or configurable in steps of occupied sub-channels.

Proposal 4:	For the shared resource pool the bandwidth of the SL-PRS shall be configurable and independent from the bandwidth selected for the PSCCH and PSSCH.

Figure 3: Wideband SL-PRS using several sub-channels and last symbols of a slot

# Resource pool configuration in scheme 2

Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

[CLOSED] Feature Lead Proposal 3.5.4-v0 For Scheme 2, with regards to Inter-UE coordination for SL-PRS, - consider the following options: - Option 1: Use Rel-17 Scheme 1 IUC procedure as a starting point - Option 2: Use Rel-17 Scheme 2 IUC procedure as a starting point - Option 3: Use both Rel-17 Scheme 1 or and 2 IUC procedure as a starting point - Note: Companies are encouraged to describe the applicability of their proposals with regards to dedicated and shared resource pool, and the changes that may be needed - In addition to the options above, - study whether also conflict indication for the already used SL PRS resource(s), or other SL PRS conflict avoidance mechanisms (e.g. SL PRS muting, comb offset coordination, and SL PRS sequence selection)

For positioning applications, SL-PRS must be coordinated between several devices. Especially for RTT methods between multiple UEs, coordination by a primary (initiator or coordinator) UE allows the coordinated transmitting of several secondary UEs (responders or other UEs used for localizing a UEs) resulting in an efficient use of the resources and a high position accuracy. Within this example two mechanisms can be considered:

- The measurements are initiated by the UE and the anchors (e.g., RSUs) respond.

- The measurements are initiated by the RSUs and a UE approaching the area covered by the RSU responds to all RSUs related to the area in parallel or sequential.

In both cases the UE initiating the measurement can suggest (and pre-reserve) already resources for the responding UEs. The related signaling can be transmitted within the detectable PSCCH/PSSCH area.  To facilitate efficient SL positioning in scheme 2 we propose the following:

Proposal 5: 	For operations in scheme 2, support that an UE can suggest (and pre-reserve) SL-PRS resources for other UE(s) (e.g. responding UEs).

The allocation strategy may take the resulting effective interference into account. It may be worthwhile to provide assistance data (AD) to the UEs supporting the selection of resources causing minimum level of interference. An area identifier is an example, according to the area identifier a UE may exclude or may prefer certain suggested resources.

Proposal 6: 	Support the exchange of assistance data (AD) for the selection of preferred resources or the exclusion of resources.

# Conclusion

We made the following proposals in this contribution:

Proposal 1: 	For the dedicated resource pool for positioning a new single-stage SCI is introduced.

Proposal 2: 	For the dedicated resource pool the number of symbols assigned to PSCCH shall be kept to minimum in order to provide more capacity for the SL-PRS.

Proposal 3: 	For the OFDM symbols including the PSCCH are allocated in a single subchannel, whereas for the SL-PRS several subchannels can be allocated.

Proposal 4:	For the shared resource pool the bandwidth of the SL-PRS shall be configurable and independent from the bandwidth selected for the PSCCH and PSSCH.

Proposal 5: 	For operations in scheme 2, support that an UE can suggest (and pre-reserve) SL-PRS resources for other UE(s) (e.g. responding UEs).

Proposal 6: 	Support the exchange of assistance data (AD) for the selection of preferred resources or the exclusion of resources.

# References

[1] RP-223549 “New WID on Expanded and Improved NR Positioning” ; Electronic Meeting, December 2022; Intel Corporation, CATT, Ericsson

[2] R1-2302162 “Moderator Summary #4 on resource allocation for SL PRS”, Qualcomm

Annex

# Evaluation scenario for bandwidth considerations

We consider the following application scenario derived from the evaluation scenarios according to (road grid sized 433m * 250m). The scenario considers multiple RSUs and a moving vehicle. The vehicle is driving in the street (Position 1) and turns right (Position 2):

- At position 1 the vehicle may receive 4 RSUs under LOS or NLOSv (TX and RX are in the same street, blocker is typical another vehicle).

- At position 1 other RSUs (in the example RSU 5 and 6 may be received under NLOS conditions.

- When the vehicle turns right at position 2 it may lose the LOS or NLOSv connection to RSU 3 and 4, but may now receive the RSU 5 and 6 under LOS condition.

Figure 4: Urban grid scenario as defined by TR37.885 with placements of RSUs

The following table gives typical values for the distance and related pathloss values (fc = 4GHz). The values in bracket may be not relevant (scenario is not applicable or RSU not considered as relevant)

Table 1: Examples for applicable pathloss values for the scenario given in Figure 4.

| RSU | UE position | Distance [m] | Pathloss LOS [dB] | Pathloss NLOSv case 2 [dB] | Pathloss NLOS |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1-4 | 1 | 209 … 223 | 89 | 98 | (119) | RSU1..4 have similar distance to the UE |
| 5, 6 | 1 | 335 … 337 | (92) | (101) | 124 | RSU5 and 6 have similar distance to Pos1 |
| 1 | 2 | 21.5 | 72 | 81 | (88) |  |
| 2 | 2 | 10 | 66.5 | 75 | 78 | RSU closed to car at position 2 |
| 5 | 2 | 240 | 89 | 98 | (119) |  |
| 6 | 2 | 254 | 90 | 99 | (120) |  |

From the Table 1 we can conclude that typical pathloss values are in the range of 66dB to 124dB for the given scenario. Assuming a TX-EIRP of 23dBm the resulting receive power is in the range of  -101dBm to -43dBm resulting in the SNR range given in Table 2 . Note: The values are examples for typical cases and don’t include worst case scenarios like very low distance between neighboring UEs.

Table 2: Resulting SNR according to Rx power in Table 1 and the utilized bandwidth

| Bandwidth | Effective Noise power | SNR |
| --- | --- | --- |
| 100Mhz | -86.9 dBm/100MHz | -14dB to 44dB |
| 20Mhz | -93.9 dBm/20MHz | -7dB to 51dB |
| 3.6MHz | -101.3 dBm/3.6MHz | 0.3dB to 58dB |

From the SNR values we can conclude:

- Assuming a PSCCH/PSSCH with a bandwidth of 3.6MHz the link budget may be sufficient for all cases.

- Depending on the SL-PRS configurations S(I)NR values of -15dB or lower are feasible. Hence, even for 100MHz bandwidth the SL-PRS from the far away RSU can be decoded