3GPP TSG RAN WG1 #112bis-e	R1- 2303417

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.2.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	NR carrier phase measurements for positioning

Document for:	Discussion & Decision

# Introduction

In the WID [1,the following objectives define the scope in the work item of the accuracy improvements when applying carrier phase measurements:

- … - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4]. - …

In this contribution we continue the discussion on measurement definitions, reporting and assistance information to enable NR carrier phase measurements.

# Carrier phase measurement report

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP

Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP

Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements

In RAN1#112, in the above agreement no conclusion was made on whether to introduce one or both of the DL carrier phase (CP) or DL carrier phase difference (CPD). Depending on the scenario one option can be preferable compared to the other. CPD can make use of the AD designed for DL-TDOA and allow for compatible RSTD and CPD reports.  On the other hand, CP offers more flexibility for reporting successive measurements within a report. This can be useful, in case of continuous position tracking of devices.  Additionally CP can be reported along with the Rx-Tx measurement which allows for resolving the UE relative speed.

Assuming N TRPs are measured by the UE, the CPD allows the reduction of the number of values to N-1. For N>2 the selection of the reference TRP may become critical. The most reliable TRP must be selected or N*(N-1)/2 values must be reported. For N=2 CPD will provide the same information as CP. For N>2 the CP method shall be preferred. CPD shall be only used, if a reliable reference is apriori known.

Proposal 1: 	For carrier phase measurements in DL support both single and difference carrier phase measurements.

Proposal 2: 	Support reporting the following measurements:

    - UL CP along with an UL-RTOA or gNB Rx-Tx measurement

    - DL CP along with an DL-RSTD or UE Rx-Tx measurement

    - DL CPD along with a DL-RSTD measurement.

# Carrier phase definition

In RAN1#109-e, the following definition for the carrier phase was agreed:

Agreement - For the purposes of discussion, for NR downlink and/or uplink carrier phase positioning, the carrier phase (CP) at a RF frequency at a receiver is a phase that is a function of the signal propagation time from an Tx antenna reference point of a transmitter (e.g., a TRP or a UE) to a Rx antenna reference point of the receiver (e.g., a UE or a TRP). - The propagation time can be expressed in a fractional part of a cycle of the RF frequency and a number of integer cycles, but the CP may be independent of the number of integer cycles.

In RAN1#112 the reference signal carrier phase (RSCP) for UL and DL was defined by the following agreements

Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE

Figure 1: Example scenario for positioning with phase measurement

Figure 1 shows an example scenario. In the example we assume a moving UE in a multipath environment. For simplicity reasons we depict in Figure 1 three paths only. Figure 2 includes other components also.

- Path 1: LOS signal: This signal has the shortest propagation path to the base station

- Path 2: Ground reflection: For the example we assume a specular reflection. The additional path length depends on the UE height, base station height and the distance

- Path 3: Reflection from a wall

An example for the resulting channel impulse response (magnitude) is given in Figure 2. The channel impulse response is a complex valued signal. The complex valued signal can be depicted as two components (I and Q) component or as “I/Q diagram”, wherein the x-axis represents the I component (real part) and the y-axis the Q component (imaginary part).

Figure 3 shows an example for the I/Q diagram of the 3 paths of the CIR, where the paths are separated and the effects resulting from the UE movement indicated in Figure 1.

Figure 2: CIR example for scenario given in Figure 1

Figure 3: I/Q diagram of complex valued CIR (3 taps only)

For the example we assume the phase is measured relative to a reference common for transmit and receive part (“ideal synchronization”). In this case the phase change depends directly on the distance change resulting from the movement. In case of non-ideal synchronization an additional “common rotation” according to the frequency offset (or phase noise) of the vectors in the I/Q diagram must be taken into account. 
In the example the path length for path 1 (LOS) and 2 (ground reflection) is increased due to movement. For path 3 (distance to the wall is reduced) the path length is reduced, resulting in an opposite sign of the phase change.

The effect of ground and near reflections becomes more relevant when bandwidth is limited. For the high bandwidth the lobes related to the LOS path and wall reflection are well separated. For 50MHz the correlation lobes overlap and the measured phase may be no longer directly related to the distance change. In LOS scenarios the multipath components will arrive at lower level. In this case the correlation peak still represents the expected phase change according to the distance change LOS UE. But depending on the additional path length of the near reflections, the AoD or AoA difference and the strength of multipath components the measurement accuracy may be reduced. Advanced signal processing methods may be able to separate the LOS and EC (early cluster = multipath components arriving with a low delay relative to LOS). But for lower bandwidth and lower delays it may become more difficult. But the phase information may help to identify such scenario. Furthermore and extended reporting (e.g. complex values reporting of the correlation lobe related to the FAP (or useful reflections) and may help to derive further information to enhance the estimator or at least a reliability indicator. 
Note:  Figure 4 shows the correlation function interpolated to a high sampling frequency. At the nominal sampling frequency according to the bandwidth of the signal only discrete points of the correlation function will be measured.

| 400MHz: | 50MHz: |
| --- | --- |
| 400MHz: | 50MHz: |

Figure 4: Examples of the magnitude of the CIR and IQ measured with 400MHz and 50MHz

Proposal 3: 	Support reporting the phase measurements for the first and additional paths (N-path reporting for the phase) associated with the LOS path.

The additional paths can be identified from the correlation function in the delay domain.

# Coherency information

To signal the applicability on the processing and transmission to support carrier phase measurements, we propose:

Proposal 4: 	Consider the signaling of the phase-coherency information related to the different PEGs at a UE/TRP.

Proposal 5: 	Consider the signaling to the UE phase-coherency information related to the different TRPs to inform on the applicability for the processing carrier measurements.

# Conclusions

Based on the discussions, we made the following observations and proposals:

Proposal 1: 	For carrier phase measurements in DL support both single and difference carrier phase measurements.

Proposal 2: 	Support reporting the following measurements:

    - UL CP along with an UL-RTOA or gNB Rx-Tx measurement

    - DL CP along with an DL-RSTD or UE Rx-Tx measurement

    - DL CPD along with a DL-RSTD measurement.

Proposal 3: 	Support reporting the phase measurements for the first and additional paths (N-path reporting for the phase) associated with the LOS path.

The additional paths can be identified from the correlation function in the delay domain.

Proposal 4: 	Consider the signaling of the phase-coherency information related to the different PEGs at a UE/TRP.

Proposal 5: 	Consider the signaling to the UE phase-coherency information related to the different TRPs to inform on the applicability for the processing carrier measurements.

# References

[1] RP-223549 “New WID on Expanded and Improved NR Positioning”; Electronic Meeting, December 2022; Intel Corporation, CATT, Ericsson