3GPP TSG RAN WG1 #112e-bis	R1-2303418

e-Meeting, 17th April – 26th April 2023

Agenda item:		9.13.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UE beamforming capability for FR1 based on beam switching among fixed directional antennas

Document for:		Discussion and Decision

# Introduction

RAN#94e approved a revised WID on NR Support for UAV (NR_UAV) [1]. In RAN#98e, the WID was revised as [2], with the following objective related to RAN WG#1:

- Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]:

  - FR1 with directional antenna at UE side

In this contribution we present our views on the UE UAV beamforming capabilities for FR1 based on beam switching among fixed directional antennas at UE side, as stated in the objective above.

# Discussion

During #112 meeting, the agreement to consider potential UE UAV beamforming capabilities for FR1 with directional antennas at UE side and that UE UAV beamforming for FR1 based on beam switching among fixed directional antennas is supported in Rel-18 were made.

RAN1 #112: Agreement RAN1 only considers potential UE UAV beamforming capabilities for FR1 with directional antennas at UE side in Rel-18 if such capabilities don’t impact RAN4 requirements. Agreement If new UE UAV beamforming capabilities for FR1 with directional antennas at UE side are supported, Rel-17 unified TCI framework is considered as baseline. Agreement Multi-TRP beamforming is not supported for UAV UEs in FR1 in Rel-18. Agreement UE UAV beamforming for FR1 based on beam switching among fixed directional antennas is supported in Rel-18. - Note 1: new UE capabilities may not be necessary to support beam switching among fixed directional antennas - Note 2: no RAN4 specification impact is assumed - FFS: whether updating (e.g. extending to FR1) legacy UE capabilities is needed, and it is not precluded if it is needed - FFS: whether/how specification may be impacted

## UE UAV beamforming capabilities for FR1 based on beam switching among fixed directional antennas at UE side

It was agreed that RAN1 only considers potential UE UAV beamforming capabilities for FR1 with directional antennas at UE side in Rel-18 if such capabilities do not impact RAN4 requirements.

Other important aspects to be considered are:

- The targeted support of beamforming capabilities for FR1 beam based on switching among fixed directional antennas at UE side have to be specific to UAV applications and scenarios only, and not be applicable in general for any type of Release 18 UE operating in FR1.

- The support for beam switching among fixed directional antennas is primarily required for Layer 3 mobility purposes, when the UE UAV performs a handover between two cells operating in FR1.

- The support of FR2-like beam management capabilities in FR1 needs to be considered at more general level, not only for UE UAVs, hence it is largely out of the scope of the discussions in this Release18 RAN1 WI.

We believe, for the current Release 18 WI, the minimum UE UAV beamforming capabilities for FR1 with directional antennas at UE side should be considered only, which are possible to specify in the available time budget, without RAN4 involvement, yet are still useful for MNOs and UAV vendors to harness the benefits of such UE UAV antenna configurations.

Observation 1: The minimum UE UAV beamforming capabilities for FR1 with directional antennas at UE side should be considered only, which are possible to specify in the available time budget.

One possible way forward which does not have any implications on RAN4/RAN5, is to (re)use the existing 2TX or 4TX RAN4 requirements and the RAN5 conductive test configurations for the UE UAV category. In the Annex section we show results with possible antenna configuration using UAV UE fixed grid of beams. Analyzing the baseline cases with 1, 2 or 4 beams, it is practical to assume that these directive antenna elements can be mapped to the 2TX or 4TX configurations i.e., also tested as such. This implies that all existing TS 38.101-1 [7] and TS 38.306 [8] specifications are kept unchanged, as long as we ensure the TX power and EIRP requirements. Furthermore, due to similarities with V2X, the specifications in TS 38.101-1 (clause 6.2E, &.5E)[7] can also be considered to be applicable. One of the key requirements is the availability of “supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [9]” as noted in TS 38.101-1 6.2E.1.1 [7]; which would be relatively easy requirement to adopt for the UEs supporting NR UAV Release 18 features.

Our proposed framework is fully in line with the WID description as it works well for FR1 (it is agnostic to frequency). While other ongoing work for multi-beam operation is currently restricted to FR2 operation and has to deal with other complications and the dynamicity of FR2 (with higher switch rate between selected beams) rather than the interference management.

Proposal 1:  RAN1 UAV Release 18 WI to consider the (re)use of the existing 2TX or 4TX antenna configurations to support UAV beamforming in FR1 based on beam switching among fixed directional antennas at UE side.

During the previous RAN1#112 meeting, Rel17 unified framework was agreed to be a baseline for UE UAV beamforming capabilities for FR1 with directional antennas at UE side. It should be noted that not all capabilities that are FR2 only might be needed for FR1 beam management procedure.

Proposal 2: RAN1 to consider extending only subset of the existing FR2 only capabilities and parameters from unified TCI state operation, for the purpose of the UE UAV beamforming capabilities for FR1 based on beam switching among fixed directional antennas at UE side.

## Indication of UAV UE Beam Characteristics

In RAN1#110bis-e the following agreement was made regarding indication of beam characteristics as UE capability to the network:

Agreement Study indication of beam characteristics, e.g., number of beams, beamwidth, beam center, radiated EIRP, etc. as UAV UE capability - FFS: Feasibility/benefit of indicating orientation of beams including height dependence - FFS: Necessary parameters, ranges of suitable values, and method of indication - FFS: Height-dependence on relevant parameters - FFS: Indication of beams as either ‘fixed’ or ‘adaptive’ Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks)

In scenarios where UAV UEs are operating in dense urban environments, UL transmissions made by those UAV UEs have the potential to create significant inter-cell interference, due to the higher LoS probability of links between UAV UEs and gNBs, particularly at high altitudes. This UL interference can negatively impact both UAV UEs and conventional ground UEs, while attempts to mitigate interference via guard bands/intervals can limit spectral efficiency and reduce network capacity. This effect can be most dramatic in scenarios where a UAV UE may be providing enhanced connectivity to ground UE and therefore have high demand for UL capacity.

UL beamforming has demonstrated the ability to reduce UL inter-cell interference, improve network capacity, and enable more flexible load balancing among neighbouring cells [3] - [6]. Benefits can be realized when UAV UEs make local beamforming decisions, however these approaches are sub-optimal compared to network-assisted approaches.

In order to evaluate further the potential benefits from using UAV beams (FR1), we show in the Annex results with more realistic assumption of fixed grid of beams at the UAV (fixed, relative to the orientation of the UAV body/ fuselage/ airframe). Our latest evaluations results show that an antenna configuration with a minimum of 2 UAV beams of 65 deg horizontal beamwidth is sufficient to harness these benefits, in the considered regular rural scenario network deployment. To conclude, for generalisation purposes, and for real-life network deployments, with denser sites etc., we recommend the use of at least 4 beams at the UAV, while a 2-beams configuration is the minimum needed to have any benefits and improvements. See Annex for details.

Observation 2: For uplink interference mitigation it is beneficial to define at least two UAV UE beam center directions (see for example Figure 1 in Annex).

This information can be beneficial in interference reduction, where UAV UEs can jointly optimize beamforming to limit interference and maximize capacity and in load-balancing, where UAV UEs may be capable of being handed over to more distant gNBs that may be less heavily loaded.

In order to achieve the benefits of network-assisted beamforming, the network must first be able to monitor the beamforming characteristics of the UAV UEs operating within the service region. While there is no current baseline for characterizing the beamforming capabilities of UEs in NR RAN, conformance requirements specified for network infrastructure in FR2 have performed a similar function in RAN4 [8].

The following parameters are specified as requirements on radiated transmissions for repeaters, IAB nodes and gNBs operating in FR2 [8]:

Beam: beam (of the antenna) is the main lobe of the radiation pattern of an antenna array.

Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam.

Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction.

Beam peak direction: direction where the maximum EIRP is found.

Beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse.

OTA coverage range: a common range of directions within which OTA requirements that are neither specified in the OTA peak directions sets nor as TRP requirement are intended to be met.

OTA peak directions set: set(s) of beam peak directions within which certain OTA requirements are intended to be met, where all OTA peak directions set(s) are subsets of the OTA coverage range.

Reference beam direction pair: Beam direction pair in the reference direction declared by the manufacturer.

While this set of parameters would be more than sufficient for the network to fully characterize directional transmissions made by a UAV UE, a more limited set of features may be sufficient to realize the benefits of network-assisted beamforming for UAV UEs. Based also on the discussion in Section 2.1, our proposal is:

Proposal 3: Specify UE FR1 capability for signalling for at least the number of beams, beam center directions and post antenna connector gain, of UAV UEs, each with a unique beam identity.

In addition to beam characteristics of the UAV UEs it may be necessary for the network to be able to monitor UAV UE orientation – hence implicitly the orientation of the UAV beams – as unlike in network infrastructure, the orientation of UAVs may change as a result of UAV mobility and flight path. This is beneficial first and foremost for operation in FR1 as explained above, and can also be utilized in FR2 when combined with the existing UE beam management solutions.

Proposal 4: Specify UE capability to report at least UAV UE orientation to the network e.g., heading/velocity vector as part of the location information

Naturally, RAN2 signalling support is needed for the above proposals. We consider this a minimum specification effort which is more or less already begin addressed in current RAN2 UAV Release 18 discussions.

Observation 3: The signalling required to convey the information about the UAV UE orientation can be RRC signalling as being discussed in RAN2 UAV Release 18.

# Conclusion

In this contribution, we discussed the UE capability signalling to indicate UAV beamforming capabilities.

The following observations have been made:

Observation 1: The minimum UE UAV beamforming capabilities for FR1 with directional antennas at UE side should be considered only, which are possible to specify in the available time budget.

Observation 2: For uplink interference mitigation it is beneficial to define at least two UAV UE beam center directions (see for example Figure 1 in Annex).

Observation 3: The signalling required to convey the information about the UAV UE orientation can be RRC signalling as being discussed in RAN2 UAV Release 18.

The following proposals have been made:

Proposal 1:  RAN1 UAV Release 18 WI to consider the (re)use of the existing 2TX or 4TX antenna configurations to support UAV beamforming in FR1 based on beam switching among fixed directional antennas at UE side.

Proposal 2: RAN1 to consider extending only subset of the existing FR2 only capabilities and parameters from unified TCI state operation, for the purpose of the UE UAV beamforming capabilities for FR1 based on beam switching among fixed directional antennas at UE side.

Proposal 3: Specify UE FR1 capability for signalling for at least the number of beams, beam center directions and post antenna connector gain, of UAV UEs, each with a unique beam identity.

Proposal 4: Specify UE capability to report at least UAV UE orientation to the network e.g., heading/velocity vector as part of the location information

# References

- 3GPP RP-213600, WID on “NR support for UAV”, RAN#94e.

- 3GPP RP-223545, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles), RAN#98e

- Yong Zeng (Editor) et al., “UAV Communications for 5G and Beyond,” Wiley 2020, ISBN: 978-1-119-57569-6.

- Tomasz Izydorczyk et al., “Achieving High UAV Uplink Throughput by Using Beamforming on Board,” in IEEE Access, vol. 8, pp. 82528-82538, 2020, doi: 10.1109/ACCESS.2020.2991658.

- R1-2209631, ” UE capability and RRC signaling for UAV beamforming”, 3GPP TSG RAN WG1 #110bis-e, 10 – 19 October, 2022.

- Ayat Z-H. et al, “Uplink coexistence for high throughput UAVs in cellular networks”, Proc. IEEE Globecom, December 2022.

- 3GPP Technical Specification 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)”, V18.0.0 (2022-12).

- 3GPP Technical Specification 38.306, “NR User Equipment (UE) radio access capabilities (Release 17)”, V17.1.0 (2022-06).

- 3GPP Technical Specification 38.106, “NR Repeater Radio Transmission and Reception (Release 17)”, V17.1.0 (2022-06).

# Annex

In reference [7] and [8] we have presented early evaluation results in terms of the terrestrial uplink user throughput improvements when using beamforming at the UAV. The main assumptions and results from [8] are reiterated here in Figure 1, for completeness.

Figure 1: Left: Simulated network deployment in FR1 for evaluating the interference impact on one hot spot cell, c.f. the scenario in [4]. Right: Example evaluation result of the simulated scenario, in terms of cell #1 terrestrial user throughput distribution [6].

The performance numbers above from [4] have been generated assuming ideal UAV beam orientation, i.e., perfect alignment with the desired direction required to minimize interference or to maximise RSRP. In order to evaluate further the potential benefits from using UAV beams (FR1), we show here results with more realistic assumption of fixed grid of beams at the UAV (fixed, relative to the orientation of the UAV body/fuselage/airframe), as depicted in Figure 2. These antenna beam configurations are similar to the setup used in the evaluation presented in [3], with additional single beam configuration.

2 x 65o beams

4 x 65o beams

6 x 65o beams

1 x 65o beams

2 x 65o beams

4 x 65o beams

6 x 65o beams

1 x 65o beams

Figure 2: UAV UE fixed grid of beams. The UAV is assumed to be a hexacopter and is depicted as a blue hexagon. The directive antenna elements in the horizontal plane are indicated with gray triangles. The antenna is assumed to be non-directional in the elevation plane.

For the evaluation of the impact of using the UAV beams configuration in Figure 2, we use the same simulator assumption as in [6]. The UAV locations at a height of 50m are selected randomly within the inner circle area in Figure 1, and for each location the UAV orientation – hence also the orientation (horizontal angle) of the fixed grid of beams in Figure 2 – is selected uniform randomly.

The main results, in terms of the terrestrial UL user throughput distribution in selected hot spot cell (cell #1, same as in [6] and Figure 1) are shown in Figure 3. The curves labelled with “(perfect)” correspond to the earlier results presented in [6], while the curves labelled “(grid)” are the new results assuming the antenna configurations in Figure 2. The main conclusion from these results confirms the benefits from using UAV beams to minimize overall uplink interference, especially when combined with the minimum interference (towards the hotspot cell) -based cell selection (same as [6] and Figure 1). The new results show that an antenna configuration with a minimum of 2 UAV beams is sufficient to harness these type of benefits (in the considered scenario):

  - The configuration with 1 beam, although show similar results, should not be used in practice due to the too strong dependency on the UAV orientation, when considering a given flight path/trajectory.

  - The configuration with 2 beams, is flexible enough, provided the beamwidths are at least 65o or 90o as evaluated in [3]

  - The configurations with 4 and 6 beams perform rather similarly as the 2 beams scenario. We believe this is mainly due to the considered deployment scenario (regular grid with 1763m inter-site distance)

Analysing in more details the benefits of UAV beams we show in Figure 4 the UAV uplink transmit power distribution for each UAV UE antenna configuration. From these results, it is evident that the use of 2-beams is mostly beneficial when RSRP based cell selection is performed, while a 4-beams configuration lowers further the required transmit power for both, RSRP and interference -based cell selection.

To conclude, for generalisation purposes, and for real-life network deployments, with denser sites etc., we recommend the use of at least 4 beams at the UAV, while a 2-beams configuration is the minimum needed to have any benefits and improvements.

Figure 3: UAV beam configuration impact on the terrestrial UL user throughput distribution in selected hot spot cell (cell #1, same as in [6] and Figure 1).

Figure 4: UAV beam configuration impact on the UAV UL transmit power distribution.