3GPP TSG RAN WG1 #112bis-e										R1-2303419

e-Meeting, April 17th – April 26th, 2023

Source:			Mavenir

Title:				On disabling HARQ feedback for IOT-NTN

Agenda Item:		9.9.3

Document for:		Discussion

# Introduction

The signalling of HARQ feedback enabling/disabling is still open for further discussion. Below working assumption is from RAN 112 [1]:

Furthermore, the followings are two other topics for further discussion:

In this contribution, we will present our view on these topics.

# Discussion

## HARQ ACK enabling/disabling indication

Signalling of HARQ ACK enabling/disabling was a topic with divergent views from different companies in the last few RAN1 meetings. The “working assumption” presented in the previous section is essentially a compromise between RRC-based configuration and DCI-based configuration.

Proposal 1: We agree with the working assumption.

The whole purpose of having additional DCI based signalling is fast enabling of DCI for particular transmissions over HARQ ACK disabled HARQ process. In other words, the major operating cases are: 1) HARQ ACK is generally disabled by RRC signalling with occasional HARQ ACK for special packets using DCI indication (e.g., MAC CE requiring ACK-based activation) , 2) HARQ ACK is enabled (No use-case for occasional disabling identified). Accordingly, we have the following observations on different choices regarding FFS #1:

Observations 1: FFS #1-Option 3 is applicable to both HARQ ACK enabled and disabled HARQ processes: In this case, RRC based enabling/disabling and RRC signalling to enable option 3 serve as two parallel mechanism for one purpose. In other words, while we can prioritize one over another, in this scenario HARQ ACK operation is configured using two different methods.

Observation 2: FFS #1-Option 3 is applicable to HARQ ACK disabled HARQ processes: This option makes more sense, as by configuring Option 3 for HARQ ACK disabled HARQ process, we configure UE for quick on-demand change of feedback state (enabling/disabling) using DCI.

Observation 3: FFS #1-Option 3 is applicable to HARQ ACK enabled HARQ processes: This option allows to enable/disable HARQ ACK feedback for HARQ ACK enabled HARQ process. We do not see much use-cases for this option.

Proposal 2: For FFS #1, we support having DCI-based overriding mechanism applicable to HARQ feedback disabled HARQ process.

Proposal 3: We are open to support “Option 3 is applicable to both HARQ ACK enabled and disabled HARQ processes”, if there is a use-case for on-demand disabling of HARQ ACK feedback for HARQ ACK enabled HARQ process.

## Re-interpreting existing fields in DCI for HARQ ACK indication

Recall the following pseudocode from the email discussion.

Let’s assume HARQ ACK is disabled by RRC signalling (downlinkHARQ-FeedbackDisabled is configured). Furthermore, dlHARQ-FeedbackDisabled-dci is also configured. In this case, it is possible that HARQ ACK is enabled for particular transmission using DCI (implicit or explicit indication). However, this indication itself in DCI needs to be done using existing fields or a new dedicated field. Re-interpreting the existing fields in DCI for HARQ ACK enabling/disabling needs further study. Because, those fields might be needed to serve their original purpose if HARQ ACK is enabled for particular transmission. For example, HARQ ACK resource fields are needed in case DCI enables HARQ ACK for a particular transmission when both downlinkHARQ-FeedbackDisabled and dlHARQ-FeedbackDisabled-dci are configured. This concept is illustrated in the figure below.

Observation 4: Using ACK resource bits in DCI for DCI indication of HARQ ACK enabling/disabling might result in conflict in the UE UL/DL operation.

The above issue can be avoided if we use NDI bit for the HARQ enabling/disabling indication, along with the assumption that no HARQ retransmission happening for RRC HARQ-feedback-disabled HARQ process. Under this scenario, NDI bit of DCI has no use-cases for transmissions that are done using an RRC HARQ-feedback-disabled HARQ process. Hence, NDI bit can be safely used for on-demand enabling of HARQ ACK for a particular transmission.

Proposal 4: NDI bit can be used for DCI indication of HARQ ACK enabling/disabling for an RRC-disabled HARQ ACK feedback.

# Conclusion

Our proposals regarding HARQ-ACK feedback indication are summarized as follows:

Proposal 1: We agree with the working assumption.

Observations 1: FFS #1-Option 3 is applicable to both HARQ ACK enabled and disabled HARQ processes: In this case, RRC based enabling/disabling and RRC signalling to enable option 3 serve as two parallel mechanism for one purpose. In other words, while we can prioritize one over another, in this scenario HARQ ACK operation is configured using two different IEs.

Observation 2: FFS #1-Option 3 is applicable to HARQ ACK disabled HARQ processes: This option makes more sense, as by configuring Option 3 for HARQ ACK disabled HARQ process, we configure UE for quick on-demand change of feedback state (enabling/disabling) using DCI.

Observation 3: FFS #1-Option 3 is applicable to HARQ ACK enabled HARQ processes: This option allows to enable/disable HARQ ACK feedback for HARQ ACK enabled HARQ process. We do not see much use-cases for this option.

Proposal 2: For FFS #1, we support having DCI-based overriding mechanism applicable to HARQ feedback disabled HARQ process.

Proposal 3: We are open to support “Option 3 is applicable to both HARQ ACK enabled and disabled HARQ processes”, if there is a use-case for on-demand disabling of HARQ ACK feedback for HARQ ACK enabled HARQ process.

Observation 4: Using ACK resource bits in DCI for DCI indication of HARQ ACK enabling/disabling might result in conflict in the UE UL/DL operation.

Proposal 4: NDI bit can be used for DCI indication of HARQ ACK enabling/disabling for an RRC-disabled HARQ ACK feedback.

# References

| [1] | 3GPP RAN1, "R1-2302067: Session notes for 9.11 NTN (Non-Terrestrial Networks) enhancements," 2023. |
| --- | --- |