3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303424

# Online, 17 – 26 April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to sps-ConfigDeactivationStateList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#112, RAN1 agreed that multicast DCI cannot be used for - joint SPS unicast and SPS multicast release - joint SPS unicast release - FFS: joint SPS multicast release Concerning FFS, it is tricky to configure a same HARQ process number for a same set of multicast SPS configurations for different UEs in the same multicast group. Moreover, how frequently synchronous end of multiple SPS transmissions for different multicast services would occur is not clear. Thus, it is proposed that sps-ConfigDeactivationStateList is not applied to multicast SPS release. The current text in 38.213 can be misleading if one or more multicast SPS configurations are configured/activated and sps-ConfigDeactivationStateList is configured for one or more unicast SPS releases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 10.2, - It is clarified that multicast DCI format 4_1 and 4_2 cannot be used for 1) joint SPS unicast and SPS multicast release, 2) joint SPS unicast release, and 3) joint SPS multicast release. Thus, multicast DCI format 4_1 and 4_2 can indicate a release for a SPS PDSCH configuration only with a same value as provided by sps-ConfigIndex (i.e. sps-ConfigDeactivationStateList is not applicable for SPS release by multicast DCI format 4_1 or 4_2). - It is clarified that unicast DCI format cannot release multicast SPS configuration(s) by sps-ConfigDeactivationStateList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the network releases multicast SPS configuration, UE may not release multicast SPS configuration. When the network does not release multicast SPS configuration, UE may release multicast SPS configuration. Therefore, misalignment of release status of multicast SPS configuration(s) between the network and the UE can occur unintentionally. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.2	PDCCH validation for DL SPS and UL grant Type 2

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI or a G-CS-RNTI provided by g-cs-RNTI, and

-	the new data indicator field in the DCI format for the enabled transport block is set to '0', and

-	the DFI flag field, if present, in the DCI format is set to '0', and

-	the time domain resource assignment field in the DCI format indicates a row with single SLIV, and

-	if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK-r16.

If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3.

If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH

-	if the UE is provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format except DCI format 4_1 and 4_2 indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations, except multicast SPS PDSCH configurations

-	if the UE is not provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList or if a SPS PDSCH release is indicated by DCI format 4_1 or 4_2, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-4.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

< End >