3GPP TSG RAN WG1 Meeting #112bis	-e                           R1-2303425

E-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.6.1

Source: 	LG Electronics

Title: 	Discussion on further UE complexity reduction for eRedCap

Document for:	Discussion and decision

- Introduction

In RAN1#112 meeting, the Re1-18 WI titled “Enhanced support of reduced capability NR devices” was discussed and the following agreements were made [1].

Initial BWP Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell” Number of PRBs Agreement: For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number Paging bandwidth Agreement: Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. RAR bandwidth and Msg3 transmission time Agreement: For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH. For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs MsgA bandwidth Agreement: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Msg4 bandwidth Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Simultaneous reception Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR. Peak data Reduction Agreement: Revise the earlier agreement by removing the square brackets like this: The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. The same value for X is used for DL and UL Agreement: For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. FFS: the value of X

- Discussion

In this contribution, we present our views on further UE complexity reduction for Rel-18 Redcap which is called eRedCap in this paper.

  - UE BB Scheduled (Transmission) Bandwidth Reduction

This chapter will deal with issues related to bandwidth reduction and mainly includes a lot of things that occur on the RACH procedure.

### Issue#1: UE minimum time relaxation in RACH procedure

At the last meeting, it was agreed that when the number of PRBs for PDSCH in Message 2 is scheduled more than the max number of PRBs for eRedCap (25 PRBs in 15 KHz SCS and 11 or 12 PRBs in 30 KHz SCS), additional time relaxation of X(X ≠ 0) is set as a single value pair depending on SCS (15KHz or 30KHz). It may be important to choose one value for X out of the three candidate values [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS as copied below.

For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS Note: Single Value pair for X is to selected for SCSs

The purpose of the eRedCap is to reduce the cost compared to the Rel-17 RedCap, and in addition to reducing the memory size, it can be said that relaxation on the UE processing time is another effective way of cost reduction. Considering that manufacturing modem solutions (modem chip platforms) can be different in DSP (digital signal processing) technology from companies to companies, it may be safer to set the UE processing time based on the views from companies potentially with some margin.

In the last meeting, many companies proposed various values on Xs from 0.5 slot to 3 slots. Each company may be different and various technologies to make baseband solutions in the respect of implementation. Among the proposed 3 candidate values for X, 2/1 ms for 15/30kHz SCS seems to be more accommodating than the other aggressive values considering the perspectives of many companies designing modem solutions. And, it is thought that the largest value of 3 candidates as much as possible here will help it operate stably in order to accommodate the unexpected worst cases. Above all, an additional relaxation time X ms can be considered, when it may possibly occur that RAR decoding of Message 2 is not completely completed before PUSCH Message 3 is transmitted at the time scheduled in Message 2 by gNodeB. In this case, since the terminal cannot transmit the PUSCH message 3 earlier than it, the UE may consider retransmission of the Message 1 again. It can be possible to respond quickly to RACH failure through restarting RACH Procedure with Message 1 retransmission from the beginning

Proposal 1: For the “FFS: value(s) of X”, X = 2/1 ms for 15/30kHz SCS is supported.

### Issue#2: 2 step (Type-2) RACH procedure

Similar to 4-step RACH procedure, on 2-step RACH procedure, there is definition on the minimum time N T, 1 + 0.5 ms between the DCI (PDCCH based on MsgB-RNTI) received for Message B PDSCH transmission and ACK/NACK PUCCH transmission for that PDSCH of Message B scheduled by PDCCH based MsgB-RNTI. Two types of RARs can be transmitted in Message B unlike Message 2. One is FallbackRAR and the other is SuccessRAR. If the number of PRBs of the scheduled PDSCH is more than the max number of PRB for eRedCap, an additional time [Y] in the minimum time N T, 1 + 0.5 + [Y] ms can be required as in the 4-step PRACH procedure. Generally, the PDU size of SuccessRAR is larger than that of UL Grant in Message 2, so it is likely that the PDSCH of Message B will be more possibly scheduled beyond the max number of PRBs for eRedCap than that of Message 2. The value [Y] can have the same value one as the value X(x) in 4-step PRACH procedure. (X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS, Note: Single Value pair for X is to selected for SCSs).

Table 1: △ Values responding to SCS.

The transmission of the ACK PUCCH for the SuccessRAR in Message B must be transmitted at the time of n + k + △ on 2-setp RACH procedure through the appropriate HARQ feedback timing (one k from {1, 2, 3, 4, 5, 6, 7, 8}) in Figure 3. And the transmission of Message 3 PUSCH for the FallbackRAR in Message B must also be transmitted with the appropriate TDRA index in Table 2 at the time n + k + △ to prevent the failure of the PUSH transmission after the reception for DCI scrambled by MsgB-RNTI. △ has values defined in the Table 1, depending on SCS (μ). However, when Message B is scheduled by PDCCH scrambled by C-RNTI (e.g., in CFRA) which is UE-specific, additional time relaxation [Y] is not needed.

Figure 3. MAC payload for Message B in 2-step PRACH procedure

MsgB-RNTI is different from C-RNTI or TC-RNTI (CS-RNTI, MCS-C-RNTI) which is scheduled for one UE of unicast PDSCH and is almost same with P-RNTI or RA-RNIT which can be scheduled for multiple UEs. Multiple UE’s MAC Control PDU contents can be multiplexed into a Message B. Then the Message B should be treated like Message 2 in the respect of RACH procedure at the initial access periods.

Proposal 2: Message B in 2 step RACH procedure should be treated in the same way as for Message 2 in 4 step RACH procedure rather than Message 4.

### Issue#3: Minimum time for Retransmission in RACH failure

If there is no MAC PDU content as a RACH preamble response in Message B or Message 2, UE will retransmit Messag1 or Message A after the minimum time between the DCI reception and retransmission. Actually, a UE expects MAC PDU corresponding to RACH preamble to send Message 3 in 4-step PRACH procedure or ACK PUCCH (NACK transmission is not necessary about Message 4 and Message B) for Message B in 2-step PRACH procedure after receiving the PDSCH for Message 2 scheduled by RA-RNTI or Message B scheduled by MsgB-RNTI. The time N T,1 + 0.75 ms was defined as the minimum time in the legacy UEs. If the PDSCH for Message 2 or Message B is allocated more than 5MHz BW PRBs like the previous Issue#1 and Issue#2, It is commonly thought that the processing time to be required for decoding Message 2 or Message B although the content related to the RACH preamble is not included or CRC error happen in the message. So an additional relaxation time [Z] can be applied as the equation N T,1 + 0.75 + [Z] ms for this case.

Proposal 3: Additional Time Z(s) in the minimum time should be “also” considered for the retransmission of Message 1 or Message A after decoding Message B or Message 2.

- The value [Z] can be different or same in each case for Message B or Message 2.

### Issue#4: Message 4 bandwidth in 4 step RACH

At the previous meeting, the working assumption was made on the number of Message 4 PDSCH resource allocation for eRedCap. The working assumption can be agreed in this e-meeting. We can’t find the reason why Message 4 is handled differently from Message 3 or unicast PDSCH scheduled by C-RNTI. Message 4 is not multiplexed with multiple UEs like Message 2 or Message B and the MAC PDU information of Message 4 is not large and constrained with limited information. It is thought that PDSCH scheduled by C-RNTI or TC-RNTI (Temporary C-RNTI) is regarded as unicast PDSCH for one specific UE. Therefore, Message 4 does not need to be allocated beyond the number of 5MHz BW PRBs regardless of being scheduled by PDCCH scrambled with TC-RNTI or C-RNTI.

Proposal 4:  Confirm the working assumption on Message 4 bandwidth without further clarification on UE behaviour.

### Issue#5: Simultaneous reception of one unicast PDSCH + one broadcast PDSCH

Regarding the reception of two PDSCHs in one slot, the following has been concluded at the last meeting as follows below. If two Common PDSCH channels partially overlap in some OFDM symbol within a slot, no additional relaxation is made in UE.

Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

Then, discussions remain at this meeting on whether to relax the requirements on simultaneous reception when one of the two PDSCH channels is unicast PDSCH. In this case, as there is neither a requirement for eRedCap to receive the broadcast PDSCH at all times, nor a strict requirement for the decoding time allowed for the broadcast PDSCH, eRedCap UEs may process the uncast PDSCH first, and then the broadcast PDSCH later based on UE implementation. Therefore, we think there is no need to relax the requirements and no need for spec update for this case.

Proposal 5: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of one unicast and one broadcast (SIB1/OSI/paging/RAR) PDSCH transmissions.

- No spec change is needed

### Issue#6: Separate Early indication in Message 1 for eRedCap

Decision has not been made yet on whether early Indication in Message 1 for an eRedCap UE is introduced or not. First of all, because of many delays (additional time: X(s), [Y(s)], [Z(s)] …...) on PRACH procedure to be caused by the minimum time, early Indication seems to be necessary for a gNodeB to signal an enough transmission time to an eRedCap UE for PUCCH or Message 3. If early Indication in Message 1 for eRedCap is introduced for eRedCap, a gNodeB can signal the appropriate timing information to an eRedCap UE against failure of Message 3 PUSCH transmission or ACK PUCCH transmission for SuccessRAR included in Message B. Without early Indication for eRedCap, a gNodeB may have to decode the uplink twice in different two time positions or signal longest timing information (K2 or K1) to an eRedCap with the consideration of worst cases. So the PRACH procedure can be always delayed more because a gNodeB does not know whether a UE is an eRedCap UE or not. In conclusion, early Indication for eRedCap can resolve the issues.

On the other hand, the additional introduction of early indication for eRedCap may trigger to complicate the operation of a gNodeB in the condition for co-existence with R17 UEs, because many R17 features (SDT, CE, and Rel-17 Redcap) that can already require many PRACH preamble/RO partitioning. Early indication in RACH preamble for eRedCap has both sides which are negative or positive on time relaxation and complication. But it is thought that the positive outweighs the negative and the complication of increasing Early indications on different features (SDT, CE, Rel-17 RedCap and so on) can be resolved by assigning more RACH resources on time and frequency. In addition, since Red-18 eRedCap can have two features which are BW3/PR3+PR1 and PR1+20MHz BW, the introduction on Early indication in Message 1 for eRedCap can help distinguish eRedCap features from RedCap or non-RedCap at an early stage. It is should be considered in complicated situations with UEs on different types or different features throughout 3GPP releases in NR.

Proposal 6: Separate Early indication in Message 1 for eRedCap should be introduced in consideration of X in RACH procedure.

- Early Indication in Message 1 for eRedCap is supported by network configuration.

Proposal 7: Early indication for PR1+20MHz eRedCap can be distinguished from Early indication for BW3/PR3+PR1 eRedCap.

  - UE Peak Data Reduction (Max T-put limitation)

This chapter will deal with issues related to Maximum data rate reduction and mainly includes X and Y Value and later considerations related to each feature for two features.

### Issue#1: X (= Either 3.0 or 3.2) value for PR1 as an add-on feature

For an eRedCap UE, the peak data rate was determined to be 10 Mbps in the 3GPP RAN1#112 meeting. Then, the X value referred to TS38.306 needs to be changed to reach the peak data 10 Mbps compared to the existing X Value. Therefore, we need to select the appropriate common X value from different candidates on all cases of SCS and the max number of PRBs for eRedCap to achieve 10 Mbps in Table 1. The candidates were 3.0 or 3.2 in the last meeting and it was agreed that one of two should be chosen in this meeting. X=3.2 was mainly supported by a lot of companies in the previous 3GPP RAN meeting.

| SCS(kHz) | 30 | 15 |
| --- | --- | --- |
| X values | BW(PRB) | BW(PRB) |
|  | 12 | 25 |
| 2.9 | 9.31 | 9.70 |
| 3.0 | 9.63 | 10.03 |
| 3.1 | 9.95 | 10.37 |
| 3.2 | 10.27 | 10.70 |
| 3.3 | 10.59 | 11.03 |
| 3.4 | 10.91 | 11.37 |

Table 1. The constraint vLayers *Qm *f ≥ (X) in Add-on Feature

The X value may vary according to Scale factor, the SCS, the Layer Number for MIMO and the Modulation order. Previously, the existing X value was not multiple with different values but one, regardless of two SCSs. Therefore, it would be better to select one common value that satisfies two cases of 30 KHz SCS and 15 KHz SCS. For 30 KHz SCS, it has already been decided that the max number of PRBs for eRedCap is 12 PRBs. With the understanding that the peak rate 10Mbs applies to eRedCap regardless of the SCS in use, we think X= 3.2 that can satisfy the 10 Mbps peak rate for both the 12 PRBs of 30 KHz SCS and 25 PRBs of 15 KHz should be supported as the new X value for eRedCap.

Proposal 8: For the relaxed constraint X when the UE peak data rate reduction is supported as an add-on to UE BB bandwidth reduction, X = 3.2 is supported.

### Issue#2: Y value for PR1 as a standalone feature

PR1 as a standalone feature is officially included in eRedCap. Y value for the standalone feature can be decided similarly to for the same reasons as above on X value for PR1 as an add-on feature.

- UE peak data rate reduction is supported at least as an Standalone to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y

Therefore, we need to select the appropriate common Y value from different candidates on all cases of SCS and the max number of PRBs for eRedCap to achieve 10 Mbps in Table 2. Y= 0.75 and 0.7 are around 10Mbps in Downlink and Uplink. For the same reasons as above we think Y= 0.75 that can satisfy both the 12 PRBs of 30 KHz SCS and 25 PRBs of 15 KHz should be supported as the new Y value for eRedCap.

| vlayers∙Qm∙f | 15kHz SCS (Mbps) | 30kHz SCS (Mbps) |
| --- | --- | --- |
| 4 | 56.7 | 54.6 |
| 1 | 14.2 | 13.6 |
| 0.8 | 11.3 | 10.9 |
| 0.75 | 10.6 | 10.2 |
| 0.7 | 9.92 | 9.97 |
| 0.4 | 5.7 | 5.5 |

Table 2. The constraint vLayers *Qm *f ≥ (X) in Standalone Feature

proposal 9: For the relaxed constraint Y when the UE peak data rate reduction is supported as a Standalone, Y = 0.75 is supported.

### Issue#3: Whether/how to differentiate 20MHz+PR1 and BW3/PR3+PR1

In the last RAN Plenary meeting, 20MHz + PR1 was introduced as another eRedCap different from BW3/PR3+PR1. The related agreements in RAN#99 plenary meeting are copied below in the rectangular box.

Rel-18 eRedcap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

According to Note 4, 20MHz BW + PR1 is same as BW3/PR2+PR1 in the initial access procedure. But, it seems to be ambiguous whether the Early Indication and the separate initial BWP for 20MHz BW + PR1 can be different from those for BW3/PR3+PR1 during the initial access procedure. For example, it can be discussed whether the 20MHz BW + PR1 eRedCap UE should always be operated in the same initial BWP same as BW3/PR3 + PR1 eRedCap UE or can be operated in the different initial BWP by RRC signaling, and whether Early Indication should be always shared with BW3/PR3+PR1 or another Early Indication can be configured for 20MHz BW + PR1 if Early Indication for eRedCap is introduced. So, the detailed operation for 20MHz BW + PR1 needs to be treated in the initial access procedure.

Proposal 10: Discuss whether the Early Indication and the separate initial BWP for 20MHz BW + PR1 can be different from those for BW3/PR3+PR1 during initial access procedure.

  - Other aspect

[Consideration on FDRA Optimization]

According to agreements in RAN1#111 meeting, the maximum number of unicast PRBs was 25 PRBs in 15KHz SCS and 11 or 12 PRBs in 30 KHz SCS (The exact number of PRBs in 30KHz SCS has not yet been decided). It means that the PRB allocation exceeding the max number of PRBs for eRedCap (5MHz PRBs) must not be allowed to an eRedCap UE for PDSCH or PUSCH scheduled C-RNTI.

Figure 1. The allocation on RBG-size 16 within 25 PRBs in 15KHz SCS

This constraint may cause an unexpected issue in the assignment of the frequency resource (PRBs) to an eRedCap UE. In the case of RA Type 0 (bitmap) or Compact RA Type 1 (RIV) in DCI X_2 based on consecutive multiple PRBs which is called RBG (Resource Block Group), more than the max of the number of PRBs for eRedCap (25 PRBs in 15 KHz SCS and 11 or 12 PRBs in 30 KHz) may be allocated depending on the situation. For instance, if RBG-size is 16 as shown in Figure 1 in 15KHz SCS and FDRA must be composed of multiples of a RBG-size (16 PRBs) for an eRedCap UE, it may not be approximately easy to reach around 25 PRBs for UE peak rate which is the max number of PRBs for eRedCap allocated to PDSCH or PUSCH PRBs in 15KHz SCS because of the restriction that 16 PRB (one RBG) must be assigned the only one RBG to an eRedCap UE although the data rate requires 25 PRBs. It may be impossible to achieve the peak data rate performance with 16 PRB (one RBG). Nevertheless, it cannot be achieved with 32 PRBs (2 REGs with 32 PRBs composed of two consecutive 16 PRBs) which violates the agreement of the FDRA allocation within 25 PRBs contiguously or not to an eRedCap UE. The similar issue could be viewed in the 30 KHz SCS with 8 RBG-size which consists of 8 consecutive multiple PRBs like Figure 2 below. It is difficult to reach around 11 or 12 PRBs with 8 REG-size in 30 KHz SCS for the peak data rate.

Figure 2. The allocation on RBG-size 8 within 11 or 12 PRBs in 30KHz SCS

Proposal 11: In the case where the RBG size for FDRA is large (e.g., 16 RBs in 15 KHz SCS, 8 RBs in 30 KHz SCS), consider enhancements to increase the efficiency in terms of DL/UL frequency resource utilization.

Message 3 PUSCH repetition was introduced in Rel-17 CE. The repetition was implemented with limited flexibility by signaling the repetition number in RAR UL Grant with 2 bits taken from the 4-bit MCS field. Now, for eRedCap, 5MHz UE BB bandwidth for PDSCH/PUSCH is being considered, in which case 9 bits out of the 14-bit FDRA field in RAR UL Grant may be sufficient for scheduling Message 3 PUSCH. Then, the spare bits from the FDRA field may be used to support other functionalities such as supporting additional repetition numbers for CE, enhancements on Message 3 PUSCH transmission/reception for coverage extension, etc.

Proposal 12: Discuss whether/how to reuse the potential spare bits from the FDRA field in RAR UL Grant for eRedCap UEs with 5 MHz UE BB bandwidth for PDSCH/PUSCH.

[Random Access Procedure]

During random access, common PUCCH resources are used for PUCCH transmission for HARQ feedback in response to Message 4/Message B. For the common PUCCH resources for eRedCap UEs, basically the same framework for common PUCCH for Rel-17 RedCap can be reused:

- In case a separate initial UL BWP is configured for eRedCap UEs, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Message 4/Message B for eRedCap UEs.

  - The frequency hopping is enabled/disabled via SIB1.

  - Disabling of frequency hopping for common PUCCH resources for eRedCap UEs is only supported for separate (not legacy) initial UL BWP.

With the introduction of eRedCap UEs in addition to Rel-17 RedCap UEs, increasing the resource efficiency of the common PUCCH resources becomes more important. Then, it is desirable that the common PUCCH resources in time and frequency domain are shared as much as possible b/w Rel-17 RedCap UEs and eRedCap UEs especially when the FH for common PUCCH transmission is disabled. From this aspect, enhancements on the user multiplexing capacity for common PUCCH can be considered. CDM (OCC), FDM, and TDM can be considered as candidate multiplexing techniques.

Proposal 13: Support enhancements on the user multiplexing capacity for common PUCCH especially when the FH for the common PUCCH resources is disabled.

    - FFS details, e.g., CDM (OCC), FDM, and TDM of the common PUCCH resources.

- Conclusion

In this contribution, we shared our views on further UE complexity reduction for eRedCap.

Proposal 1: For the “FFS: value(s) of X”, X = 2/1 ms for 15/30kHz SCS is supported.

Proposal 2: Message B in 2 step RACH procedure should be treated in the same way as for Message 2 in 4 step RACH procedure rather than Message 4.

Proposal 3: Additional Time Z(s) in the minimum time should be “also” considered for the retransmission of Message 1 or Message A after decoding Message B or Message 2.

- The value [Z] can be different or same in each case for Message B or Message 2.

Proposal 4:  Confirm the working assumption on Message 4 bandwidth without further clarification on UE behaviour.

Proposal 5: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of one unicast and one broadcast (SIB1/OSI/paging/RAR) PDSCH transmissions.

- No spec change is needed

Proposal 6: Separate Early indication in Message 1 for eRedCap should be introduced for consideration on X in RACH procedure.

- Early Indication in Message 1 for eRedCap is supported by network configuration.

Proposal 7: Early indication for PR1+20MHz eRedCap can be distinguished from Early indication for BW3/PR3+PR1 eRedCap.

Proposal 8: For the relaxed constraint X when the UE peak data rate reduction is supported as an add-on to UE BB bandwidth reduction, X = 3.2 is supported.

proposal 9: For the relaxed constraint Y when the UE peak data rate reduction is supported as a Standalone, Y = 0.75 is supported.

Proposal 10: Discuss whether the Early Indication and the separate initial BWP for 20MHz BW + PR1 can be different from those for BW3/PR3+PR1 during initial access procedure.

Proposal 11: In the case where the RBG size for FDRA is large (e.g., 16 RBs in 15 KHz SCS, 8 RBs in 30 KHz SCS), consider enhancements to increase the efficiency in terms of DL/UL frequency resource utilization.

Proposal 12: Discuss whether/how to reuse the potential spare bits from the FDRA field in RAR UL Grant for eRedCap UEs with 5 MHz UE BB bandwidth for PDSCH/PUSCH.

Proposal 13: Support enhancements on the user multiplexing capacity for common PUCCH especially when the FH for the common PUCCH resources is disabled.

    - FFS details, e.g., CDM (OCC), FDM, and TDM of the common PUCCH resources.

- Reference

R1-2301885, RAN1 agreements for Rel-18 NR RedCap

RP-230778, Proposal for PR1 in eRedCap