3GPP TSG RAN WG1 #112bis-e			            R1-2303426

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.7.1

Source: 	LG Electronics

Title: 	Discussion on NES techniques in spatial and power domains

Document for:	Discussion and decision

- Introduction

In RAN#98-e, a new WI [1] of network energy savings for NR was approved, and one of WID objectives is related to techniques in spatial and power domains as shown below.

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

In this contribution, we discuss and provide our views on techniques for network energy savings (NES) in spatial and power domains.

- CSI framework

In RAN1#112 meeting, the following agreement was made to define two types of spatial domain adaptation.

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

During RAN1#112 discussion, it was clarified that two types will not be captured in NR specification but which type of spatial domain adaptation can be applied depends on gNB’s implementation. Therefore, we prefer to consider both types when spatial domain NES techniques is being developed, even though eventually the common CSI framework might be able to cover both of them.

Proposal #1: Both of the following two types are supported for NES spatial domain adaptation.

- Type 1: All antenna elements associated to a logical antenna port are disabled/enabled

- Type 2: Part/subset of antenna elements associated to a logical antenna port are disabled/enabled

For the purpose of NES, gNB can shut down some of spatial elements (e.g. antenna elements, antenna ports, active transceiver chains) and/or lower transmission power. However, without knowledge of the CSI corresponding to the less number of spatial elements and/or reduced power, gNB may not successfully communicate with the associated UEs right after turning off some of spatial elements and/or reducing transmission power, which leads to system performance degradation. Therefore, CSI framework needs to be enhanced so that UE can feed back CSI corresponding to different numbers of CSI-RS antenna ports (APs) and/or power offset values between CSI-RS and PDSCH.

Currently, a single CSI reporting setting (i.e., CSI-ReportConfig IE) has the following limitations:

- A single CSI-RS resource set can be associated with each CMR/IMR, and all CSI-RS resources within the set are restricted to be configured with the same number of APs (except for the NZP CSI-RS resources used for interference measurement). Thus, gNB cannot obtain the CSI corresponding to different numbers of CSI-RS APs for a CSI reporting.

- Power offset value between PDSCH and CSI-RS is configured per CSI-RS resource and a CSI-RS resource set can be composed of more than one CSI-RS resource for which the power offset values can be different. However, for a CSI reporting, UE feeds back the CSI corresponding to a specific CRI out of multiple CSI-RS resources within a CSI-RS resource set (except for m-TRP operation), thus gNB cannot acquire the CSI corresponding to multiple power offset values between PDSCH and CSI-RS.

To overcome above limitations of current CSI framework for NES, RAN1 discussed how to enhance CSI framework and made the following agreements in RAN1#112 meeting.

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns” Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included

From our understanding, A2-1 reuses current CSI framework thus it requires more CSI report configurations compared to A2-2 since independent/separate CSI report configurations should be configured for each spatial adaptation pattern. Thus, between A2-1 and A2-2, A2-1 can be deprioritized.

As detailed methods of sub-configuration in A2-2, A1-1, A1-2, or combination of A1-1 and A1-2 can be considered. To be specific, we suggest the following approaches for efficient adaptation of spatial elements which can be also applicable to adaptation of power offset values between PDSCH and CSI-RS. In addition, it can be considered that the approach applied for NES techniques in spatial domain is different from the approach applied for those in power domain (e.g., Approach 1 for spatial domain and Approach 3 for power domain).

- Approach 1: One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration is associated with one CSI-RS resource set (for CMR) and corresponds to one spatial or power adaptation pattern. Each CSI-RS resource/resource set can be associated with only one spatial or power adaptation pattern.

- Approach 2: One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration is associated with a subset of CSI-RS resources within a CSI-RS resource set and corresponds to one spatial or power adaptation pattern. CSI-RS resource set can be associated with more than one spatial or power adaptation patterns but each CSI-RS resource within the CSI-RS resource set can be associated with only one spatial or power adaptation pattern.

- Approach 3: One CSI report configuration contains multiple CSI report sub-configurations where multiple sub-configurations are associated with a same CSI-RS resource and correspond to different spatial or power adaptation patterns. Each CSI-RS resource can be associated with one or more spatial or power adaptation patterns.

(a)

(b)

(c)

Figure 1. Examples of CSI framework for NES in spatial domain: (a) Approach 1, (b) Approach 2, and (c) Approach 3

Figure 1 depicts examples corresponding to above 3 approaches of CSI framework particularly for spatial domain adaptation. For Approach 1 (as in Figure 1(a)), a CSI report configuration can include CSI-RS resource set #1 with 32-port NZP CSI-RS resources and CSI-RS resource set #2 with 16-port NZP CSI-RS resources (, as CMR or IMR). For Approach 2 (as in Figure 1(b)), a CSI-RS resource set (as CMR or IMR) configured in a CSI report configuration can include NZP CSI-RS resources #1/#2/#3 with 32 APs and NZP CSI-RS resources #3/#4 with 16 APs. For Approach 3 (as in Figure 1(c)), each NZP CSI-RS resource within a CSI-RS resource set (as CMR or IMR) configured for a CSI report configuration is configured with 32 APs and 16 APs.

It is noted that spatial adaptation pattern in the above approaches represents a number of antenna ports for type 1 or a number of enabled antenna elements associated to a logical antenna port for type 2. It is also noted that power adaptation pattern in the above approaches represents a power offset value between PDSCH and CSI-RS.

Proposal #2: To enable efficient adaptation of spatial elements and power offset values between PDSCH and CSI-RS, the following approaches can be taken into account for CSI framework enhancement.

- Approach 1: One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration is associated with one CSI-RS resource set (for CMR) and corresponds to one spatial or power adaptation pattern. Each CSI-RS resource/resource set can be associated with only one spatial or power adaptation pattern (e.g., for a CSI reporting setting, CSI-RS resource set #1 with 32-APs NZP CSI-RS resource(s) and CSI-RS resource set #2 with 16-APs NZP CSI-RS resource(s), as CMR).

- Approach 2: One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration is associated with a subset of CSI-RS resources within a CSI-RS resource set and corresponds to one spatial or power adaptation pattern. CSI-RS resource set can be associated with more than one spatial or power adaptation patterns but each CSI-RS resource within the CSI-RS resource set can be associated with only one spatial or power adaptation pattern (e.g., for a configured CSI-RS resource set in a CSI reporting setting, CSI-RS resources #1/#2 with 32 APs and CSI-RS resources #3/#4 with 16 APs).

- Approach 3: One CSI report configuration contains multiple CSI report sub-configurations where multiple sub-configurations are associated with a same CSI-RS resource but correspond to different spatial or power adaptation patterns. Each CSI-RS resource can be associated with one or more spatial or power adaptation patterns (e.g., a CSI-RS resource within a configured CSI-RS resource set for a CSI reporting setting, is configured with multiple power offset values of {-3, 3} dB, or with 32 and 16 APs).

- Note 1: Spatial adaptation pattern represents a number of antenna ports for type 1 or a number of enabled antenna elements associated to a logical antenna port for type 2.

- Note 2: Power adaptation pattern represents a power offset value between PDSCH and CSI-RS.

For above all approaches, if multiple numbers of APs can be configured for a CSI reporting setting, separate CodebookConfig higher layer parameters need to be configured for each number of APs. For instance, if Type I Single-Panel Codebook is configured and 32-port & 16-port spatial adaptation patterns are configured for a CSI report configuration (as in Figure 1), four-four-TypeI-SinglePanel-Restriction and four-two-TypeI-SinglePanel-Restriction higher layer parameters can be configured for 32-port and 16-port spatial adaptation patterns, respectively.

Proposal #3: For a CSI report configuration that contains multiple CSI report sub-configurations where each sub-configuration corresponds to a different number of antenna ports, independent/separate CodebookConfig higher layer parameters are configured for each sub-configuration.

Furthermore, for Approach 3, if multiple numbers of APs can be associated with a same NZP CSI-RS resource (as in Figure 1(c)), there seem to be additional issues to be resolved.

The first issue is how to signal on/off status for each AP. A simple signaling method could be to use bitmap where ‘1’ and ‘0’ indicates ON and OFF status, respectively, for an antenna port or for multiple antenna ports (corresponding to one or more CDM group, or corresponding to parts of CDM group).

The second issue is how to adjust antenna port mapping considering codebook construction based on APs with ON status.

Figure 2. Example of antenna port mapping for 32-port CSI-RS configured with cdm4-FD2-TD2

Figure 3. Example of antenna port layout for 32-port CSI-RS configured with (N1,N2)=(4,4)

Figure 2 depicts an example of AP mapping for 32-port CSI-RS with:

- cdm4-FD2-TD2

- firstOFDMSymbolInTimeDomain configured as 3

- firstOFDMSymbolInTimeDomain2 configured as 10

- frequencyDomainAllocation configured as 110011

Figure 3 depicts an example of AP layout for 32-port CSI-RS configured with (N1,N2)=(4,4). For instance, 16-port NZP CSI-RS can be used for CSI report sub-configuration by muting CDM group #2/3/6/7, as shown in Figure 4(a). Figure 4(b) depicts an antenna port layout for codebook construction which corresponds to (N1,N2)=(2,4). However, it should be noted that this (N1,N2) =(2,4) combination is not supported in current NR specification. To resolve this problem, the corresponding antenna port layout can be changed to (N1,N2)=(4,2) by re-indexing AP in ascending order or by rotating them 90 degrees.

(a)                                        (b)

Figure 4. Example of 16-port CSI-RS which is a subset of 32-port CSI-RS: (a) antenna port mapping and (b) antenna port layout

In addition, we may consider to change AP mapping order such that the remaining AP indexes can be well-aligned with current codebook structure (e.g., frequency first, time second, CDM group third AP indexing rather than CDM group first, frequency second, time third AP indexing).

Proposal #4: For a CSI report configuration that contains multiple CSI report sub-configurations where multiple sub-configurations are associated with a same CSI-RS resource but correspond to different number of antenna ports, discuss at least following issues.

- How to signal ON/OFF status for each antenna port

- How to adjust antenna port mapping considering codebook construction based on antenna ports with ON status

- CSI measurement and report

Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account. Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included

In RAN1#112 meeting, it was agreed to further study enhancements that report multiple CSIs corresponding to the spatial adaptation pattern for each CSI. It was also agreed to discuss how to configure and indicate multiple power offset values to UE to adapt power offset values between PDSCH and CSI-RS, and how to measurement/reporting. In order to report multiple CSIs corresponding to the multiple spatial adaptation patterns or multiple power offset values to one CSI report, it is necessary to consider the CSI calculation complexity of the UE and the overhead reduction method of the CSI feedback.

From the gNB's point of view, it may be the best to perform spatial and power domain adaptation by receiving as much CSI information on the spatial or power adaptation patterns as possible, but from the UE's point of view, it is too burdensome to calculate and report CSI for all configured spatial or power adaptation patterns. Therefore, in order to reduce the amount of CSI information to be calculated and reported, a method of reducing the candidate number of spatial or power adaptation pattern is needed. In one way, the UE can directly select CSI corresponding to the spatial or power adaptation pattern to be calculated and reported on its own, which is a risk that it is not suitable in terms of the operation of the gNB.

Therefore, it is possible to consider a method in which the gNB indicates the CSI corresponding to the spatial or power adaptation pattern to be calculated and reported directly to the UE. For example, gNB can indicate the index of CSI-RS resource set (e.g., for Approach 1 in Section 2), or the group index of CSI-RS resources (e.g., if CSI-RS resources with the same number of APs are grouped for Approach 2 in Section 2), or the number of APs and/or the AP ON/OFF information, offset or absolute value for power offset (Approach 3 in Section 2) to obtain CSI information corresponding to the desired spatial or power adaptation pattern. This method can reduce the computational complexity of the UE and the overhead of the CSI report compared to calculating and reporting CSI for all configured spatial or power adaptation patterns.

In another way, it is possible to configure a specific condition/threshold (e.g., target CQI) to the UE in advance and calculate and report only multiple CSI information corresponding to multiple spatial or power adaptation patterns only when the condition/threshold is satisfied. This method can reduce the signalling overhead of the gNB and provide the UE the option to select CSI information to report, but the computational complexity can be increased to find spatial or power adaptation patterns that satisfy the condition/threshold.

Proposal #5: The following options can be considered to reduce the UE's CSI computational complexity and to find the optimal spatial or power adaptation patterns from the gNB perspective.

  - Option 1: gNB indicates the spatial or power adaptation pattern(s) to be calculated and reported, and UE calculates and reports CSI information corresponding to indicated spatial or power adaptation pattern(s).

  - Option 2: gNB configures a specific condition/threshold (e.g., target CQI) to the UE in advance and UE calculates and reports only multiple CSI information corresponding to multiple spatial or power adaptation patterns only when the condition/threshold is satisfied.

Meanwhile, after determining the spatial or power adaptation patterns to be calculated and reported, it is also necessary to consider an overhead reduction method particularly for the case of transmitting the corresponding multiple CSIs in one CSI report. If there is a CSI content (e.g., RI) that remains unchanged across adaptation patterns, the CSI content (e.g. RI) may be included only once without including the CSI content for each adaptation pattern, and the remaining CSI contents for each adaptation pattern can be contained. At this time, the CSI report may be configured with a differential value compared to a specific baseline/reference CSI value to further reduce the overhead. For example, since CSI calculated for different power offset values may be composed of the same RI/PMI information and only different CQI values, RI/PMI information can be included only once across the power offset values, and the CQI values corresponding to each power offset value can be included individually in CSI report. In addition, the CQI value can be reported as a differential value by using the CQI value of a specific spatial or power adaptation pattern as a baseline/reference.

In another way, based on the condition/threshold (e.g., CQI index) configured by the gNB, both wideband CSI and sub-band CSI information are included in the CSI report for the best performance (e.g., highest CQI) spatial or power adaptation patterns, and only wideband CSI information are included in the CSI report for other adaptation patterns. In addition, for the purpose of preventing UCI payload size mismatch with the UE or informing the adaptation pattern selected by the UE, the UE can report which spatial or power adaptation pattern(s) is contained/selected in the CSI report.

Proposal #6: When multiple CSIs corresponding to multiple spatial or power adaptation patterns are reported as one CSI report, the following options can be considered to reduce feedback overhead.

  - Option A: If there is a CSI content (e.g., RI) that remains unchanged across adaptation patterns, the CSI content (e.g. RI) may be included only once without including the CSI content for each adaptation pattern, and the remaining CSI contents for each adaptation pattern can be contained.

  - Option B: Based on the condition/threshold (e.g., CQI index) configured by the gNB, both wideband CSI and sub-band CSI information are included in the CSI report for the best performance (e.g., highest CQI) spatial or power adaptation pattern(s), and only wideband CSI information are included in the CSI report for other adaptation patterns.

In addition, the following issues need to be considered to report one or more CSIs in one CSI report.

- Composition of CSI parts 1 and 2

- CSI omission and priority rule

- CSI processing unit (CPU) occupation and computation time

- Application delay for spatial/power adaptation pattern change

For CSI part 1, CSI report #n can be composed of CRI + RI + LI + PMI + CQI for 1st TB according to Table 6.3.1.1.2-7 in TS 38.212. If multiple CSI information corresponding to multiple spatial or power adaptation patterns is included in one CSI report, the mapping order should be aligned with the gNB in advance of which adaptation pattern the CSI information in the CSI report corresponds, or the UE needs to indicate this information directly.

For example, when configuring CSI report for different power offset values configured in one CSI-RS resource, the following options can be considered.

1) CRI + RI#1 + LI#1 + PMI#1 + CQI#1 for 1st TB + RI#2 + LI#2 + PMI#2 + CQI#2 for 1st TB

2) CRI + RI#1 + RI#2 + LI#1 + LI#2 + PMI#1 + PMI#2 + CQI#1 for 1st TB + CQI#2 for 1st TB

For CSI part 2, it may be necessary to define priority reporting levels between multiple CSI information corresponding to multiple spatial or power adaptation patterns. For example, Part 2 sub-band CSI of even sub-band for a specific spatial or power adaptation pattern can be prioritized over other spatial or power adaptation patterns. Once the priority reporting level is configured for each spatial or power adaptation pattern, if the amount of resources allocated to the CSI report is less than the payload size of the multiple CSI to be reported, it can be configured to drop from Part 2 CSI information corresponding to the lower priority according to the pre-configured rule.

Regarding priority rule, if the Priority rule (e.g., PriiCSIy,k,c,s defined in Section 5.2.5 of TS 38.214) is configured in advance for each spatial or power adaptation pattern, when collisions occur in the time-domain between CSI reports (if there is overlap even one symbol), only a CSI report containing CSI information corresponding to a high priority spatial or power adaptation pattern is transmitted, and a low CSI report can be dropped.

Regarding CPU occupation, as part/subset of CSI-RS resources configured for a CSI reporting setting can be used (especially for Approach 1 or 2), the number of CSI-RS resources corresponding to spatial/power adaptation pattern(s) to be measured can be considered. Or, multiple hypothesis on the number of APs (or power offset values) for a single CSI-RS resource (especially for Approach 3) can be used, CPU occupation for this case can be greater than the number of CSI-RS resources configured in the CSI report configuration.

- Beam management

As described in WID, gNB’s adaptation of spatial elements can necessitate the enhancement of beam management (BM). When UE performs BM-related measurements such as L1-RSRP and L1-SINR, it is up to UE’s implementation whether or not to search and change UE’s RX beam each time it receives each CSI-RS for BM. However, if gNB will turn on or off spatial elements for NES, UE may need to adjust the RX beam to a specific CSI-RS for BM which could be suitable for the on/off status of gNB’s spatial elements in the future. For instance, gNB can inform a CSI-RS resource (set) index to UE, then UE is enforced to adjust the RX beam when receiving the corresponding CSI-RS resource (set), which could require processing time relaxation for beam reporting.

CSI-RS for BM can be configured with up to 2 APs. gNB’s spatial element on/off may lead to the change of the number of APs for CSI-RS for BM (or the change of the number of antenna elements associated with an AP). To handle this situation, enhancements for CSI framework and reporting (proposed in Sections 2 and 3, respectively) can be also applied to BM case. For instance, in a CSI-ReportConfig configuration, CSI-RS resource set #1 with 2-APs CSI-RS resource(s) and CSI-RS resource set #2 with 1-AP CSI-RS resource(s) can be configured (as in Approach 1) and one of two CSI-RS resource sets can be activated based on gNB’s indication (as in Option 1). It should be noted that similar methods can be applicable to radio link monitoring and link recovery procedures as well (e.g., 2 candidate sets for RLM/BFD/candidate beam RS where one set is configured with 2-APs CSI-RS resources while the other set is configured with 1-AP CSI-RS resources).

In RAN1#112 meeting [2], there was a discussion that when CSI-RS TX beam becomes wider due to the reduced number of spatial elements at gNB, a smaller number of CSI-RS TX repetitions is needed for UE RX beam sweeping (i.e., for a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'on'). However, it should be noted that the same principle can be applicable to gNB TX beam sweeping (i.e., for a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'off'). Therefore, considering wider beam width for CSI-RS transmission due to the reduced number of spatial elements at gNB, the number of repetitions can be adjusted for CSI-RS for UE RX beam sweeping as well as gNB TX beam sweeping.

Proposal #7: Consider at least the following issues for beam management enhancement.

- How to inform UE to adjust the RX beam when receiving a specific CSI-RS for beam management

- How to handle the case where CSI-RS configured for beam management, radio link monitoring or link recovery procedures is affected by gNB’s adaptation of spatial elements

- How to adjust the number of repetitions for a CSI-RS resource with the higher layer parameter repetition set to 'on' or 'off'

- TCI configuration

For a TCI state (or TCI UL state) configuration, one or two DL RSs (i.e., SSB or CSI-RS) are configured as reference signal for QCL information. Then, a TCI state index can be configured/indicated for the reception of PDCCH, PDSCH or CSI-RS. For UL transmission such as SRS or PUCCH, CSI-RS can be configured as reference signal for spatial relation information. Therefore, if a CSI-RS that is configured as reference signal for QCL information or for spatial relation information, can be affected by gNB’s adaptation of spatial elements (e.g., gNB may indicate the change of TX power or the number of APs for the CSI-RS via group-common L1/L2 signaling), we may need to discuss how UE performs DL/UL reception/transmission associated with the CSI-RS. To be specific, the following methods can be considered:

- Method 1: Configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on gNB’s adaptation of spatial elements

- Method 2: Configure multiple candidate sets of TCI state(s) associated with DL/UL signal/channel and switch one of them based on gNB’s adaptation of spatial elements (e.g., TCI state #1 and TCI state #2 are configured for CORESET index 1 and one of them is applied for receiving CORESET index 1 based on gNB’s adaptation of spatial elements)

- Method 3: Invalidate DL/UL signal/channel related to a CSI-RS resource that is deactivated or affected due to gNB’s adaptation of spatial elements

It is noted that the above methods are more efficient than current DCI or MAC-CE based QCL information update since those methods don’t require gNB to transmit L1/L2 signaling individually.

Proposal #8: Consider the following methods for TCI configuration enhancement.

- Method 1: Configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling

- Method 2: Configure multiple candidate sets of TCI state(s) associated with DL/UL signal/channel and switch one of them based on L1/L2 signaling

- Method 3: Invalidate DL/UL signal/channel related to a CSI-RS resource that is deactivated or affected due to gNB’s adaptation of spatial elements

- Conclusions

In this contribution, NES techniques in spatial and power domains were discussed, and the followings were proposed.

CSI framework

Proposal #1: Both of the following two types are supported for NES spatial domain adaptation.

- Type 1: All antenna elements associated to a logical antenna port are disabled/enabled

- Type 2: Part/subset of antenna elements associated to a logical antenna port are disabled/enabled

Proposal #2: To enable efficient adaptation of spatial elements and power offset values between PDSCH and CSI-RS, the following approaches can be taken into account for CSI framework enhancement.

- Approach 1: One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration is associated with one CSI-RS resource set (for CMR) and corresponds to one spatial or power adaptation pattern. Each CSI-RS resource/resource set can be associated with only one spatial or power adaptation pattern (e.g., for a CSI reporting setting, CSI-RS resource set #1 with 32-APs NZP CSI-RS resource(s) and CSI-RS resource set #2 with 16-APs NZP CSI-RS resource(s), as CMR).

- Approach 2: One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration is associated with a subset of CSI-RS resources within a CSI-RS resource set and corresponds to one spatial or power adaptation pattern. CSI-RS resource set can be associated with more than one spatial or power adaptation patterns but each CSI-RS resource within the CSI-RS resource set can be associated with only one spatial or power adaptation pattern (e.g., for a configured CSI-RS resource set in a CSI reporting setting, CSI-RS resources #1/#2 with 32 APs and CSI-RS resources #3/#4 with 16 APs).

- Approach 3: One CSI report configuration contains multiple CSI report sub-configurations where multiple sub-configurations are associated with a same CSI-RS resource but correspond to different spatial or power adaptation patterns. Each CSI-RS resource can be associated with one or more spatial or power adaptation patterns (e.g., a CSI-RS resource within a configured CSI-RS resource set for a CSI reporting setting, is configured with multiple power offset values of {-3, 3} dB, or with 32 and 16 APs).

- Note 1: Spatial adaptation pattern represents a number of antenna ports for type 1 or a number of enabled antenna elements associated to a logical antenna port for type 2.

- Note 2: Power adaptation pattern represents a power offset value between PDSCH and CSI-RS.

Proposal #3: For a CSI report configuration that contains multiple CSI report sub-configurations where each sub-configuration corresponds to a different number of antenna ports, independent/separate CodebookConfig higher layer parameters are configured for each sub-configuration.

Proposal #4: For a CSI report configuration that contains multiple CSI report sub-configurations where multiple sub-configurations are associated with a same CSI-RS resource but correspond to different number of antenna ports, discuss at least following issues.

- How to signal ON/OFF status for each antenna port

- How to adjust antenna port mapping considering codebook construction based on antenna ports with ON status

CSI measurement and report

Proposal #5: The following options can be considered to reduce the UE's CSI computational complexity and to find the optimal spatial or power adaptation patterns from the gNB perspective.

  - Option 1: gNB indicates the spatial or power adaptation pattern(s) to be calculated and reported, and UE calculates and reports CSI information corresponding to indicated spatial or power adaptation pattern(s).

  - Option 2: gNB configures a specific condition/threshold (e.g., target CQI) to the UE in advance and UE calculates and reports only multiple CSI information corresponding to multiple spatial or power adaptation patterns only when the condition/threshold is satisfied.

Proposal #6: When multiple CSIs corresponding to multiple spatial or power adaptation patterns are reported as one CSI report, the following options can be considered to reduce feedback overhead.

  - Option A: If there is a CSI content (e.g., RI) that remains unchanged across adaptation patterns, the CSI content (e.g. RI) may be included only once without including the CSI content for each adaptation pattern, and the remaining CSI contents for each adaptation pattern can be contained.

  - Option B: Based on the condition/threshold (e.g., CQI index) configured by the gNB, both wideband CSI and sub-band CSI information are included in the CSI report for the best performance (e.g., highest CQI) spatial or power adaptation pattern(s), and only wideband CSI information are included in the CSI report for other adaptation patterns.

Beam management

Proposal #7: Consider at least the following issues for beam management enhancement.

- How to inform UE to adjust the RX beam when receiving a specific CSI-RS for beam management

- How to handle the case where CSI-RS configured for beam management, radio link monitoring or link recovery procedures is affected by gNB’s adaptation of spatial elements

- How to adjust the number of repetitions for a CSI-RS resource with the higher layer parameter repetition set to 'on' or 'off'

TCI configuration

Proposal #8: Consider the following methods for TCI configuration enhancement.

- Method 1: Configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling

- Method 2: Configure multiple candidate sets of TCI state(s) associated with DL/UL signal/channel and switch one of them based on L1/L2 signaling

- Method 3: Invalidate DL/UL signal/channel related to a CSI-RS resource that is deactivated or affected due to gNB’s adaptation of spatial elements

- References

RP-223540, “New WID: Network energy savings for NR, ” Huawei, RAN#98-e

RAN1#112 chairman’s note