3GPP TSG RAN WG1 #112bis-e		                        R1-2303427

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.7.2

Source: 	LG Electronics

Title: 	Discussion on cell DTX/DRX mechanism

Document for:	Discussion and decision

- Introduction

In RAN#98-e, a new WI [1] of network energy savings for NR was approved, and one of WID objectives is related to cell DTX/DRX mechanism as shown below.

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In this contribution, we discuss and provide our views on cell DTX/DRX mechanism for network energy savings (NES).

- Enhancements to UE C-DRX configuration

UE DRX in RRC_CONNECTED mode (called UE C-DRX hereafter) is introduced to reduce UE’s power consumption by configuring the periodic ON/OFF duration. The UE can save power by entering the sleep state during the OFF duration in which PDCCH needs not to be monitored. The UE needs to wake up periodically in ON duration to monitor PDCCH for receiving/transmitting DL/UL transmission. Configuring UE C-DRX with short ON-duration and long OFF-duration can save more power, whereas scheduling latency for DL/UL transmission is longer, so gNB should properly adjust it through UE C-DRX parameters such as drx-LongCycleStartOffset, drx-SlotOffset, drx-onDurationTimer.

UE C-DRX is also beneficial for reducing gNB’s power consumption because the periodic CSI report or SRS transmission is not allowed during the OFF duration. However, the gNB may not enter the sleep state during the OFF duration because UE is allowed to initiate UL transmission according to the configured resources (e.g., using PUCCH, RACH, SR, or CG-PUSCH) during the OFF duration. In addition, since UE C-DRX is configured in a UE-specific manner, ON/OFF duration of UEs in the cell may not be aligned with each other. For instance, the gNB may perform PDCCH transmission by waking up every unaligned ON duration of the UEs in which the C-DRX is configured, power consumption of the gNB may not be reduced.

To reduce gNB power consumption, the various techniques related to the adaptation of DTX/DRX are considered including aligning or omitting UE C-DRX pattern across multiple UE or design of Cell DTX/DRX patterns/timers/parameters/procedure during the study item as captured in TR 38.864 [2]. Currently, alignment/omission of DRX cycles or offsets at different UEs can be achieved by RRC (re)configuration, but depending on whether the cell stays active or inactive, a mechanism with low delay is needed to dynamically adjust the UE C-DRX configuration. The DRX parameters (e.g., start offsets, DRX cycle, and length of ON duration) can be dynamically indicated by L1/L2 signaling (e.g., group-common DCI or MAC-CE) for UEs configured with C-DRX within the cell. In addition, the duration or timer of the DRX parameters can be also indicated when the gNB indicates the DRX parameters to align/omit the UE C-DRX configuration. The UE can return to the previously configured C-DRX configuration when the duration or timer is ended or expired or can maintain the indicated DRX parameter until a further indication is given if there is no separately configured duration/timer value. The gNB can configure the candidates of multiple DRX parameters and duration/timer combinations in advance and dynamically indicate one of them via L1/L2 signaling.

Proposal #1: To align DRX active time for different UEs in the cell, the DRX parameters (e.g., start offsets, DRX cycle and length of ON duration) and its application time (during which indicated DRX parameters are applied) can be jointly indicated by L1/L2 signaling (e.g., group-common DCI or MAC-CE).

- Cell DTX/DRX mechanism

Meanwhile, even if DRX cycle or offsets for different UEs in the cell are aligned, the UE may still initiate UL transmission according to the configured resources (e.g., using PUCCH, RACH, SR, or CG-PUSCH) during the OFF duration. In order to reduce the gNB power consumption, it is needed to turn off the transmission/reception in the OFF duration to make a cell stay in the inactive state as long as possible. In this regards, Cell DTX/DRX mechanism can be utilized to turn off transmission/reception of a specific signal(s)/channel(s) at cell inactive time.

The Cell DTX/DRX time-domain pattern consisting of active time and inactive time can be preconfigured to UEs and activated through dynamic indication (e.g., group-common DCI). In addition, the Cell DTX/DRX behaviour during inactive time can be also (pre)configured or indicated, for example, gNB can configure to turn off its dynamic data transmission/reception (i.e., dynamically scheduled PDSCH and PUSCH) during Cell DTX/DRX inactive time (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). As another example, gNB can configure to turn off all transmission and reception for data traffic during Cell DTX/DRX non-active periods except for the reference signals (e.g., SSB and CSI-RS) and SR/RACH. Furthermore, these Cell DTX/DRX behaviours during inactive time can be also applied to the OFF duration of UE C-DRX.

Proposal #2: For network energy saving, a signal/channel to be turned off from the Cell DTX/DRX non-active period can be configured for each signal/channel.

Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

In RAN1#112 meeting [3], the set of candidate signals/channels that UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX were agreed for further discussion in RAN1. In this section, we examine the signals/channels that can be turned off in the Cell DTX/DRX non-active period, and the impacts and potential solutions of when they are actually turned off for network energy saving.

First of all, in the case of P/SP CSI-RS, it is necessary to separately consider CSI-RS for CQI (channel quality indicator) acquisition and CSI-RS for BFR (beam failure recovery)/RRM (radio resource management)/RLM (radio link monitoring). In the case of CSI-RS for CQI acquisition, since it is used to adjust the modulation and coding scheme (MCS) to be used for DL transmission according to channel quality, it may be degraded due to incorrect MCS selection, but it may not be fatal.

However, if the CSI-RS for BFR/RRM/RLM is not transmitted in the Cell DTX non-active period, the BFR/RRM/RLM operation based on it may have a fatal problem, for example, repeated beam failure detection and recovery request transmission if the non-active period is relatively long. As a result, the gNB can separately configure/indicate the RS signals to be monitored for each time period, for example, in the Cell DTX inactive period, SSB can be configured to be used for BFR/RRM/RLM while CSI-RS is configured in Cell DTX active period. In another way, parameters related to performing BFR/RRM/RLM can be configured by relaxation (e.g., by temporarily holding the beamFailureDetectionTimer value, increasing the maximum value in the non-active period, or adjusting Q_in and/or Q_out values).

In the case of SPS-PDSCH, it may be necessary to first discuss how to handle the SPS-PDSCH resources included in the Cell DTX non-active period. For example, we can drop all or part of the SPS-PDSCH resources included in the non-active period or UE can be configured to still monitor it. And in the case of HARQ-ACK PUCCH for SPS-PDSCH, if the SPS-PDSCH is turned off in the Cell DTX non-active period, it can be automatically configuring to turn off together. In addition, it may be necessary to discuss whether or not to allow PUCCH transmission for SPS-PDSCH received just before the non-active period. In addition, for the case of CG-PUSCH, it is necessary to discuss how to handle the CG-PUSCH resources included in the Cell DRX non-active period, similar with the SPS-PDSCH mentioned above.

In the case of SR transmission, it may be necessary to first discuss whether to allow transmission in the Cell DTX non-active period. If PDCCH scrambled with C-RNTI is turned off in the Cell DTX non-active period, even if the gNB receives the SR transmitted by the UE, it may not be able to transmit PDDCH in response. Therefore, if SR transmission is allowed in the Cell DTX non-active period, PDCCH transmission can be temporarily allowed after SR is transmitted in Cell DTX non-active period. In addition, if the UE transmits SR, PUSCH can be scheduled through UL grant, and SRS may be required to obtain UL channel information. Therefore, periodic/semi-persistent-SRS can be transmitted even if periodic/semi-persistent-SRS is configured to be turned off in the Cell DTX (or Cell DRX) non-active period.

Furthermore, if the UE transmits RACH for BFRQ (beam failure recovery request), PDCCH transmission can be allowed even if PDCCH scrambled with UE specific RNTI is configured to be turned off in the Cell DTX non-active period as an exception.

Proposal #3: At least following signals/channels for connected mode UEs can be expected to not transmit or receive during non-active periods of cell DTX/DRX.

- DL

  - Periodic/Semi-persistent CSI-RS

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Furthermore, Cell DTX/DRX configuration and the UE C-DRX configuration can be applied simultaneously to the connected mode UE in the cell. In this case, it may be necessary to define the UE behaviours for each time interval (a combination of the active/inactive time of Cell DTX/DRX and the ON/OFF duration of UE-DRX). For example, a UE may perform PDCCH reception only during a time interval in which the active time in a cell DTX/DRX configuration and the ON duration in a UE C-DRX configuration intersect.

Proposal #4: It is necessary to discuss UE behaviour when both Cell DTX/DRX and UE C-DRX are configured simultaneously.

- Conclusions

In this contribution, Cell DTX/DRX mechanism were discussed, and the followings were proposed.

Proposal #1: To align DRX active time for different UEs in the cell, the DRX parameters (e.g., start offsets, DRX cycle and length of ON duration) and its application time (during which indicated DRX parameters are applied) can be jointly indicated by L1/L2 signaling (e.g., group-common DCI or MAC-CE).

Proposal #2: For network energy saving, a signal/channel to be turned off from the Cell DTX/DRX non-active period can be configured for each signal/channel.

Proposal #3: At least following signals/channels for connected mode UEs can be expected to not transmit or receive during non-active periods of cell DTX/DRX.

- DL

  - Periodic/Semi-persistent CSI-RS

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Proposal #4: It is necessary to discuss UE behaviour when both Cell DTX/DRX and UE C-DRX are configured simultaneously.

- References

RP-223540, “New WID: Network energy savings for NR,” Huawei, RAN#98-e

TR 38.864, “Study on network energy savings for NR”, 3GPP

RAN1#112 Chairman’s note