3GPP TSG-RAN WG1 Meeting #112bis-e	R1- 2303440

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.4.2

Source:	NVIDIA

Title:	AI and ML for positioning enhancement

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to finalize representative sub use cases and assess potential specification impact:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In the previous RAN1 meetings, initial agreements were made to provide directions for further investigation. In this contribution, we discuss relevant aspects of AI/ML for positioning enhancement.

# 2	Representative sub use cases

5G positioning services aim to support verticals and applications with high positioning accuracies. High accuracy positioning is characterized by ambitious system requirements for positioning accuracy in many verticals and applications, including regulatory needs.

In location based services and eHealth, higher accuracy is instrumental to new services and applications, both outdoor and indoor. For example, on the factory floor, it is important to locate assets and moving objects such as forklifts, or parts to be assembled. Similar needs exist in transportation and logistics, e.g., rail, road and use of UAVs. In some road use cases, UEs supporting V2X application(s) are also applicable to such needs. In cases such as guided vehicles (e.g. industry, UAVs) and positioning of objects involved in safety-related functions, availability needs to be very high.

NR offers a variety of positioning technologies. Release-15 NR supports positioning, e.g., by using LTE positioning in NSA operation. Release-16 NR much enhances the positioning support with a range of positioning methods, including both downlink-based and uplink-based positioning. Release-17 NR introduces additional enhancements to reduce latency for time-critical use cases such as remote control, deliver positioning accuracy down to the level of 20-30 cm for use cases such as factory automation, and improve integrity protection of the location information. 3GPP Release 18 is investigating solutions to further improve accuracy, integrity, and power efficiency in positioning, study sidelink positioning, and investigate positioning support for RedCap devices.

Traditional DL/UL-TDOA, AOA, AOD, multi-RTT positioning methods may not provide satisfactory performance in environments with heavy NLOS conditions, such as in industrial environments.

Traditional “RF fingerprinting” requires a database that maps RF measurements to position. Construction of an accurate map of “RF fingerprints” for a real environment is challenging. Besides, the database requires regular updates to adapt to varying environment.

A promising approach may go as follows:

- Create a digital twin of the environment (e.g., industry)

- Simulate a 5G network in the digital twin to collect synthetic data that maps RF measurements to position

- Use the synthetic data (and real data if available) to train an AI/ML model that learns the mapping of RF measurements to position

- Deploy the pre-trained AI/ML model in the actual environment for positioning inference

- Train online to augment the AI/ML model to adapt to the actual environment

- Retrain offline when needed (e.g., due to major change in the environment)

In our companion contribution [2], we present evaluation results for AI/ML based fingerprinting, which show that the AI/ML-based approach can significantly improve positioning estimation accuracy when compared to a benchmark method using TOA based positioning estimation.

Observation 1: AI/ML techniques can be used to learn the mapping of RF measurements to position.

Proposal 1: High accuracy positioning in heavy NLOS scenarios should be the target of using AI/ML for positioning enhancement.

Proposal 2: AI/ML techniques used to learn the mapping of RF measurements to position (i.e., direct AI/ML positioning) should be studied for positioning enhancement.

Besides using AI/ML based algorithms for direct AI/ML positioning such as RF fingerprinting, AI/ML algorithms can also be used for assisted positioning, where the output of the AI/ML model provides intermediate estimates such as LOS/NLOS classification, timing estimates, and angular estimates. These intermediate estimates become input to another algorithm (AI/ML based or non-AI/ML based) to derive the final position estimate.

In our companion contribution [2], we present evaluation results for AI/ML assisted positioning, where the output of the AI/ML model provides LOS/NLOS classification. The results show that AI/ML assisted positioning can provide high-fidelity intermediate estimate for LOS/NLOS classification, which in turn can be used to derive final position estimate.

Proposal 3: AI/ML techniques used to provide intermediate estimates such as LOS/NLOS classification (i.e., AI/ML assisted positioning) should be studied for positioning enhancement.

At RAN1#109-e, the meaning of “sub use case” was discussed with several options left on the table for further investigation.

- Option 1: by scenario

- Option 2: by {input, output} of an AI/ML model

- Option 3: for estimation, tracking, and prediction etc. as different sub use cases

- Option 4: by functionality that the AI/ML model is intended to fulfil where LOS/NLOS classification and fingerprinting to directly estimate UE’s position as different sub use cases

- Other option(s), please elaborate

The above two positioning methods are general categorization. In principle, all potential positioning sub use cases can be put under either of the two categories of positioning methods. Therefore, this categorization does not lead to selection of sub use cases and/or down selection of selected sub use cases.

In our view, it may be easier to list the specific examples proposed by different companies, which is more in line with Option 4. Then representative sub use cases can be selected from the list of specific examples. Some examples that have attracted much attention include using LOS/NLOS classification and fingerprinting.

At RAN1#110, it was agreed that for characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected:

- Direct AI/ML positioning

- AI/ML assisted positioning

For AI/ML based positioning accuracy enhancement, direct AI/ML positioning and AI/ML assisted positioning can be selected as representative sub-use cases.

The positioning methods can be UE-based, UE-assisted/LMF-based, and NG-RAN node assisted. For the study of benefit(s) and potential specification impact for AI/ML based positioning accuracy enhancement, one-sided model whose inference is performed entirely at the UE or at the network can be prioritized in Rel-18 SI.

At RAN1#110bis, it was agreed to investigate benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement:

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

The detailed specification impact heavily depends on the selected sub use cases. It would be desirable to agree on at least one sub use case as early as possible so that the corresponding specification impact can be carried out. The specification impact analysis carried out for one exemplary sub use case would shed light on the specification impact analysis for other sub use cases if agreed later.

# 3	Potential specification impact

Although it is likely that most of the AI/ML algorithms for positioning enhancement can be up to implementation, there are several aspects deserving studies in 3GPP. These include

- Signalling support for the training and execution of AI/ML models for positioning enhancement

- Data required by the AI/ML models for positioning enhancement (e.g., data reported by UE to gNB, assistance data from gNB to UE)

- Outputs generated by the AI/ML models for positioning enhancement to be delivered from gNB to UE or from UE to gNB

AI/ML model training can occur at gNB or UE side. In either case, the training entity can benefit from assistance from the other entity for training data collection. Relevant areas for discussion include training data type/size, training data source determination, and assistance signalling and procedure for training data collection.

Specifically, regarding data collection for AI/ML model training for AI/ML based positioning, request and report of training data should consider aspects including ground truth label, measurement corresponding to model input, associated information of ground truth label and/or measurement corresponding to model input. Regarding assistance signalling and procedure to facilitate generating training data, one should consider reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier, and assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc.

Proposal 4: For AI/ML model training for positioning enhancements, study potential specification impact related to training data type/size, training data source determination, and assistance signalling and procedure for training data collection at UE side or network side.

Most of the evaluation results presented by companies are based on supervised learning, which require label for each training example. How to obtain ground truth labels is an important aspect that deserves further investigation, including the corresponding specification impact. We need to study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data. In particular, the feasibility study on the entity to obtain ground truth label and/or other training data needs to take into account at least the availability of the entity to obtain label and/or other training data.

Besides, the potential specification impact analysis needs to cover (1) the details of request/report of label and/or other training data, (2) delivery of the collected label and/or other training data to the training entity when the training entity is not the same entity that obtained label and/or other training data, and (3) assistance signaling indicating reference signal configuration(s) to derive label and/or other training data.

For direct AI/ML positioning, ground truth label is UE location, which can be PRU with known location, UE-generated location based on non-NR and/or NR RAT-dependent positioning methods, LMF-generated UE location based on positioning methods, or known PRU location used by LMF. For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameters corresponding to AI/ML model output. To this end, PRU can generate label directly or calculate based on measurement/location, UE can generate label directly or calculate based on measurement/location, or network entity can generate label directly or calculate based on measurement/location.

In real scenarios, the ground truth labels are measured/estimated and thus they are inevitably noisy. When the noisy ground truth is provided by one entity to another (e.g., from network to UE or from UE to network), it would be beneficial for the receiving entity to know the noisy level of the provided noisy ground truth. More generally, it would be beneficial to have certain quality indicators for ground truth labels.

Proposal 5: For AI/ML model training for positioning enhancements, study potential specification impact related to ground truth label determination and noisy level of the ground truth labels.

In our companion contribution [2], we present evaluation results for AI/ML based fingerprinting, which show that the AI/ML-based approach can significantly improve positioning estimation accuracy when compared to a benchmark method using TOA based positioning estimation. Nonetheless, it should be noted that the improvement of positioning accuracy comes at the cost of higher requirement on the training dataset for supervised training. It is not always possible to obtain a dataset with high user density in real deployments. One promising solution is to utilize a digital twin network – a digital replica of a corresponding physical network – to generate synthetic data, which is annotated information that a simulation running in a digital twin network generates as an alternative to real-world data [3]. Put another way, synthetic data is created in a cyber sibling of the physical world rather than being measured or collected from the real world. It may be artificial, but synthetic data reflects real-world data in a statistical sense. Digital twin networks capture the precise geometry and material properties of objects in the environment to produce datasets for training AI/ML models used for high-accuracy positioning.

Proposal 6: Study potential specification impact of different data collection methods (e.g., utilizing digital twin technology) for obtaining training data set with high user density.

For generating other training data such as measurement corresponding to model input, one should consider entities including PRU and UE for UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b), and TRP for NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b).

Similar to many other functionalities in 3GPP systems, the usage of AI/ML model for a certain functionality should be under network control, if the functionality at one side cannot be made transparent to the other side. Therefore, assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, model selection, etc. should be investigated.

For model configuration, relevant aspects on conditions/criteria of using AI/ML to enhance positioning accuracy include (1) validity conditions (e.g., applicable area/scenario/environment, time interval), (2) model capability (e.g., positioning accuracy quality, model inference latency), and (3) conditions and requirements (e.g., required assistance signalling and/or reference signals configurations, dataset information).

Proposal 7: For AI/ML based positioning enhancements, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

AI/ML models are data driving. They are trained to learn patterns from data. But the environment of a mobile communication system is dynamic and changes over time, and thus the data also keeps changing. As the data from the environment changes, the AI/ML model performance may be degraded. Therefore, it is essential to monitor the AI/ML model performance and regularly update the model to maintain satisfactory model performance. To this end, assistance signalling and procedure for model performance monitoring and model update/tuning should be investigated.

The first question related to AI/ML model monitoring is what performance metrics should be used for the monitoring. If the AI/ML model performance degrades according to the monitored performance metrics, under what conditions the AI/ML model should be updated require study. Besides, performance monitoring may be facilitated by dedicated reference signals and measurement feedback/report.

The analysis needs to consider UE-side model vs. network-side model. The former requires investigation of assistance signalling and procedure, while the latter requires studying report/feedback and procedure at least for network-side model. It should cover both cases: (1) model inference and model monitoring at the same entity, and (2) entity performing model monitoring is not the same entity performing model inference.

Specifically, if monitoring is based on model output, data used for calculating monitoring metric should cover estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning. If monitoring is based on model input, data used for calculating monitoring metric should cover measurement corresponding to model inference input.

If a given type of data is necessary for calculating monitoring metric, we need to study how an entity can be used to provide the given type of data for calculating monitoring metric, potential signalling for provisioning of the given type of data for calculating associated monitoring metric, potential assistance signalling and procedure to facilitate an entity providing data for calculating monitoring metric, and potential UE-network interaction (e.g., model monitoring decision indication between UE and network).

Proposal 8: For AI/ML based positioning enhancements, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning, including monitored metrics, triggers for model update, dedicated reference signals, measurements, and feedback report.

When it comes to AI/ML model inference input, report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) may be needed. In general, the type of model input, and model input acquisition and pre-processing may have potential specification impact. Similarly, when it comes to AI/ML model inference output, outputs generated by an AI/ML model may need to be delivered from gNB to UE or from UE to gNB. The post-processing of AI/ML model inference output may have potential specification impact as well.

Regarding AI/ML model inference, we need to consider types of measurement as model inference input (e.g., new measurement, existing measurement). To this end, UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b, and TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b. In particular, for LMF-side model (Case 2b and Case 3b), report of measurements as model inference input to LMF is needed. For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF should be considered for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a). Besides, new and/or enhancement to existing assistance signalling and procedure to facilitate model inference for both UE-side and Network-side model should be considered.

Specifically, for direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input may include potential new measurement (e.g., CIR/PDP) and existing measurement (e.g., RSRP/RSRPP/RSTD). For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF may include  new measurement report (e.g., ToA, path phase), existing measurement report (e.g., RSTD, LOS/NLOS indicator, RSRPP), and enhancement of existing measurement report (e.g., soft information/high resolution of RSTD).

Proposal 9: For AI/ML based positioning enhancements, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing for UE side or network side inference.

Proposal 10: For AI/ML based positioning enhancements, study potential specification impact related to report/feedback of model inference output and post-processing for UE side or network side inference.

Different UEs may have different capabilities when it comes to the support of AI/ML algorithms for positioning. Therefore, UE capability for AI/ML based positioning including model training, model inference and model monitoring needs to be investigated and defined.

Proposal 11: For AI/ML based positioning enhancements, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: AI/ML techniques can be used to learn the mapping of RF measurements to position.

Based on the discussion in the previous sections we propose the following:

Proposal 1: High accuracy positioning in heavy NLOS scenarios should be the target of using AI/ML for positioning enhancement.

Proposal 2: AI/ML techniques used to learn the mapping of RF measurements to position (i.e., direct AI/ML positioning) should be studied for positioning enhancement.

Proposal 3: AI/ML techniques used to provide intermediate estimates such as LOS/NLOS classification (i.e., AI/ML assisted positioning) should be studied for positioning enhancement.

Proposal 4: For AI/ML model training for positioning enhancements, study potential specification impact related to training data type/size, training data source determination, and assistance signalling and procedure for training data collection at UE side or network side.

Proposal 5: For AI/ML model training for positioning enhancements, study potential specification impact related to ground truth label determination and noisy level of the ground truth labels.

Proposal 6: Study potential specification impact of different data collection methods (e.g., utilizing digital twin technology) for obtaining training data set with high user density.

Proposal 7: For AI/ML based positioning enhancements, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

Proposal 8: For AI/ML based positioning enhancements, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning, including monitored metrics, triggers for model update, dedicated reference signals, measurements, and feedback report.

Proposal 9: For AI/ML based positioning enhancements, study potential specification impact related to report/feedback of model input for inference, type of model input, and model input acquisition and pre-processing for UE side or network side inference.

Proposal 10: For AI/ML based positioning enhancements, study potential specification impact related to report/feedback of model inference output and post-processing for UE side or network side inference.

Proposal 11: For AI/ML based positioning enhancements, study potential specification impact related to UE capability for AI/ML based beam prediction including model training, model inference and model monitoring.

# Reference

RP-221348, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #96, June 2022

R1-2303439, “Evaluation of AI and ML for positioning enhancements,” RAN1#112-bis-e, NVIDIA.

X. Lin, L. Kundu, C. Dick, E. Obiodu, and T. Mostak, “6G Digital Twin Networks: From Theory to Practice,” arXiv preprint arXiv:2212.02032, Dec. 2022.