3GPP TSG RAN WG1 #112	bis-e		                                    		   R1-2303442

e-Meeting, April 17th – 26th, 2023

Agenda Item:	5

Source:	InterDigital, Inc.

Title:	Discussion on LS reply regarding error source distributions

Document for:	Discussion and Decision

# Introduction

In RAN1#121, the following LS was sent to RAN1 from RAN2 to inquire details about error sources [1].

During the discussion on RAT-dependent positioning integrity, RAN2 has reached the following agreement regarding the distribution of error sources: Agreements: RAN2 anticipate that the error sources are overbounded by a Gaussian distribution. RAN2 would like to confirm with RAN1 on the above agreement regarding the distribution of error sources and respectfully ask RAN1 to provide the parameters (e.g. mean and standard deviation) for the overbound Gaussian distribution.

In this contribution, we discuss how the parameters for error sources can be characterized.

# Discussion

Error sources considered in the LS reply

During the study item, candidates of distributions for error sources listed in Table 6.1.1-2 in TR 38.859 have been identified. Since the LS from RAN2 concern error source distribution, it is proposed to limit the scope of the discussion to the error sources indicated in Table 6.1.1-2 in TR 38.859.

Proposal 1: For the discussion related to LS reply for R2-2302271, limit the scope of the discussion to the error sources listed in Table 6.1.1-2 in TR 38.859.

Overbounding an error source with a Gaussian distribution

In the LS, RAN2 asked RAN1 whether the RAN2’s agreement on the use of a Gaussian distribution to overbound error sources can be confirmed. As shown in Table 6.1.1-2 in TR 38.859 [2], candidates for the distribution of error sources include Gaussian. Therefore, error sources with identified distribution(s) during the study item can be overbounded by Gaussian distribution.

Proposal 2: RAN1 can confirm that error sources with options for distributions listed in Table 6.1.1-2 in TR 38.859 can be overbounded by a Gaussian distribution.

Table 1 “Table 6.1.1-2: Identified candidates for distributions to model the errors due to different error sources” from TR 38.859 [2]

| Error source | Candidate(s) for distribution for error source |
| --- | --- |
| Timing measurement errors (NOTE 1, 2, 3) | Gaussian distribution |
| Inter-TRP synchronization errors | - Uniform distribution (NOTE 4) - Gaussian distribution |
| TRP location error (e.g., NR-TRP-LocationInfo in [16]) | - Uniform distribution (NOTE 5) - Gaussian distribution |
| TRP location error (e.g., Geographical coordinates in [17]) | - Uniform distribution - Gaussian distribution |
| ARP location error (e.g., ARPLocationInformation in [17]) | - Uniform distribution - Gaussian distribution |
| NOTE 1: Timing measurement errors are applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurements. NOTE 2: It is assumed that the timing measurement error is associated with the first path. NOTE 3: It is assumed that the timing measurement error contains TEG related TX/RX timing error if the TEG related information is provided NOTE 4: This may already be consistent with the uncertainty related to NR-RTD-Info in [16]. NOTE 5: This may already be consistent with the uncertainty related to NR-TRP-LocationInfo in [16]. |  |

Details for error sources

Mean and standard deviation can be used to characterize a Gaussian distribution. RAN2 has inquired about necessary parameters for a Gaussian distribution. As in Table 8.1.2.1b-1 in TS 38.305 [3], mean and standard deviation can be indicated in an LPP message for UE-based integrity for GNSS. Thus, from RAN1’s perspective, it is sufficient to identify references to determine candidate values for mean and standard deviation.

Mean value for error sources

Details of timing measurement errors have been discussed in RAN1 during the Rel. 18 study item [2]. The timing measurement errors considered in RAN1 can be errors in RSTD, UE/gNB Rx-Tx time difference, RTOA. The presence of unknown timing delay in timing measurements have been known since Rel. 17[4]. Such an unknown element in the measurement which may make the mean of the measurement error non-zero. Thus the following observation is made.

Observation 1: Mean for timing measurement error can be zero nor non-zero

Similarly, for TRP location error, inter-TRP synchronization error or ARP location error, mean of the error can be based on implementation. For example, depending on the method used to determine the location of TRP (e.g., GNSS), the error may contain unknown offset. Thus the following observation is made.

Observation 2: Mean for TRP location error, inter-TRP synchronization error or ARP location error can be zero or non-zero.

Based on the above observations, the following proposal is made.

Proposal 3: Mean for the error sources shown in Table 6.1.1-2 in TR 38.859 can be zero or non-zero depending on implementations at the UE or network.

Standard deviation values for error sources

RAN1 can identify information elements found in the current specification from which value ranges for the standard deviation of the error source can be determined. Examples area shown below.

- Quality indication for the timing measurement (e.g., nr-TimingQuality for TDOA/RTT measurements in TS 37.355, Measurement Quality in TS 38.355) can be used to quantify spread in the timing measurement error.

- Location uncertainty for TRP location (e.g., locationUNC in TS 37.355, location uncertainty in TS 38.857 [5]) can be used to estimate spread in the error statistics of the location error of the TRP.

- Quality of the timing of reference TRP (e.g., rtd-Quality-r16 in NR-RTD-Info in TS 37.355) can be used to estimate the spread in inter-TRP synchronization error.

Thus, the following proposal is made:

Proposal 4: References for derivation of the value range for the standard deviation for error sources shown in Table 6.1.1-2 in TR 38.859 can be uncertainty or quality indicator in specification, e.g., nr-TimingQuality for timing measurement error, locationUNC for TRP/ARP location error and NR-RTD-Info for inter-TRP synchronization error.

# Conclusion.

In this contribution, the following proposals and observations are made. The draft LS reply is shown in our companion contribution [6].

Observation 1: Mean for timing measurement error can be zero nor non-zero

Observation 2: Mean for TRP location error, inter-TRP synchronization error or ARP location error can be zero or non-zero.

Proposal 1: For the discussion related to LS reply for R2-2302271, limit the scope of the discussion to the error sources listed in Table 6.1.1-2 in TR 38.859.

Proposal 2: RAN1 can confirm that error sources with options for distributions listed in Table 6.1.1-2 in TR 38.859 can be overbounded by a Gaussian distribution.

Proposal 3: Mean for the error sources shown in Table 6.1.1-2 in TR 38.859 can be zero or non-zero depending on implementations at the UE or network.

Proposal 4: References for derivation of the value range for the standard deviation for error sources shown in Table 6.1.1-2 in TR 38.859 can be uncertainty or quality indicator in specification, e.g., nr-TimingQuality for timing measurement error, locationUNC for TRP/ARP location error and NR-RTD-Info for inter-TRP synchronization error.

# Reference.

[1] R1-2302282 (R2-2302271), “LS on error source distributions,” CATT, RAN2#121, Athens, Greece, Feb. 2023.

[2] TR 38.859, “Study on expanded and improved NR positioning,” ver. 18.0.0., Dec. 2022.

[3] TS 38.805, “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN,” ver. 17.3.0., Dec. 2022.

[4] TR 38.857, “Study on NR Positioning Enhancements,” ver. 1.1.0., March, 2021.

[5] TS 38.455, “NR Positioning Protocol A (NRPPa),” ver. 17.3.0., Dec., 2022.

[6] R1-2303441, “Draft Reply LS to RAN2 on error source distributions,” InterDigital, RAN1#112bis-e, Apr. 2023.