3GPP TSG RAN WG1 #112bis-e 		                                    		   R1-2303446

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.5.2

Source:	InterDigital, Inc.

Title:	Discussion on positioning based on NR carrier phase measurement

Document for:	Discussion and Decision

# Introduction

In RAN1#112, agreements related to definitions of phase measurements and report contents were made [1]. In addition, the following items are to be considered or specified related to phase-based positioning [2].

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this contribution, enhancements necessary to support positioning based on carrier phase measurement are discussed.

# Potential enhancements

## Positioning using phase measurements

Measurement reporting:

In RAN1#112, the following agreement related to measurement was made [1].

Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements

In NR, both timing and angle-based positioning have been specified. Phase measurements can serve as assisting measurements which means that phase-based positioning is configured jointly with other positioning methods, e.g., DL-TDOA. Since phase information can be obtained through timing measurements, it is straightforward to associate RSTD and phase measurements. In addition, similar to GNSS, carrier phase only measurements-based positioning should also be supported for scenarios where both TX and RX are calibrated. Thus, the following proposal is made.

Proposal 1: For DL-based positioning or DL & UL based positioning, support reporting of the carrier phase measurements together with the at least RSTD, UE Rx-Tx time difference measurements and RSRP measurements

In addition, phase measurements can be used as a stand-alone measurement if inter-gNB synchronization is accomplished. It should be noted that discussions in [3] show that synchronization loss between gNBs can be kept minimal in indoor or factory-like environment. In addition, techniques to achieve high synchronization accuracy between gNBs is presented in [3] which enable phase-only positioning without dependence on existing RAT dependent positioning methods such as DL-TDOA. Thus the following proposal is made.

Proposal 2 : Support reporting of the carrier phase-based measurements alone.

RSTD and carrier phase measurements can supplement each other to precisely determine the propagation time difference between two TRPs (e.g., the target and the reference TRPs). In our view, the reference TRP for RSTD measurement also can be used as a reference TRP for carrier-phase difference measurement since both measurements (RSTD and phase difference) target to measure the propagation time of the reference signal. Therefore, it is beneficial to use the same reference TRP for both RSTD and phase difference measurement.

Proposal 3: Support using the same reference TRP for joint carrier phase and DL-RSTD measurement reporting.

Assistance data:

In phase-based positioning, differential or double-differential of phase measurements is used for positioning to eliminate unknown phase offset present in the measurements. For this purpose, for the UE to compute phase difference, transmitted PRS resources targeted for phase-difference based positioning, should share the common or similar unknown phase offset characteristics. Thus, a mechanism to include information indicating that PRS resources share similar unknown phase characteristics should be studied.

One possible approach is to specify a phase error group (PEG). PRS resources in the same PEG may share similar unknown phase offset (e.g., PRSs are transmitted from the same amplifier). Knowing that the PRS resources share similar unknown phase offset characteristics may allow the receiver to implement differential or double-differential phase calculation to mitigate residual phase in the measurements. In addition, any necessary assistance information should be studied for both UE-based and UE-assisted carrier phase measurement based positioning.

Proposal 4:  Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

Phase measurement resource:

In RAN1#112, the following agreement was made.

Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

For phase measurement report from the DL-PRS signal(s) of a TRP, one issue is to decide which subcarrier(s) to measure and report carrier phase measurement. The UE is aware of the channel condition (e.g., frequency response of the channel, LOS/NLOS condition). Therefore, it is more flexible to allow the UE to determine which subcarrier to measure the carrier phase. In addition, the network may also can request the number of resources to report and the channel condition to request to facilitate the derivation of the UE’s position. Thus we make the following proposals:

Proposal 5: Support Option 1 “Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF “

Proposal 6: Support the UE to determine the subcarrier(s) to measure and report the carrier phase.

In addition, there may be a frequency offset present in subcarriers. To remove the offset, differential processing can be applied across subcarriers within a PFL or carrier. Therefore, we make the following proposal.

Proposal 7: Support Option 2 “Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier “

Phase difference measurement:

In RAN1#112, the following agreement related to phase difference measurement was made.

Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP

Regarding phase difference measurement, the UE can be configured with a reference TRP. In this case the UE does not need to report the reference. However, it is possible that the configured reference TRP is not suitable (e.g., having low RSRP or small LOS indicator), which may result in inaccurate phase reports of all resources. Therefore, it is beneficial to allow the UE to select the reference for phase difference measurement based on channel condition. In addition, to guarantee the quality of the phase reference, against which the phase difference is calculated, the network can configure conditions to be a phase reference (e.g., based on RSRP, LOS/NLOS).

Proposal 8: Support the UE to select and report the reference TRP for phase difference measurements.

Measurement and report for integer ambiguity

The propagation time between the transmitter and the receiver can be determined as the summation of the number of cycles and the fractional part of a cycle. However, for phase measurement, the UE can measure the fractional part of a cycle only and the number of cycles is not measurable. Precisely determine the number of cycles is crucial for the accuracy of the carrier phase positioning since if the number of cycles is not determined accurately, the carrier phase measurement at the UE, which measures the fractional part of the cycle, is not meaningful.

In addition, in the presence of integer ambiguity [4, 5] due to transmission sources located beyond one cycle of the wavelength of the carrier, the UE can estimate the number of cycles based on other positioning techniques, which can be within or outside 3GPP. Therefore, it is beneficial to allow the UE to report the estimated number of cycles or range for the integer ambiguity for one PRS resource.

Proposal 9: Support Option 3 “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF”

To solve the integer ambiguity problem, the UE can be configured to measure multiple frequencies in a measurement window. For each frequency of DL-PRS, the UE may acquire one carrier phase measurement (e.g., one fractional part of a cycle). The UE then can determine the propagation time by combining multiple carrier phase measurement results in multiple frequencies of DL-PRS (e.g., by solving a linear programming problem).

Proposal 10: Support the UE to perform PRS measurement in multiple frequencies in one measurement window.

For UE-based carrier phase positioning, the UE can derive the propagation time and the distance to the TRP based on the set carrier phases measured in different frequencies in one measurement window. However, for UE-assisted, LMF needs to derive the location of the UE. Therefore, the phase measurement in multiple frequencies should be reported to the LMF. Specifically, the UE should report the phase measurement of DL-PRS transmitted in multiple frequencies in one measurement report.

Proposal 11: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting.

Measurement and report for multipath or LOS/NLOS conditions:

In RAN1#112, the following agreement was made to support LOS/NLOS indicator for carrier phase measurement.

Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Phase measurement is sensitive to multipath and NLOS PRS transmission since a small change in a propagation path may result in a large difference in phase measurement. For a NLOS path, the propagation time can vary based on the reflection path. Therefore, the phase measurement can vary significantly as well. In this case, the phase measurement is not beneficial to determine the position of the UE. Therefore, it is necessary to study in which condition the UE should report phase measurement.

Proposal 12: Specify the condition under which carrier phase should be measured and reported based on

- LOS/NLOS indicator associated PRS resource or TRP

- RSRP

## Use of PRU

During Rel. 17, a use of PRU (Positioning Reference Unit) [4] was discussed such that timing/antenna parameters can be calibrated to eliminate unknow phase/time offset in measurements. In TS 38.305, PRU is defined as a UE with a known location [4]. Using the measurements (e.g., RSTD, phase measurements) returned by PRU, double differential processing can be used to eliminate unknown timing/phase offset. Furthermore, if PRUs are located nearby the UE, it is possible to resolve or eliminate integer ambiguity in the measurement based on the measurements from the PRU. Thus, the following proposal is made.

Proposal 13: Support PRU for NR carrier phase-based positioning.

In UE-assisted NR positioning, the UE makes measurements on the received PRS and reports the measurements to the LMF.  In UE-based NR positioning, the UE makes measurements on the received PRS and determines its position based on the measurements. Similarly, for NR carrier phase positioning, UE-based positioning is feasible if the UE obtains phase measurements from the received PRS. The measurements from PRU can be useful if there are unknown phase offset present/integer ambiguity in the received PRS. Thus, the following proposals are made:

Proposal 14: Study a mechanism to transfer measurements from the PRU to the UE in UE-based positioning.

In addition, the following conclusions and agreement were made in RAN1#112. In the agreement, it introduces a potential mechanism to study simultaneous transmission of SRS and simultaneous releasements of DL-PRS.

Conclusion Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report. Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact.

For precise measurements of phase, mechanism to guarantee simultaneous reception of PRS between UE and PRU or simultaneous transmission of SRS between PRU and UE should be studied. Thus, the following proposal is made:

Proposal 15: Study mechanisms to support simultaneous transmission of SRS between UE and PRU for UL-based positioning methods and DL & UL positioning method

Proposal 16: Study mechanisms to support simultaneous reception of PRS between UE and PRU for DL-based positioning methods and DL & UL positioning method

# Conclusion.

In this contribution, the following proposals are made.

Proposal 1: For DL-based positioning or DL & UL based positioning, support reporting of the carrier phase measurements together with the at least RSTD, UE Rx-Tx time difference measurements and RSRP measurements

Proposal 2 : Support reporting of the carrier phase-based measurements alone.

Proposal 3: Support using the same reference TRP for joint carrier phase and DL-RSTD measurement reporting.

Proposal 4:  Study assistance information that allows the UE or network to mitigate unknown phase offset in phase measurements, e.g., phase error group.

Proposal 5: Support Option 1 “Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF “

Proposal 6: Support the UE to determine the subcarrier(s) to measure and report the carrier phase.

Proposal 7: Support Option 2 “Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier “

Proposal 8: Support the UE to select and report the reference TRP for phase difference measurements.

Proposal 9: Support Option 3 “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF”

Proposal 10: Support the UE to perform PRS measurement in multiple frequencies in one measurement window.

Proposal 11: Support the UE to report the phase measurement in multiple frequencies in one measurement reporting.

Proposal 12: Specify the condition under which carrier phase should be measured and reported based on

- LOS/NLOS indicator associated PRS resource or TRP

- RSRP

Proposal 13: Support PRU for NR carrier phase-based positioning.

Proposal 14: Study a mechanism to transfer measurements from the PRU to the UE in UE-based positioning.

Proposal 15: Study mechanisms to support simultaneous transmission of SRS between UE and PRU for UL-based positioning methods and DL & UL positioning method

Proposal 16: Study mechanisms to support simultaneous reception of PRS between UE and PRU for DL-based positioning methods and DL & UL positioning method

# Reference

[1] RAN1 chairman notes, RAN1#121, Athens, Greece, Feb. 2023.

[2] RP-230328, “New WID on Expanded and Improved NR Positioning, ” RAN#99, Rotterdam, Netherlands, Feb. 2023.

[3] R1-2211259, “Experiment and Simulation Result on Carrier Phase Based Positioning, ” Localia, RAN1#111, Toulouse, France, Nov., 2022.

[4] F. V. Diggelen, “A-GPS: Assisted GPS, GNSS and SBAS,” Artech House, 2009.

[5] P. Henkel, “Reliable Carrier Phase Positioning”, Ph.D. Dissertation, Technical University of Munich, July 2010