3GPP TSG RAN WG1 #112bis-e 		                                    		   R1-2303448

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.5.4

Source:	InterDigital, Inc.

Title:	Bandwidth aggregation for positioning measurements

Document for:	Discussion and Decision

# Introduction

In RAN#98, the following descriptions for bandwidth aggregation for WID were agreed [1]. In addition, agreements were made in RAN1#112 [2].

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

In this contribution, enhancements necessary to support bandwidth aggregation for positioning are discussed.

# Potential enhancements

DL related measurements

It should be noted that from the agreements in RAN4 that PRS/SRS resources in different PFLs or carriers should be transmitted simultaneously. Thus, for DL positioning, the UE can be configured with a measurement gap during which the UE can measure PRS across different PFLs and process the measurements. In addition, PRS processing window is supported in Rel. 17. Both measurement gap and PRS processing window can be configured for bandwidth aggregation. Thus, the following proposal is made.

Proposal 1: Support both PRS processing window and measurement gap based measurement processing for bandwidth aggregation

Conditions for aggregation

In the current configuration mechanism, the LMF indicates which PFL to measure to the UE. Therefore, using the similar mechanism, in case of aggregated PFLs, the LMF should indicate to the UE which PFLs to measure. According to the RAN4 agreement [3], the same numerologies should be assumed across different PFLs. In addition, according to the RAN4 agreement, the different bandwidth can be assumed for different PFL. RAN1 should discuss whether there are any PRS related configurations that can be different across PFLs.

In RAN1#112, the following agreements were made.

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers

One of the FFS points was whether PRS resources with the same UE Rx TEG and gNB Tx TEG across PFLs should be combined. Since the TEG margin for a PRS resource indicates maximum timing error for the resource, the same characteristics should be maintained across PRS resources over different PFLs. A similar argument can be made for SRSp where SRS resource with the same UE Tx TEG and/or gNB Rx TEG should be combined. It should be noted that single RF chain is assumed for PFL aggregation. Therefore, RF characteristics should be the same across all frequency layers.

Observation 1: A single RF chain is assumed to be used for aggregated bandwidths for transmission and reception

Any timing error characteristics (e.g., timing error margin) inherent to the RF chain should appear in the measurements of aggregated PFLs. Thus, we make the following proposal.

Proposal 2: For PRS bandwidth aggregation, aggregated PRS resources from a TRP across the aggregated PFLs should have the same gNB Tx TEG and the same UE Rx TEG.

Proposal 3: For SRS bandwidth aggregation, aggregated SRS resources from a TRP across the aggregated carriers should have the same UE Tx TEG and the same gNB Rx TEG.

On-demand framework

The following agreement was made in RAN1#112.

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

In the current on-demand framework, the number of frequency layers can be requested by the UE. Depending on measurement conditions, it will be beneficial for the UE to request PFL aggregation. In addition ,the UE can request a set of frequency layer IDs that can be combined to the network.

Proposal 4: Support on-demand request for the UE to request PFL indices for bandwidth aggregation

Measurements

Although aggregated PFL/carriers is supported for timing based positioning methods, the measurements for timing based positioning methods include power measurements. Thus, as optional measurement, the power measurement for aggregated PFLs should be supported

Proposal 5: As an optional measurement, power measurements for PFL (RSRP, RSRPP) should be supported

When PRS resources are aggregated across PFLs, the UE can also be indicate priority of measurements. For example, the network may determine that the UE should make measurements on indicated PFLs at higher priority. This enables efficient measurement at the UE.

Proposal 6: Support priority level associated with PFL(s) for measurement when PRS bandwidth aggregation is enabled.

Co-existence between bandwidth aggregation between PRS and data

In RAN1#112, the following agreement was made.

Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

It should be discussed whether carrier aggregation for data communication is an enabler for frequency layer aggregation for PRS. In Rel. 17, introduction of a PRS processing window allowed co-existence between data communication and positioning where the window allowed prioritization by the UE between PRS and DL channels in an active BWP. Thus, when data aggregation is enabled, it should also enable frequency layer aggregation for PRS. For this automatic enablement, the UE should be provided with association between Scells and PRS frequency layers (PFL).

Proposal 7: When carrier aggregation for data communication is enabled, it should automatically enable frequency layer aggregation of PRS, provided that the UE is given association between Scells and PRS frequency layers.

Default frequency layer

In addition, fallback behavior after PRS aggregation is disabled should be discussed. For example, after PRS bandwidth aggregation is disabled, the UE needs to know the default frequency layer to make measurement. Thus, the UE needs to receive a configuration for the default PRS frequency layer ID to make measurement.

Proposal 8: Support configuration of the default frequency layer to perform measurements on after PRS bandwidth aggregation is disabled

# Conclusion.

In this contribution, the following proposals and observations are made.

Observation 1: A single RF chain is assumed to be used for aggregated bandwidths for transmission and reception

Proposal 1: Support both PRS processing window and measurement gap based measurement processing for bandwidth aggregation

Proposal 2: For PRS bandwidth aggregation, aggregated PRS resources from a TRP across the aggregated PFLs should have the same gNB Tx TEG and the same UE Rx TEG.

Proposal 3: For SRS bandwidth aggregation, aggregated SRS resources from a TRP across the aggregated carriers should have the same UE Tx TEG and the same gNB Rx TEG.

Proposal 4: Support on-demand request for the UE to request PFL indices for bandwidth aggregation

Proposal 5: As an optional measurement, power measurements for PFL (RSRP, RSRPP) should be supported

Proposal 6: Support priority level associated with PFL(s) for measurement when PRS bandwidth aggregation is enabled.

Proposal 7: When carrier aggregation for data communication is enabled, it should automatically enable frequency layer aggregation of PRS, provided that the UE is given association between Scells and PRS frequency layers.

Proposal 8: Support configuration of the default frequency layer to perform measurements on after PRS bandwidth aggregation is disabled

# Reference

[1] RP-230328, “New WID on Expanded and Improved NR Positioning, ” RAN#99, Rotterdam, Netherlands, Feb. 2023.

[2] 3GPP, RAN1 Chairman’s note, Athens, Greece, RAN1#112, Feb. 2023.

[3] R4-2220438, “WF on expanded and improved NR positioning, ” RAN4#105, Toulouse, France Nov. 2022.