3GPP TSG RAN WG1 #112bis-e	                          			R1-2303455

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.10.1

Source:	InterDigital, Inc.

Title:	Discussion on L1 enhancements for inter-cell beam management

Document for:	Discussion and Decision

# Introduction

In RAN1 #112, L1 enhancements for inter-cell beam management were discussed and the following agreements were made [1]. In this contribution, further discussion on L1 enhancements is presented.

Agreement - RAN1 shares the same understanding as RAN2 on agreement: - The LTM mobility trigger information is conveyed in a MAC CE - The same MAC CE is used for the LTM triggering. Agreement The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following: - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2 Agreement For L1-RSRP measurement RS configuration - For SSB based L1-RSRP measurement: - As a starting point, at least the following information needs to be provided to a UE, e.g. - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst) - For inter-frequency: frequency domain location (e.g. center frequency), SCS - FFS: transmission power (for pathloss calculation) - Note: other parameters included in the configuration can be further discussed - Including above agreement into the LS - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration. - Following RAN1 understanding will be provided in the LS - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112: - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there. - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells] - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations. - Note RAN2 has a full flexibility to design the whole RRC structure design. - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters. - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations. Agreement Send an LS to RAN2,3,4 on the RAN1 agreements in this meeting - All agreements in AI 9.12.1 and 9.12.2 in RAN1#112 are included - The LS contents agreed in AI 9.12.1 (on L1 measurement configuration) and AI 9.12.2 (on RAR) are also included Agreement - At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s) - FFS: beam indication for mTRP case

# Discussion

## L1 measurement configuration and reporting

It has been agreed in RAN2 that a L1/L2 inter-cell mobility candidate configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. If L1 measurements are enabled for all configured candidates, the associated overhead is large in terms of measurement configuration, measurement resource configuration, and reporting resources overhead. This implies a limit in terms of the number of candidates that can feasibly be preconfigured. Furthermore, if the LTM area includes cells controlled by multiple DU’s, provisioning all resources by RRC leads to complications at the network side if such resources are managed at DU level.

In order to allow sufficient candidates to be pre-configured while keeping measurements and the associated overhead minimized, the number of candidate cells measured should be smaller than the number of stored candidate configurations. An option to limit the number of LTM candidates which are considered for measurement is to enable certain measurement configurations only when using a particular SpCell. That is, the network may pre-configure all measurement configurations which will be needed in all of the configured LTM candidates, but only the relevant ones should be in use at a particular time.

Proposal 1: A subset of the configured candidate cells are actively measured.

### CSI-RS based measurement

One discussion that needs to be concluded is whether to support CSI-RS based L1 measurement in addition to SSB based measurement. Our view is that CSI-RS can be supported as measurement RS due to certain advantages they provide such as narrower beams and higher scheduling flexibility. However, CSI-RS measurement may incur higher overhead on the UE and techniques to reduce the overhead should be considered if CSI-RS are supported.

Proposal 2: Consider supporting CSI-RS as measurement RS for inter-cell beam management.

When CSI-RS are supported, there has been proposals to also support L1-SINR as a measurement quantity in addition to L1-RSRP. Since the benefit of L1-SINR over L1-RSRP for mobility is not clear, and L1-SINR measurement will further increase UE complexity, we do not think that it is necessary to support L1-SINR at this point.

Proposal 3: L1-SINR as a measurement quantity is not supported in Rel-18.

### Cell subset selection/activation

As noted above, measuring all of the configured cells would cause a significant burden on the UE which is further increased with CSI-RS measurements. So, selection and consequent activation of a subset of the RRC configured candidate cells should be considered. This can be achieved by the UE detecting (possibly autonomously) and reporting the strongest SSBs. The serving cell, upon receiving this information, may consequently activate measurement of a subset of the configured cells where the measurements can now include measurements of CSI-RS from the activated cells. This approach, as well as reducing CSI-RS measurement overhead, would also enable the serving cell to choose and activate CSI-RS associated to the strongest SSBs.

Such reporting of the SSBs would be best carried by MAC CE given the large amount of information to include in the report. One can also safely assume that autonomous detection of SSB is possible since L3 reporting relies on this, for both intra-frequency and inter-frequency measurements.

Proposal 4: Support reporting of strongest detected SSB resources in a MAC CE.

As the UE moves within the LTM area, the set of SSB resources relevant for inclusion in a set of L1 measurement resources or TCI state changes. The network needs to be informed relatively quickly at least when a new SSB resource is detected. One possibility is to configure the MAC CE to be transmitted periodically. However, this could result in large number of unnecessary transmissions when the UE is not moving. A more appropriate principle is that the UE triggers transmission of the report when there is a change in the set of strongest detected SSBs.

Proposal 5: Support triggering of MAC CE measurement report when there is a change in the set of strongest detected SSB resources.

To further reduce configuration overhead, we may allow L1 measurement resources to be configured without involving RRC signaling; signaling by MAC CE is an attractive option to achieve this. For example, in the case of a CSI-SSB resource set, the MAC CE can associate a CSI-SSB resource set ID to a set of SSB resources, including respective SSB indices and associated PCI’s.

Proposal 6: Support dynamic update of L1 measurement resource configuration without RRC signaling.

### Report contents (Reporting of inter-frequency measurements)

In RAN1 #111, RAN1 agreed to support gNB-scheduled L1 measurement reporting by UCI, and further that in a single report instance, report for serving cell and candidate cell(s) for intra-frequency and/or inter-frequency can be included. Reporting for a variable number of L1-RSRP measurements for different beams/cells can be supported by e.g., including information on the number of reported measurements, which frequencies, and one measurement result in CSI part 1, and the remaining measurement results in CSI part 2. However, given that there are typically many beams/cells that can be measured, and that CSI payload is limited, it is necessary to specify rules to report only the relevant results for mobility.

The UE should always indicate results for resource(s) on same frequency as the serving cell since the network may prioritize intra-frequency mobility. For resources on different frequencies (inter-frequency), the information may only be useful if the best result on a frequency is higher than a threshold relative to the best intra-frequency result. In addition, when the best intra-frequency result is sufficiently high it is typically not necessary to report for other frequencies.

Proposal 7: Support that inter-frequency L1 measurement results are included in a L1 report under a condition that the best result on the frequency is higher than a threshold. (FFS the threshold).

Proposal 8: Support that inter-frequency L1 measurement results are included in a L1 report under a condition that the best intra-frequency result is lower than a threshold (FFS the threshold).

For identifying a resource in the report, the two following options could be considered:

- Report a SSBRI (or CRI) indicating a resource out of a single set of resources where each individual resource can be configured with frequency identity (e.g., ARFCN)

- Report the identity of a set of resources and the identity of a resource within the set (SSBRI or CRI), where a set of resources is configured with frequency identity.

Option (a) extends the ICBM solution by associating an ARFCN in addition to a PCI. Option (b) implies configuration of separate resource sets for different frequencies. Since in many cases reporting for inter-frequencies is not required, option (a) may result in unnecessary overhead since it would likely require additional bits for each SSBRI or CRI. On the other hand, option (b) may only require a few bits indicating whether results are present for each frequency (resource set) in the CSI part 1. Therefore, option (b) is preferred.

Proposal 9: RRC configures a separate set of resources for channel measurement for each frequency.

Proposal 10: L1 measurement report indicates which set(s) of inter-frequencies are included.

### Event triggered report

Currently only L3 measurements support cell detection based on e.g., a provided carrier frequency, and only L3 measurements support event trigged report. L3 reporting is not efficient enough to support LTM execution, e.g., due to relatively long filtering delay and reporting delay. Therefore, in order to improve the mobility latency while keeping CSI reporting overhead to a reasonable level, L1/L2 based reporting can be used.

Proposal 11: L1 UE triggered reporting is supported at least for reporting detected cells in a carrier.

Since MAC CE is transmitted on PUSCH, this would be the most reasonable choice for transmission of a UE triggered L1 report.

## Beam indication

In RAN1 #111, the following was agreed for beam indication:

- For beam indication timing for Rel-18 LTM, - Support Scenario 2: Beam indication together with cell switch command, - For Rel-17 unified TCI framework, - Beam indication indicates TCI state for each target serving cell - FFS: Scenario 1: Beam indication before cell switch command - FFS: Scenario 3: Beam indication after cell switch command - FFS: Activation of TCI state(s) of target serving and/or candidate cell(s).

From the above scenarios, Scenario 3 does not help reduce latency so can be excluded from further consideration. Regarding Scenario 1, it can already be supported if ICBM is configured in the cell. If ICBM is not configured, additional signaling (either a MAC CE or PDCCH) has to be supported for beam indication before cell switch. It can be argued that if beam is indicated before the cell switch command, the UE may be able to estimate the QCL properties associated to the beam, and this may help reduce latency after cell switch. However, if a subset of configured TCI states is activated, then the UE can estimate the QCL properties of the active TCI states in anticipation of using one of those states after cell switch. So, our view is that the benefit of indicating the beam before cell switch is not clear to warrant additional signaling. So, the following are proposed:

Proposal 12: Scenario 1 (beam indication before cell switch command) is not supported if ICBM is not configured.

Proposal 13: TCI state activation is supported for candidate cells.

Proposal 14: To indicate TCI states for multiple target cells, existing mechanisms are reused.

The beam application time in LTM may be different than the application time specified in Rel-17 since scenarios including inter-DU interaction and inter-frequency cells may need longer beam application time. So, extension of the beam application time may be needed as compared to Rel-17.

Proposal 15: Beam application time may be different from that for Rel-17 ICBM, FFS the exact value(s).

## Preparation phase

In RAN1 #111, RAN1 agreed to support DL synchronization for candidate cell(s) based on at least SSB before cell switch command. Considering that DL synchronization is anyway required for the UE to perform measurement on the candidate cell, satisfying this requirement should not significantly increase UE complexity under the reasonable assumption that LTM is performed only on a target cell that has previously been measured and reported by the UE. It should also be feasible for inter-frequency candidate cells if such cells have been measured and reported by the UE. Regarding UE behavior, we think synchronization can be used to establish and maintain frame/symbol boundary. Finer canonization (e.g., using the TRS) for the purpose of PDCCH/PDSCH reception can be achieved after activation of the corresponding TCI state(s).

Proposal 16: Support DL synchronization (symbol/frame boundary detection using SSB) before reception of cell switch for both intra-frequency and inter-frequency candidate cell(s).

Proposal 17: Support fine synchronization with candidate cells for PDSCH/PDCCH reception after TCI state activation.

Acquisition of the target cell before cell switch has also been proposed to reduce the delay after cell switch until the network can maximize throughput in the target cell. On the other hand, this could potentially also significantly increase the burden of the UE unless CSI acquisition can be done in a selective manner for a small number of candidate cells to which the UE is most likely to switch within a short period of time. In addition, it may not be feasible if the candidate cells are on a frequency for which the UE requires measurement gaps.

Proposal 18: Support CSI acquisition at least for intra-frequency candidate cell(s) before reception of cell switch command.

## Cell switch command

After the UE performs cell switch, it may need uplink resources to send a confirmation to the target cell. The uplink resource assignment can be included in the cell switch command.

Proposal 19: Cell switch command indicates an uplink resource for the target cell.

As discussed above, CSI acquisition of the target cell before cell switch may be beneficial. If this feature cannot be supported in Rel-18 (e.g., due to lack of time), an alternative solution may be to trigger aperiodic CSI by including a field in the cell switch command.

Proposal 20: Support aperiodic CSI trigger in cell switch command.

## Beam failure recovery enhancements

When LTM is used, the UE can be dynamically indicated (with a MAC CE) to perform change of serving cell. This means that the network is prepared to allocate resources for that UE without RRC reconfiguration from any serving cell within the LTM mobility area. In case of beam failure detection, this opens possibility of recovering beam for PDCCH from a non-serving cell to avoid radio link failure when no beam from the serving cell can be properly received, or to select a better beam. This enhancement would significantly increase robustness of the connection.

Proposal 21: Support beam failure recovery on resources of non-serving cell.

# Conclusion

This contribution proposed the following L1 enhancements to support L1/L2 triggered mobility in Rel-18:

Proposal 1: A subset of the configured candidate cells are actively measured.

Proposal 2: Consider supporting CSI-RS as measurement RS for inter-cell beam management.

Proposal 3: L1-SINR as a measurement quantity is not supported in Rel-18.

Proposal 4: Support reporting of strongest detected SSB resources in MAC CE.

Proposal 5: Support triggering of MAC CE measurement report when there is a change in the set of strongest detected SSB resources.

Proposal 6: Support dynamic update of L1 measurement resource configuration without RRC signaling.

Proposal 7: Support that inter-frequency L1 measurement results are included in a L1 report under a condition that the best result on the frequency is higher than a threshold. (FFS the threshold).

Proposal 8: Support that inter-frequency L1 measurement results are included in a L1 report under a condition that the best intra-frequency result is lower than a threshold (FFS the threshold).

Proposal 9: RRC configures a separate set of resources for channel measurement for each frequency.

Proposal 10: L1 measurement report indicates which set(s) of inter-frequencies are included.

Proposal 11: L1 UE triggered reporting is supported at least for reporting detected cells in a carrier.

Proposal 12: Scenario 1 (beam indication before cell switch command) is not supported if ICBM is not configured.

Proposal 13: TCI state activation is supported for candidate cells.

Proposal 14: To indicate TCI states for multiple target cells, existing mechanisms are reused.

Proposal 15: Beam application time may be different from that for Rel 17 ICBM, FFS the exact value(s).

Proposal 16: Support DL synchronization (symbol/frame boundary detection using SSB) before reception of cell switch for both intra-frequency and inter-frequency candidate cell(s).

Proposal 17: Support fine synchronization with candidate cells for PDSCH/PDCCH reception after TCI state activation.

Proposal 18: Support CSI acquisition at least for intra-frequency candidate cell(s) before reception of cell switch command.

Proposal 19: Cell switch command indicates an uplink resource for the target cell.

Proposal 20: Support aperiodic CSI trigger in cell switch command.

Proposal 21: Support beam failure recovery on resources of non-serving cell.

# References

[1] Draft Report of 3GPP TSG RAN WG1 #112