3GPP TSG RAN WG1 #112bis-e	R1-2303459

e-Meeting, April 17th – April 26th, 2023

Source:	Sharp

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

# Background

Several agreements have been achieved in RAN1#112 meeting [1], which could serve as basis for discussion.

Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable. Agreement Study whether or not a slot can consist of both SBFD and non-SBFD symbols including - Benefits - Use cases - Scheduling flexibility - Implementation complexity - Compatibility with legacy TDD DL/UL configuration Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP. Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH Agreement For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used - Option 2: - Part of the DL RBG inside the DL subband cannot be used - Part of the UL RBG inside the UL subband cannot be used FFS: The part of the RBG outside. Agreement For SBFD-aware UEs, study at least the following issues for PDSCH: - PRG(s) with size of 2 and 4 that overlaps with subband boundary - Wideband precoder in case of non-contiguous DL subbands Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols Agreement: Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation

# SBFD operation

# Semi-static SBFD operation

At the last meeting, we have made agreement on dynamic SBFD. On the other hand, semi-static SBFD should be also discussed, or even prioritized over dynamic SBFD.

At the last meeting, it has been discussed that two interpretations of semi-static SBFD operations exists as follows:

- SBFD UEs can assume one transmission direction for SBFD symbols in semi-static manner. Transmission direction in SBFD symbols is configured separately from SBFD configuration.

- SBFD UE cannot assume one transmission direction for SBFD symbols in semi-static manner. Transmission direction in SBFD symbols is determined by scheduling DCI.

Figure 1 shows an example of Operation 1. Figure 1a is an example of semi-static SBFD configuration in one TDD cycle. Figure 1b and 1c shows actual TDD configuration for UE#1 with DL heavy traffic and UE#2 with UL heavy traffic. Thus, in Operation 1, one UE can assume one transmission direction in SBFD symbols. There are no dynamic ways to derive the transmission direction. In this context, the configuration of the transmission direction should be UE-specific (like tdd-UL-DL-ConfigDedicated), and the gNB should still be able to allocate DL and UL simultaneously on SBFD symbols.

In Operation 2, UE assumes SBFD region is potentially available for DL and UL, and transmission direction is determined by scheduling DCI. This behaviour is similar to operation in semi-static flexible symbols without DCI format 2_0 configuration. The UE is unaware of transmission direction in SBFD symbols until they are scheduled transmission or reception by scheduling DCI. Operation 2 is more efficient since it can adapt to the UE traffic condition dynamically.

On the other hand, Operation 1 has benefit in terms of UE implementation. This is because the UE is not required to implement dynamic adaptation of RF settings (if necessary) by scheduling DCI. Therefore, we suggest keeping both options on the table, and verify if Operation 1 has benefit over static TDD.

Proposal 1: For semi-static SBFD, further study, in terms of performance, implementation complexity, switching latency, the following two options:

- SBFD UEs can assume one transmission direction for SBFD symbols in semi-static manner. Transmission direction in SBFD symbols is configured separately from SBFD configuration.

- SBFD UE cannot assume one transmission direction for SBFD symbols in semi-static manner. Transmission direction in SBFD symbols is determined by scheduling DCI.

Figure 1a: Example of semi-static SBFD configuration in one TDD cycle

Figure 1b: Example of TDD configuration for UE with DL heavy traffic

Figure 1c: Example of TDD configuration for UE with UL heavy traffic

# Dynamic SBFD operation

In the agreement, for dynamic SBFD, performance, implementation complexity, and switching latency should be considered. In this contribution, we provide analysis for implementation complexity. Performance comparison should be made once key parameters including UE in-channel sensitivity in system level simulation assumptions have been provided by RAN4.

Firstly, as discussed in the previous section, Option 1 (Semi-static) agreed for a symbol configured as DL in TDD-UL-DL-ConfigCommon, has two interpretations. One is that SBFD UEs can assume one transmission direction for SBFD symbols in semi-static manner. The other is that SBFD UEs can not assume one transmission direction for SBFD symbols in semi-static manner. Therefore, as a first step, we suggest the following modifications to the agreement to capture both operations for semi-static SBFD.

Proposal 2: Suggest the following change to the previous agreement:

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 0 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed, and SBFD UEs can assume one transmission direction for SBFD symbols in semi-static manner

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed, and SBFD UEs can not assume one transmission direction for SBFD symbols in semi-static manner

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

Option 0 is implementation friendly the most. In Option 0, the UE is not required to dynamically change its transmission direction settings. In Option 1, the UE is required to dynamically change its transmission direction settings based on scheduling DCI. On the other hand, In Option 2, the UE is required to dynamically change the RF/digital filtering bandwidth for DL if it is required. However, at the current stage, RAN4 has not decided if the UE is necessary to implement RF filtering adaptation based on SBFD subbands. If RAN4 decided the UE is not required to adapt RF filtering bandwidth for SBFD subbands, Option 1 and Option 2 is the same from RF perspective. Difference is only on scheduling resource limitation that, in Option 1, the UE can assume UL subband is not necessary to receive. It may relax digital filtering requirements to some extent. However, digital filtering is implementation matter.

Observation 1: For SBFD-aware UEs, for a symbol configured as DL in TDD-UL-DL-ConfigCommon, difference in implementation complexity in terms of RF filtering adaptation aspect among Option 1 and Option 2 is not observed in a case that RAN4 decided that RF filtering bandwidth adaptation to SBFD subbands are not required.

If RF filtering adaptation is required, Option 1 is also required to its filtering bandwidth dynamically when DL/UL switching happens within SBFD region. For example, when the UE is operated with DL subbands in slot n, and the UE is scheduled with UL subbands in slot n+1 based on scheduling DCI, the UE needs to change its filtering bandwidth and transmission direction setting simultaneously. Therefore, both Option 1 and Option 2 requires dynamic adaptation of transmission direction and/or filtering bandwidth. Therefore, we propose:

Proposal 3: For SBFD-aware UEs, for a symbol configured as DL in TDD-UL-DL-ConfigCommon, performance in terms of system level simulations should be considered for further down-selection of Option 0, 1 and 2.

For dynamic SBFD for a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the agreement that transmission direction of all RBs outside UL subband (and guard band if any) should be the same, as agreed at RAN1#111 as should be kept. Therefore, for dynamic SBFD in flexible symbols, DDU, DUU, UDD, and DDU should not be considered. Those requires additional filtering patterns in gNB and UE if necessary.

Proposal 4: For dynamic SBFD in flexible symbols, DDU, DUU, UDD, and DDU should not be considered.

Therefore, we only consider DDD, UUU, DUD, and UDU for dynamic SBFD in flexible symbols. On the other hand, in Option 2 where UL transmission outside the UL subband is not allowed, implementation complexity in terms of filtering adaptation is the same as the Option 2 for DL symbols. Only difference is that the UE needs to monitor DCI format 2_0 for Option 2 for flexible symbols, which has been already supported in NR.

Observation 2: For dynamic SBFD, UE implementation complexity in terms of filtering bandwidth adaptation (if necessary) is the same for Option 2 for DL symbols and for Option 2 for flexible symbols.

Proposal 5: For SBFD-aware UEs, performance in terms of system level simulations should be considered for further down-selection of Option 0, 1 and 2 for DL symbols and Option 2 for flexible sybmols.

# SBFD indication

For semi-static configuration of subband time locations for SBFD operation, it has been agreed that explicit configuration within a period is the baseline. One remaining issue on the agreement is the definition of the period.

Intuitively, the period could be the same as the period of TDD configuration. Therefore, we think the period can be provided by TDD-UL-DL-ConfigCommon.

On the other hand, the TDD configuration provided by TDD-UL-DL-ConfigCommon is optional field in SIB1 for TDD cells, which implies the network may or may not provide the TDD configuration. However, when TDD-UL-DL-ConfigCommon is not provided, all the symbols are flexible symbols. As discussed above, flexible symbols should be utilized for transparent SBFD operations, and therefore we don’t think optimization specific to non-transparent SBFD operations in flexible symbols is not of high priority. Therefore, at least explicit/implicit configuration of length of the period for SBFD subband time location indication should be considered as a baseline.

Proposal 6: For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period configured by TDD-UL-DL-ConfigCommon is the baseline.

Further, SBFD operation in initial access should be discussed. In our understanding, msg3 PUSCH repetition should support transmission on UL subbands. Therefore, SBFD subband indication should be available before initial access. Thus, we propose:

Proposal 7: SBFD subband indication should be available before initial access.

Proposal 8: Msg3 PUSCH repetition should support transmission on UL subbands.

One remaining detail is whether or not explicit guard band indication is necessary. According to RAN4 LS reply, they are still discussing Rx model for victim UE from co-channel aggressor.

If RAN4 made conclusion that guard bands between DL subband and UL subband is not necessary for victim UE, neither guard band indication nor DL subband indication is necessary. If RAN4 made conclusion that guard bands between DL subband and UL subband is necessary for victim UE, either guard band indication or DL subband is necessary. Therefore, we think the discussion should be highly dependent on RAN4 conclusion.

Proposal 9: Whether or not explicit guard band indication is necessary or not should be discussed after RAN4 made conclusion on necessity of guard bands between subbands.

# Potential enhancements to procedure for physical channels and signals

# Downlink

### PDCCH

PDCCH resource configuration except for CORESET 0 is via RBG level bitmap where each RBG comprises of 6 RBs. In NR Rel-15, RBG grid is fixed with reference to Point A. On the other hand, in Rel-16 NR-U, RB-level offset has been introduced for more flexible frequency resource configuration. Therefore, we propose:

Proposal 10: PDCCH enhancement other than CORESET 0 is not prioritized in SBFD SI.

CORESET 0 is configured via information in MIB, and the configuration flexibility is limited. Therefore, overlap of CORESET 0 with UL subbands in time and/or frequency domain may be discussed.

### PDSCH

RA type-0

Figure 2 shows an example of RBG assignment for RA type-0. As an illustrative example, it is assumed that the number of resource blocks in the active BWP is 24, and the RBG size is 4. In this case, RBG#X is comprised of resource block with index 4X to index 4X+3. Further, it is assumed that the subband#0 is comprised of resource block with index 0 to index 6, the subband#1 is comprised of resource block with index 9 to index 13, and the subband#2 is comprised of resource block with index 16 to 23.

System performance is not degraded if RBG#2 and RBG#3 are handled by gNB implementation. On the other hand, system performance degradation should be observed when RBG#1 is handled solely by gNB implementation since it implies PRB#4, 5 and 6 becomes unavailable.

To handle the issue, modification to RA type-0 can be considered. That is, the gNB may indicate allocation of RBGs which are partially mapped outside of DL subbands, and the UE recognizes only the PRBs in the DL subbands are allocated when the PDSCH is scheduled in SBFD symbols.

Proposal 11: The following modified RA type-0 should be considered:

- The gNB indicates allocation of RBGs which are partially mapped outside of DL subbands

- The UE recognizes only the PRBs in the DL subbands are allocated when the PDSCH is scheduled in SBFD symbols

Figure 2: Example of RBG assignment for RA type-0

RA type-1

We had extensive discussion on whether to study resource allocation for PDSCH. In our view, type-1 RA with DCI format 1_0 in USS is of the highest priority. DCI format 1_0 in USS only supports type-1 RA, and it can allocate wideband carrier of 100 MHz. On the other hand, maximum bandwidth for type-1 RA with DCI format 1_0 in CSS is limited to the size of CORESET 0 as long as the CORESET 0 is configured in the cell. Maximum bandwidth of CORESET 0 is less than 20 MHz for FR1. Therefore, the maximum bandwidth for type-1 RA can be confined in any subband. Therefore, we propose:

Proposal 12: Enhancement to RA type-1 at least for DCI format 1_0 monitored in USS should be studied in SBFD.

# Uplink

According to TS38.213, any uplink transmission is prohibited on downlink symbols. In this section, we assume that the restriction on uplink transmission is relieved such that at least some uplink transmissions can be performed on uplink subbands in downlink symbols.

### PUCCH

Single slot PUCCH (including PUCCH for msg4 HARQ-ACK) triggered by a DCI format has flexibility in time and/or frequency domain enough to avoid overlapping with DL subbands. Therefore, we propose:

Proposal 13: Enhancement to single slot PUCCH triggered by a DCI format is not prioritized in SBFD.

PUCCH repetition is mapped to available uplink slots continuously starting at the first slot indicated or configured by the gNB where the UE refers to TDD pattern to determine availability of uplink slots.

Figure 3 to 5 show examples of 8 repetitions for PUCCH repetition. Figure 3 shows an example in which the repetitions are transmitted in both the SBFD region and the legacy UL region. Repetitions are mapped continuously in the SBFD region and the legacy UL region, and they are not mapped in the legacy DL region. In this example, Rep#4 and Rep#5 leads to uplink resource fragmentation which has been discussed and addressed in Rel-17 RedCap. To reduce fragmentation, different frequency resource allocation for each region like Figure 5 may be considered further. In Figure 4, different starting RBs and different hopping offsets (i.e., Offset#0 and Offset#1) are provided for each region such that the repetitions are placed on the edge of each available uplink resource. Figure 5 is another example to reduce fragmentation in the legacy region, where the repetitions are mapped only in the SBFD region. In this example, we can keep the simplicity of transmission/reception procedure for repetition due to the aligned frequency resource.

There could be at least three alternatives for enhancements to PUCCH repetition as follows:

- PUCCH repetitions are mapped to SBFD symbols and uplink symbols where a frequency resource allocation is applied to both symbol types

- PUCCH repetitions are mapped to SBFD symbols and uplink symbols where different frequency resource allocations are applied to each symbol type

- PUCCH repetitions are mapped only to SBFD symbols

Alt.1 has the best potential coverage enhancement and latency reduction considering availability of joint channel estimation. Therefore, we slightly prefer Alt.1. On the other hand, uplink resource fragmentation is severe since the repetition in the uplink symbols are at the middle of the carrier. Alt.2 solves the uplink resource fragmentation issue. However, it cannot maximize joint channel estimation opportunity due to different frequency resource in each symbol type. Alt.3 also solves the uplink resource fragmentation issue. However, latency would increase.

Proposal 14: The following 3 alternatives could be further studied for enhancement to PUCCH repetition:

- PUCCH repetitions are mapped to SBFD symbols and uplink symbols where a common frequency resource allocation is applied to both symbol types

- PUCCH repetitions are mapped to SBFD symbols and uplink symbols where different frequency resource allocations are applied to each symbol type

- PUCCH repetitions are mapped only to SBFD symbols

Figure 3: Repetitions transmitted in both the SBFD region and the legacy UL region

Figure 4: Repetitions transmitted in both the SBFD region and the legacy UL region with different frequency resource for the SBFD region and the legacy UL region

Figure 5: Repetitions transmitted only in SBFD region

Configured PUCCH (e.g., SR PUCCH, PUCCH for SPS HARQ-ACK) has the same issues on fragmentation. Like PUCCH repetition case, there could be at least three alternatives for enhancements to configured PUCCH as follows:

- A common frequency resource allocation is applied to each symbol type

- Different frequency resource allocations are applied to each symbol type

- Configured PUCCH transmission is restricted to only one symbol type

Proposal 15: The following 3 alternatives could be further studied for enhancement to configured PUCCH:

- A common frequency resource allocation is applied to each symbol type

- Different frequency resource allocations are applied to each symbol type

- Configured PUCCH transmission is restricted to only one symbol type

### PUSCH

Single slot PUSCH (including msg3 PUSCH) scheduled by a DCI format has flexibility in time and/or frequency domain enough to avoid overlapping with DL subbands. Therefore, we propose:

Proposal 16: Enhancement to single slot PUSCH scheduled by a DCI format is not prioritized in SBFD.

PUSCH repetitions has similar issues with PUCCH repetition. Therefore, we propose:

Proposal 17: The following 3 alternatives could be further studied for enhancement to PUSCH repetition:

- PUSCH repetitions are mapped to SBFD symbols and uplink symbols where a common frequency resource allocation is applied to both symbol types

- PUSCH repetitions are mapped to SBFD symbols and uplink symbols where different frequency resource allocations are applied to each symbol type

- PUSCH repetitions are mapped only to SBFD symbols

Configured PUSCH (e.g., for type-1/2 configured scheduling) has similar issues with configured PUCCH. Therefore, we propose:

Proposal 18: The following 3 alternatives could be further studied for enhancement to configured PUSCH:

- A common frequency resource allocation is applied to each symbol type

- Different frequency resource allocations are applied to each symbol type

- Configured PUSCH transmission is restricted to only one symbol type

# Conclusion

In this contribution, we have the following observations:

Observation 1: For SBFD-aware UEs, for a symbol configured as DL in TDD-UL-DL-ConfigCommon, difference in implementation complexity in terms of RF filtering adaptation aspect among Option 1 and Option 2 is not observed in a case that RAN4 decided that RF filtering bandwidth adaptation to SBFD subbands are not required.

Observation 2: For dynamic SBFD, UE implementation complexity in terms of filtering bandwidth adaptation (if necessary) is the same for Option 2 for DL symbols and for Option 2 for flexible symbols.

In this contribution, we have the following proposals:

Proposal 1: For semi-static SBFD, further study, in terms of performance, implementation complexity, switching latency, the following two options:

- SBFD UEs can assume one transmission direction for SBFD symbols in semi-static manner. Transmission direction in SBFD symbols is configured separately from SBFD configuration.

- SBFD UE cannot assume one transmission direction for SBFD symbols in semi-static manner. Transmission direction in SBFD symbols is determined by scheduling DCI.

Proposal 2: Suggest the following change to the previous agreement:

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 0 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed, and SBFD UEs can assume one transmission direction for SBFD symbols in semi-static manner

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed, and SBFD UEs can not assume one transmission direction for SBFD symbols in semi-static manner

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

Proposal 3: For SBFD-aware UEs, for a symbol configured as DL in TDD-UL-DL-ConfigCommon, performance in terms of system level simulations should be considered for further down-selection of Option 0, 1 and 2.

Proposal 4: For dynamic SBFD in flexible symbols, DDU, DUU, UDD, and DDU should not be considered.

Proposal 5: For SBFD-aware UEs, performance in terms of system level simulations should be considered for further down-selection of Option 0, 1 and 2 for DL symbols and Option 2 for flexible sybmols.

Proposal 6: For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period configured by TDD-UL-DL-ConfigCommon is the baseline.

Proposal 7: SBFD subband indication should be available before initial access.

Proposal 8: Msg3 PUSCH repetition should support transmission on UL subbands.

Proposal 9: Whether or not explicit guard band indication is necessary or not should be discussed after RAN4 made conclusion on necessity of guard bands between subbands.

Proposal 10: PDCCH enhancement other than CORESET 0 is not prioritized in SBFD SI.

Proposal 11: The following modified RA type-0 should be considered:

- The gNB indicates allocation of RBGs which are partially mapped outside of DL subbands

- The UE recognizes only the PRBs in the DL subbands are allocated when the PDSCH is scheduled in SBFD symbols

Proposal 12: Enhancement to RA type-1 at least for DCI format 1_0 monitored in USS should be studied in SBFD.

Proposal 13: Enhancement to single slot PUCCH triggered by a DCI format is not prioritized in SBFD.

Proposal 14: The following 3 alternatives could be further studied for enhancement to PUCCH repetition:

- PUCCH repetitions are mapped to SBFD symbols and uplink symbols where a common frequency resource allocation is applied to both symbol types

- PUCCH repetitions are mapped to SBFD symbols and uplink symbols where different frequency resource allocations are applied to each symbol type

- PUCCH repetitions are mapped only to SBFD symbols

Proposal 15: The following 3 alternatives could be further studied for enhancement to configured PUCCH:

- A common frequency resource allocation is applied to each symbol type

- Different frequency resource allocations are applied to each symbol type

- Configured PUCCH transmission is restricted to only one symbol type

Proposal 16: Enhancement to single slot PUSCH scheduled by a DCI format is not prioritized in SBFD.

Proposal 17: The following 3 alternatives could be further studied for enhancement to PUSCH repetition:

- PUSCH repetitions are mapped to SBFD symbols and uplink symbols where a common frequency resource allocation is applied to both symbol types

- PUSCH repetitions are mapped to SBFD symbols and uplink symbols where different frequency resource allocations are applied to each symbol type

- PUSCH repetitions are mapped only to SBFD symbols

Proposal 18: The following 3 alternatives could be further studied for enhancement to configured PUSCH:

- A common frequency resource allocation is applied to each symbol type

- Different frequency resource allocations are applied to each symbol type

- Configured PUSCH transmission is restricted to only one symbol type

# References

- RAN1 chairman’s note at RAN1#112 meeting, February 2023