3GPP TSG RAN WG1 Meeting#112bis-e	R1-2303462

e-Meeting, April 17th – 26th, 2023

Title:				Draft reply LS to RAN4 on Rel-18 multi-carrier enhancements for NR

Response to: 				R4-2303507

Release:				Release 18

Work Item:				NR_MC_enh-Core

Source:				[RAN1], Apple

To:				RAN4

Cc:				RAN2

Contact Person

Name:				Ankit Bhamri

Email Address:  				a.bhamri@apple.com

Send any reply LS to:	 	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:				none

# Overall description

RAN1 thanks RAN4 for the LS on Rel-18 Multi-carrier enhancement for NR.

RAN1 would like to provide answer to the following question:

From RAN1 perspective, is it possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period? Two examples are given below: - Example #1: In the case of 3-band Tx switching, the switching is performed from 1T+1T on band A and B to 2T on band C. - Example #2: In the case of 4-band Tx switching, the switching is performed from 1T+1T on band A and B to 1T+1T on band C and D.

Reply to the question:

From RAN1 perspective, for the cases involving switching of the two Tx chains between two different bands (example#1: A(1Tx)+B(1Tx) -> C(2Tx) and example#2: A(1Tx)+B(1Tx) -> C(1Tx)+D(1Tx)), there can be two possibilities:

- If a single switching instance is determined for the switching of the two Tx chains, then a single switching period (maximum among the band pairs) is applied, i.e. there is no overlapping switching period. Within that single switching period, whether the two Tx chains switch concurrently between the two different bands pairs or not is a UE implementation.

- If two switching instances are determined for the switching of the two Tx chains, then two corresponding switching periods are applied. The overlap of the two switching periods should not be possible and consequently, the two Tx chains are not switched concurrently between the two different band pairs

# Actions

To RAN WG4:

ACTION:  RAN1 kindly asks RAN4 to take above information into account for their future work.

# Dates of Next TSG-RAN WG1

TSG RAN WG1 Meeting #113          22 – 26 May, 2023         		Incheon, Korea

TSG RAN WG1 Meeting #114          21 – 25 August, 2023         	Toulouse, France