3GPP TSG RAN WG1 #112bis-e			R1-2303465

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	5

Source:	Apple

Title:	Discussion on RAN2 LS on Comparison of SL-RSRP and SD-RSRP Measurements

Document for:	Discussion/Decision

# Introduction

RAN2 sent a reply LS on comparison of SL-RSRP and SD-RSRP measurements [1]. In the LS, a question is asked: “Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?”

In this contribution, we discuss whether the comparison of SL-RSRP and SD-RSRP measurements can be used for the purposes of triggering a measurement report.

# Discussion

In RAN2 LS on comparison of SL-RSRP and SD-RSRP measurements [1], it is mentioned that RAN2 has not reached consensus on whether the measured RSRP of a candidate L2 U2N relay UE becoming an offset better than the measured RSRP of the serving L2 U2N relay UE can serve as a new event to trigger measurement report for session continuity enhancements.

In NR sidelink, the PSSCH transmission power is given by the following formula [2]:

PPSSCHi=min⁡(PCMAX, PMAX, CBR,minPPSSCH,Di, PPSSCH, SLi),

where PPSSCH,Di is the power control based on the Uu link pathloss and PPSSCH, SLi is the power control based on the sidelink pathloss. Note that PPSSCH,Di is applicable for all sidelink cast types, and is effectively valid when the parameter dl-P0-PSSCH-PSCCH is configured, while PPSSCH, SLi is applicable for sidelink unicast only, and is effectively valid when the parameter sl-P0-PSSCH-PSCCH is configured.

It is clear that for sidelink broadcast and sidelink unicast, the PSSCH transmission power is subjective to Uu link pathloss. For sidelink unicast, the PSSCH transmission power may be additionally   subjective to sidelink link pathloss.

Observation 1: The PSSCH transmit power for sidelink broadcast and sidelink unicast is subjective to Uu link pathloss, while the PSSCH transmit power for sidelink unicast may be additionally subjective to sidelink pathloss.

A UE’s Uu link pathloss is between the UE and gNB, while a UE’s sidelink pathloss is between the UE and pair UE. Due to different locations of gNB and pair UE, a UE’s Uu link pathloss is independent of this UE’s sidelink pathloss to pair UE.

Observation 2: A UE’s Uu link pathloss is independent of this UE’s sidelink pathloss to pair UE.

When candidate relay UE broadcasts discovery message, its PSSCH transmission power is subject to Uu link pathloss. When candidate relay UE unicasts sidelink data to target UE, its PSSCH transmission power may be additionally subject to sidelink pathloss.

Due to the independence between candidate relay UE’s Uu link pathloss and candidate relay UE’s sidelink pathloss to target UE, candidate relay UE’s broadcast PSSCH transmission power for discovery messages could be much larger than its unicast PSSCH transmission power for sidelink data.

Hence, the event of measured SD-RSRP for candidate relay UE’s discovery messages being an offset better than measured SL-RSRP for serving relay UE does not necessarily imply the potential measured SL-RSRP for candidate relay UE’s sidelink data is better than measured SL-RSRP for serving relay UE.

Observation 3: Even if the measured SD-RSRP of candidate relay UE is an offset better than the measured SL-RSRP of serving relay UE, the potential measured SL-RSRP of candidate relay UE is likely worse than the measured SL-RSRP of serving relay UE.

Based on the above discussions, it is it is reasonable to not introduce new event Z2.

Proposal 1: The comparison of SL-RSRP measurements and SD-RSRP measurements is not used for the purpose of triggering a measurement report at higher layer.

The corresponding draft reply LS on comparison of SL-RSRP and SD-RSRP measurements is in [3].

# Conclusion

In this contribution, we provided our views on comparison of SL-RSRP and SD-RSRP measurements. Our observations and proposal are as follows:

Observation 1: The PSSCH transmit power for sidelink broadcast and sidelink unicast is subjective to Uu link pathloss, while the PSSCH transmit power for sidelink unicast may be additionally subjective to sidelink pathloss.

Observation 2: A UE’s Uu link pathloss is independent of this UE’s sidelink pathloss to pair UE.

Observation 3: Even if the measured SD-RSRP of candidate relay UE is an offset better than the measured SL-RSRP of serving relay UE, the potential measured SL-RSRP of candidate relay UE is likely worse than the measured SL-RSRP of serving relay UE.

Proposal 1: The comparison of SL-RSRP measurements and SD-RSRP measurements is not used for the purpose of triggering a measurement report at higher layer.

# References

- R2-2302234, “LS to comparison of SL-RSRP and SD-RSRP measurements,” Mar. 2023.

- 3GPP TS 38.213, NR; Physical layer procedures for control, v17.4.0, Jan. 2023.

- R1-2303466, “Draft reply LS to RAN2 on comparison of SL-RSRP and SD-RSRP measurements,” Apr. 2023.