3GPP TSG RAN WG1 Meeting #112bis-e                                                    R1-2303470

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.1.3.1

Source:	Apple

Title:	Views on supporting increased number of orthogonal DMRS ports

Document for:	Discussion/Decision

# Introduction

In RAN1#112, following agreements related to DMRS enhancements in MIMO have been made [1]:

Conclusion

Dynamic switching between R15 DMRS port and R18 DMRS port by a scheduling DCI is not supported in Rel-18

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows:

- For 1 CW,

  - 1) Rows 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1)

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, at least support the following rows:

- For 1 CW,

  - 2) Row 9-11

    - For the above rows, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Working Assumption

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s):

- For one CW, support at least row 30 in the following table.

  - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

- FFS: other rows are not precluded

Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 30 | 2 | 0,2,3 |

Agreement

For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, support Alt.1 for PTRS RE mapping.

- Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4

Table 4 Different RE offsets set for different Rel.18 DMRS port indexes

| DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DM-RS Configuration type 1 |  |  |  | DM-RS Configuration type 2 |  |  |  |
|  | resourceElementOffset |  |  |  | resourceElementOffset |  |  |  |
|  | offset00 | offset01 | offset10 | offset11 | offset00 | offset01 | offset10 | offset11 |
| 0 | 0 | 2 | 6 | 8 | 0 | 1 | 6 | 7 |
| 1 | 2 | 4 | 8 | 10 | 1 | 6 | 7 | 0 |
| 2 | 1 | 3 | 7 | 9 | 2 | 3 | 8 | 9 |
| 3 | 3 | 5 | 9 | 11 | 3 | 8 | 9 | 2 |
| 4 | - | - | - | - | 4 | 5 | 10 | 11 |
| 5 | - | - | - | - | 5 | 10 | 11 | 4 |
| 8 | 4 | 6 | 10 | 0 | - | - | - | - |
| 9 | 6 | 8 | 0 | 2 | - | - | - | - |
| 10 | 5 | 7 | 11 | 1 | - | - | - | - |
| 11 | 7 | 9 | 1 | 3 | - | - | - | - |
| 12 | - | - | - | - | 6 | 7 | 0 | 1 |
| 13 | - | - | - | - | 7 | 0 | 1 | 6 |
| 14 | - | - | - | - | 8 | 9 | 2 | 3 |
| 15 | - | - | - | - | 9 | 2 | 3 | 8 |
| 16 | - | - | - | - | 10 | 11 | 4 | 5 |
| 17 | - | - | - | - | 11 | 4 | 5 | 10 |

Working Assumption

To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

- Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

  - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported.

- FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not.

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 1 | 8 |
| 7 | 1 | 9 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4 | 1 | 8,9 |
| 5 | 2 | 8,9 |
| 6 | 2 | 10,11 |
| [7] | [2] | [8,10] |
| 8-15 | Reserved | Reserved |

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| [1] | [2] | [8-10] |
| 2 | 1 | 0,1,8 |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| [1] | [2] | [8-11] |
| 2 | 1 | 0,1,8,9 |
| 3 | 2 | 0,1,8,9 |
| 4 | 2 | 2,3,10,11 |
| 5-15 | Reserved | Reserved |

Agreement

For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update

- Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2.

  - FFS: Association with The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW with the higher MCS |
| 1 | 2nd scheduled DMRS port the CW with the higher MCS |
| 2 | 3rd scheduled DMRS port the CW with the higher MCS |
| 3 | 4th scheduled DMRS port the CW with the higher MCS |

In this contribution, we provide our views on following aspects for DMRS enhancements:

- MU-MIMO restrictions/limitations between legacy and new DMRS antenna ports

- MU-MIMO scheduling within a CDM group

- DMRS antenna port indication

- PTRS to DMRS enhancements

# MU-MIMO between legacy and new DMRS ports

In RAN#110 [2], following agreement related to MU-MIMO has been made:

Agreement

Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports.

- For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed).

- For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH.

  - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs.

- Note: PUSCH above is CP-OFDM waveform.

Furthermore, in RAN1#112, following proposal has been considered (but not agreed) in the moderator’s summary for DMRS enhancements [3]:

FL Proposal 2.5A

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note: MU-MIMO restriction in Rel.17 is applied.

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - UE does not expect such MU-MIMO in a CDM group.

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: UE does not expect such MU-MIMO in a CDM group.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

    - Alt.3: It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group (i.e. no spec impact and no additional restriction).

In our view, for PDSCH, for legacy UEs, it should be clear that no multiplexing of other UEs with the new DMRS antenna ports within the same CDM group is supported. As there is no possibility to indicate to the legacy UEs, therefore, it should not be considered. For the case of Rel-18 UE, there could be two possibilities. One possibility is that the UE is only configured with legacy DMRS antenna ports and the other possibility is that the UE is configured with legacy as well as new DMRS antenna ports. For the first possibility, the default/baseline capability should be also that there is no multiplexing of other UEs with the new DMRS antenna ports within the same CDM group. Additionally, it could be an optional UE capability to indicate the support of multiplexing between legacy and new DMRS antenna ports within the same CDM group for PDSCH. For the second possibility, it can be a default capability that multiplexing between legacy and new DMRS antenna ports within the same CDM group for PDSCH is supported.

Proposal 1: For a legacy UE, multiplexing with a Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported

Proposal 2: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported as a default capability

Proposal 3: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH can be supported as an optional UE capability

# MU-MIMO within a CDM group

In RAN#112 [3], following proposal (not yet agreed) has been under discussion for MU-MIMO scheduling restriction within a CDM group:

FL Proposal 2.6A

- Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

  - If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

From UE’s channel and interference estimation’s complexity point of view, it is important to restrict MU-MIMO scheduling when a UE is scheduled with DMRS ports in more than one CDM group. With FD-OCC 4,  if a UE is scheduled with DMRS ports in more than 1 CDM group, then the UE’ estimator needs to consider a minimum of 8 layers even in case of MU-MIMO. Such an impact on UE’s implementation is not reasonable and should be avoided. Also, in our view, such restriction is already considered for legacy DMRS tables with such entries. Therefore, FL proposal 2.6A from RAN1#112 should be supported for both single symbol and double symbol DMRS.

Proposal 4: Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH for both single symbol and double symbol DMRS:

- If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

# DMRS antenna port indication

For DMRS antenna port indication, it is quite straightforward to add new entries to the existing tables and consequently increase the bitfield size, wherever the current bitfield size is not sufficient. Based on this, no new field is needed in the DCI.

Proposal 5: For DMRS antenna port indication, add new entries to the existing tables and increase the bitfield size, wherever required

- No new field related to DMRS is needed

For specific entries for enhanced DMRS type 1 and type 2 for both PDSCH and PUSCH, still the majority of the discussion is pending. We have currently agreed on very limited tables/entries. For further discussion, we shall focus on entries that are beneficial to enhance the support for MU-MIMO rather than any optimization for SU-MIMO cases. This would be beneficial to limit the discussion only to relevant entries and avoid excessively large size DMRS tables for enhanced DMRS types.

Proposal 6: As a principle, for further discussion on new entries for DMRS antenna port indication for enhanced DMRS types, we should not consider entries that optimize SU-MIMO cases

- We should consider entries that are only beneficial for MU-MIMO scenario

Another high-level aspect that has been discussed is whether to support a separate table for M-TRP or not. Currently, for M-TRP, a separate table is supported since Rel-16. However, now that we are discussing DMRS entries for enhanced DMRS types for both S-TRP and M-TRP, we should consider a single table with entries for M-TRP as well. Moreover, we think that the entries required for M-TRP are very limited and shall not increase the DMRS table size significantly. Therefore, having a separate table for just a few additional entries for M-TRP is not reasonable.

Proposal 7: For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, common DMRS port table is specified for S-DCI based M-TRP and S-TRP

One aspect to consider in case of common DMRS port table for S-DCI based M-TRP and S-TRP is how to handle the case when UE may not support M-TRP functionality. In this case, it can simply to specified that UE is not expected to be indicated with the entry for M-TRP.

Proposal 8: For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, if a common DMRS port table is specified for S-DCI based M-TRP and S-TRP, then UE should not be expected to be indicated with a DMRS entry for S-DCI based M-TRP if UE doesn’t support such M-TRP functionality

# PTRS to DMRS enhancements

In the current specification, we only support up to 2 port PTRS, the 2 bits “PTRS-DMRS association” field is supported in DCI. The current design supports up to 2 independent phase noise and up to 4-layer PUSCH transmission, for example. 4 Tx partial coherent codebook operation with two panels (port groups). In Rel-18, it was agreed to support 8 Tx UL operation, we need to consider the following agreed antenna architecture

Table 1: Illustration of UE antenna layout

If we assume each antenna port group has its own independent phase noise in the worst case, and if we support up to 8 Tx UL partial-coherent operation over 4 port groups (panels) with up to 8 layers, we need the following

- 4 port PTRS

- 4 bit “PTRS-DMRS association” field in DCI

Proposal 9: To support 8 Tx UL operation, support 4 port PTRS and 4 bit “PTRS-DMRS association” field in DCI

# Conclusion

In this contribution, we have discussed our views on DMRS enhancements and made following proposals:

Proposal 1: For a legacy UE, multiplexing with a Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported

Proposal 2: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH is not supported as a default capability

Proposal 3: For a Rel-18 UE that is only configured with legacy DMRS antenna ports, multiplexing with another Rel-18 UE on new DMRS antenna ports within the same CDM group for PDSCH can be supported as an optional UE capability

Proposal 4: Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH for both single symbol and double symbol DMRS:

- If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

Proposal 5: For DMRS antenna port indication, add new entries to the existing tables and increase the bitfield size, wherever required

- No new field related to DMRS is needed

Proposal 6: As a principle, for further discussion on new entries for DMRS antenna port indication for enhanced DMRS types, we should not consider entries that optimize SU-MIMO cases

- We should consider entries that are only beneficial for MU-MIMO scenario

Proposal 7: For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, common DMRS port table is specified for S-DCI based M-TRP and S-TRP

Proposal 8: For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, if a common DMRS port table is specified for S-DCI based M-TRP and S-TRP, then UE should not be expected to be indicated with a DMRS entry for S-DCI based M-TRP if UE doesn’t support such M-TRP functionality

Proposal 9: To support 8 Tx UL operation, support 4 port PTRS and 4 bit “PTRS-DMRS association” field in DCI

# Reference

- 3GPP RAN1#112, “Chairman Notes”

- 3GPP RAN1#110, “Chairman Notes”

- R1-2301774, “FL summary on DMRS#1”, Moderator (NTT DOCOMO)