3GPP TSG RAN WG1 #112bis-e			R1-2303471

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.3.2

Source:	Apple Inc.

Title:	Views on Rel-18 MIMO SRS enhancement

Document for:	Discussion/Decision

# Introduction

In Rel-18, as part of the approved WID in RP-213598, [1], the following two objectives are considered for SRS enhancement.

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off. - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences. - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices. - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In last RAN1 meeting #111 [2], the following agreement/conclusion were reached regarding Rel-18 SRS enhancement.

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured. - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity . - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index within a radio frame and OFDM symbol index , and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index . - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) . - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame. Agreement For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), - Option 2: For comb 4, do not support 4 comb offsets. Agreement For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2. - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols. Conclusion No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18. Conclusion No consensus to support the following for TDD CJT SRS enhancement in Rel-18: - Further enhancements to frequency hopping - Sequence hopping/randomization, per-hop sequence from a long SRS sequence - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment. - Precoded SRS for DL CSI acquisition - Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Configuration of v (sequence index within a group) per SRS resource - Multiplying mask sequence to the legacy SRS sequence

In contribution, we provide our views on SRS enhancement in the above two directions.

# SRS Enhancement for 8 Tx UL Operation

NR supports 4 different usages of SRS, i.e., “beamManagement”, “codebook”, “nonCodebook”, “antennaSwitching”. 8 Tx UL operation requires SRS enhancement on the following 3 usages, i.e., “codebook”, “nonCodebook”, “antennaSwitching”.

For “codebook” and “antennaSwitching” usage, in RAN#111 meeting, it was agreed to support the case of 8 ports mapped onto the more than 1 OFDM symbols (i.e., TDM). It is important to note that, NR also supports intra-slot frequency hopping for SRS. When TDM is used to distribution 8 SRS ports across multiple slots, specification shall ensure that 8 SRS ports are contained within each intra-slot frequency hop segment.

Proposal 1, To support the case of 8 ports mapped onto more than 1 OFDM symbol (i.e., TDM), when SRS intra-slot frequency hopping is configured, 8 SRS ports shall be contained within each intra-slot frequency hop segment, i.e., R symbols.

Proposal 2, For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, select Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

- Also consider patterns with more than 2 consecutive {1,2,..,s} for infra-slot frequency hopping.

Proposal 3, To support the case of 8 ports mapped onto more than 1 OFDM symbol (i.e., TDM), consider restriction that all SRS-Resource configured in the same SRS-ResourceSet have the same TDM pattern, i.e., in terms of how many symbols is used to carry complete 8 SRS ports.

Proposal 4, For SRS enhancement to support 8 Tx UL operation, for “codebook” usage, the number of  SRS-Resource per SRS-ResourceSet can follow Rel-15/16 design.

- When full power mode 2 is not configured, maximum 2 SRS-Resource per SRS-ResourceSet with usage = “codebook”

- When full power mode 2 is configured, maximum 4 SRS-Resource per SRS-ResourceSet with usage = “codebook”

# SRS Enhancement for Interference Randomization

For SRS enhancement for interference randomization, we agreed in RAN1#112 meeting that both cyclic shift hopping and comb offset hopping are supported.

To determine the hopping pattern for SRS comb offset hopping and/or cyclic shift hopping, RAN1#111 agreed that the design shall be based on the pseudo-random sequence c(i), initialized with a network-configured ID. We also agreed to support the hopping based on the slot index. For infra-slot hopping, we have the following proposal.

Proposal 5, For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', select.

- Option 2: Within a slot, hopping based on only the symbol index  .

For the pseudo-random sequence initialization, it is important to note that, within a cell, NW can multiplex SRS from multiple users orthogonally based on the assignment of comb offset and cyclic shift. When different user starts to perform comb offset hopping and/or cyclic shift hopping, the design shall ensure that the orthogonality among users is still maintained. Therefore, we think SRS sequence identity, i.e., nIDSRS may not be a good idea considering that different users can be configured with different SRS sequence identity. We have the following proposal.

Proposal 6, For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with a new ID configured by the NW via RRC per SRS-Resource.

# Conclusion

In this contribution, we provide our views on how Rel-18 MIMO SRS enhancement. We have the following proposals.

Proposal 1, To support the case of 8 ports mapped onto more than 1 OFDM symbol (i.e., TDM), when SRS intra-slot frequency hopping is configured, 8 SRS ports shall be contained within each intra-slot frequency hop segment, i.e., R symbols.

Proposal 2, For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, select Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

- Also consider patterns with more than 2 consecutive {1,2,..,s} for infra-slot frequency hopping.

Proposal 3, To support the case of 8 ports mapped onto more than 1 OFDM symbol (i.e., TDM), consider restriction that all SRS-Resource configured in the same SRS-ResourceSet have the same TDM pattern, i.e., in terms of how many symbols is used to carry complete 8 SRS ports.

Proposal 4, For SRS enhancement to support 8 Tx UL operation, for “codebook” usage, the number of  SRS-Resource per SRS-ResourceSet can follow Rel-15/16 design.

- When full power mode 2 is not configured, maximum 2 SRS-Resource per SRS-ResourceSet with usage = “codebook”

- When full power mode 2 is configured, maximum 4 SRS-Resource per SRS-ResourceSet with usage = “codebook”

- mode 2 is configured, maximum 4 SRS-Resource per SRS-ResourceSet with usage = “codebook”

Proposal 5, For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', select.

- Option 2: Within a slot, hopping based on only the symbol index  .

Proposal 6, For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with a new ID configured by the NW via RRC per SRS-Resource.

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021

[2] Chairman Notes, 3GPP TSG RAN WG1 #112, Athens, Greece, February 27th – March 3rd  2023