3GPP TSG RAN WG1 #112bis-e	                                         	           R1-2303473

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.4.2

Source:	Apple Inc.

Title:	Views on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion/Decision

# Introduction

In Rel-18, a work item was approved for further MIMO enhancements (WID in RP-213598 [1]), and the study and possible specification of 8 Tx UL operation is included as one of the objectives.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we discuss the remaining issues on SRI and TPMI enhancements for enabling 8 Tx UL operation.

# Discussions

## Codebook based transmission

UE antenna configurations and reporting

We have had the following agreements on antenna configurations:

Agreement

For 8TX PUSCH, at least support

- Ng=1, 2, 4

Note: The above does not restrict the Ng for the non-coherent case

Agreement

Support the following cases for codebook design for 8TX precoders

- Full coherent precoders with Ng=1

  - FFS: Full coherent precoders with Ng=2, Ng=4

- Partial coherent precoders with Ng=2 and Ng=4

  - This does not imply any relation with the number of TPMI indications for 8TX precoder

- Non-coherent precoders

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported,

- (N1, N2) = (4, 1)

- (N1, N2) = (2, 2)

A pair of (N1, N2) can be configured with subject to UE capability.

The targeted antenna configurations are essential for codebook design, and it is critical that the gNB knows what kind of antenna configurations the UE has, or alternatively, what codebooks the UE supports, so that the proper codebook can be configured for the UE. Therefore, UE capability reporting of support of codebook(s) for full coherent, partial coherent, or non-coherent codebook is necessary, similar to the legacy systems.

In addition, for full coherency or partial coherency, there can be multiple possible codebooks that are designed for different antenna layout. This should also be reported by the UE.

Proposal 1: For the support of 8 Tx UL with codebook based transmission scheme, UE reports:

- Whether it supports full coherent, partial coherent, and/or non-coherent codebook.

- For a UE supporting a full-coherent codebook, it further reports whether it supports (N1, N2) = (2, 2) or (N1, N2) = (4, 1).

- For a UE supporting a partial coherent codebook, it further reports the number of non-coherent antenna groups supported, i.e., Ng = 2 or 4.

At the same time, the gNB may configure which codebook set to use, depending on UE capability. In legacy 4 Tx design, the codebook for a UE supporting coherent codebook includes all the full coherent, partial coherent and non-coherent precoders. The question is whether we follow the same approach for 8 Tx. The drawback of following the same approach is larger DCI overhead for TPMI indication, given that there can be many more precoders compared to 4Tx. In terms of the benefit, the main benefit is that the gNB can schedule using less number of antennas (<= 8) in case e.g. some antennas are blocked and experience poor channel condition. The blocking issue might be common for handheld devices such as smart phones even though the antennas are coherent. But for the devices of interest for 8 Tx, which are CPE or vehicle-type of devices, it is more unlikely to have some of the antennas being blocked among coherent antennas. If we want to have more flexibility, it can be left to gNB to configure what precoders are included based on UE capability. The necessity needs some more discussions and this will bring more complication in TPMI indication.

Proposal 2: For a UE with 8 Tx UL configured with full coherent codebook, by default partial/non-coherent precoders are not included. For a UE with 8 Tx UL configured with partial coherent codebook, by default non-coherent precoders are not included.

- FFS whether gNB can configure whether partial and/or non-coherent precoders are included

Full coherent codebook

For full coherent codebook, the following was agreed in RAN1#112:

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported,

- (N1, N2) = (4, 1)

- (N1, N2) = (2, 2)

A pair of (N1, N2) can be configured with subject to UE capability.

Agreement

Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook

- Precoding matrices generated according to (O1, O2) = (1, 1) is supported

- Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1

  - Subject to UE capability

- FFS: Different O1, O2 values for different ranks

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1),

- Study whether/how to support (O1, O2) = (2,1), (2,2)

  - whether for all rank, or rank 1-2, or rank 3-8

  - applicability of different (O1, O2) values per agreed (N1, N2)

  - companies are encouraged to submit simulation results

Regarding additional O1 and O2 values, the oversampling ratio of 2 provides limited or very little gain compared to oversampling ratio of 1 based on the evaluation results provided by companies. Therefore, supporting oversampling ratio of 1 seems sufficient already.

Proposal 3: For 8Tx full coherent codebook design based on R15 single DL Type I codebook, do not support precoding matrices generated according to (O1, O2) where O1>1 and/or O2>1.

Partial coherent codebook

For partial coherent codebook with Ng = 2 or 4, the following was agreed in RAN1#112:

Agreement

For partially coherent uplink precoding by an 8TX UE codebook,

- When Ng=2

  - Precoding design is based on Rel-15 UL 4TX codebook,

    - Full-coherent precoders are used

      - FFS whether partial-coherent precoders are needed

- When Ng=4, down-select from,

  - Alt1:

    - Precoding design is based on Rel-15 UL 2TX codebook,

      - Full-coherent precoders are used

  - Alt2:

    - Precoding design is based on Rel-15 UL 4TX codebook,

      - Partial-coherent precoders are used

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=2,

- Following rank and layer splitting cases are supported

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 1 | (1,0), (0,1) |  |
| 8 |  | (4,4) |

- Select from the following cases based on the performance and overall DCI overhead

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (2,1), (1,2) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2), (3,1), (1,3) |
| 5 |  | (4,1), (1,4), (2,3), (3,2) |
| 6 |  | (4,2), (2,4), (3,3) |
| 7 |  | (4,3), (3,4) |

Note: Above is not relevant to how precoders are indicated.

For Ng = 2, it has been agreed to have precoding design based on Rel-15 UL 4Tx codebook. The remaining question is how to indicate the precoder, and whether we support all combinations of layer splitting. There is also discussion on whether one codeword should be mapped to one antenna group.

On the combinations of layer splitting, we think all the combinations can be supported. The reason is that different antenna groups may be located in different places on the UE and/or face different directions. This means the number of layers that can be supported by each group can be quite different, which justifies the support of all the combinations.

From overhead perspective, if we support all the combinations, it can be done by 2 TPMI indications, each with 5 bits, so the total overhead is 5+5= 10 bits.

If we support only some of the more balanced combinations, in our view the set should at least include (1, 0), (0, 1), (2, 0), (0, 2), (1, 1), (3, 0), (0, 3), (1, 2), (4, 0), (0, 4), (2, 2), (2, 3), (3, 3), (3, 4), (4, 4). The number of precoders is 16+16+8+8+256+4+4+128+2+2+64+32+16+8+4=568, which also require 10 bits. Even though the number of precoders is reduced, it does not reduce the number of bits needed. It requires careful consideration whether such optimization is needed.

Proposal 4: For partial coherent codebook design with Ng = 2, each antenna group is indicated with a NR Rel-15 UL full coherent 4TX precoder, with separate TPMI field provided in the DCI. Each TPMI field is 5 bit.

On whether one codeword should be mapped to one antenna group, it is unclear to us why such restriction is necessary. At least for rank <=4, only 1 codeword is supported, and layers may be split between two antenna groups already. There is no immediate reason why layer splitting cannot be done similarly in case of two codewords.

In addition, we already agreed to reuse DL Rel-15 codeword to layer mapping. This mapping is fixed. E.g. in case of 5 layers, the 1st codeword is mapped to the first 2 layers, and the 2nd codeword is mapped to the remaining 3 layers. By enforcing one codeword mapping to one antenna group, we can only support layer splitting of (2, 3), not any other splitting. Such constraint greatly limits what kind of combinations can be supported for layer splitting.

Proposal 5: For partial coherent codebook design with Ng = 2, there is no restriction that one codeword is mapped to one antenna group only.

For Ng = 4, the main question is whether to have the design based on 2 Tx full coherent codebook (Alt 1) or 4 Tx partial coherent codebook (Alt 2). Either way works. The tradeoff is between the flexibility provided by a larger precoder set and the DCI overhead. Alt 1 requires more bits in DCI (1 or 2 more bits, depending on how signaling is done), while providing more precoders. At this point, the exact use cases for 4 antenna groups are not very clear, and it is somewhat difficult to judge the performance impact. For example, if 4 antenna groups are in 4 different locations facing 4 different directions, it may be beneficial to allow more flexibility in precoders. But this may not be true in some other cases. Since it is unclear at this point, it may be reasonable to allow more flexibility. So we prefer Alt 1.

Proposal 6: For partial coherent codebook design with Ng = 4, adopt Alt 1 (precoding design is based on Rel-15 UL 2TX codebook).

Non-coherent codebook

For non-coherent codebook, it is essentially antenna port selection, and the number of antenna ports selected correspond to the number of layers for transmission. The following was agreed for rank-1 transmission:

Agreement

For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission.

with the scaling factor of .

The same mechanism as used for SRI for non-codebook based transmission can provide full flexibility in antenna port selection to support up to 8 layers. This requires 8 bits in the worst case.

Proposal 7: For non-coherent codebook design, TPMI indication reuses the mechanism for SRI for non-codebook based transmission, which requires up to 8 bits.

## Non-codebook based transmission

SRI indication

For SRI indication for non-codebook based transmission, the following was agreed in RAN1#112:

Agreement

For NCB-based 8TX PUSCH transmission with , where  is the number of configured single-port SRS resources in a resource set,

- All SRS port combinations are supported

- For SRI indication, down-select from,

  - Option 1: Use an  bit length bitmap

  - Option 2: Use a legacy-based solution

- Consideration of Lmax for SRI indication

For , Rel-15 SRI indication is reused

Option 1 always requires NSRS bits, regardless of the maximum rank. On the other hand, Option 2 allows smaller overhead in some cases when the maximum rank is smaller than NSRS.  Given that Option 2 is also a straightforward extension of the legacy mechanism, Option 2 is preferred.

Proposal 8: For non-codebook based transmission scheme with 8Tx UL, the existing SRI indication mechanism is extended to support up to 8 single-port SRS resources in a resource set.

## Support of two codewords

For the support of two codewords, the following was agreed in RAN1#112:

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support

- Alt.2: A second MCS field (5 bits) is indicated for the second codeword

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated.

- FFS: Details on how to signal

Agreement

To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q{0,1},

where , and  are defined similar to the legacy single CW PUSCH transmission.

Agreement

To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from,

- Alt1: First CW

- Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

For NDI and RV, there is still FFS on the detailed signaling mechanism. NDI has a full bit information, and RV has a full 2 bit information. It is unclear how joint indication with other fields can save overhead. Any kind of joint indication also increase specification effort and implementation effort. Therefore we propose:

Proposal 9: For 8 Tx PUSCH, new fields are added in the DCI to indicate NDI and RV for the second codeword.

For UCI multiplexing, among the two options, we prefer Alt 2.

Proposal 10: For 8 Tx PUSCH with two codewords, UCI is always multiplexed only on the CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW).

There has been some discussion on the enabling/disabling of the second codeword. Our understanding is that two codewords are used whenever rank > 4, so the decision can be made based on rank (from TPMI/SRI signaling), and no separate signaling is necessary.

Proposal 11: For 8 Tx PUSCH, two codewords are used whenever rank > 4, and no additional signaling is necessary.

# Conclusion

In contribution, we have discussed the SRI/TPMI enhancements for 8 Tx UL, and have the following proposals:

Proposal 1: For the support of 8 Tx UL with codebook based transmission scheme, UE reports:

- Whether it supports full coherent, partial coherent, and/or non-coherent codebook.

- For a UE supporting a full-coherent codebook, it further reports whether it supports (N1, N2) = (2, 2) or (N1, N2) = (4, 1).

- For a UE supporting a partial coherent codebook, it further reports the number of non-coherent antenna group, i.e., Ng = 2 or 4.

Proposal 2: For a UE with 8 Tx UL configured with full coherent codebook, by default partial/non-coherent precoders are not included. For a UE with 8 Tx UL configured with partial coherent codebook, by default non-coherent precoders are not included.

- FFS whether gNB can configure whether partial or non-coherent precoders are included

Proposal 3: For 8Tx full coherent codebook design based on R15 single DL Type I codebook, do not support precoding matrices generated according to (O1, O2) where O1>1 and/or O2>1.

Proposal 4: For partial coherent codebook design with Ng = 2, each antenna group is indicated with a NR Rel-15 UL full coherent 4TX precoder, with separate TPMI field provided in the DCI. Each TPMI field is 5 bit.

Proposal 5: For partial coherent codebook design with Ng = 2, there is no restriction that one codeword is mapped to one antenna group only.

Proposal 6: For partial coherent codebook design with Ng = 4, adopt Alt 1 (precoding design is based on Rel-15 UL 2TX codebook).

Proposal 7: For non-coherent codebook design, TPMI indication reuses the mechanism for SRI for non-codebook based transmission, which requires up to 8 bits.

Proposal 8: For non-codebook based transmission scheme with 8Tx UL, the existing SRI indication mechanism is extended to support up to 8 single-port SRS resources in a resource set.

Proposal 9: For 8 Tx PUSCH, new fields are added in the DCI to indicate NDI and RV for the second codeword.

Proposal 10: For 8 Tx PUSCH with two codewords, UCI is always multiplexed only on the CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW).

Proposal 11: For 8 Tx PUSCH, two codewords are used whenever rank > 4, and no additional signaling is necessary.

# Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021