3GPP TSG- RAN WG1 Meeting #112-bis-e                                           R1- 2303476

e-Meeting, Apr 17th – Apr 26th, 2023

Agenda Item:	9.2.2.2

Source:	Apple Inc.

Title:	Discussion on other aspects of AI/ML for CSI enhancement

Document for:	Discussion/Decision

# Introduction

AI/ML for physical layer has gained tremendous interest in academic and industry research in recent years. The first 3GPP SI will study the use of AI/ML technology in air interface design, through three carefully selected use cases [1]. In addition to evaluation the potential gain of AI/ML based approach, potential specification impact will be identified through the study.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

AI based CSI enhancement is one of the key use cases which provide unique view on AI/ML for air interface framework. In this paper, we discuss the use case selection and potential specification impact for CSI compression and CSI prediction.

# Potential specification impact for CSI compression

## Training collaboration

In RAN1 110, three different training collaboration has been defined.

For training collaboration type 2, the UE generate target CSI and share the target CSI dataset to the NW for training purpose. For each epoch, the UE side and UW side needs to update the gradients. If the training procedure is done over the air interface, large amount of traffic and complicated co-ordination of training update is required. It is not necessary nor feasible to perform the type 2 training collaboration over 3GPP specified interface. Proprietary solutions based on multi-vendor agreement can always be done, but it is outside of 3GPP scope.

In RAN1 111, it was concluded:

Conclusion

In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI.

Conclusion

In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI.

For model update, smaller data set can be used to update the previous trained model. However, for training collaboration type 2, the exchange over the air is the gradients which linearly scale with the number of epochs. Therefore even smaller dataset is used, the complexity and overhead for over the air update of training collaboration type II is still prohibitive.

Proposal 1: Model update using training collaboration type 2 over 3GPP air interface incur high complexity and large overhead. It can be deprioritized for R18 study.

For type 1 and type 3 training collaboration type, different variations can be further discussed. Each approach has its pros/cons.

- For training collaboration type 1,

  - 1a: the model is trained at the UE-side

  - 1b: the model is trained at the NW-side

- For training collaboration type 3,

  - 3a: UE first training. The UE train the CSI generation model and CSI reconstruction model together and generate the CSI generation model output and CSI reconstruction model output data set for the NW to train the reconstruction model. The quantization method is determined by the UE.

  - 3b: NW first training. The NW train the CSI generation model and CSI reconstruction model together, generate the CSI generation model input and CSI generation model output data set for the UE to train the generation model. The quantization method is determined by the NW.

Proposal 2: To facilitate future discussion on necessity and benefit of each training collaboration type,

- Further categorize the training collaboration type 1 as: 1a-training at UE side, 1b-training at NW side.

- Further categorize the training collaboration type 3 as: 3a-UE first and 3b-NW first.

In RAN1 112, a set of metrics are captured as conclusion for pros/cons analysis of different training collaboration type.

Conclusion

In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects:

- Whether model can be kept proprietary

- Requirements on privacy-sensitive dataset sharing

- Flexibility to support cell/site/scenario/configuration specific model

- gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware

- Model update flexibility after deployment

- feasibility of allowing UE side and NW side to develop/update models separately

- Model performance based on evaluation in 9.2.2.1

- Whether gNB can maintain/store a single/unified model

- Whether UE device can maintain/store a single/unified model

- Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use

- Whether training data distribution can be matched to the device that will use the model for inference

- Whether device capability can be considered for model development

- Other aspects are not precluded

- Note: training data collection and dataset/model delivery will be discussed separately

Conclusion

In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects:

- Whether model can be kept proprietary

- Requirements on privacy-sensitive dataset sharing

- Flexibility to support cell/site/scenario/configuration specific model

- gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware

- Model update flexibility after deployment

- feasibility of allowing UE side and NW side to develop/update models separately

- Model performance based on evaluation in 9.2.2.1

- Whether gNB can maintain/store a single/unified model

- Whether UE device can maintain/store a single/unified model

- Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use

- Whether training data distribution can be matched to the device that will use the model for inference

- Whether device capability can be considered for model development

- Other aspects are not precluded

- Note: training data collection and dataset/model delivery will be discussed separately

Table I summarizes the pros/cons of each training collaboration.

Observation 1:

Table I: Comparison of different training collaboration

|  | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | UE side | NW side |  | UE first | NW first |
| Model proprietary | No | No | Yes | Yes | Yes |
| Require privacy data sharing | No | UE share dataset to NW | UE share target CSI to NW | UE share target CSI and encoder output to NW | UE share dataset to NW. NW share encoder training dataset |
| Flexibility to support config specific model | Yes, with NW assisted info | Yes | No. | Yes, with NW assisted info | Yes |
| Model upgrade flexibility | Flexible | Most flexible | Not flexible | Flexible | Flexible |
| Develop separately | Yes | Yes | No | Yes | Yes |
| Single model at gNB | No | Yes | Difficult | Yes | Yes |
| Single model at UE | Yes | Yes via frequent download | Difficult | Yes | Performance might degrade |
| Extendibility | Limited | Limited | Limited | Limited | Limited |
| Device specific training data | Yes | Yes, with UE assisted info | Yes | Yes | Yes, with UE assisted info |
| Device capability considered in training | Yes | Yes | Yes | Yes | Yes |

## Inference operation

UCI configuration and report:

Agreement

In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report:

- NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration.

- How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

Agreement

In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report:

- NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration.

- How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

For CSI compression inferencing, the NW can activate the CSI compression using additional signaling on top of current MIMO RRC signaling, such as a new report type or a new codebook type in reportConfig.  When the new report type for AI based CSI compression is configured, it is activated. When the report type is de-configured, it is de-activated.

Proposal 3: NW can configure AI based CSI compression with enhanced MIMO related RRC configuration.

In legacy e-typeII codebook, the maximum payload size per configuration scales with RI. When RI=2,3 and 4, roughly similar feedback bits are generated, which is about twice the size of RI=1 feedback. This feature is also desirable for AI based CSI feedback, to limit the maximum UCI size.

For AI based CSI compression, network can configure the maximum number of payload size per configuration. UE can determine the RI, select the proper CSI construction model based on RI and the max payload size configured by the network.

- When layer specific model is used, different model can be used for each layer, and corresponding model ID and model output will be sent as UCI.

- When layer common model is used, the UE can still select different model for different layer so different payload size can be generated for different layer. The UE can choose one AI model for each layer, or separate AI model for different layers, for example, the 1st and/or 2nd layer can choose AI model, while 3rd and/or 4th layer can choose a different AI model with a fewer number of latent space bit.

The below figure shows an example of CSI report format.

Figure 2. Example of CSI report content and format.

To reduce the size of neutral network ID (NN ID), NW can configure a list of NN IDs through RRC, and UE can choose one or multiple of ID from the RRC configured list.

Proposal 4: For eigen-vector based CSI compression, NW configure the maximum UCI size and list of candidates NN IDs via RRC configuration.

Proposal 5: For eigen-vector based CSI compression, the UE determine which AI model to use based on rank and include the model ID as part of the CSI report.

In legacy codebook design, up to 38 subbands can be configured for e-type II codebook. The number of subbands is the same as the number of frequency basis. For AI based CSI compression, when eigen-vectors are used, the size of input CSI to the AI model is related to the configured subbands. One simple way that have been evaluated is design AI model input size corresponding to the largest supported subbands, and when configured subband is smaller than max, padding zeros for model inferencing. This might not be an efficient solution since it results in the same UCI bits and higher model complexity. Another method is to allow size of subband be configurable to ensure the total number of subband is the same as input CSI dimension.

Quantization:

Agreement

In CSI compression using two-sided model use case, further study at least use cases of the following potential specification impact on quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB:

- Alignment of the quantization/dequantization method and the feedback message size between Network and UE

Agreement

In CSI compression using two-sided model use case, further study at least use cases of the following potential specification impact on quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB:

- Alignment of the quantization/dequantization method and the feedback message size between Network and UE

For training collaboration type 3, the quantizer and de-quantizer need to be known between the two parties involved in the training, so proper training can be done. Taken UE first training for example, the UE will generate the training input, which is binary quantized value. If NW does not know the quantization method used in the encoder and try to train a de-quantizer and reconstruction model jointly, the training problem is very complex as the dimension of the binary input is large.

With a given UCI bit size, different combination of floating-point latent space dimension, different codebook size can be all be designing choices. Also notice that for vector quantization, due to the large latent space dimension, the floating-point output needs to be segmented into smaller pieces to allow smaller VQ codebook to be trained. An example with UCI 60 bits, CSI construction part output before quantization model is 30 floating point real value is shown in Fig. 2.

Fig. 2: Example of VQ quantization

To enable multi-vendor training, for example UE first training, where one CSI reconstruction model is trained with multiple datasets generated from different vendors, the size of the latent dimension should be aligned to facilitate the aggregation of multi-vendor training dataset. For example, with 60 bits UCI, if one UE vendor design with CSI reconstruction output size as 30, one vendor with 20 and anther vendor with 40, the corresponding dataset aggregation CSI reconstruction model training can be difficult. The same observation goes for NW first training.

In addition, it is desirable to specify the VQ codebook size, to enable reasonable VQ implementation. In addition, CSI generation model output puncturing has been proposed and evaluated as one method to adapt the UCI payload size, when VQ is used, it is desirable to align the puncturing granularity with the VQ codebook size.

Proposal 6: To enable quantization alignment in AI based CSI compression training type 3, specify at least the size/dimension of CSI generation model output before quantization.

Proposal 7: If vector quantization is supported in AI based CSI feedback, the input/output size of vector quantization codebook should be specified.

Support of legacy CSI report principles

In RAN1 112 meeting, it was agreed to further study whether some legacy MIMO feedback principles can be supported by AI based approach.

Agreement

In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least:

- The priority rule regarding CSI collision handling and CSI omission

- Codebook subset restriction

- CSI processing Unit

Agreement

In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least:

- The priority rule regarding CSI collision handling and CSI omission

- Codebook subset restriction

- CSI processing Unit

For AI based CSI compression with domain transformation pre-processing, where the input CSI is the precoding matrix represented using angular-delay domain projection, legacy CSI principle related to prioritization and CBSR can be applied to the input CSI directly.

For AI based CSI compression without domain transformation pre-processing, different approaches are needed. For prioritization rule, in legacy codebook priority rule prioritize the W2 coefficients following layer > spatial basis > frequency basis order. When UCI payload size is limited, frequency basis with large index is first dropped, then spatial basis with large index, then layer. For AI based CSI compression, the frequency basis and spatial basis are essentially part of the AI model, where different payload size configuration will automatically extract different frequency and spatial basis, so CSI size adjustment can be achieved by adapting to different models.

For codebook subset restriction, the eigen-vector can be projected to a subspace orthogonal to the restricted subspace, the projected eigen-vectors will be input to CSI generation model, to achieve the functionality of codebook subset restriction.  Note UCI payload size is not affected by the application of codebook subset restriction.

Proposal 8: When domain transformation pre-processing is used, legacy CSI report principle can be applied to input CSI directly.

Proposal 9: When domain transformation pre-processing is not used,

- Prioritization rule is indirectly support by selecting different AI model with different UCI bit size.

- CBSD can be supported by projecting the input CSI in the subspace orthogonal to restricted sub-space before AI model.

# Potential specification impact for CSI prediction

AI based CSI prediction is one sided model. When CSI prediction is performed at the UE side, training data set, model selection, training, inferencing can be up to UE implementation. For performance monitoring, the UE can monitor the prediction performance by comparing predicted channel versus next CSI-RS measurement.

To enable CSI prediction, potential specification impact include:

- UE report capability of support AI based CSI prediction.

- UE request to activate/de-activate of CSI prediction based on inferencing accurate and UE speed.

- NW configure/activate CSI prediction, potentially with desired prediction time window.

## Proposal 10: For CSI prediction use case, potential specification impact including UE capability signaling, UE request and NW activation/de-activation signaling.

# Conclusion

In the paper, we discuss the potential specification impact on CSI compression and CSI prediction use case. The proposals are:

Proposal 1: Model update using training collaboration type 2 over 3GPP air interface incur high complexity and large overhead. It can be deprioritized for R18 study

Proposal 2: To facilitate future discussion on necessity and benefit of each training collaboration type,

- Further categorize the training collaboration type 1 as: 1a-training at UE side, 1b-training at NW side.

- Further categorize the training collaboration type 3 as: 3a-UE first and 3b-NW first.

Proposal 3: NW can configure AI based CSI compression with enhanced MIMO related RRC configuration.

Proposal 4: For eigen-vector based CSI compression, NW configure the maximum UCI size and list of candidates NN IDs via RRC configuration.

Proposal 5: For eigen-vector based CSI compression, the UE determine which AI model to use based on rank and include the model ID as part of the CSI report.

Proposal 6: To enable quantization alignment in AI based CSI compression training type 3, specify at least the size/dimension of CSI generation model output before quantization.

Proposal 7: If vector quantization is supported in AI based CSI feedback, the input/output size of vector quantization codebook should be specified.

Proposal 8: When domain transformation pre-processing is used, legacy CSI report principle can be applied to input CSI directly.

Proposal 9: When domain transformation pre-processing is not used,

- Prioritization rule is indirectly support by selecting different AI model with different UCI bit size.

- CBSD can be supported by projecting the input CSI in the subspace orthogonal to restricted sub-space before AI model.

## Proposal 10: For CSI prediction use case, potential specification impact including UE capability signaling, UE request and NW activation/de-activation signaling.

Observation 1:

Table I: Comparison of different training collaboration

|  | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | UE side | NW side |  | UE first | NW first |
| Model proprietary | No | No | Yes | Yes | Yes |
| Require privacy data sharing | No | UE share dataset to NW | UE share target CSI to NW | UE share target CSI and encoder output to NW | UE share dataset to NW. NW share encoder training dataset |
| Flexibility to support config specific model | Yes, with NW assisted info | Yes | No. | Yes, with NW assisted info | Yes |
| Model upgrade flexibility | Flexible | Most flexible | Not flexible | Flexible | Flexible |
| Develop separately | Yes | Yes | No | Yes | Yes |
| Single model at gNB | No | Yes | Difficult | Yes | Yes |
| Single model at UE | Yes | Yes via frequent download | Difficult | Yes | Performance might degrade |
| Extendibility | Limited | Limited | Limited | Limited | Limited |
| Device specific training data | Yes | Yes, with UE assisted info | Yes | Yes | Yes, with UE assisted info |
| Device capability considered in training | Yes | Yes | Yes | Yes | Yes |

# Reference

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN meeting #94, Electronic meeting, Dec 6-17, 2022

- Chairman’s notes, 3GPP TSG RAN1 #109-e Meeting, May 2022.

- Chairman’s notes, 3GPP TSG RAN1 #110, Aug 2022.

- Chairman’s notes, 3GPP TSG RAN1 #110bis-e, Oct 2022

- Chairman’s notes, 3GPP TSG RAN1 #112, Feb 2023