3GPP TSG RAN WG1 #112	                                                                                  R1-2303479

Athens, Greece, February 27th – March 3rd, 2023

Agenda Item:	9.2.4.1

Source:	Apple Inc.

Title:	On Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion/Decision

# Introduction

In the SID governing the AI/ML study, an initial set of use cases has been decided on for positioning accuracy enhancements in different scenarios including, e.g., those with heavy NLOS conditions, with  the following objective:

- evaluate the performance benefits of AI/ML-based algorithms with a methodology based on statistical models for link and system simulations.

In this contribution, we provide some preliminary performance evaluation results for the direct AI ML positioning method based on the evaluation methodology and parameters agreed on in RAN1.

# Discussion

## Evaluation Methodology

In RAN1 #109-e  and RAN #110, the following agreements were made on the evaluation methodology [4]:

Agreement Companies are encouraged to provide evaluation results for: - Direct AI/ML positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - AI/ML assisted positioning - Companies are encouraged to describe at least the following implementation details for the evaluation - details of the channel observation used as the input of the AI/ML model inference (e.g., type and size of model input), model input acquisition and pre-processing - details of the output of the AI/ML model inference, how the AI/ML model output is used to obtain the UE’s location Agreement When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used. - If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference. - If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference. Agreement As a starting point, the training, validation and testing dataset are from the same large-scale and small-scale propagation parameters setting. Subsequent evaluation can study the performance when the training dataset and testing dataset are from different settings. Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyse are: {50%, 67%, 80%, 90%}. Agreement For evaluation of AI/ML based positioning, the computational complexity can be reported via the metric of floating point operations (FLOPs). - Note: For AI/ML assisted methods, computational complexity for the AI/ML model is only one component of the overall complexity for estimating the UE’s location. - Note: Other metrics to measure the computational complexity are not precluded. Agreement For evaluation of AI/ML based positioning, the model complexity is reported via the metric of “number of model parameters”.

In RAN1 #111, the following agreement was made:

Agreement Study how AI/ML positioning accuracy is affected by: user density/size of the training dataset. Note: details of user density/size of training dataset to be reported in the evaluation. Agreement For both direct and AI/ML assisted positioning methods, investigate at least the impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model. - The fine-tuning data is the training dataset from the target deployment scenario. Agreement For AI/ML based positioning, company optionally evaluate the impact of at least the following issues related to measurements on the positioning accuracy of the AI/ML model. The simulation assumptions reflecting these issues are up to companies. - SNR mismatch (i.e., SNR when training data are collected is different from SNR when model inference is performed). - Time varying changes (e.g., mobility of clutter objects in the environment) - Channel estimation error Agreement For AI/ML assisted approach, study the performance of model monitoring metrics at least where the metrics are obtained from inference accuracy of model output. Agreement For AI/ML assisted positioning, evaluate the three constructions: - Single-TRP, same model for N TRPs - Single-TRP, N models for N TRPs - Multi-TRP (i.e., one model for N TRPs) Note: Individual company may evaluate one or more of the three constructions.

In RAN1 #112, the following agreements were made :

Agreement For both direct AI/ML positioning and AI/ML assisted positioning, companies include the evaluation area in their reporting template, assuming the same evaluation area is used for training dataset and test dataset.

## Data Set Generation

We generate the dataset based on agreements in RAN1 #109-e using the IIoT  InF-DH scenario based on the following agreements with detailed assumptions in the Appendix:

Agreement The IIoT indoor factory (InF) scenario is a prioritized scenario for evaluation of AI/ML based positioning. Agreement For evaluation of AI/ML based positioning, at least the InF-DH sub-scenario is prioritized in the InF deployment scenario for FR1 and FR2. Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area. Agreement If spatial consistency is enabled for the evaluation, companies model at least one of: large scale parameters, small scale parameters and absolute time of arrival, where - the large scale parameters are according to Section 7.5 of TR 38.901 and correlation distance = for InF (Section 7.6.3.1 of TR 38.901) - the small scale parameters are according to Section 7.6.3.1 of TR 38.901 - the absolute time of arrival is according to Section 7.6.9 of TR 38.901 Agreement If spatial consistency is enabled for the evaluation of AI/ML based positioning, the baseline evaluation does not incorporate spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). - It is optional to implement spatially consistent UT/BS mobility modelling (Section 7.6.3.2 of TR 38.901). Agreement When providing evaluation results for AI/ML based positioning, participating companies are expected to describe data labelling details, including: - Meaning of the label (e.g., UE coordinates; binary identifier of LOS/NLOS; ToA) - Percentage of training data without label, if incomplete labeling is considered in the evaluation - Imperfection of the ground truth labels, if any Agreement For evaluation of AI/ML based positioning, study the performance impact from availability of the ground truth labels (i.e., some training data may not have ground truth labels). The learning algorithm (e.g., supervised learning, semi-supervised learning, unsupervised learning) is reported by participating companies.

## AI Model and Input for Direct AI/ML positioning

We evaluate the direct AI/ML positioning technique in which parameters from multiple gNBs serve as inputs into a neural network that directly estimates the UE position coordinates. Examples of these inputs could include the Channel Impulse Response (CIR), the Power Delay Profile (PDP), and/or the Layer 1 Reference Signal Received Power (L1-RSRP) and what (if any) pre-processing is needed. The specific input  may have an impact on the specification. Figure 1 illustrates a block diagram with the AI model, its input and output

Figure 1: High Level AI model

We assume a one-sided AI-model with model inference at either the UE-side (UE-based positioning) or at the network side (LMF based/UE assisted positioning or LMF based/NG-RAN assisted positioning).  Note that the training can take place at either side with the resulting model used at the side at which it is trained or  transferred to the other side.

The complexity of the AI model is shown in the Table below:

Table 1: Model Complexity

| AI model | FLOPs | Trainable Parameters |
| --- | --- | --- |
| CNN | 2.75G | 1,480,140 |

## AI Model and Input AI/ML-Assisted Positioning

We evaluate the AI/ML-assisted positioning technique in which parameters from multiple gNBs serve as inputs into a neural network that estimates the TOA of the different channels. The input is the normalized Channel Impulse Response (CIR). Figure 1 illustrates a block diagram with the AI model, its input and output

Figure 2: High Level AI model

We assume a one-sided AI-model with model inference at either the UE-side (UE-based positioning) or at the network side (LMF based/UE assisted positioning or LMF based/NG-RAN assisted positioning).  Note that the training can take place at either side with the resulting model used at the side at which it is trained or  transferred to the other side. We assume a multi-TRP model with 1 model for 18 TRPs.

The complexity of the AI model is shown in the Table below:

Table 2: Model Complexity

| AI model | FLOPs(×106) | Trainable Parameters (×106) |
| --- | --- | --- |
| CNN | 2.75G | 1,480,140 |

## KPIs or Metrics

In RAN1 #109-e, the following agreement was made

Agreement For all scenarios and use cases, the main KPI is the CDF percentiles of horizonal accuracy. - Companies can optionally report vertical accuracy. Agreement The CDF percentiles to analyze year: {50%, 67%, 80%, 90%}.

In RAN1 #110-bis-e, the following agreement was made:

Agreement For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - LOS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a link between UE and TRP; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - Companies provide info on how LOS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.)

## Generalization and Finetuning

In RAN1 #110, to study the generalization and finetuning of the AI model, the following agreements have been made

Agreement To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time. Agreement For both direct and AI/ML assisted positioning methods, investigate at least the impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model. - The fine-tuning data is the training dataset from the target deployment scenario.

Additional agreements were made in RAN1 #110-bis-e, as follows:

Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: (e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH) Agreement For AI/ML based positioning, if an InF scenario different from InF-DH is evaluated for the model generalization capability, the selected parameters (e.g., clutter parameters) are compliant with TR 38.901 Table 7.2-4 (Evaluation parameters for InF). - Note: In TR 38.857 Table 6.1-1 (Parameters common to InF scenarios), InF-SH scenario uses the clutter parameter {20%, 2m, 10m} which is compliant with TR 38.901.

In RAN1 #111, the following agreement was made:

Agreement For AI/ML assisted approach, for a given AI/ML model design (e.g., input, output, single-TRP vs multi-TRP), identify the generalization aspects where model fine-tuning/mixed training dataset/model switching is necessary

# Evaluation Results

The synthetic dataset used in this contribution is generated with the following options:

Table 3: General Assumptions/Options

| Scenario | InF-DH |
| --- | --- |
| Training/Validation Data Set Size | 47500, 23750, 9500 |
| Testing Data Set Size | 2500, 1250, 500 |
| UE Distribution | uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area |
| Spatial Consistency | We model spatial consistency for the Large Scale Parameters, Small scale Parameters and Absolute Time of Arrival We do not model spatial consistency for UT/BS mobility modeling |
| Input Data | Direct AI Model: CIR ~ [18 x 256 x 2] ~ [# gNBs x # taps x Real/Imaginary] The # of gNBs indicates the channel from each gNB. The # of taps is truncated if greater than 256 The third dimension captures the real and imaginary values of the channel taps NOTE: CIR for each gNB is the CIR of {1Tx x 1Rx} pair from the {32 Tx x 4 Rx} channel based on the channel assumptions AI assisted model: Same input but normalized to a power of 1. |
| Ground Truth Label | Perfect UE coordinates with 100% of the training data labeled Perfect TOA of UE from gNB |
| Synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |
| Learning Algorithm | Supervised learning |
| Generalization | - Different drops - Different clutter parameters - gNB synchronization error - InF scenario (InF-SH) One aspect at a time is evaluated |

## Direct AI/ML Positioning: Baseline, Generalization and Finetuning

In the table below, we evaluate the baseline direct AI/ML positioning performance without model generalization.

Table 4: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, with a CNN, UE distribution area = 100x40 m

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 | Drop 1 | 47500 | 2500 | 1,480,140 | 2.75G | 0.884m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 | Drop 1 | 23750 | 1250 | 1,480,140 | 2.75G | 1.234m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 | Drop 1 | 9500 | 500 | 1,480,140 | 2.75G | 1.860m |

In the table below, we evaluate the direct AI/ML positioning performance with model generalization for scenarios with different drops, clutter, model and synchronization.

Table 5: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE or Network side, with model generalization, with a CNN, UE distribution area = 100x40 m

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 | Drop 2 | 47500 | 2500 | 1,480,140 | 2.75G | 4.665m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | 60%,6,2 | 40%,2,2 | 47500 | 2500 | 1,480,140 | 2.75G | 5.076m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Ideal n/w synch | Non-ideal network sync | 47500 | 2500 | 1,480,140 | 2.75G | 10.9103m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | InF-DH | InF-SH | 47500 | 2500 | 1,480,140 | 2.75G | 6.037m |

In the table below, we evaluate the direct AI/ML positioning performance with model finetuning for scenarios with different drops, clutter, model and synchronization

Table 6: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE or Network side, with model finetuning, with a CNN, UE distribution area = 100x40 m

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop1 | Drop 2 | Drop2 | 47500 | 5000 | 2500 | 1,480,140 | 2.75G | 3.69m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop1 | Drop 2 | Drop2 | 47500 | 2500 | 2500 | 1,480,140 | 2.75G | 4.02m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | 60%,6,2 | 40%,2,2 | 40%,2,2 | 47500 | 5000 | 2500 | 1,480,140 | 2.75G | 4.41m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | 60%,6,2 | 40%,2,2 | 40%,2,2 | 47500 | 2500 | 2500 | 1,480,140 | 2.75G | 4.81m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Ideal n/w synch | Non-ideal network sync | Non-ideal network sync | 47500 | 5000 | 2500 | 1,480,140 | 2.75G | 8.22m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Ideal n/w synch | Non-ideal network sync | Non-ideal network sync | 47500 | 2500 | 2500 | 1,480,140 | 2.75G | 8.62m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | InF-DH | InF-SH | InF-SH | 47500 | 5000 | 2500 | 1,480,140 | 2.75G | 5.08m |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | InF-DH | InF-SH | InF-SH | 47500 | 2500 | 2500 | 1,480,140 | 2.75G | 4.8m |

Observation 1

- Direct AI based positioning shows good performance in the baseline case

- Direct AI ML does not show good generalization performance

- Finetuning improves the performance with more improvement as the data size increases.

## AI/ML Assisted Positioning: Baseline, Generalization and Finetuning

In the table below, we evaluate the baseline AI/ML-assisted Positioning (TOA estimation) performance without model generalization.

Table 7:  AI/ML-assisted Positioning (TOA estimation): Evaluation results for AI/ML model deployed on UE or Network-side, without model generalization, with a CNN, UE distribution area = 100x40 m

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Drop 1 | Drop 1 | 47500 | 2500 | 1.6 | 3.1 | 1.138m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Drop 1 | Drop 1 | 23750 | 1250 | 1.6 | 3.1 | 1.396m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Drop 1 | Drop 1 | 9500 | 500 | 1.6 | 3.1 | 1.748m |

For the AI-ML assisted method, the mean absolute TOA error with a data set of size 47500 is 1.22 ns equivalent to 0.36 meters.

In the table below, we evaluate the AI/ML-assisted Positioning (TOA estimation) performance with model generalization for scenarios with different drops, clutter, model and synchronization.

Table 8: AI/ML-assisted Positioning (TOA estimation): Evaluation results for AI/ML model deployed on UE/network-side, model generalization, with a CNN

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Drop 1 | Drop 2 | 47500 | 2500 | 1,480,140 | 2.75G | 3.3219m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | 60%,6,2 | 40%,2,2 | 47500 | 2500 | 1,480,140 | 2.75G | 3.9532m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Ideal n/w synch | Non-ideal network sync | 47500 | 2500 | 1,480,140 | 2.75G | 11.0172m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | InF-DH | InF-SH | 47500 | 2500 | 1,480,140 | 2.75G | 3.8203m |

In the table below, we evaluate the AI/ML-assisted Positioning (TOA estimation) performance with model finetuning for scenarios with different drops, clutter, model and synchronization

Table 9: AI/ML-assisted Positioning (TOA estimation):  Evaluation results for AI/ML model deployed on UE or Network side, with model finetuning, with a CNN, UE distribution area = 100x40 m

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Drop1 | Drop 2 | Drop2 | 47500 | 2500 | 2500 | 2.43 | 5.12 | 1.5884m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Drop1 | Drop 2 | Drop2 | 47500 | 2500 | 2500 | 2.43 | 5.12 | 1.8724m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | {60%,6,2} | {40%,2,2} | {40%,2,2} | 47500 | 5000 | 2500 | 2.43 | 5.12 | 2.5522m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | {60%,6,2} | {40%,2,2} | {40%,2,2} | 47500 | 2500 | 2500 | 2.43 | 5.12 | 2.7036m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Ideal n/w synch | Non-ideal network sync | Non-ideal network sync | 47500 | 5000 | 2500 | 2.43 | 5.12 | 4.3411m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | Ideal n/w synch | Non-ideal network sync | Non-ideal network sync | 47500 | 2500 | 2500 | 2.43 | 5.12 | 5.3007m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | InF-DH | InF-SH | InF-SH | 47500 | 5000 | 2500 | 2.43 | 5.12 | 1.8826m |
| CIR [18,1,256,2] | Direct path ToA [1x1] | Ideal TOA 100% labeled | InF-DH | InF-SH | InF-SH | 47500 | 2500 | 2500 | 2.43 | 5.12 | 2.1166m |

Observation 2

- AI-ML assisted positioning shows good performance in the baseline case

- AI-ML assisted positioning shows better generalization performance than the direct AI/ML case

- Finetuning improves the performance with more improvement as the data size increases with

## Direct AI/ML Positioning: Data Size

In RAN1 #112, the following agreement was made [8]:

Agreement For both the direct AI/ML positioning and AI/ML assisted positioning, study the model input, considering the tradeoff among model performance, model complexity and computational complexity. - The type of information to use as model input. The candidates include at least: time-domain CIR, PDP. - The dimension of model input in terms of NTRP, Nt, and Nt’. - Note: For the direct AI/ML positioning, model input size has impact to signaling overhead for model inference.

We evaluate the direct AI/ML positioning performance assuming the N TRPs with the highest RSRP are used in the model where N = 4,9 or 18.

Table 10: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE or Network side, using different # of TRPs , with a CNN, UE distribution area = 100x40 m

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 18 TRP | Drop 1 18 TRP | 47500 | 2500 | 1,480,140 | 2.75G | 0.884m |
| CIR [9,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 9 TRP | Drop 1 9 TRP | 47500 | 2500 | 1,480,140 | 2.75G | 1.0629 |
| CIR [4,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 4 TRP | Drop 1 4 TRP | 47500 | 2500 | 1,480,140 | 2.75G |  |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 18 TRP | Drop 1 18 TRP | 23750 | 1250 | 1,480,140 | 2.75G | 1.234m |
| CIR [9,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 9 TRP | Drop 1 9 TRP | 23750 | 1250 | 1,480,140 | 2.75G | 1.2960 |
| CIR [4,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 4 TRP | Drop 1 4 TRP | 23750 | 1250 | 1,480,140 | 2.75G | 1.8235 |
| CIR [18,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 18 TRP | Drop 1 18 TRP | 9500 | 500 | 1,480,140 | 2.75G | 1.860m |
| CIR [9,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 9 TRP | Drop 1 9 TRP | 9500 | 500 | 1,480,140 | 2.75G | 1.9717m |
| CIR [4,1,256,2] | UE coordinates [1x2] | 2-D UE position 100% labeled | Drop 1 4 TRP | Drop 1 4 TRP | 9500 | 500 | 1,480,140 | 2.75G | 2.7403m |

The # of TRPs affects the performance with improved performance with an increased # of TRPs.

- The difference between 18 and 9 TRPs is minimal

- The difference between 9 and 4 TRPs is quite large

Note that the performance loss can be regained by using a mix of CIRs and RSRPs as demonstrated in

Observation 3: There are trade-offs in the size/type of input to the model and the positioning performance.

Proposal: Study the performance using mixed inputs

# Conclusion

In this contribution, we have evaluation methodology for AI_ML for positioning and presented some preliminary results showing the benefits of AI-based positioning in high NLOS scenarios. We also have made the following observations:

Observation 1

- Direct AI based positioning shows good performance in the baseline case

- Direct AI ML does not show good generalization performance

- Finetuning improves the performance with more improvement as the data size increases.

Observation 2

- AI-ML assisted positioning shows good performance in the baseline case

- AI-ML assisted positioning shows better generalization performance than the direct AI/ML case

- Finetuning improves the performance with more improvement as the data size increases with

Observation 3: There are trade-offs in the size/type of input to the model and the positioning performance.

Proposal: Study the performance using mixed inputs

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- 3GPP TR 38.857, Study on NR Positioning Enhancements; (Release 17)

- 3GPP TR 398.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

- Chairman’s Notes, RAN1 #109-e, May 2022.

- Chairman’s Notes, RAN1 #110, August 2022

- Chairman’s Notes, RAN1 #110-bis-e, October 2022

- Chairman’s Notes, RAN1 #111, November 2022

- Chairman’s Notes, RAN1 #112, February 2023

- R1-2204242, Evaluation on AI/ML for positioning accuracy enhancement, May 2022, Apple

# Simulation Assumptions

|  |  | FR1 Specific Values |
| --- | --- | --- |
| Channel model |  | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |
|  | Room height | 10m |
| Total gNB TX power, dBm |  | 24dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation |
| gNB antenna radiation pattern |  | Single sector – Note 1 |
| Penetration loss |  | 0dB |
| Number of floors |  | 1 |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area is the convex hull of the horizontal BS deployment. |
| UE antenna height |  | Baseline: 1.5m |
| UE mobility |  | 3km/h |
| Min gNB-UE distance (2D), m |  | 0m |
| gNB antenna height |  | Baseline: 8m |
| Clutter parameters: {density , height ,size } |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} - Note: an individual company may treat {40%, 2m, 2m} as optional in their evaluation considering their specific AI/ML design. |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |