3GPP TSG RAN WG1 #112bis-e                                                                             R1-2303480

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.4.2

Source:	Apple Inc.

Title:	On Other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion/Decision

# Introduction

In the SID governing the AI/ML study, an initial set of use cases has been decided on for positioning accuracy enhancements in different scenarios including, e.g., those with heavy NLOS conditions with  the following objective [1]:

- finalize representative sub use cases for each use case for characterization and baseline performance evaluations.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

In this contribution, we look at the the potential specification impact of AI/ML for positioning accuracy enhancement.

# Specification Impact for AI/ML for Positioning Accuracy Enhancement

In this contribution, we look at the potential specification impact of AI/ML for positioning accuracy enhancement based on the steps that would have to occur in an AI/ML procedure and on the use cases for AI/ML positioning accuracy enhancemnets agreed to in RAN1 #110-bis-e as follows:

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

From the agreements so far in the study we can look at an AI/ML procedure as consisting of the following:

- Model Capability, Indication and  Configuration

- Model training and Data collection

- Model Inference,  Monitoring and Monitoring Response

We will study the potential specification impact of AI/ML for positioning accuracy enhancement based on these steps.

# Specification Impact of an AI/ML procedure

## Model Capability, Indication and Configuration

The AI/ML Model Capability, Indication and Configuration step performs the initial AI/ML model(s) setup and the potential specification impacts need to be identified and includes assistance signalling and procedures (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection). Discussion of the details should wait till a framework has been decided on in the general session.

Proposal 1: Discussion of the details Model Capability, Indication and Configuration   should wait till the a framework has been decided on in the general session.

## Model Training and Data Collection

Model training issues have been captured in the following agreements.

In RAN1 #109-e, the following agreement was made [3]:

Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection

In RAN1 #110, the following agreement were made [4]:

Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded

In RAN1 110-bis-e, the following agreements were made [5]:

Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signalling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

In RAN1 111, the following agreements were reached [6]:

Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified for further study - For direct AI/ML positioning, ground truth label is UE location - PRU with known location - UE generates location based on non-NR and/or NR RAT-dependent positioning methods - LMF generates UE location based on positioning methods - LMF with known PRU location - Note: user data privacy needs to be preserved - For AI/ML assisted positioning, ground truth label is one or more of the intermediate parameter(s) corresponding to AI/ML model output - PRU generates label directly or calculates based on measurement/location - UE generates label directly or calculates based on measurement/location - Network entity generates label directly or calculates based on measurement/location - The following options of entity to generate other training data at least measurement corresponding to model input are identified for further study - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: other options of entity to generate other training data are not precluded - Note: Existing PRU definition is in 38.305 Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, study benefits, feasibility and potential specification impact (including necessity) for the following aspects - Request/report of training data - Ground truth label - Measurement corresponding to model input - Associated information of ground truth label and/or measurement corresponding to model input - Assistance signaling and procedure to facilitate generating training data - Reference signal (e.g., PRS/SRS) configuration(s) and configuration identifier - Assistance information, e.g., between LMF and UE/PRU, for label calculation/generation, and label validity/quality condition, etc. - Note1: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed - Note2: Study may consider different entity to generate training data as well as different types of training data when applicable - Note3: study considers both of the following cases when applicable - when the training entity is the same entity to generate training data - when the training entity is not the same entity to generate training data

In RAN1 #112, the following agreement was made [7]:

Agreement Regarding training data generation for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods - Network entity generates ground truth label based on positioning methods - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study Agreement Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects - Associated information of training data - Quality indicator at least for ground truth label (if needed) - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc. - Assistance signaling and procedure to facilitate generating/collecting training data - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signaling other than above 2 for data collection - E.g., requested quality of training data

Model Training and Data Collection

- training data type/size: Given the current sub-use cases selected, RAN1 should allow for flexibility in the data type needed e.g. for direct AI/ML based positioning, the CIR, L1-RSRP and PDP can be used.

- Measurement Entity (e.g., UE/PRU/TRP): Training data may be sourced from different entitiesin the network. The specification impact depends on if the training/inference is at the UE or at the LMF/gNB as for example, it may determine the types of reference signals configured to acquire the training data. It may also depend on beam correspondence i.e. if channel is reciprocal and model is at the LMF/gNB. In the case of beam correspondence, the channel estimates at the gNB based on DMRS/SRS may be sufficient. In the case of no beam correspondence, new feedback types may be needed e.g. PDP, TOA.  Support may be needed to enable a central data collection entity transfer data to the training entity.(e.g. from PRU to LMF to UE). NOTE: the quality of the measurement should be signaled only if the measurement entity is different from the inferencing/monitoring entity.

- Assistance signalling and procedure for training data collection : This depends on if the training/inference is at the UE or at the LMF/gNB. It may need LPP/NRPPa based signaling to trigger feedback of training data to the training device. In addition, an update to the LPP/NRPPa protocols may be needed to transfer any additional assistance information (e.g. scenario,  availability and quality of noisy ground truth labels etc.)

- Ground Truth Quality: The quality of the ground truth labels should be signaled to assist the selection and monitoring of the AI/ML model.  Typically, AI/ML training assumes perfect labeling but in reality, the labels may be noisy. For example, the use of large-scale image data labelled by crowdsourcing  or speech/phone data  labelled by users who may not have the expertise (or may be in a specific mood). For AI/ML based positioning, with Direct AI/ML positioning, the position label may depend on the accuracy of the UEs position. If the UE is mobile or the UE position estimate is wrong, the associated label may be wrong. For AI/ML assisted positioning, the  label may depend on the specific type. For LOS/NLOS identification, the NLOS probability of each tap may be in error. For TDoA/ToA/AoA/AoD estimation  identification, there may be an error in the estimate sent to the gNB. To mitigate the effect of a noisy ground truth, the measuring entity may feed back information on the quality /accuracy of the ground truth label (similar to the feedback for GNS/RAT-dependent Integrity assistance data). This information may be used to select the right model (model trained based on accuracy estimate) or right type of model (e.g. supervised, unsupervised, semi-supervised model).

- Overall Data Collection Procedure (All cases): Data collection entity is Positioning reference Unit (PRU) or UE with location finding capability.

  - Data collection entity may be stationary or mobile

  - Step 1: Configuration/setup indicates PRU capability and label accuracy

  - Step 2: LMF/AI-MF configures entity for data collection. This includes at least the reference symbol configuration, labels required, and assistance information required e.g. label accuracy. Note that entity (if UE) has to be explicitly configured and should opt into the data collection procedure.

  - Step 3: LMF triggers data collection instance

    - Data collection Instance type:

      - aperiodic, periodic, semi-persistent

      - opportunistic i.e. if UE is sending positioning feedback to data collection entity, it also sends additional data collection information

      - For positioning with a mobile UE, can occur at fixed location points

    - Data collection instance consists of three steps:

      - Measurement Signal: configuration, trigger, type (DL-PRS, UL-SRS, SL-PRS), detailed configuration (BW, time duration, additional parameters)

      - Measurement and processing: measurement gaps at UE, muting from different gNBs, measurement type/metric (Channel Impulse Response {CIR}, Power Delay Profile {PDP}, LOS/NLOS property - Line of Sight/Non-line of sight), TOA {Time of Arrival}, Angle of arrival), associated ground truth label

      - Feedback: to data aggregator (e.g. the LMF).

  - Step 4: Training entity collects training information from the data aggregator.

For Model Training and Data Collection

- Proposal 2: training data type/size: Given the current sub-use cases selected, RAN1 should allow for flexibility in the data type needed

- Proposal 3: Measurement Data (e.g. UE/PRU/TRP): Training data may be sourced from the actual network. The specification impact depends on if the training/inference is at the UE or at the LMF/gNB. It may also depend on beam correspondence  Support may be needed to enable a central data collection entity transfer data to the training entity.(e.g. from PRU to LMF to UE).

- Proposal 4: assistance signalling and procedure for training data collection : The specification impact depends on if the training/inference is at the UE or at the LMF/gNB. It may need positioning protocol based signaling to trigger feedback of training data to the training device. In addition, some assistance information may be needed for the availability and quality of noisy ground truth labels.

- Proposal 5: Ground Truth Quality: The quality of the ground truth labels should be signaled to assist the selection and monitoring of the AI/ML model.

- Proposal 6: Summary of specification impact for data collection:

|  | Ground Truth Label generation entity (RAN1 #112) | Measurement entity (RAN1 #112) | Measurement Data | Measurement quality (Ground Truth, measurement) |
| --- | --- | --- | --- | --- |
| Case 1 (Direct) | PRU | PRU/UE | CIR, PDP, L1-RSRP | Ground Truth |
| Case 1 (UE-assisted) | PRU | PRU/UE | CIR, PDP (TOA), CIR (NLOS) | Ground truth |
| Case 2a | PRU | PRU/UE | CIR (TOA, NLOS) | Ground Truth |
| Case 2b | LMF with know PRU location | PRU/UE | CIR, PDP, L1-RSRP | Ground Truth and measurement |
| Case 3a | Network entity with known PRU location | TRP | CIR (TOA, NLOS) | Ground Truth |
| Case 3b | LMF with know PRU location | TRP | CIR, PDP, L1-RSRP | Ground Truth and measurement |

## Model Inference,  Model Monitoring and Model Monitoring Response

In RAN1 #109-e, the following agreement was made [3]:

Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output

In RAN1 #110, the following agreement were made [4]:

[KO: Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded

In RAN1 110-bis-e, the following agreements were made [5]:

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded

In RAN1 111, the following agreements were reached [6]:

Agreement Regarding AI/ML model inference, to study and provide inputs on potential specification impact (including necessity and applicability of specifying AI/ML model input and/or output) at least for the following aspects for each of the agreed cases (Case 1 to Case 3b) in AI/ML based positioning accuracy enhancement - Types of measurement as model inference input - new measurement - existing measurement - UE is assumed to perform measurement as model inference input for Case 1, Case 2a and Case 2b; TRP is assumed to perform measurement as model inference input for Case 3a and Case 3b - Report of measurements as model inference input to LMF for LMF-side model (Case 2b and Case 3b) - For AI/ML assisted positioning, new measurement report and/or potential enhancement of existing measurement report as model output to LMF for UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a) - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - New and/or enhancement to existing assistance signaling - Note: whether such assistance signaling and procedure can be applied to other aspect(s) of AI/ML model LCM can also be discussed Agreement - Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on feasibility, potential benefits (if any) and potential specification impact at least for the following aspects - At least the following are identified for further study as potential data for calculating monitoring metric - If monitoring based on model output - E.g. , estimated UE location corresponding to model output for direct AI/ML positioning, estimated intermediate parameter(s) corresponding to model output for AI/ML assisted positioning, ground truth label corresponding to model inference output for both direct and AI/ML assisted positioning - If monitoring based on model input - E.g., measurement corresponding to model inference input - Note1: other type of potential data for model monitoring is not precluded - Note2: combination of one or more type of potential data for monitoring is not precluded - If a given type of data is necessary for calculating monitoring metric, study whether and if so - How an entity can be used to provide the given type of data for calculating monitoring metric - Companies are requested to report their assumption of the entity (or entities) used to provide the given type of data for calculating monitoring metric for each case - Potential signalling for provisioning of the given type of data for calculating associated monitoring metric - Potential assistance signaling and procedure to facilitate an entity providing data for calculating monitoring metric - Potential UE-network interaction - E.g., model monitoring decision indication between UE and network

In RAN1 #112, the following agreement was made [7]:

Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

The following discusses each of these elements in some detail:

- Monitoring Entity and Metric:  This may differ for direct AI positioning and AI-assisted positioning.

  - For direct AI positioning or AI-assisted monitoring based on the estimated position, an accurate position ground truth obtained by the traditional location services of a UE,  a PRU or a UE with GPS-based location services may be used  to calibrate the AI-based location and vice versa. As an example, the Monitoring location [X,Y] can be based on

    - any of the RAT-independent techniques (e.g. GNSS)

    - RAT-dependent techniques (e.g. TDOA) defined in 3GPP Rel-16 and Rel-17 or

    - a fixed UE at a known location.

  - For AI-assisted positioning, in addition to the estimated position, intermediate KPI outputs with a known intermediate KPI value such as a TOA of a PRU/UE with a known location m may be used as the monitoring reference.

  - For both direct AI positioning and AI-assisted positioning, the monitoring may be based on the properties and characteristics of the input e.g. a Doppler estimate on an input CIR may indicate the model is not appropriate.

- Input for Inferencing and Monitoring : model input acquisition and pre-processing for both inferencing and monitoring will depend on if the AI model is UE based, network based and on beam correspondence. In a simple example, for the direct in case 1, the LMF may indicate PRS configurations for the gNBs to transmit to the UE as shown in Figure 1. An additional input capturing the parameters needed for monitoring (e.g. the ground truth) may be needed.

Figure 1: Channel Acquisition for UE based inference/Monitoring

- Monitoring Response: The Monitoring error may serve as input into Monitoring response. This response may be result in an action by the system including fallback to traditional methods, model retraining, model re-tuning or model switching. It could also result in a network-UE interaction model monitoring decision indication between UE and network. For example, if the Monitoring error > Monitoring error threshold for time > Monitoring time duration, then an AI model update can be requested. The update signaling may have specification impact.

Model Inference,  Model Monitoring and Model Monitoring Response

- Proposal  7:  Monitoring Entity and Metric : For direct AI positioning or AI-assisted monitoring based on the estimated position, an accurate position ground truth obtained by the traditional location services of a UE,  a PRU or a UE with GPS-based location services may be used  to calibrate the AI-based location and vice versa.  For AI-assisted positioning, intermediate KPI outputs may be used as the monitoring reference. For both direct AI positioning and AI-assisted positioning, the monitoring may be based on the properties and characteristics of the input e.g. a Doppler estimate on an input CIR may indicate the model is not appropriate.

|  | Entity to derive monitoring metric (RAN1 #112) | Metric | Others (1) | Others (2) |
| --- | --- | --- | --- | --- |
| Case 1 (Direct) | UE, PRU | Ground Truth Difference |  | General statistics of input/output/scenario |
| Case 1 (UE-assisted) | UE, PRU | Ground Truth Difference | Location Difference |  |
| Case 2a | UE, PRU | Ground Truth Difference | Location Difference |  |
| Case 2b | LMF | Ground Truth Difference |  |  |
| Case 3a | gNB | Ground Truth Difference | Location Difference |  |
| Case 3b | LMF (no gNB) | Ground Truth Difference |  |  |

- Proposal 8: Input for Inferencing and Monitoring : model input acquisition and pre-processing will depend on if the AI model is UE based, network based and on beam correspondence. An additional input capturing the parameters needed for monitoring (e.g. the ground truth) may be needed.  A summary of the RS configuration and Feedback for the different cases is found below:

|  | RS Configuration | Feedback |
| --- | --- | --- |
| Case 1 (Direct) | PRS configuration | None |
| Case 1 (UE-assisted) | PRS configuration | None |
| Case 2a | PRS configuration | TOA, NLOS |
| Case 2b | PRS configuration | CIR, PDP, L1-RSRP, quality |
| Case 3a | SRS configuration | TOA, NLOS |
| Case 3b | SRS configuration | CIR, PDP, quality |

- Proposal 9: Update/ UE-network interaction: Monitoring Response: The Monitoring error may serve as input into Monitoring response. This response may be result in an action by the system including fallback to traditional methods, model retraining, model re-tuning or model switching. It could also result in a network-UE interaction model monitoring decision indication between UE and network.

# Conclusion

In this contribution, we provided our views on use cases and potential specification impacts on the  enhancement on AI/ML for positioning accuracy enhancement. Based on the discussion, we have the following proposals:

Proposal 1: Discussion of the details Model Capability, Indication and Configuration   should wait till the a framework has been decided in the general session.

For Model Training and Data Collection

- Proposal 2: training data type/size: Given the current sub-use cases selected, RAN1 should allow for flexibility in the data type needed

- Proposal 3: Measurement Data (e.g. UE/PRU/TRP): Training data may be sourced from the actual network. The specification impact depends on if the training/inference is at the UE or at the LMF/gNB. It may also depend on beam correspondence  Support may be needed to enable a central data collection entity transfer data to the training entity.(e.g. from PRU to LMF to UE).

- Proposal 4: assistance signalling and procedure for training data collection: The specification impact depends on if the training/inference is at the UE or at the LMF/gNB. It may need positioning protocol based signaling to trigger feedback of training data to the training device. In addition, some assistance information may be needed for the availability and quality of noisy ground truth labels.

- Proposal 5: Ground Truth Quality: The quality of the ground truth labels should be signaled to assist the selection and monitoring of the AI/ML model.

- Proposal 6: Summary of specification impact for data collection:

|  | Ground Truth Label generation entity (RAN1 #112) | Measurement entity (RAN1 #112) | Measurement Data | Measurement quality (Ground Truth, measurement) |
| --- | --- | --- | --- | --- |
| Case 1 (Direct) | PRU | PRU/UE | CIR, PDP, L1-RSRP | Ground Truth |
| Case 1 (UE-assisted) | PRU | PRU/UE | CIR, PDP (TOA), CIR (NLOS) | Ground truth |
| Case 2a | PRU | PRU/UE | CIR (TOA, NLOS) | Ground Truth |
| Case 2b | LMF with know PRU location | PRU/UE | CIR, PDP, L1-RSRP | Ground Truth and measurement |
| Case 3a | Network entity with known PRU location | TRP | CIR (TOA, NLOS) | Ground Truth |
| Case 3b | LMF with know PRU location | TRP | CIR, PDP, L1-RSRP | Ground Truth and measurement |

Model Inference,  Model Monitoring and Model Monitoring Response

- Proposal  7:  Monitoring Entity and Metric : For direct AI positioning or AI-assisted monitoring based on the estimated position, an accurate position ground truth obtained by the traditional location services of a UE,  a PRU or a UE with GPS-based location services may be used  to calibrate the AI-based location and vice versa.  For AI-assisted positioning, intermediate KPI outputs may be used as the monitoring reference. For both direct AI positioning and AI-assisted positioning, the monitoring may be based on the properties and characteristics of the input e.g. a Doppler estimate on an input CIR may indicate the model is not appropriate.

|  | Entity to derive monitoring metric (RAN1 #112) | Metric | Others (1) | Others (2) |
| --- | --- | --- | --- | --- |
| Case 1 (Direct) | UE, PRU | Ground Truth Difference |  | General statistics of input/output/scenario |
| Case 1 (UE-assisted) | UE, PRU | Ground Truth Difference | Location Difference |  |
| Case 2a | UE, PRU | Ground Truth Difference | Location Difference |  |
| Case 2b | LMF | Ground Truth Difference |  |  |
| Case 3a | gNB | Ground Truth Difference | Location Difference |  |
| Case 3b | LMF (no gNB) | Ground Truth Difference |  |  |

- Proposal 8: Input for Inferencing and Monitoring : model input acquisition and pre-processing will depend on if the AI model is UE based, network based and on beam correspondence. An additional input capturing the parameters needed for monitoring (e.g. the ground truth) may be needed. A summary of the RS configuration and Feedback for the different cases is found below:

|  | RS Configuration | Feedback |
| --- | --- | --- |
| Case 1 (Direct) | PRS configuration | None |
| Case 1 (UE-assisted) | PRS configuration | None |
| Case 2a | PRS configuration | TOA, NLOS |
| Case 2b | PRS configuration | CIR, PDP, L1-RSRP, quality |
| Case 3a | SRS configuration | TOA, NLOS |
| Case 3b | SRS configuration | CIR, PDP, quality |

- Proposal 9: Update/ UE-network interaction: Monitoring Response: The Monitoring error may serve as input into Monitoring response. This response may be result in an action by the system including fallback to traditional methods, model retraining, model re-tuning or model switching. It could also result in a network-UE interaction model monitoring decision indication between UE and network.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R1-2301341, Evaluation on AI/ML for positioning accuracy enhancement, Apple, RAN1 #112, February 2023

- Chairman’s Notes RAN1 #109-e, May 2022

- Chairman’s Notes RAN1 #110, August 2022

- Chairman’s Notes RAN1 #110-bis-e, October 2022

- Chairman’s Notes RAN1 #111, November 2022

- Chairman’s Notes RAN1 #112, February 2023