3GPP TSG RAN WG1 #112bis-e                                                              R1-2303481

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.3.1

Source:	Apple Inc.

Title:	On evaluations for NR duplex evolution

Document for:	Discussion/Decision

# Introduction

In RAN#94 e-meeting, a new study item, ‘Study on Evolution of NR Duplex Operation’, was approved to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum with some of objectives as follows [1]:

- Identify applicable and relevant deployment scenarios (RAN1)

- Develop evaluation methodology for duplex enhancement (RAN1)

In RAN1#109-e, evaluation scenarios and assumptions were discussed, where as a result of such discussions an LS was sent to RAN4, [2], on how to identify and model different sources of interference to the victim UE, due to UE-to-UE CLI, and victim gNB, due to gNB-to-gNB CLI and gNB self-interference. In the past RAN1 meetings, more detailed simulation assumptions were discussed whereas such of discussions couple of agreements are made. In this contribution, we share our evaluation results on duplex evolution for NR TDD, where the goal is to achieve an upper bound on the claimed system enhancements like coverage enhancement, and UL/DL system throughput.

# On system comparison between SBFD and non-SBFD

## Impact of SBFD antenna configuration on SBFD performance

In previous meetings and for the purpose of evaluations, RAN1 achieved couple of agreements on BS antenna configuration for SBFD vs legacy TDD operation. For instance, it was agreed in RAN1#109 that

Agreement

For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used.

- Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD.

- Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD.

- Companies report which option is assumed in their simulation.

Later in RAN1#110, in addition to a new option, more details were agreed for the above options, for BS antenna configuration for the purpose of evaluation and comparison between SBFD and legacy TDD. Here we should note that antenna configuration at the BS may directly impact the SBFD performance. More precisely, depending on which option is implemented at the BS, we may have loss of channel reciprocity, and wider beams in DL transmission and UL reception (which results lower effective SINR, worse DL/UL coverage, etc). In our view, any SBFD performance gain shall also precisely consider and model such loss due to BS antenna configuration.

Proposal 1: RAN1 to consider the impact of separate BS antennas for simultaneous RX/TX operation on SBFD performance, like loss of channel reciprocity, wider beams in DL transmission and UL reception, etc.

## UL transmission in SBFD versus non-SBFD slots

As it will be discussed in our companion contribution in [3], aggressive UE in uplink transmission within SBFD slots may have to apply different uplink power adjustment parameters (like P0, etc), than uplink transmission in legacy non-SBFD slots, just to manage better CLI to the victim nearby UE. Reduced PUSCH transmit power has direct impact on coverage and throughput.

Proposal 2: RAN1 to consider the impact of potentially reduced transmit power for uplink transmission on SBFD slots to SBFD performance.

# Deployment Scenario

Regarding the deployment cases, in RAN1#109-e it was agreed that

Agreement

For discussion purpose for evaluation, define the following deployment cases for SBFD:

- Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

- Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations.

- Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration.

  - Deployment Case 3-1: Only 1-layer is considered

  - Deployment Case 3-2: 2-layer is considered

- Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration.

Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2

Note: SBFD subband configuration is from gNB perspective.

In RAN1#110, it was agreed that

Agreement

For SBFD evaluation from RAN1 perspective, the evaluation assumptions that are specific for Deployment Case 2 and Case 3-1 can be discussed with low priority.

In RAN1#110b-e, it was agreed that

Agreement

For evaluation of dynamic/flexible TDD for the single operator case, consider the following scenarios:

- FR1

  - 1-layer scenario: Indoor office with dynamic TDD UL/DL assignment

  - (Optional) 1-layer scenario: Urban Macro with dynamic TDD UL/DL assignment

  - 2-layer Scenario B

    - Layer 1: Urban Macro

    - Layer 2: Indoor office or Indoor factory (companies to report which one is used)

      - Indoor factory is optional (Companies are to report the used layout.)

      - Regarding the Indoor office layer, reuse the Indoor office (InH) scenario (i.e., open office in Table 7.2-2 in TR38.901) and relevant channel model in TR38.901.

      - Regarding the Indoor factory layer, reuse the Indoor factory (InF) scenario (i.e., Table 7.2-4 in TR38.901) and relevant channel model in TR38.901.

      - FFS: consider only one indoor office/factory dropped in the whole network

  - Regarding 2-layer scenario, the two layers are deployed in the same carrier

    - Layer 1 uses legacy static TDD operation with DL dominant static TDD UL/DL configuration

    - Layer 2 uses one of the following options (companies to report which option is used)

      - Option 1: All gNBs in layer 2 use legacy static TDD operation with the same UL dominant static TDD UL/DL configuration

      - Option 2: All gNBs in layer 2 use dynamic TDD UL/DL assignment

      - Other options are not precluded and can be reported by companies

- FR2-1

  - 1-layer scenario: Indoor office with dynamic TDD UL/DL assignment

  - (Optional) 1-layer scenario: Dense Urban Macro layer with dynamic TDD UL/DL assignment

- For above scenarios, the following is assumed:

  - DL dominant static TDD UL/DL configuration: {DDDSU}, where S=[12D:2G:0U]

  - UL dominant static TDD UL/DL configuration: {DSUUU}, where S=[12D:2G:0U]

  - dynamic TDD UL/DL assignment: {FFFFF}, companies to report the guard symbols assumed in their simulation

    - other configurations for dynamic TDD are not precluded and can be reported by companies

Companies can submit results for other scenarios

Given that many aspects regarding full-duplex operation at gNB shall be evaluated, it is more beneficial to narrow down discussions based on a deployment case which majority of companies agree on it. In our view, deployment case 1, under FR-1 with 1-layer scenario, i.e., indoor office, should be prioritized. Solutions to enhance UL performance, the feasibility of duplex operation in a TDD band, the CLI impact to victim UE/gNB, and the achievable UL enhancement gain versus DL performance loss, shall focus on a scenario under Deployment Case 1 for which current specification fails.

Proposal 3: Prioritize scenarios for Deployment Case 1, for which assuming the current signaling available at the scheduler to avoid CLI, UE-to-UE CLI is still the most severe case.

Dynamic TDD was considered earlier in NR, where handling CLI and RIM were introduced in Rel-16. As part of the CLI WI in Rel-16, and for the adjacent channel co-existence, RAN4 investigated the performance degradation on the victim UE/gNB due to UE-toUE CLI and gNB-to-gNB CLI as a result of dynamic TDD operation at the aggressor node [3]. The evaluations reported in [3] are made for different scenarios: Urban Macro (both FR1 and FR2), Indoor (both FR1 and FR2), and Dense Urban (FR2 only), based on the simulation assumption given by TR 38.828, Sec. 5.2. It is observed that for adjacent channel co-existence, when higher BS power is assumed and/or the distance between aggressor UE and serving gNB is large (so aggressive UE has to transmit with high power), we may have performance degradation, respectively due to BS-to-BS CLI and UE-to-UE CLI [2].

Given that the CLI issue is more severe for the co-channel scenario, and the inter/intra-subband CLI is a common problem for both dynamic TDD and full-duplex operation at gNB, we have the following proposal:

Proposal 4: Full-duplex operation shall not be supported for macro-to-macro scenarios, at least for FR1.

# Initial results for co-channel CLI

## Same UL/DL resource ratio between Legacy TDD and SBFD

In this section, we run some system evaluations based on system level simulation assumptions provided in [4]. More precisely, we assume the office model with 12 BSs, each serving almost 10 UEs (UE to BS association is based on geometry). Total number of UEs in the system are assumed to be 120 UEs. We assume almost 60% of UEs are randomly marked as UL UEs, i.e., UEs with uplink traffic, while the rest of UEs are UEs marked as DL UEs. The purpose of this evaluation is to study an upper bound for system enhancement (in terms of UL throughput, coverage, etc) so in this setup we don’t mandate any special UE clustering where we assume UE dropping is done randomly, as clustering UEs increase the chance of severe UE-to-UE CLI. Aligned with the same purpose, we assume a TRP operating at SBFD is capable to keep the same number of Tx/Rx antenna elements as operating in legacy TDD (so no loss to SBFD operation in terms of antenna arrays). Further, to the favour of an upper bound, we assume no UE-to-UE nor gNB-to-gNB CLI. BS to BS distance is assumed to be 20m, following indoor office model in [2] and [4]. For FR1 and for indoor scenario, we assume no antenna gain, and no beamforming/directionality array gain, neither at UE nor at the BSs (same assumption is applied to both SBFD and legacy TDD). Further system assumptions are given in Appendix, mainly from [4], with some modifications to [4] shown in italic font, as explained in Appendix.

In the following we compare achievable UL and DL throughputs for between SBFD and legacy TDD, where we assume no UE-to-UE CLI, no gNB-to-gNB CLI and no SI at gNB for SBFD case (that is why we refer to the observations in this sub-section as ideal). For SBFD, we assume deployment Case 1 as agreed in RAN1#109-e for evaluation purpose (thus there is no intra sub-band CLI) that is

- (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

We also assume 60% of resources are allocated to UL UEs and 40% of resources are allocated to DL UEs for both SBFD configuration and legacy TDD configuration. For the sake of simplicity, we do not assume guard band for SBFD or guard time for legacy TDD.

Figure 1: Average (left) and 5% (right) DL Throughput

Figure 2: Average (left) and 5% (right) UL Throughput

From Fig. 1 and 2, the most significant gain in throughput is achieved for uplink cell-edge UE, which is almost 10%. For average uplink and downlink throughput no meaningful gain in throughput is achieved, which is expected given that same ratio of resources between legacy TDD and SBFD is assumed. It shall be noted though the observed 10% gain in UL system throughput is ideal, as no interference is modelled.

Observation: For indoor scenario with no CLI/SI at UE or gNB, UL throughput enhancement for cell-edge UEs is limited to 10%.

# Conclusion

In this contribution, we provided some evaluation results for co-channel UE-to-UE CLI. Based on what we discussed, the following proposals and observation are made:

any SBFD performance gain shall also precisely consider and model such loss due to BS antenna configuration.

Proposal 1: RAN1 to consider the impact of separate BS antennas for simultaneous RX/TX operation on SBFD performance, like loss of channel reciprocity, wider beams in DL transmission and UL reception, etc.

Proposal 2: RAN1 to consider the impact of potentially reduced transmit power for uplink transmission on SBFD slots to SBFD performance.

Proposal 3: Prioritize scenarios for Deployment Case 1, for which assuming the current signaling available at the scheduler to avoid CLI, UE-to-UE CLI is still the most severe case.

Proposal 4: Full-duplex operation shall not be supported for macro-to-macro scenarios, at least for FR1.

Observation: For indoor scenario with no CLI/SI at UE or gNB, UL throughput enhancement for cell-edge UEs is limited to 10%.

# References

- RP-213591, Moderator (CMCC), “SID Study on evolution of NR duplex operation”, Dec. 2021

- R1-2205543, Moderator (CMCC), “LS on interference modelling for duplex evolution”, May 2022

- R1-2303483, “Views on potential enhancements on dynamic TDD”, Apple, April 2023

- TR 38.828, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)

- TR 38.308, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release14)

# Appendix

Table 1. System layout for indoor (from [3, Table 5.2.1.3-1] with some modifications)

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | 2 BS, 2 UEs |
| Inter-site distance |  | 20m |
| BS antenna height |  | 3 m |
| UE location | Outdoor/indoor | Indoor |
|  | LOS/NLOS | See the description |
|  | UE antenna height | 1 m |
| UE distribution (horizontal) |  | See the description |
| Minimum BS - UE distance (2D) |  | 0 m |
| Channel model |  | Indoor Office |
| Shadowing correlation |  | NA |

Table 2. Pathloss model (from [3], Table 5.2.2.1-1)

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |

Table 3. Other assumptions

| Minimum UE-UE (2D) distance | Indoor-to-Indoor: 1m |
| --- | --- |
| Carrier frequency | 4GHz |
| Channel bandwidth | 100MHz |
| BS TX power | 24 dBm |
| UE TX power | 23 dBm |
| Path-loss model | BS-to-BS: InH-office [TR 38.803] BS-to-UE: InH-office [TR 38.803] UE-to-UE: InH-office [TR 38.803] |
| UE antenna | Omni |
| gNB antenna | [3, Table 5.2.3.2.3-1] |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |

Figure A.1: LLS for 230kbps UL Throughput