3GPP TSG- RAN WG1 Meeting #112-bis-e                                                      R1- 2303484

e-Meeting,  Apr 17th – Apr 26th, 2023

Agenda Item:	9.4.1.1

Source:	Apple Inc.

Title:	On channel access mechanism for sidelink on FR1 unlicensed spectrum

Document for:	Discussion/Decision

# Introduction

Sidelink in FR1 unlicenced bands (n46 and n96/n102) was approved [1]. The CCA aspects have been extensively discussed.

In this paper, we discuss remaining design aspects related to channel access.

# Discussion

## Type 1 channel access

Type 1 channel access is used for initiating UE to start a COT for sidelink transmission. 6 steps of type 1 channel access procedure can reuse similar procedure defined for Uu link.

Energy detection threshold is one key component of type 1 channel access procedure. In [2], EDT is defined as

In current 37.213, the energy detection threshold adaptation procedure is defined in section 4.2.3.

Similar procedure can be defined for sidelink including higher layer configured max EDT value, and max EDT calculated based on PC, MAX.

Proposal 1:    Type 1 EDT determination can use UL EDT as starting point. Consider both NW configured EDT and UE autonomously determined EDT based on PC,MAX.

## Type 2A/2B/2C channel access mechanism

Within COT, whether >25us gap is allowed has been discussed. For DL COT, greater than 25us gap is allowed, and transmission after gap can resume after type 2 LBT. However, for UL COT, greater than 25 us gap is not allowed and transmission after this gap need to perform type-1 LBT.

In RAN1 110, it was agreed that ≥25us gap is allowed for shared COT, where type 2A channel access procedure can be used. Other scenarios are FFS.

For sidelink unlicensed band operation, allowing gap greater than 25us within SL-COT should not be limited to COT sharing. When UE stop transmission temporarily, due to resource pool configuration, or SL SSB transmission configuration, or gap created to allow FDMed UEs to perform LBT, or new traffic arrival within the SL-COT, the UE should be able to resume transmission within the SL-COT after type 2A/2B/2C channel access procedure.

Proposal 2:    Type 2A/2B/2C SL channel access can be used for the COT initiating UE to resume transmission after gap within the COT, based on gap length.

## Resource selection mode 2 enhancement with type 1 CCA

It has been agreed that the existing mode 1 and mode 2 resource selection schemes are supported with type 1 CCA procedure. In legacy resource selection mode 2, the resource selection window and candidate resource set does not consider CCA impact as the system is designed for dedicated band. When type 1 CCA procedure is used together with resource selection mode 2, enhancement is required to avoid frequent CCA failure and packet drops.

For type-1 channel access procedure, the UE can start sensing the channel any time after traffic arrival at the buffer. For mode-2 resource selection, there are two options for resource selection and CCA interaction.

- Option 1: the UE can perform type 1 CCA first, then select resource on the RB sets that finish the CCA procedure.

- Option 2: the UE select resource first, then perform type 1 CCA on the RB sets that includes the selected resource.

There are pros/cons of each option, and different enhancement schemes can be used to improve performance.

### Resource selection on successful RB sets after type 1 CCA

Figure 1 shows an example of option 1 for single channel access, where the resource pool contains 3 RB sets. The UE can start independent type 1 CCA procedure on each RB sets and depending on which RB set finish the type 1 CCA first, then select resource on this RB set. The UE can perform one short LBT right before the transmission on the selected resource. The scheme has the benefit of higher CCA successful rate particularly if the resource pool contains multiple RB sets.

Figure 1:   Example of option 2, CCA first, and resource selection after type 1 CCA success.

For multi-channel case, similar options are observed. Since the PSSCH and PSCCH transmission only starts if all RB sets of the resource selection is cleared, option 1 can have higher transmission opportunity comparing to single channel case.

Proposal 3: For mode 2, UE can perform type 1 CCA first, then perform resource selection on RB sets with backoff counter N=0.

### Enhancement for resource selection before type 1 CCA

Figure 2 shows an example of option 2 for single channel access, where the resource pool contains 3 RB sets. The UE will first select a transmission resource following legacy resource selection procedure, then perform type 1 CCA on the RB sets that include the selected resource. The type 1 LBT can take long time to finish depending on contention window size and channel condition. If the selected resource is close in time, the chance of type 1 CCA failure is high. The type 1 CCA time should be considered in the resource selection window determination.

Figure 2:   Example of option 1. Resource selection first, then perform CCA

In legacy model 2 resource selection, the resource selection window is between (n+T1, n+T2), where 0≤T1≤Tproc,1; T2,min≤T2≤ Remaining packet delay budget (PDB). To consider the CCA time, T1 can be enhanced to include minimum CCA time, CCAmin≤T1≤ CCAmin + Tproc,1, where CCAmin is the CCA time required if the channel is clear, i.e,, TCCA_min = deferral duration + N*9us, where N is the random number generated for type-1 CCA.

T2,min can be enhanced to include expected average CCA time, so the UE choose ending window length large enough to accommodate CCA time. Since type 1 CCA procedure depends on many factors such as CW size, random number, and interference condition, the time to finish CCA procedure is random. In LAA and NR-U, channel occupancy rate was introduced to measure how busy the channel is statistically. The channel occupancy can be used to estimate the expected CCA time for resource selection window. The higher the channel occupancy rate is, the longer the expected CCA time.

Proposal 4: For mode 2, when resource selection is performed first, considering CCA time in the resource selection window determination.

Consistent LBT failure has been discussed within RAN1 and RAN2. RAN 2 decided that the L1 LBT failure indication granularity is per RB set. If an RB set trigger consistent LBT failure, MAC layer can indicate the RB set to PHY, and PHY can exclude the RB set with consistent LBT failure from the candidate resource set SA.

Proposal 5: For mode 2, when resource selection is performed first, exclude the RB set with consistent LBT failure from the candidate resource set SA.

## COT sharing

In RAN1 112, the following agreement are captured:

Agreement

- A responding UE over a shared COT can be:

  - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator

    - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE

    - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE

  - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator

    - FFS Limitations on what additional IDs may be included and how they may be indicated

Whether additional ID are supported in the COT sharing information is FFS.  The concept of additional ID assumes the COT initiating UE knows the traffic condition of other UEs, therefore can share the COT with those by adding additional IDs. There are many issues with the additional ID:

- The COT initiating UE does not know the traffic condition of other UEs, since there is no SR or BSR sent to the COT initiating UE.

- The additional ID are overhead in the SCI. But the UEs indicated by the additional ID may or may not transmit using the shared COT. Therefore, the benefit is not clear.

- There can be multiple COT initiating UEs (FDMed transmission). This will result in many UEs to share the COT, increasing collision probability.

- UEs with additional ID transmit with COT sharing might not meet regulation requirement.

Proposal 6: For COT sharing, do not support additional IDs in COT sharing information.

To enable COT sharing, the channel access information can be included as part of SCI stage 1 or stage 2. For COT sharing, the responding device should transmit only the traffic with priority class higher than the priority class used to acquire the COT, the transmission power to be used is also lower than the Tx power used in EDT calculation for channel access. In addition, the remaining COT duration to be shared should be also included in SCI so that the responding UE knows how long the transmission can be. In addition, SCI might include the CCA type and CP extension, so the responding UE can transmit with CP extension to ensure either 16us or 25us gap is used between transmissions.

Proposal 7: SCI includes sidelink COT sharing information such as access priority class/MCOT, remaining COT length, EDT, shared COT CCA type and CP extension index etc.

When UE receive the COT sharing information, UE will perform resource selection within the shared COT before transmission. The resource selection window length should be within the COT duration. In legacy model 2 resource selection, the resource selection window is between (n+T1, n+T2), where 0≤T1≤Tproc,1; T2,min≤T2≤ Remaining packet delay budget (PDB). In case of resource selection for COT sharing, max T2 should be within the remaining COT duration, i.e., T2≤ min(remaining COT duration, Remaining packet delay budget (PDB)).

Proposal 8: When the responding UE transmit within the shared COT, the resource selection window should be within the remaining COT length indicated in the initiating UE’s SCI.

## CPE and starting position

CPE starting position in SL-U operation as agreed in RAN1 111.

Agreement

A CPE can be transmitted from a CPE starting position before SL transmission for the following two options:

- Option 1: within the symbol just before the next AGC symbol

- Option 2:

  - within the symbol just before the next AGC symbol for 15 kHz SCS

  - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS

- FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

In eLAA/NR-U configured grant design, the starting point after type 1 channel access procedure is successful depends on whether this is full BW configuration, or partial BW configuration. In case of full BW configuration, random starting position is used, so if one UE starts the transmission, it will naturally block other UEs to start therefore avoiding collision. The random starting position and corresponding CPE length can follow similar design as NR-U, i.e., the maximum CPE length is one OFDM symbol length with 15KHz SCS. In case of partial BW, to avoid one UE transmission block another FDMed UE in the same 20MHz CCA bandwidth, the same starting position is used. For sidelink UE, similar mechanism can be used. For channels that FDMed together, one configured starting position can be used.

Proposal 9: One default CPE length is pre-configured in the resource pool, and the default CPE length is used in:

- PSCCH/PSSCH/PSFCH transmission in shared COT sharing

- PSFCH transmission outside of shared COT

- PSCCH/PSSCH transmission with partialBW outside of shared COT

Proposal 10: Multiple CPE can be used for UE initiated COT with fullBW transmission (fullBW).

- The starting position is randomly chosen from the sensing slot (9us) boundary with 1 (15KHz SCS) or 2 OFDM (30KHz and 60KHz) symbols.

In Uu link, channel access and CPE is signaled in DL and UL DCI for UL transmission. Channel access model such as type 1, type 2A/2B/2C and CPE is jointed configured using ChannelAccess-CPext field. Similar concept can be used in sidelink to enable dynamic indication of CPE length. If not indicated in SCI, the default configured CPE can be used.

Proposal 11:  CPE length and CCA type can be dynamically updated by SCI for shared COT.

The gap symbol in slot format is used for Tx/Rx transition and to perform CCA. The gap symbol length needs to be longer than the CCA type 2A to enable COT sharing. If 60KHz SCS is supported in SL-U, the OFDM symbol length of 60KHz SCS is shorter than 25us. To enable type 2A CCA, 2 gap symbol should be enabled for 60KHz SCS, as shown in Fig. 3.

Fig. 3: 2 gap symbol length for 60KHz SCS to allow 25us CCA time

Proposal 12:  For 60KHz SCS, to allow 25us CCA, extend the gap symbol to 2 symbol length.

## Multi-slot transmission

Support of multi-consecutive slot transmission is agreed in RAN1 110.

Agreement

Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U.

Multi-TTI PUSCH transmission in Rel-15 enables repetition of PUSCH transmission in time domain to support URLLC or extend UL coverage. For NR-U, multi-TTI PUSCH transmission targets to schedule multiple slots with different transport blocks using a single UL grant. The main NR-U multi-PUSCH design focus is to enable higher efficiency for larger traffic and increase UL transmission probability. For SL unlicensed multi-slots transmission, similar to NR-U, multi-TB transmission should be prioritized.

Proposal 13: Multi-slot transmission should prioritize multi-TB transmission.

Since multi-TTI SL transmission is optimized for large traffic, for SL transmission targeting for smaller payload size, it is better to transmit the smaller payload in one slot with larger frequency domain resource allocation, instead of extending the transmission in time domain. If multi-slot transmission is enabled with FDMed UEs, additional sensing gap needs to be created after each slot to allowed FDMed UE to sense the channel. However, the UE performing multi-slot transmission will have to sense the channel as well, and might fail the CCA, therefore cancelling the ongoing multi-slot transmission.  To avoid all the complicated design without clear potential benefit, we propose to limit the multi-slot transmission to the case where transmission on all the resource blocks of an RB set.

Proposal 14: For model 1 RA with CG and mode 2 RA, multi-slot transmission is enabled only for full BW transmission where all the resource blocks within an RB set is configured.

Options for PHY and MAC interaction have been discussed in RAN1 110bis-e.  Since multi-slot transmission is to enable continuous transmission in consecutive slots, Option 1 can be used when L1 is triggered for reporting a subset of candidate resources for MCSt, and option A is recommended when L1 reports a subset of candidate resources for MCSt

Proposal 15: For multi-slot transmission, option 1 and option A for L1 reporting of a subset of candidate resources.

## CW update

When two starting position, PSSCH transmission can be either one slot or partial slot. For CW computation, it is desirable to have a full slot as the reference duration to have consistent update.  Therefore, when multi-slot transmission is enabled, reference duration is from the starting point until the 1st full slot where PSSCH transmission happens, or burst end, whichever comes first (i.e., only partial slot transmission happens).

Proposal 16: If multi-slot transmission is enabled, reference duration is from the starting point until the 1st full slot where PSSCH transmission happens, or burst end, whichever comes first (i.e., only partial slot transmission happens).

# Conclusion

In the paper, we discuss the sidelink unlicensed band channel access design aspects. The proposals are:

Proposal 1:    Type 1 EDT determination can use UL EDT as starting point. Consider both NW configured EDT and UE autonomously determined EDT based on PC,MAX

Proposal 2:    Type 2A/2B/2C SL channel access can be used for the COT initiating UE to resume transmission after gap within the COT, based on gap length.

Proposal 3: For mode 2, UE can perform type 1 CCA first, then perform resource selection on RB sets with backoff counter N=0.

Proposal 4: For mode 2, when resource selection is performed first, considering CCA time in the resource selection window determination.

Proposal 5: For mode 2, when resource selection is performed first, exclude the RB set with consistent LBT failure from the candidate resource set SA.

Proposal 6: For COT sharing, do not support additional IDs in COT sharing information.

Proposal 7: SCI includes sidelink COT sharing information such as access priority class/MCOT, remaining COT length, EDT, shared COT CCA type and CP extension index etc.

Proposal 8: When the responding UE transmit within the shared COT, the resource selection window should be within the remaining COT length indicated in the initiating UE’s SCI.

Proposal 9: One default CPE length is pre-configured in the resource pool, and the default CPE length is used in:

- PSCCH/PSSCH/PSFCH transmission in shared COT sharing

- PSFCH transmission outside of shared COT

- PSCCH/PSSCH transmission with partialBW outside of shared COT

Proposal 10: Multiple CPE can be used for UE initiated COT with fullBW transmission (fullBW).

- The starting position is randomly chosen from the sensing slot (9us) boundary with 1 (15KHz SCS) or 2 OFDM (30KHz and 60KHz) symbols.

Proposal 11:  CPE length and CCA type can be dynamically updated by SCI for shared COT.

Proposal 12:  For 60KHz SCS, to allow 25us CCA, extend the gap symbol to 2 symbol length.

Proposal 13: Multi-slot transmission should prioritize multi-TB transmission.

Proposal 14: For model 1 RA with CG and mode 2 RA, multi-slot transmission is enabled only for full BW transmission where all the resource blocks within an RB set is configured.

Proposal 15: For multi-slot transmission, option 1 and option A for L1 reporting of a subset of candidate resources.

Proposal 16: If multi-slot transmission is enabled, reference duration is from the starting point until the 1st full slot where PSSCH transmission happens, or burst end, whichever comes first (i.e., only partial slot transmission happens).

# Reference

- RP-221798, “WID revision: NR sidelink evolution,” June. 2022.

- ETSI EN 301 893, 5 GHz RLAN; Harmonized standard covering the essential requirements of article 3.2 of directive 2014/53/EU, v2.1.1, May 2017.

- TS 37.213, Physical layer procedures for shared spectrum channel access.