3GPP TSG RAN WG1 #112bis-e			R1-2303485

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.2

Source:	Apple

Title:		On Sidelink Physical Channel Design Framework for Unlicensed Spectrum

Document for:	Discussion/Decision

# Introduction

The physical channel design framework for sidelink on FR1 unlicensed spectrum has been extensively discussed in the past several RAN1 meetings [1]-[6].

In this contribution, we provide our views on the sidelink physical channel design framework on FR1 unlicensed spectrum.

# Discussion

## Resource pool

RAN1 #109-e Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

It was agreed to at least support one sidelink resource pool including integer number of RB sets. It is open whether to support one sidelink resource pool including subset of PRBs of one RB set.

If a resource pool includes a subset of PRBs of one RB set, then the PSCCH/PSSCH transmission may be via part of the RB set, which may violate the OCB requirement where the transmission needs to occupy at least 80% of the LBT channel bandwidth. Hence, it is preferred not to support that one sidelink resource pool is (pre-)configured to include a subset of RB set.

Proposal 1: Do not support that one sidelink resource pool is (pre-)configured to include a subset of RB set.

The time domain configuration of a resource pool was discussed in RAN1 #109-e meeting. It is open which slots belong to resource pool.

The time resources in the same carrier may be shared by multiple sidelink resource pools. For example, one resource pool occupies a part of slots in a DFN cycle (i.e., 10240 ms), while another resource pool occupies another part of slots in a DFN cycle. Hence, it is necessary to use a bitmap to indicate the time resources of a resource pool, in the same way as in NR sidelink on licensed spectrum.

Proposal 2: Use a bitmap to indicate the time domain resources of a resource pool.

### Intra-cell guard band

RAN1 #110b-e Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

It was agreed that PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets. It was further agreed that PRBs within intra-cell guard band of two adjacent RB sets can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels in multi-channel case and the UE uses both RB sets for PSSCH transmission. The details are open, including handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets.

In our view, the PRBs within intra-cell guard band can be allocated to the interlaces in one of the two adjacent RB sets, e.g., the RB set with lower frequency. These PRBs can be treated as an extension of the RB set when associating them with the corresponding interlaces. Figure 1 shows an example of the association of these PRBs with interlaces. There are 3 PRBs in the intra-cell guard band in the figure, which are allocated to RB set 1. There are 5 interlaces in RB set 1, where the last PRB (i.e., before intra-cell guard band) in RB set 1 belongs to interlace 1 (block box). Then the first PRB in the intra-cell guard band belongs to interlace 2 of RB set 1 (red box), the second PRB in the intra-cell guard band belongs to interlace 3 of RB set 1 (blue box), and the third PRB in the intra-cell guard band belongs to interlace 4 of RB set 1 (green box).

Figure 1: Illustration of intra-cell guard band

Proposal 3: If a resource pool contains two adjacent RB sets, all the PRBs within intra-cell guard band of two adjacent RB sets are associated with the interlaces in one of the two adjacent RB sets.

- These PRBs are treated as an extension of the allocated RB set when associating them with the corresponding interlaces.

When the PRBs within intra-cell guard band are used for PSSCH transmission, the rate matched bits of PSSCH are mapped to these PRBs within intra-cell guard band. It is worth examining whether the rate matched bits for SCI stage 2 are also mapped to these PRBs within intra-cell guard band.

Proposal 4: When the PRBs within intra-cell guard band are used for PSSCH transmission, the rate matched bits of PSSCH are mapped to these PRBs.

- FFS whether SCI stage 2 rate matched bits are mapped to these PRBs.

The association of PRBs within intra-cell guard band with interlaces causes certain interlaces having more PRBs than other interlaces. This leads to unequal sub-channel size. If initial transmission and re-transmission use two interlaces with different numbers of PRBs, then the sidelink TBS calculation based on the actual number of PRBs of an interlace will lead to TBS misalignment between initial transmission and re-transmission. To avoid this issue, a reference number of PRBs per interlace is used for the sidelink TBS determination. This reference number is (pre-)configured by resource pool.

Proposal 5: In calculating sidelink TBS, a reference number of PRBs per interlace is used, where this number is (pre-)configured by resource pool.

It is open whether the PRBs within intra-cell guard band are used for PSFCH transmission. In our view, each PSFCH transmission is restricted to a single RB set, and hence, the PRBs within intra-cell guard band are not used for PSFCH transmission.

Proposal 6: The PRBs within intra-cell guard band are not used for PSFCH transmission.

## Slot structure for 2 candidate starting symbols

RAN1 #110b-e Working assumption: Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission. - RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol - The candidate starting symbol(s) are intended for AGC purpose - FFS: other potential uses of the candidate starting symbol(s) - FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc. - FFS whether 2 candidate starting symbols is also supported for slots with PSFCH RAN1 #111 Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding the location of 1st starting symbol, down-select one of the followings: - Option 1: it is fixed as symbol#0 - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL - Regarding the location of 2nd starting symbol, down-select one of the followings: - Option A: it is a fixed location - FFS the location, e.g., symbol#4, #7, etc. - Option B: it is a (pre-)configured location per resource pool - FFS the details of candidate locations - Note: assume symbol index in a slot starts from #0 RAN1 #111 Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference symbol length, down-select one of the followings: - Option 1: The reference symbol length is dynamically indicated by Tx UE - Option 2: The reference symbol length is determined based on 1st starting symbol - Option 3: The reference symbol length is determined based on 2nd starting symbol - Option 4: The reference symbol length is (pre-)configured RAN1 #111 Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot RAN1 #111 Agreement Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH. RAN1 #112 Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 RAN1 #112 Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

It is working assumption [4] that the candidate starting symbol(s) are intended for AGC purpose. It is open whether a second candidate starting symbol is for AGC purpose if PSCCH/PSSCH transmission starting from the first candidate starting symbol.

If a PSCCH/PSSCH transmission starts from the first candidate starting symbol, then the chance that another PSCCH/PSSCH transmission starts from the second candidate starting symbol in the same slot is low since the LBT for the latter PSCCH/PSSCH transmission will likely fail.

However, this situation may not be fully avoided, e.g., in hidden node case. Suppose UE1 makes PSCCH/PSSCH transmission starting from the first candidate starting symbol. It is possible that UE2 makes PSCCH/PSSCH transmission starting from the second candidate starting symbol in the same slot, in case UE1 is a hidden node to UE2 and UE2’s LBT is successful without detecting UE1’s transmission. UE2’s transmission will lead to AGC issue to other UEs’ reception if the second candidate starting symbol is not used for AGC purpose.

Besides hidden node case, the situation where UE2 makes PSCCH/PSSCH transmission starting from the second candidate starting symbol while UE1 makes PSCCH/PSSCH transmission starting from the first candidate starting symbol may also occur if a resource pool contains more than 1 RB set.

Suppose UE1 and UE2 make PSCCH/PSSCH transmissions on two different RB sets of a resource pool. UE1 makes PSCCH/PSSCH transmission starting from the first candidate starting symbol and UE2 makes PSCCH/PSSCH transmission starting from the second candidate starting symbol of the same slot. This is possible since UE2’s LBT operation is performed only on the second RB set and it does not detect UE1’s PSCCH/PSSCH transmission on the first RB set. Considering a receiver UE’s AGC operation is performed per resource pool or per sidelink BWP, UE2’s transmission will lead to AGC issue to other UEs’ reception if the second candidate starting symbol is not used for AGC purpose. This is illustrated in Figure 2.

Figure 2: Illustration of AGC issue over different RB sets

To avoid this AGC issue which occurs only if a resource pool has more than 1 RB set, we propose that for a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from the first starting symbol, then a single AGC symbol is applied for PSCCH/PSSCH transmission in a slot in case a resource pool contains 1 RB set. Two AGC symbols are applied for PSCCH/PSSCH transmission in a slot in case a resource pool contains more than 1 RB set, where the second candidate starting symbol is used as the second AGC symbol. The second AGC symbol is the copy of its following symbol.

Proposal 7: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from the first starting symbol, then

- a single AGC symbol is applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains 1 RB set;

- two AGC symbols are applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains more than 1 RB set.

  - The second AGC symbol is the second candidate starting symbol, with contents being the copy of its following symbol.

It is working assumption that if a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, TBS is determined based on a reference number of symbols which is (pre-)configured, at least for COT initiation.

It is open whether a different reference number of symbols is needed in a shared COT. The main argument of using a different reference number of symbols in a shared COT is that PSCCH/PSSCH transmission in a shared COT is likely starting from the first candidate starting symbol. Hence, a more suitable reference number of symbols could be used.

However, applying Type 2 LBT before PSCCH/PSSCH does not guarantee there is no LBT failure. In other words, it is possible that PSCCH/PSSCH transmission in a shared COT starts from the second candidate starting symbol. Also, a receiver UE may not always know whether a PSCCH/PSSCH transmission is for COT initiation or in a shared COT. For example, it is possible that PSCCH/PSSCH initial transmission is for COT initiation and PSCCH/PSSCH re-transmission in a shared COT. This ambiguity at receiver UE prevents using a different reference number of symbols for COT sharing in sidelink TBS determination. Hence, we think a single reference number of symbols is (pre-)configured for sidelink TBS determination, no matter whether the PSCCH/PSSCH is for COT initiation or COT sharing.

The value of reference number of symbols is open. In our view, the reference number of symbols is lower bounded by 6, which is the minimum number of supported PSSCH symbols in a slot. The upper bound of reference number of symbols could be 12, which excludes AGC symbol and GAP symbol in a slot.

Proposal 8: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, sidelink TBS is determined based on a (pre-)configured reference number of symbols for both COT initiation and COT sharing. The value range of reference number of symbols is between 6 and 12.

In Rel-16/17 NR sidelink, the PSFCH occasion is configured to be at least 2 or 3 slots after PSCCH/PSSCH transmission. If PSCCH/PSSCH transmission starts from the second candidate starting symbol position, then the processing time constraints need to be examined. In this case, PSCCH decoding can only start after the second candidate starting symbol position, which will delay the corresponding PSSCH decoding, as well as the subsequent preparation of PSFCH transmission. To ensure there are enough processing time, it is preferred that if a resource pool supports 2 candidate starting symbol positions, then the PSFCH occasion is at least 3 slots after the corresponding PSCCH/PSSCH transmission.

Proposal 9: If a resource pool supports 2 candidate starting symbol positions, then the PSFCH occasion is at least 3 slots after the corresponding PSCCH/PSSCH transmission.

## PSCCH and PSSCH

### Sub-channel in interlace RB-based resource pool

RAN1 #110 Agreement For PSCCH and PSSCH in SL-U: - For interlace RB-based transmission - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission - 1 sub-channel equals K interlace - FFS: whether K is fixed as 1 or (pre-)configured - Discuss whether one or both of the following alternatives are supported - Alt 1: 1 sub-channel is confined within 1 RB set - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool RAN1 #110b-e Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding 1 sub-channel equals K interlace(s) - At least K=1 and K=2 is supported for 15 kHz SCS - At least K=1 is supported for 30 kHz SCS - FFS: details related to multiple RB sets RAN1 #110b-e Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details RAN1 #110b-e Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - Down-select one of the followings - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Option 2: Support explicitly indicating at least the used sub-channel index(s) - At least RB set index(s) is not explicitly indicated - FFS details RAN1 #111 Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, RAN1 further study the followings: - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set - FFS: whether/how to use intra-cell guardband PRBs RAN1 #112 Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

For interlace RB-based PSCCH/PSSCH transmission, it was agreed that a sub-channel is defined and indexed with 1 RB set and is periodically indexed across different RB sets within the resource pool.

With this definition, it is straightforward that a frequency domain resource indication for interlace RB-based PSSCH transmission is explicitly indicated by the used sub-channel index(s) and RB set index(s). This is similar to the NR-U rule.

Proposal 10: For interlace RB-based PSSCH transmission, the frequency domain resource indication is explicitly indicated by the used sub-channel index(s) and RB set index(s).

It is open when more than one RB set is used for PSCCH/PSSCH transmission, whether the used interlace indices in different RB sets are always the same or can be different.

In our view, if a resource pool contains multiple RB sets, a PSCCH/PSSCH transmission does not necessarily use a sub-channel over all the RB sets. On the other hand, if a PSCCH/PSSCH transmission uses a sub-channel in a subset of RB sets in a resource pool, then the sub-channel index(s) in the RB sets with PSCCH/PSSCH transmission are always the same.

Consider an example that a resource pool is composed of 3 RB sets. A PSCCH/PSSCH transmission may occupy a total of 4 sub-channels. In one case, these 4 sub-channels could be across 2 RB sets, where 2 sub-channels in each RB set is used. As illustrated in Figure 3, a PSCCH/PSSCH transmission uses sub-channel # 1 and #2 in RB set #0 and sub-channel #1 and #2 in RB set #1. This selection simplifies the frequency domain resource indication.

In principle, the selected sub-channels in an RB set has contiguous sub-channel index(s) and the selected RB sets for a PSCCH/PSSCH transmission has contiguous RB set index(s).

Figure 3: Illustration of interlace RB-based PSCCH/PSSCH transmissions.

Proposal 11: If a resource pool has multiple RB sets and a PSCCH/PSSCH transmission occupies more than one sub-channel, these sub-channels are allocated in a subset of RB sets in the resource pool, where these subset of RB sets has contiguous index(s).

- All these subset of RB sets have the same contiguous sub-channel index(s) for the PSCCH/PSSCH transmission.

In Rel-16/17 NR sidelink, PSCCH is restricted in a single sub-channel even if PSSCH transmissions occupy more than one sub-channel. This design is to facilitate PSCCH decoding. It was agreed that PSCCH is transmitted within a single sub-channel for SL-U.

The number of PRBs for PSCCH is configurable per resource pool to support different types of applications.

Proposal 12: The number of PRBs for PSCCH is configurable per resource pool.

### Sub-channel in contiguous RB-based resource pool

RAN1 #110b-e Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission RAN1 #111 Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, further study the following options: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size - FFS: whether/how to use intra-cell guardband PRBs - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size - FFS: how to use intra-cell guardband PRBs - FFS: how to use the subchannel including PRBs in guardband RAN1 #112 Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

It was agreed that both contiguous RB-based PSCCH/PSSCH transmissions and interlace RB-based PSCCH/PSSCH transmissions are supported. For contiguous RB-based PSCCH/PSSCH transmission, the mapping between sub-channel and PRBs was agreed in the way that sub-channel aligns with resource pool boundary.

Based on this definition, it is possible that the intra-cell guard band PRBs can be at every location of a sub-channel. In general, PSCCH occupies the lowest N PRBs of a sub-channel.

Based on the agreement [4] that intra-cell guard band PRBs are not used for PSCCH transmission, if the lowest N PRBs of a sub-channel have overlap with intra-cell guard band, then this sub-channel cannot be used to carry PSCCH. This implies that such a sub-channel cannot be used or selected as the first sub-channel (or, the sub-channel with the lowest sub-channel index for PSCCH/PSSCH transmission) for a PSCCH/PSSCH transmission.

Figure 4 illustrates sub-channel definition for contiguous RB-based resource pool. It is clear that sub-channel #5 cannot be selected as the first sub-channel since its lowest N PRBs have overlap with intra-cell guard band. It is possible that sub-channel #4 cannot be selected as the first sub-channel, depending on whether there is overlap between its lowest N PRBs and intra-cell guard band.

Figure 4: illustration of sub-channel definition for contiguous RB-based resource pool

Based on the above discussions, the resource selection procedure needs to be modified to avoid selecting a sub-channel as the first sub-channel for a PSCCH/PSSCH transmission if the lowest PRBs of this sub-channel have overlap with intra-cell guard band.

For example, if a UE is to select 2 sub-channels for its PSCCH/PSSCH transmission, it should avoid selecting sub-channels (#4, #5) or sub-channels (#5, #6) from the resource pool in Figure 4.

Proposal 13: For contiguous RB-based PSCCH/PSSCH transmission, resource selection procedure is modified to ensure that a sub-channel is not selected as the first sub-channel for PSCCH/PSSCH transmission, if the lowest N PRBs of this sub-channel have overlap with intra-cell guard band.

- N is the (pre-)configured number of PRBs for PSCCH transmission.

Within the above definition of sub-channel, it is open how to deal with the remaining PRBs, e.g., for meeting OCB requirements. In our view, the remaining RPBs should not be used so that all sub-channels will have the same size, which follows Rel-16 NR sidelink design.

Proposal 14: For contiguous RB-based PSCCH/PSSCH transmission, the remaining PRBs of a resource pool are not used.

### Sub-channel for 60 kHz SCS

The sub-channel definition for 60 kHz SCS was discussed. It was proposed by FL that for PSCCH/PSSCH/PSFCH/S-SSB, RAN1 to further study how to meet OCB and PSD requirement under 60 kHz SCS. An RB set is of 20 MHz, which contains only 24 PRBs under 60 kHz SCS. If an interlace is composed of 10 or 11 PRBs, like 15 kHz or 30 kHz SCS, then an RB set contains 2 interlaces. Subsequently, the bandwidth of unused PRBs in an RB set could be large under 60 kHz SCS.

To mitigate frequency resource waste, it is proposed that each interlace under 60 kHz SCS contains more PRBs than an interlace under 15 kHz or 30 kHz SCS.

Proposal 15: To meet OCB and PSD requirement under 60 kHz SCS, RB-based interlace is supported, where each interlace includes more PRBs than an interlace under 15 kHz or 30 kHz SCS.

### Multi-consecutive slot transmission

RAN1 #110 Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details RAN1 #110 Agreement For PSCCH and PSSCH resource indication in time/frequency domain: - For time domain: R16 NR SL TRIV is reused as baseline - For frequency domain: - further study sub-channel indexing and resource indication - FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

For PSCCH and PSSCH resource indication in time/frequency domain, it was agreed that Rel-16 NR sidelink TRIV is reused as baseline. It is open whether any enhancement is needed if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission. Additionally, it was agreed that multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U.

In Rel-16 NR sidelink, TRIV is determined by two time gap variables, t1 and t2, where t1 is the time gap between the initial transmission slot and the first re-transmission slot, t2 is the time gap between the initial transmission slot and the second re-transmission slot. Since the resource reservation window by SCI is 32 slots, the value range of t1 is 1≤t1≤31 and the value range of t2 is t1<t2≤31. With MCSt, if Rel-16 NR sidelink TRIV is reused, then the value ranges of t1 and t2 need to be redefined such that there is no time overlap between initial transmission and re-transmissions.

Proposal 16: In supporting multi-consecutive slots transmission, the value ranges of t1 and t2 in deriving TRIV are redefined to avoid time overlap between initial transmission and re-transmissions.

## PSFCH

### Time domain

RAN1 #110 Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded FFS details of above alternatives RAN1 #110b-e Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives RAN1 #111 Agreement To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - FFS other details, e.g., HARQ-ACK timeline - Alt 2: PSFCH occasions are dynamically indicated - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc. - FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH - FFS: Whether/how to address PSFCH collision if any - FFS: Whether/how to handle the linearly decreased PSFCH capacity RAN1 #112 Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

In Rel-16/17 NR sidelink, PSFCH occasion is either 2 or 3 slots (by configuration) after the PSSCH transmission. In other words, the timing of PSFCH is associated with the timing of the corresponding PSSCH transmission. If the similar timing mapping rule is applied in SL-U, it is possible that PSFCH occasion may or may not be in the same COT as for PSSCH transmission. If PSFCH occasion is outside the COT duration for PSSCH transmission, then type 1 LBT is required before PSFCH transmission. It is likely that PSFCH transmission is dropped due to LBT failure. Even if PSFCH occasion is inside the COT duration for PSSCH transmission, it is possible that the COT is lost when there is no contiguous PSSCH transmissions. Then type 1 LBT is required before PSFCH transmission.

It was agreed that more than 1 PSFCH occasion per PSCCH/PSSCH transmission is supported to address PSFCH transmission dropping due to LBT failure. Such PSFCH occasion(s) are (pre-)configured. It is open whether or not the usage of such (pre-)configured PSFCH occasion(s) is dynamically indicated.

One motivation of dynamically indicate which (pre-)configured PSFCH occasion(s) to be used for PSFCH transmission is to ensure there is no interruption of continuous transmissions so that COT can be maintained or MCSt can be conducted. However, in our view, when PSFCH occasion(s) are (pre-)configured, they should not be reusable for PSCCH/PSSCH transmissions. Hence, the time occasion(s) for PSFCH will be empty in case they are dynamically indicated to be not used. This does not avoid COT lose or MCSt interruption. Furthermore, even if the (pre-)configured PSFCH occasion(s) are dynamically indicated to be reused for PSCCH/PSSCH transmissions, a receiver UE may continue to transmit or receive PSFCH in the (pre-)configured PSFCH occasion(s) due to the large processing time of such dynamic indication decoding.

Proposal 17: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, such PSFCH occasion(s) are (pre-)configured (i.e., Option 1).

The details of (pre-)configuration of multiple PSFCH occasion(s) are still open.

The PSFCH initial transmission is 2 or 3 slots after PSCCH/PSSCH transmission in NR sidelink. It is preferred to set PSFCH re-transmission occasion(s) after PSFCH initial transmission occasion. The time gap between PSFCH re-transmission and PSFCH initial transmission can be (pre-)configured per resource pool. Different frequency domain PSFCH resources (i.e., interlace) or code domain PSFCH resources (i.e., cyclic shift pairs) can be used for PSFCH re-transmission. This is illustrated in Figure 5.

Figure 5: Illustration of PSFCH re-transmission

Proposal 18: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, the time gap between PSFCH initial transmission time occasion and PSFCH re-transmission time occasion is (pre-)configured by resource pool.

It is open on the HARQ-ACK timeline for the case of supporting more than 1 PSFCH occasion per PSCCH/PSSCH transmission. In Rel-16/17 NR sidelink, when a UE selects the resources for a single TB transmission, it ensures a minimum time gap Z = a +b between any two selected resources of a TB where a HARQ feedback for the first resource is expected, where a is the time gap between the end of the last symbol of PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception and b is the time required for PSFCH reception and processing plus PSSCH re-transmission preparation and is determined by UE implementation.

In the case of supporting multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission, when a UE selects resources for a single TB transmission, it ensures a minimum time gap Z = a +b+c between any two selected resources of a TB where a HARQ feedback for the first resource is expected, where c is the time gap between the first PSFCH occasion and the last PSFCH occasion for the corresponding PSSCH transmission. This is illustrated in Figure 6.

Figure 6 Illustration of HARQ-ACK timeline

Proposal 19: If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, when a transmitter UE selects resources for a single TB transmission, it ensures a minimum time gap Z = a +b+c between any two selected resources of a TB where a HARQ feedback for the first resource is expected,

- a is the time gap between the end of the last symbol of PSSCH transmission of the first resource and the start of the first symbol of the first corresponding PSFCH reception,

- b is the time required for PSFCH reception and processing plus PSCCH/PSSCH re-transmission preparation,

- c is the time gap between the first PSFCH occasion and the last PSFCH occasion for the corresponding PSCCH/PSSCH transmission.

If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, then the receiver UE’s behavior of using multiple PSFCH transmission occasions needs to be specified. In our view, a receiver UE only needs to make a single PSFCH transmission among multiple PSFCH transmission occasions for a TB’s (re)transmission. If a receiver UE fails transmitting PSFCH on any precedent PSFCH occasion corresponding to the same TB due to LBT failure, then it attempts to make a PSFCH transmission.

Consider an example of groupcast with ACK/NACK feedback. If there are two PSFCH transmission occasions for PSCCH/PSSCH initial transmission and there are two PSFCH transmission occasions for PSCCH/PSSCH re-transmission (i.e., same TB), then a receiver UE only needs to make a single PSFCH transmission. Specifically, it attempts to make a PSFCH transmission in the first PSFCH transmission occasion for PSCCH/PSSCH initial transmission. If it succeeds, then it stops sending PSFCH in the remaining 3 PSFCH transmission occasions. Otherwise, the receive UE attempts to make a PSFCH transmission in the second PSFCH transmission occasion for PSCCH/PSSCH initial transmission, and so on.

Proposal 20: When multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is (pre-)configured, a receiver UE attempts to make a PSFCH transmission if it has not transmitted on any precedent PSFCH occasion corresponding to the same TB.

### Frequency domain

RAN1 #111 Agreement Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc. - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 interlace - Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE uses different cyclic shifts on different PRBs in the interlace - Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - The above dedicated PRB and common PRBs are within 1 interlace - FFS: value of K2 - Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth - FFS: whether IBE issue exists and whether/how to address it - Note: in the above descriptions - The dedicated PRB/cyclic shift conveys ACK/NACK information - Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS. RAN1 #111 Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details RAN1 #112 Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case - FFS: whether/how to reduce PAPR of PSFCH transmission

It was agreed that at least interlace RB-based PSFCH transmission is supported to meet OCB and PSD requirement. If each PSFCH transmission occupies one interlace (e.g., 10 PRBs), then the frequency domain resources of PSFCH will be limited. Note that in Rel-16/17 NR sidelink, each PSFCH transmission occupies only a single PRB. The details about RB-based interlace PSFCH transmission are open, where five alternatives are identified.

We shall compare different alternatives in terms of PSFCH capacity and PSFCH reliability.

In the comparison, we consider an example of an RB set composed of 5 interlaces where each interlace has 10 PRBs. The configured number of cyclic shift pairs used for PSFCH transmissions is 6. Hence, this RB set has a total of 5*10*6 = 300 PSFCH resources.

In Alt 1-1a, a common interlace is used to meet OCB requirement. This results in 60 common PSFCH resources, which leaves 300-60 = 240 remaining PSFCH resources available to deliver sidelink HARQ-ACK information. Since each PSFCH transmission uses K3 dedicated PRB(s), one RB set supports a total of 240/K3 PSFCH transmissions.

In Alt 2-2a, each PSFCH transmission occupies 1 interlace. Among the 6 cyclic shift pairs in this interlace, one cyclic shift pair is common which leaves 5 remaining dedicated cyclic shift pairs to deliver sidelink HARQ-ACK information. Since each PSFCH transmission uses K1 dedicated PRB(s),

one RB set supports a total of (300- 5*10)/K1 = 250/K1 PSFCH transmissions.

In both Alt 2-3a and Alt 2-4a, each PSFCH transmission occupies 1 dedicated interlace. Hence, one RS set supports a total of 300/10=30 PSFCH transmissions.

In Alt 3-2a, each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs. Hence, one RB set supports a total of (300-6*K2)/K4 PSFCH transmissions, which is equal to 288/K4 for K2= 2.

Overall, Alt 2-3a and Alt 2-4a have limited PSFCH capacity, while Alt 1-1a, Alt 2-2a and Alt 3-2a have configurable PSFCH capacity, depending on the values of parameters K3, K1 and K4, respectively. The maximum allowable PSFCH capacity in Alt 3-2a is the largest among the alternatives.

On the other hand, Alt 2-3a and Alt 2-4a use 10 dedicated PRBs to transmit sidelink HARQ-ACK information, and they achieve the highest reliability among the alternatives. The number of dedicated PRBs for transmitting sidelink HARQ-ACK information is configurable as K3, K1 and K4 in Alt 1-1a, Alt 2-2a and Alt 3-2a, respectively. In general, the value range of K3, K1 and K4 can be between 1 and 10. Hence, Alt 1-1a, Alt 2-2a and Alt 3-2a have the flexibility to balance between PSFCH capacity and PSFCH transmission reliability and hence, they could be considered.

In Alt 1-1a, all UEs transmit on the common interlace, which increases the total power on this common interlace. This may have IBE issue between common interlace and other resources.

Considering that Alt 3-2a allows the largest PSFCH capacity, we prefer Alt 3-2a. The value of K4 is (pre-)configured by resource pool, while the value of K2 is always set to 2 to meet the OCB requirement. To address PSD limit issue, it is preferred that the K2 common PRBs and K4 dedicated PRBs are within 1 interlace. To mitigate the IBE issue on K2 common PRBs, we prefer that different PSCCH/PSSCH transmissions have different common PRBs.

Proposal 21: Each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRB(s) where K2 common PRBs locate at the two edges of an RB set (Option 3-2a).

- K2 common PRBs and K4 dedicated PRBs are within 1 interlace to address PSD limit.

- K2 is set to 2 and K4 is (pre-)configured by resource pool.

- Different PSFCH transmissions can use different common PRB(s) to mitigate IBE issue.

The PSFCH transmission under 60 kHz SCS was discussed and two alternatives are identified for further study. We think the similar PSFCH transmission as for 15 kHz or 30 kHz SCS could be applied to 60 kHz SCS as well. Specifically, each PSFCH transmission occupies K dedicated PRBs and two common PRBs, where the value K is (pre-)configured per resource pool and two common PRBs are at the two edges of an RB set.

Proposal 22: For 60 kHz SCS, each PSFCH transmission occupies K dedicated PRBs and two common PRBs, where

- the value K is (pre-)configured per resource pool,

- two common PRBs are at the two edges of an RB set.

## S-SSB

### Time domain

RAN1 #110b-e Agreement At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool. - Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer RAN1 #110b-e Agreement Regarding additional candidate S-SSB occasions: - Their number and time domain locations are (pre-)configured or pre-defined RAN1 #111 Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number RAN1 #111 Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details RAN1 #111 Agreement Regarding S-SSB, RAN1 further study the following: - How to transmit S-SSB when a SL BWP contains multiple RB sets. RAN1 #112 Agreement Down-select one or support both of the followings: - Option 1: Additional candidate S-SSB occasions are excluded from resource pool - Option 2: Additional candidate S-SSB occasions belong to resource pool - Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

It was agreed that if LBT is performed for S-SSB, in addition to the S-SSB occasions in Rel-16/17 NR sidelink design, additional candidate S-SSB occasions are supported.

In our view, if a UE successfully transmits S-SSB on the initial S-SSB occasion, then the UE does not use the additional candidate S-SSB occasions. Even in this case, the additional candidate S-SSB occasions should not be reused for PSCCH/PSSCH transmission. Specifically, the slots containing additional candidate S-SSB occasions are excluded from sidelink resource pool.

Although this semi-static configuration of resource pool increases S-SSB resource overhead, it largely simplifies UE’s operations on PSCCH/PSSCH transmission. Since otherwise, UE needs to dynamically determine whether a slot is used for S-SSB transmission or used for PSCCH/PSSCH transmission. In case a S-SSB transmission occurs in a slot belonging to resource pool, all the reserved PSCCH/PSSCH resources will be either pre-empted or delayed, resulting in performance degradation.

Proposal 23: If additional candidate S-SSB occasions are supported, then the slots containing additional candidate S-SSB occasions are excluded from resource pool.

It was agreed that the number and time domain locations of additional candidate S-SSB occasions are (pre-)configured or pre-defined. Furthermore, there are five identified options on the number and locations of additional candidate S-SSB occasions.

In our view, the number of additional candidate S-SSB occasions is (pre-)configured as part of the IE SL-SyncConfig to keep the flexibility. The locations of additional candidate S-SSB occasions are (pre-)configured by a time offset to the initial S-SSB occasion. This is illustrated in Figure 7.

Figure 7: Illustration of S-SSB occasions

Proposal 24: Regarding the number and location(s) of additional candidate S-SSB occasions, each Rel-16/17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion (i.e., Option 2), where

- the value of K is (pre-)configured,

- the gap between additional candidate S-SSB occasions and Rel-16/17 NR SL S-SSB slots is (pre-)configured.

A UE attempts to transmit S-SSB on a Rel-16/17 NR S-SSB slot. If the UE successfully transmits S-SSB in a Rel-16/17 NR S-SSB slot, then it does not need to make any additional S-SSB transmissions until the next Rel-16/17 NR S-SSB slot. Otherwise, the UE attempts to transmit S-SSB on the subsequent additional candidate S-SSB occasion. If a UE successfully transmits S-SSB in an additional candidate S-SSB occasion, then it does not make any additional S-SSB transmissions until the next Rel-16/17 NR S-SSB slot. Otherwise, the UE attempts to transmit S-SSB on the subsequent additional candidate S-SSB occasion.

Proposal 25: A UE attempts to transmit on an additional candidate S-SSB occasion, if it has not transmitted on the precedent Rel-16/17 NR SL S-SSB slot and its other corresponding additional candidate S-SSB occasions.

### Frequency domain

RAN1 #111 Agreement For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS - Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2) - FFS gap of 0 - Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K - FFS gap of 0 - FFS PSBCH resource - Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - For Option 1-1 and 1-2 above - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set - FFS: whether transient period issue exists and whether/how to address it RAN1 #112 Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS. RAN1 #112 Agreement RAN1 further study the followings: - Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

In Rel-16/17 NR sidelink, each S-SSB occupies 11 contiguous PRBs. This design does not meet OCB and PSD requirement for S-SSB transmission. It was agreed in RAN1 #112 meeting to down select one or more of three options. In Option 1-1, the interlaced RB transmission is applied to S-SSB, in a similar way as for PSCCH/PSSCH transmission. In Option 3-1, S-SSB is repeated in frequency domain such that it occupies enough bandwidth. In Option A, OCB exemption is applied to S-SSB.

Regarding the temporary exemption of OCB requirement, the following limits are copied from regulation [7].

Figure 8: Limitation of occupied channel bandwidth [7]

During a COT, UE may operate temporarily with an OCB of less than 80% of its nominal channel bandwidth with a minimum of 2 MHz. The periodicity of S-SSB transmission is 160 ms, which implies an infrequent channel occupancy. If the temporary exemption of OCB requirement is applicable for S-SSB transmission, there is no need to modify the legacy S-SSB structure for the operations on unlicensed spectrum.

In our view, the simplest way of designing S-SSB for SL-U is to apply OCB exemption to S-SSB (i.e., Option A). The legacy S-SSB occupies 11 PRBs in frequency, which is equal to 1.98 MHz bandwidth for sub-carrier spacing (SCS) of 15 kHz or 3.96 MHz bandwidth for sub-carrier spacing of 30 kHz.

It is open how to meet the minimum of 2 MHz requirement under 15 kHz SCS. One way to meet the temporary exemption of OCB requirement is to append an additional PRB to the legacy S-SSB structure, on the symbols of PSBCH. This is illustrated in Figure 9. The contents of the additional PRB can be identical to its neighbor PRB in the legacy PSBCH. In this way, the legacy S-SSB structure is largely reused and there is no impact on the reception of S-SSB.

Figure 9: Temporary exemption of OCB requirements for S-SSB transmission

Proposal 26: For sidelink operations on unlicensed spectrum to meet OCB and PSD requirement, apply temporary exemption of OCB requirement for S-SSB transmission (i.e., Option A).

- For 15 kHz SCS, an additional PRB is appended to legacy S-SSB structure on the symbols of PSBCH.

It was agreed to study whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s). In our view, a UE could transmit S-SSB repetitions in more than one RB set to maintain the COT in this case.

Proposal 27: It is supported that a UE transmits S-SSB repetitions over multiple RB sets to maintain the COT.

## Power control

RAN1 #112 Agreement RAN1 further study the followings: - Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

For sidelink operations on unlicensed spectrum, the PSD limit needs to be considered. In some region, the PSD limit is 10 dBm/MHz.

NR sidelink has open loop power control. The transmission power of sidelink channels is subject to downlink or sidelink pathloss. With the total UE transmission power limit of 23 dBm, the PSD limit of 10 dBm/MHz is satisfied if the total transmission bandwidth is 20 MHz or 1 RB set. If the sidelink transmission (e.g., S-SSB) occupies a smaller bandwidth, then additional power control limit should be imposed on this sidelink transmission to meet the PSD limit. If multiple PSFCH transmissions can condensed in 1 MHz, then additional power control limit should be imposed on the PSFCH transmissions to meet the PSD limit.

Proposal 28: If a S-SSB transmission occupies less than 20 MHz bandwidth, then the PSD limit is considered in determining the transmission power of S-SSB. If multiple PSFCH transmissions are condensed in 1 MHz bandwidth, then the PSD limit is used in determining the transmission power of PSFCH.

# Conclusion

In this contribution, we provided our views on physical channel design framework for sidelink on FR1 unlicensed spectrum. Our proposals are as follows:

Proposal 1: Do not support that one sidelink resource pool is (pre-)configured to include a subset of RB set.

Proposal 2: Use a bitmap to indicate the time domain resources of a resource pool.

Proposal 3: If a resource pool contains two adjacent RB sets, all the PRBs within intra-cell guard band of two adjacent RB sets are associated with the interlaces in one of the two adjacent RB sets.

- These PRBs are treated as an extension of the allocated RB set when associating them with the corresponding interlaces.

Proposal 4: When the PRBs within intra-cell guard band are used for PSSCH transmission, the rate matched bits of PSSCH are mapped to these PRBs.

- FFS whether SCI stage 2 rate matched bits are mapped to these PRBs.

Proposal 5: In calculating sidelink TBS, a reference number of PRBs per interlace is used, where this number is (pre-)configured by resource pool.

Proposal 6: The PRBs within intra-cell guard band are not used for PSFCH transmission.

Proposal 7: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, if PSCCH/PSSCH transmission starts from the first starting symbol, then

- a single AGC symbol is applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains 1 RB set;

- two AGC symbols are applied for PSCCH/PSSCH transmission in a slot, in case a resource pool contains more than 1 RB set.

  - The second AGC symbol is the second candidate starting symbol, with contents being the copy of its following symbol.

Proposal 8: If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, sidelink TBS is determined based on a (pre-)configured reference number of symbols for both COT initiation and COT sharing. The value range of reference number of symbols is between 6 and 12.

Proposal 9: If a resource pool supports 2 candidate starting symbol positions, then the PSFCH occasion is at least 3 slots after the corresponding PSCCH/PSSCH transmission.

Proposal 10: For interlace RB-based PSSCH transmission, the frequency domain resource indication is explicitly indicated by the used sub-channel index(s) and RB set index(s).

Proposal 11: If a resource pool has multiple RB sets and a PSCCH/PSSCH transmission occupies more than one sub-channel, these sub-channels are allocated in a subset of RB sets in the resource pool, where these subset of RB sets has contiguous index(s).

- All these subset of RB sets have the same contiguous sub-channel index(s) for the PSCCH/PSSCH transmission.

Proposal 12: The number of PRBs for PSCCH is configurable per resource pool.

Proposal 13: For contiguous RB-based PSCCH/PSSCH transmission, resource selection procedure is modified to ensure that a sub-channel is not selected as the first sub-channel for PSCCH/PSSCH transmission, if the lowest N PRBs of this sub-channel have overlap with intra-cell guard band.

- N is the (pre-)configured number of PRBs for PSCCH transmission.

Proposal 14: For contiguous RB-based PSCCH/PSSCH transmission, the remaining PRBs of a resource pool are not used.

Proposal 15: To meet OCB and PSD requirement under 60 kHz SCS, RB-based interlace is supported, where each interlace includes more PRBs than an interlace under 15 kHz or 30 kHz SCS.

Proposal 16: In supporting multi-consecutive slots transmission, the value ranges of t1 and t2 in deriving TRIV are redefined to avoid time overlap between initial transmission and re-transmissions.

Proposal 17: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, such PSFCH occasion(s) are (pre-)configured (i.e., Option 1).

Proposal 18: For more than 1 PSFCH occasion per PSCCH/PSSCH transmission, the time gap between PSFCH initial transmission time occasion and PSFCH re-transmission time occasion is (pre-)configured by resource pool.

Proposal 19: If multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is supported, when a transmitter UE selects resources for a single TB transmission, it ensures a minimum time gap Z = a +b+c between any two selected resources of a TB where a HARQ feedback for the first resource is expected,

- a is the time gap between the end of the last symbol of PSSCH transmission of the first resource and the start of the first symbol of the first corresponding PSFCH reception,

- b is the time required for PSFCH reception and processing plus PSCCH/PSSCH re-transmission preparation,

- c is the time gap between the first PSFCH occasion and the last PSFCH occasion for the corresponding PSCCH/PSSCH transmission.

Proposal 20: When multiple PSFCH transmission occasions for a PSCCH/PSSCH transmission is (pre-)configured, a receiver UE attempts to make a PSFCH transmission if it has not transmitted on any precedent PSFCH occasion corresponding to the same TB.

Proposal 21: Each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRB(s) where K2 common PRBs locate at the two edges of an RB set (Option 3-2a).

- K2 common PRBs and K4 dedicated PRBs are within 1 interlace to address PSD limit.

- K2 is set to 2 and K4 is (pre-)configured by resource pool.

- Different PSFCH transmissions can use different common PRB(s) to mitigate IBE issue.

Proposal 22: For 60 kHz SCS, each PSFCH transmission occupies K dedicated PRBs and two common PRBs, where

- the value K is (pre-)configured per resource pool,

- two common PRBs are at the two edges of an RB set.

Proposal 23: If additional candidate S-SSB occasions are supported, then the slots containing additional candidate S-SSB occasions are excluded from resource pool.

Proposal 24: Regarding the number and location(s) of additional candidate S-SSB occasions, each Rel-16/17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion (i.e., Option 2), where

- the value of K is (pre-)configured,

- the gap between additional candidate S-SSB occasions and Rel-16/17 NR SL S-SSB slots is (pre-)configured.

Proposal 25: A UE attempts to transmit on an additional candidate S-SSB occasion, if it has not transmitted on the precedent Rel-16/17 NR SL S-SSB slot and its other corresponding additional candidate S-SSB occasions.

Proposal 26: For sidelink operations on unlicensed spectrum to meet OCB and PSD requirement, apply temporary exemption of OCB requirement for S-SSB transmission (i.e., Option A).

- For 15 kHz SCS, an additional PRB is appended to legacy S-SSB structure on the symbols of PSBCH.

Proposal 27: It is supported that a UE transmits S-SSB repetitions over multiple RB sets to maintain the COT.

Proposal 28: If a S-SSB transmission occupies less than 20 MHz bandwidth, then the PSD limit is considered in determining the transmission power of S-SSB. If multiple PSFCH transmissions are condensed in 1 MHz bandwidth, then the PSD limit is used in determining the transmission power of PSFCH.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 #112 Meeting, Mar. 2023.

- R1-2301930, “FL summary#3 for AI 9.4.1.2 SL-U physical channel design framework,” Mar. 2023.

- Chairman’s Notes, 3GPP TSG RAN WG1 #111 Meeting, Nov. 2022.

- Chairman’s Notes, 3GPP TSG RAN WG1 #110bis-e Meeting, Oct. 2022.

- Chairman’s Notes, 3GPP TSG RAN WG1 #110 Meeting, Aug. 2022.

- Chairman’s Notes, 3GPP TSG RAN WG1 #109-e Meeting, May 2022.

- ETSI EN 301 893, 5 GHz RLAN; Harmonized standard covering the essential requirements of article 3.2 of directive 2014/53/EU, v2.1.1, May 2017.