3GPP TSG RAN WG1 #112bis-e                                                                             R1-2303490

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.3

Source:	Apple Inc.

Title:	On Resource allocation for SL positioning reference signal

Document for:	Discussion/Decision

# Introduction

A SID defining objectives for a study on expanded and improved NR positioning was agreed in RAN #94 [1],  with the following justification for sidelink positioning:

- There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified for V2X, public safety, commercial and IIOT use cases.

and the following objective:

- Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc., reusing existing reference signals, procedures, etc. from sidelink communication and from positioning as much as possible [RAN1]

- Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc., reusing existing reference signals, procedures, etc. from sidelink communication and from positioning as much as possible [RAN1]

The conclusion to the SID was captured in a Technical Report [2]. A Work Item description with the following objective was made [3]:

- Specify support of resource allocation for SL PRS:

  - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

    - For resource allocation mechanism for SL PRS in Scheme 2:

      - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

      - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

  - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

    - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

- Specify signaling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

In this contribution, we discuss resource allocation  and grouping for the SL positioning reference signal.

# Discussion

### Dedicated and Shared Resource Pools

In  NR V2X, only certain slots are (pre-)configured to accommodate SL PSCCH/PSSCH/PSFCH transmissions. This is known as a “resource pool” and consists of contiguous PRBs and contiguous or non-contiguous slots that have been (pre-)configured for SL transmissions and defined within a SL BWP with a bitmap of size X. S-SSB are transmitted in dedicated slots.

For the SL positioning reference signal transmissions, it is possible that the slots for transmitting the SL positioning reference signal are within the slots configured for the legacy resource pool (shared resource pool). Alternatively, the slots for transmitting SL positioning reference signal can be separate from the slots configured for the resource pool, e.g., in a similar manner as in for S-SSB transmission (a dedicated resource pool).

With regards to the SL Positioning resource allocation, either dedicated resource pool(s) and/or shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS.

Note that it is be necessary to clarify if a slot dedicated for SL-PRS transmission within a SL communication resource pool or the SL-PRS requires its own dedicated RP to be defined as  a dedicated RP.

Proposal 1: It is necessary to clarify if a slot dedicated for SL-PRS transmission within a SL communication resource pool or the SL-PRS requires its own dedicated RP to be defined as  a dedicated RP.

#### Dedicated Resource Pools and Slot Structure

In the study item the following relevant agreements were made:

Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication Agreement A dedicated SL-PRS resource pool is (pre-)configured in the only SL BWP of a carrier.

Frame Structure: SL positioning reference signal can be configured in independent/dedicated SL positioning slots. In this case, a new SL positioning slot structure is needed (for example as shown in Figure 1) with the possible need for AGC and gap symbols and an independent resource allocation method (agreed to as Scheme 1 and Scheme 2 in RAN1 #110).

Figure 1: SL Positioning Signal configured in independent slot

Resource Allocation: With regards to which channels can be included in the resource pool in addition to SL-PRS, RAN1 is considered  the following options:

- Opt. 1: No other channel can be included beyond SL-PRS

- Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

- Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

In RAN1 #112, the following agreement was made:

- Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Option 2 will require a new SCI in the SL PRS resource pool for reserving/indicating one or more SL-PRS resource(s) in the dedicated PRS resource pool. In this case, a single stage SCI can be designed. For  scheme 2 in this option, the sensing-based resource allocation needs to support (pre-)configuration of a sensing window with dynamic, periodic  and semi-persistent scheduling and procedures to select the PRS resource and a new PSSCH resource.  SL-PRS transmissions from multiple UE pairs may be supported as shown in Figure 2 below.

Figure 2: SCI and SL-PRS transmission for multiple SL-UE pairs in Option 2

Option 3 (PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included) may be considered for any feedback with IUC.

Observation 1: With regards to which channels can be included in the resource pool in addition to SL-PRS, the following options require some specification updates:

- Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Observation 1-1: should support a single stage SCI, and sensing-based and  random resource allocation.

- Option 3 (PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included )

  - Observation 1-2: may be needed to support IUC feedback.

Proposal 2: With regards to which channels can be included in the resource pool in addition to SL-PRS, at least Option 2 should be supported. Option 3 is supported if Inter-UE coordination is supported in the dedicated resource pool.

Proposal 3: Single Stage SCI should be supported for PSSCH signaling for SL-PRS.

### Shared Resource Pools and Slot Structure

In the study item the following relevant agreements were made:

Agreement With regards to the SL Positioning resource allocation, for SL Positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication (if supported), backward compatibility with legacy Rel-16/17 UEs should be ensured.

Frame Structure: For the shared resource, a discussion is needed on the modifications to the SL-slot frame structure including:

- the P-SRS placement within the slot (e.g., the PSSCH region, the PSFCH region or in an independent region of the SL slot). The SL PRS can be transmitted as a few symbols or occupy the entire slot. They can be:

  - transmitted within the PSSCH

  - transmitted within dedicated slots within an existing resource pool containing the  PSSCH/PSSCH/PSFCH

  - transmitted within the PSFCH

  - transmitted in an independent section within the slot similar to the PSFCH

- The need for a  new AGC symbol and additional gap symbols between the different signals/channels

In Figure 3, we show a scenario in which the positioning reference signal is placed in the PSSCH region of the slot (immediately after the DMRS symbol). Note that in this case, the positioning reference signal has the same frequency resources as the PSSCH and as such there is no need for a gap or AGC symbol. If they are set to different frequency resource sizes, additional symbols may be needed.

Resource Allocation: For the shared resource, the SL positioning symbols can use the same resource allocation method as PSSCH/PSCCH/PSFCH i.e.  the legacy Mode 1 or Mode 2 solution. Modifications may be needed to enable signaling of the SL-PRS transmission within the current signaling framework e.g. an update to the SCI.

Backwards Compatibility: For Rel-16 and Rel-17 SL UEs, the current physical sidelink shared channel (PSSCH), occupies the entire time-frequency resource after the sidelink control information (SCI) and before the physical sidelink feedback channel (PSFCH). Thus, if a shared resource is used when transmitting to a Rel-16/Rel-17 UE, there may be a backwards compatibility issue as the Rel16/el-17 UE expects to use the entire PSCCH resource till the gap before the PSFCH and no there is no resource to transmit a sidelink positioning reference signal (SL-PRS). This can be resolved by one or more of the following:

- specifying that only slots that are used for transmission to Rel-18 and above UEs may use a shared resource pool

  - signaling to indicate the resources that the PSSCH/PSFCH should avoid due to use by the positioning reference signal

- limiting shared resources to aperiodic SL-PRS transmission and/or

- modifying the behaviour of shared periodic or semi-persistent SL-PRS in shared resources.

Figure 3: SL Positioning Signal Configured in same slot in PSSCH region

Proposal 4: For the shared resource, a discussion is needed on the modifications to the SL-slot frame structure including:

- the P-SRS placement within the slot (e.g., the PSSCH region, the PSFCH region or in an independent region of the SL slot). The SL PRS can be transmitted as a few symbols or occupy the entire slot. They can be:

  - transmitted within the PSSCH

  - transmitted within dedicated slots within an existing resource pool containing the  PSSCH/PSSCH/PSFCH

  - transmitted within the PSFCH

  - transmitted in an independent section within the slot similar to the PSFCH

- The need for a  new AGC symbol and additional gap symbols between the different signals/channels

Proposal 5: For the shared resource, the SL positioning symbols can use the same resource allocation method as PSSCH/PSCCH/PSFCH

Proposal 6: To support backwards compatibility for Rel-17/Rel-17 UEs for the shared resource pool, the following options should be considered:

- specifying that only slots that are used for transmission to Rel-18 and above UEs may use a shared resource pool

  - signaling to indicate the resources that the PSSCH/PSFCH should avoid due to use by the positioning reference signal

- limiting shared resources to aperiodic SL-PRS transmission and/or

- modifying the behavior of shared periodic or semi-persistent SL-PRS in shared resources

### Resource Allocation Details

Resource allocation can be Scheme 1 (i.e., Network-centric operation SL-PRS resource allocation) or Scheme 2 (i.e. UE autonomous SL-PRS resource allocation). In the study item the following relevant agreements were made:

Agreement Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS Agreement With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre-)configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility Agreement Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable Agreement For Scheme 2, with regards to Resource allocation mechanism for SL-PRS, pick one or both of the following options: - Option 1: A sensing-based resource allocation should be introduced - Option 2: A random resource selection should be introduced - In either option 1 or 2, the legacy designs for UE autonomous resource allocation should be used as a starting point. Study if/what enhancements may be needed.

In RAN1 #112, the following agreements have been made [8]:

- Agreement - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further). Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications. - Study if any changes are needed Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

Scheme 1 Allocation: For Scheme 1 SL-PRS resource allocation based on the agreement in RAN1 #112,  resource allocation can be through Dynamic grant, or through configured grant type 1/type 2 from gNB similar to the legacy Mode 1 solution for sidelink communications (Option 1). This may require an update to the DCI format.

For dynamic resource allocation and type 2 configured grant resource allocation for SL-PRS in Scheme 1, when the  gNB signals a  resource for the SL-PRS, it indicates the SL-PRS resource index within a slot. The SL-PRS grant can be indicated by:

- Option 1: Updating DCI 3_0 with a new field such as a “SL-PRS resource index”. This may be suitable for the shared resource pool.

- Option 2: Introducing a new DCI format. This may be suitable for the dedicated resource pool.

For type 1 configured grant resource allocation for SL-PRS, the “sl-ConfiguredGrantConfig” may have a new field such as “sl-PRS_resources”, which indicates the SL-PRS resources in the grant.

Scheme 2 Allocation: For Scheme 2 SL-PRS resource allocation, the working assumption from RAN1 #112 shown below should be agreed to:

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

We do not see a need for any enhancements for coexistence of random selection and sensing-based resource selection in a resource pool similar to the Rel-16/Rel-17 design.

For the shared resource pool, the sensing-based resource pool may reuse the Stage 1 (legacy) SCI with the introduction of a new Stage 2 SCI format to support the indication of both SL communications and SL-PRS parameters.

For the dedicated resource pool, the design of the scheme 2 resource allocation procedure is dependent on the frame structure of the slot(s) in the dedicated resource pool.

For the Rel-16 type sensing, the following options should be considered:

- Option 1: SL-PRS and PSCCH are allocated in the same section of the slot and can be scheduled together: a single resource allocation similar to the Rel-16 mode 2 allocation is sufficient. Note that due to the resources required by the SL-PRS and the PSSCH, it may be more efficient to separate them especially if the SL-PRS are either time or comb multiplexed.

- Option 2: SL-PRS and the PSCCH are allocated in separate sections of the slot and cannot be scheduled together (e.g. a PSSCH section and a SL-PRS section): the resource allocation will depend on the mapping between the PSCCH and the SL-PRS.

  - Option 2-1: if an implicit mapping between the PSSCH resource and the SL-PRS resource is used, then a single resource allocation similar to the Rel-16 mode 2 allocation is sufficient.

  - Option 2-2: if explicit mapping in which the PSSCH indicates the SL-PRS resource is used, then the PSSCH and the SL-PRS may need separate resource allocations. Note that the resource allocation for each may be sensing based or random access based.

As a simple example, the following procedure shows the steps needed for Option 2-2 in the case that both resources use sensing-based resource allocation:

- Step 1: UE receives resource pool (pre)configurations for dedicated resource pool for SL positioning.

- Step 2: UE receives/decodes SCI in the SCI section of the resource pool and measures the  PSCCH RSRP

- Step 3: The UE selects the SL-PRS resource from the SL-PRS section of the resource pool based on SCI decoding and RSRP of PSCCH.

  - For sensing based selection, this procedure is similar to the legacy SL PSCCH/PSSCH resource selection procedure (as in Section 8.1.4 of TS 38.214)

- Step 4: The UE selects an SCI resource from the SCI section of a slot. Note that a single stage SCI can be used with no backwards compatibility issue.

  - This is also based on a sensing based resource selection

- Step 5: The UE transmits the  SCI in the selected SCI resource, which indicates the selected SL-PRS resource

- Step 6: The UE transmits SL-PRS in the selected SL-PRS resource

For Rel-17 type sensing, the IUC schemes supported will be determined by the  channels can be included in the resource pool in addition to SL-PRS in the dedicated resource pool:

- IUC Scheme 1 in which the set of preferred and/or non-preferred resources are indicated, requires that a PSSCH is present in the dedicated resource pool for the IUC message contents.

- IUC Scheme 2 in which the presence of expected/potential resource conflict is signaled, requires the presence of a PSFCH in the dedicated resource pool for the IUC message contents.

This is directly related to the decision on Option 3 : PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

Proposal 7: For dynamic resource allocation and type 2 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the SL-PRS grant can be indicated by:

- Option 1: Updating DCI 3_0 with a new field such as a “SL-PRS resource index” at least for the shared resource pool.

- Option 2: Introducing a new DCI format at least for the dedicated resource pool.

Proposal 8: For type 1 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the “sl-ConfiguredGrantConfig” may have a new field such as “sl-PRS_resources”, which indicates the SL-PRS resources in the grant.

Proposal 9: For Scheme 2 SL-PRS resource allocation, the working assumption from RAN1 #112 shown below should be agreed to:

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

We do not see a need for any enhancements for coexistence of random selection and sensing-based resource selection in a resource pool similar to the Rel-16/Rel-17 design.

Proposal 10: For Scheme 2 SL-PRS allocation in the shared resource pool, the sensing-based resource pool may reuse the Stage 1 (legacy) SCI with the introduction of a new Stage 2 SCI format to support the indication of both SL communications and SL-PRS parameters.

Proposal 11: For the dedicated resource pool, the design of the scheme 2 resource allocation procedure is dependent on the frame structure of the slot(s) in the dedicated resource pool.

Proposal 12: For Scheme 2 SL-PRS allocation with Rel-16 type sensing in the dedicated resource pool, the following options should be considered:

- Option 1: SL-PRS and PSCCH are allocated in the same section of the slot and can be scheduled together: a single resource allocation similar to the Rel-16 mode 2 resource allocation is sufficient.

- Option 2: SL-PRS and the PSCCH are allocated in separate sections of the slot and cannot be scheduled together (e.g. a PSSCH section and a SL-PRS section): the resource allocation will depend on the mapping between the PSCCH and the SL-PRS.

  - Option 2-1: if an implicit mapping between the PSSCH resource and the SL-PRS resource is used , then a single resource allocation similar to the Rel-16 mode 2 allocation is sufficient.

  - Option 2-2: if explicit mapping in which the PSSCH indicates the SL-PRS resource is used, then the PSSCH and the SL-PRS may need separate resource allocations.

Propsal 13: For Rel-17 type sensing, the IUC schemes supported will be determined by the  channels can be included in the resource pool in addition to SL-PRS in the dedicated resource pool:

- IUC Scheme 1 in which the set of preferred and/or non-preferred resources are indicated, requires that a PSSCH is present in the dedicated resource pool for the IUC message contents.

- IUC Scheme 2 in which the presence of expected/potential resource conflict is signaled, requires the presence of a PSFCH in the dedicated resource pool for the IUC message contents.

This is directly related to the decision on Option 3 : PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included.

Proposal 14: For Scheme 2 SL-PRS resource allocation,  support both congestion control and inter-UE coordination.

### Frequency Domain resource allocation

In the study item the following relevant agreements were made:

Agreement At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

In  NR V2X, a resource pool is divided into a (pre-)configured number of L sub channels, where a sub-channel is a group of consecutive PRBs in a slot. For SL positioning, the frequency resources allocated to the SL positioning reference signal should also be (pre-)configured based on sub-channels. Overall, the SL positioning reference signal should be in a wide band to achieve high positioning accuracy.

The frequency resources used for SL positioning  can be set equal or smaller than the  shared channel frequency domain allocation size for both the shared and dedicated resource pools.

Proposal 15: The frequency resources used for SL positioning  can be set equal or smaller than the  shared channel frequency domain allocation size for both the shared and dedicated resource pools.

### Group-based resource allocation

In RAN1 110-bis-e, the following agreement were made with regards to resource allocation [6]:

Agreement From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one) - Broadcast (as a working assumption). - FFS: Applicability of the above cast types

In RAN1 #112, the following agreement was made :

- Agreement - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

For broadcast and multicast SL PRS in the shared resource pool, RAN1 should down-select from the following two options in the case of shared resource allocation:

- Option 1: Grouping is the same as PSSCH i.e. the PSSCH and the PRS are bundled together

- Option 2: Grouping is independent of the PSSCH

Note that for the second option, signaling may be needed to indicate the UEs that may need the PRS independent of the PSSCH signaling.

Broadcast and multicast transmission in the dedicated resource pool can use the design for SL communications as a starting point in the new SL-PRS SCI.

Proposal 16: The SL PRS may be unicast, broadcast or multi-cast. For broadcast and multicast SL PRS/SRS in the shared resource pool, RAN1 should study the following two options:

- Option 1: Grouping is the same as PSSCH i.e. the PSSCH and the PRS are bundled together

- Option 2: Grouping is independent of the PSSCH

Proposal 17: Broadcast and multicast transmission in the dedicated resource pool can use the design for SL communications as a starting point in the new SL-PRS SCI.

# Conclusion

In this contribution, the following observations and proposals were made:

Proposal 1: It is necessary to clarify if a slot dedicated for SL-PRS transmission within a SL communication resource pool or the SL-PRS requires its own dedicated RP to be defined as  a dedicated RP.

Observation 1: With regards to which channels can be included in the resource pool in addition to SL-PRS, the following options require some specification updates:

- Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Observation 1-1: should support a single stage SCI, and sensing-based and  random resource allocation.

- Option 3 (PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included )

  - Observation 1-2: may be needed to support IUC feedback.

Proposal 2: With regards to which channels can be included in the resource pool in addition to SL-PRS, at least Option 2 should be supported. Option 3 is supported if Inter-UE coordination is supported in the dedicated resource pool.

Proposal 3: Single Stage SCI should be supported for PSSCH signaling for SL-PRS.

Proposal 4: For the shared resource, a discussion is needed on the modifications to the SL-slot frame structure including:

- the P-SRS placement within the slot (e.g., the PSSCH region, the PSFCH region or in an independent region of the SL slot). The SL PRS can be transmitted as a few symbols or occupy the entire slot. They can be:

  - transmitted within the PSSCH

  - transmitted within dedicated slots within an existing resource pool containing the  PSSCH/PSSCH/PSFCH

  - transmitted within the PSFCH

  - transmitted in an independent section within the slot similar to the PSFCH

- The need for a  new AGC symbol and additional gap symbols between the different signals/channels

Proposal 5: For the shared resource, the SL positioning symbols can use the same resource allocation method as PSSCH/PSCCH/PSFCH

Proposal 6: To support backwards compatibility for Rel-17/Rel-17 UEs for the shared resource pool, the following options should be considered:

- specifying that only slots that are used for transmission to Rel-18 and above UEs may use a shared resource pool

  - signaling to indicate the resources that the PSSCH/PSFCH should avoid due to use by the positioning reference signal

- limiting shared resources to aperiodic SL-PRS transmission and/or

- modifying the behavior of shared periodic or semi-persistent SL-PRS in shared resources

Proposal 7: For dynamic resource allocation and type 2 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the SL-PRS grant can be indicated by:

- Option 1: Updating DCI 3_0 with a new field such as a “SL-PRS resource index” at least for the shared resource pool.

- Option 2: Introducing a new DCI format at least for the dedicated resource pool.

Proposal 8: For type 1 configured grant resource allocation for SL-PRS in Scheme 1 PRS allocation, the “sl-ConfiguredGrantConfig” may have a new field such as “sl-PRS_resources”, which indicates the SL-PRS resources in the grant.

Proposal 9: For Scheme 2 SL-PRS resource allocation, the working assumption from RAN1 #112 shown below should be agreed to:

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

We do not see a need for any enhancements for coexistence of random selection and sensing-based resource selection in a resource pool similar to the Rel-16/Rel-17 design.

Proposal 10: For Scheme 2 SL-PRS allocation in the shared resource pool, the sensing-based resource pool may reuse the Stage 1 (legacy) SCI with the introduction of a new Stage 2 SCI format to support the indication of both SL communications and SL-PRS parameters.

Proposal 11: For the dedicated resource pool, the design of the scheme 2 resource allocation procedure is dependent on the frame structure of the slot(s) in the dedicated resource pool.

Proposal 12: For Scheme 2 SL-PRS allocation with Rel-16 type sensing in the dedicated resource pool, the following options should be considered:

- Option 1: SL-PRS and PSCCH are allocated in the same section of the slot and can be scheduled together: a single resource allocation similar to the Rel-16 mode 2 resource allocation is sufficient.

- Option 2: SL-PRS and the PSCCH are allocated in separate sections of the slot and cannot be scheduled together (e.g. a PSSCH section and a SL-PRS section): the resource allocation will depend on the mapping between the PSCCH and the SL-PRS.

  - Option 2-1: if an implicit mapping between the PSSCH resource and the SL-PRS resource is used , then a single resource allocation similar to the Rel-16 mode 2 allocation is sufficient.

  - Option 2-2: if explicit mapping in which the PSSCH indicates the SL-PRS resource is used, then the PSSCH and the SL-PRS may need separate resource allocations.

Propsal 13: For Rel-17 type sensing, the IUC schemes supported will be determined by the  channels can be included in the resource pool in addition to SL-PRS in the dedicated resource pool:

- IUC Scheme 1 in which the set of preferred and/or non-preferred resources are indicated, requires that a PSSCH is present in the dedicated resource pool for the IUC message contents.

- IUC Scheme 2 in which the presence of expected/potential resource conflict is signaled, requires the presence of a PSFCH in the dedicated resource pool for the IUC message contents.

This is directly related to the decision on Option 3 : PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included.

Proposal 14: For Scheme 2 SL-PRS resource allocation,  support both congestion control and inter-UE coordination.

Proposal 15: The frequency resources used for SL positioning  can be set equal or smaller than the  shared channel frequency domain allocation size for both the shared and dedicated resource pools.

Proposal 16: The SL PRS may be unicast, broadcast or multi-cast. For broadcast and multicast SL PRS/SRS in the shared resource pool, RAN1 should study the following two options:

- Option 1: Grouping is the same as PSSCH i.e. the PSSCH and the PRS are bundled together

- Option 2: Grouping is independent of the PSSCH

Proposal 17: Broadcast and multicast transmission in the dedicated resource pool can use the design for SL communications as a starting point in the new SL-PRS SCI.

# Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning

- TR 3GPP TR 38.859 V1.0.0 (2022-12), Technical Report for Study on expanded and improved NR positioning (Release 18), December 2022

- RP-223549, New WID on Expanded and Improved NR Positioning, December 2022

- Chairman’s Notes, RAN1 #109-e, May 2022

- Chairman’s Notes, RAN1 #110, August 2022

- Chairman’s Notes, RAN1 #110-bis-e, October 2022

- Chairman’s Notes, RAN1 #111, November 2022

- Chairman’s Notes, RAN1 #112, February 2022