3GPP TSG RAN WG1 #112bis-e		R1-2303495

e-Meeting, April 17th – April 26th, 2023

Source: 	Apple Inc.

Title:                     Further RedCap UE complexity reduction

Agenda item:	9.6.1

Document for:	Discussion and Decision

# 1 Introduction

This contribution presents our views on the remaining issues to progress the new WID ‘Enhanced support of reduced capability NR devices’ [1].

# Discussions

- 2.1  On UE Peak Data Reduction

The peak data rate supported by an NR UE is computed using the following expression from TS 38.306 [2]:

### The peak data rate supported by an NR UE is computed using the following expression from TS 38.306:4.1.2 Supported max data rate for DL/UL For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. is the maximum RB allocation in bandwidth with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where is the UE supported maximum bandwidth in the given band or band combination. is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL […] The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations. For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅f(j) is no smaller than 4. NOTE: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and f(j)=1.

As per the approved WID [1], it was agreed to remove the highlighted restriction for Rel-18 eRedcap UE, which can achieve ~4% cost reduction benefit compared to Rel-17 Redcap device. The exact value of relaxed constraint for eRedcap UEs need to be studied to complete the design.

In RAN1 111 meeting, different options were discussed and the following was agreed on this issue [2]:

Agreement - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary.

In RAN1 112 meeting, the following was agreed for the support of PR1:

Agreement For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

During RAN Plenary 99 meeting, there was heated debating regarding the necessary to support PR1 as ‘stand-alone’ feature and the following was endorsed [4]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps …. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate.

The target peak data rate of Rel-18 eRedcap UEs is kept as 10Mbps per RAN plenary 98 meeting discussions, which should serve as the metric to determine the feasible value of ‘X’ and ‘Y’ for ‘add-on’ option and ‘standalone’ option. The achievable peak data rate for two alternatives are roughly estimated in Table 1 and Table 2, respectively.

Table 1: Achievable peak data rate for ‘standalone’ PR1 with 20MHz BW

| vlayers∙Qm∙f | 15kHz SCS (Mbps) | 30kHz SCS (Mbps) |
| --- | --- | --- |
| 4 | DL: 56.7 Mbps UL: 60.7 Mbps | DL: 54.6 Mbps UL: 58.4 Mbps |
| 1 | DL: 14.2 Mbps UL: 15.2 Mbps | DL: 13.6 Mbps UL: 14.6 Mbps |
| 0.8 | DL: 11.3 Mbps UL: 12.1 Mbps | DL: 10.9 Mbps UL: 11.7 Mbps |
| 0.75 | DL: 10.6 Mbps UL: 11.4 Mbps | DL: 10.2 Mbps UL: 10.9 Mbps |
| 0.4 | DL: 5.7 Mbps UL: 6.1 Mbps | DL: 5.5 Mbps UL: 5.8 Mbps |

Table 2: Achievable peak data rate for ‘add-on’ PR1 with Rel-18 BW3

| vlayers∙Qm∙f | 15kHz SCS, 25 PRBs (Mbps) | 30kHz SCS, 12 PRBs (Mbps) |
| --- | --- | --- |
| 4 | DL: 13.4 UL: 14.3 | DL: 12.8 UL: 13.7 |
| 3.2 | DL: 10.7 UL: 11.4 | DL: 10.27 UL: 10.98 |
| 3.0 | DL: 10.0 UL: 10.7 | DL: 9.6 UL: 10.3 |

Referring to Table 1 above assuming PR1 is specified as ‘standalone’ feature, it is feasible to modify the restriction of vLayers(j)⋅Qmj⋅f(j) from 4 to 0.75 for eRedcap UE, which can reduce BB cost and still meet the 10Mbps peak data rate requirement. In the RAN1 112 meeting, the value ‘1’ was proposed by several companies without detailed reasoning. The supported peak data rate with the vLayers(j)⋅Qmj⋅f(j) = ‘1’ is ~14 Mbps for FR1 DL with 15kHz SCS, which is far beyond the target data rate ‘10Mbps’, which unnecessarily increases the soft bits buffer size and the cost of eRedcap device.

As shown in Table 2, if PR1 is introduced as an ‘add-on’ feature for Rel-18 eRedcap UEs with reduced 5MHz BB BW, relaxing  vLayers(j)⋅Qmj⋅f(j) to be ‘3.2’ can be considered to meet the target peak data rate of Rel-18 eRedcap UEs.

Therefore, we made the following proposal:

Proposal 1:

- For a standalone PR1 feature, support to relax the component (vLayers·Qm·f ) in peak data rate calculation from 4 to 0.75.

- For an ‘add-on’ PR1 feature, support to relax the component (vLayers·Qm·f ) in peak data rate calculation from 4 to 3.2.

- 2.3  On early Indication of Rel-18 eRedcap UE

In RAN plenary 98 meeting, the WID was updated to include the following objective for early indication [1]:

- UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2]

The following was agreed in RAN1 111 meeting [2] for eRedcap:

Agreement For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

To ensure the Msg3 PUSCH is scheduled within a 5MHz BW for Rel-18 eRedcap UE as agreed above, it seems necessary to introduce mechanism that provides the possibility for network to identify eRedcap UE before Msg2 and then schedule Msg3 accordingly. In addition, processing timing relaxation for RAR is introduced for Rel-18 eRedcap. It is desirable to identify the Rel-18 eRedcap UE based on the preamble and then allocate FDRA resource properly.

In RAN1 112 meeting, concerns on the standard and implementation complexity were raised on support of Msg1-based early identification. First, it should be noted that NW has full flexibility not to use Msg-1 based early identification and therefore implementation complexity is not a real concern. Regarding the standard effort, the Rel-17 RACH split framework can be fully used to allocate dedicated resource for Rel-18 eRedcap identification with almost zero specification impact.

Having said that, we proper the following:

Proposal 2:

- Support a sperate early indication using Msg1 for Rel-18 eRedcap UE.

# 3. Conclusion

In the previous sections, we have discussed different open issues to further reduce Redcap device complex, including reducing BB bandwidth for PDSCH and PUSCH from 20MHz to 5MHz and relaxing the restriction for peak data rate reduction.

Based on the discussions above, the following proposals were made:

Proposal 1:

- For a standalone PR1 feature, support to relax the component (vLayers·Qm·f ) in peak data rate calculation from 4 to 0.75.

- For an ‘add-on’ PR1 feature, support to relax the component (vLayers·Qm·f ) in peak data rate calculation from 4 to 3.2.

Proposal 2:

- Support a sperate early indication using Msg1 for Rel-18 eRedcap UE.

# References

- RP-223544  New WID on enhanced support of reduced capability NR devices

- RAN1 111 meeting Chairman notes

- RAN1 112 meeting Chairman notes

- RAN Plenary 99 Chairman notes