3GPP TSG RAN WG1 #112bis-e		R1-2303497

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.7.1

Source:	Apple

Title:	Discussion on cell DRX/DTX mechanism

Document for:	Discussion/Decision

Introduction

In RAN1 #112 meeting, while it was the first meeting and waiting for the clearer definition of cell DTX/DRX from RAN2, RAN1 made the following agreements:

Agreement

- RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects

  - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX

    - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX

- Further discussions on other aspects are not precluded

R1-2302131	Discussion Summary #2 for enhancements on cell DTX/DRX mechanism	Moderator (Intel Corporation)

Agreement

At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS.

- DL

  - Periodic/Semi-persistent CSI-RS (including TRS)

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - PDCCH in Type-3 CSS

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Other signals/channels are not precluded.

In this contribution, we provide our consideration on the above study points.

On UE behaviors in Cell DTX / DRX non-active duration

For the monitoring of PDCCH, there were discussions in the last meeting that it would be preferred for the network to maintain some flexibility especially for low latency traffic. While we consider it important to support low latency traffic, especially voice and emergency calls, we think there could be other options to support such traffic and can also take into UE power consumption into account, for example, with some exceptional SPS/CG/SR in non-active duration of Cell DTX / DRX, gNB could wake up to transmit low latency traffic in these SPS occasions during non-active duration of Cell DTX or receive SR associated with low latency traffic in these CG occasions during the non-active duration of Cell DRX. Therefore, we do not see the need to maintain UE-specific PDCCH monitoring during the non-active duration.

Proposal 1: UE does not monitor PDCCH including UE-specific RNTI and Type-3 CSS in cell DTX non-active duration,

- For DG-PDSCH/PUSCH scheduled by PDCCH received during on duration, up to gNB scheduling.

- HARQ-ACK still allowed based on gNB scheduling for PDSCH scheduled by PDCCH in ON duration.

For SPS/CG/SR reception/transmission, it was already under discussion in RAN2. Therefore, we propose to wait for their agreement. We have the following observations and proposals for reference, more detailed discussion can be referred to our RAN2 contribution [1].

Observation 1: Because gNB has to wake up for preamble reception in all ROs in non-active duration of Cell DRX, marginal NES loss is expected if the occasions of exceptional CG/SR are configured close to ROs.

Observation 2: Because gNB has to wake up for SSB/SIB/paging in non-active duration of Cell DTX, marginal NES loss is expected if the exceptional SPS are configured close to occasions to transmit SSB/SIB/paging.

Thus, we propose to allow some exceptional SPS/CG/SR in non-active duration of Cell DTX / DRX, that is, UE may be configured a list of SPS-ConfigIndex, ConfiguredGrantConfigIndex and schedulingRequestID, that can still be received or monitored during the cell DTX/DRX non-active duration.

Proposal 2: A list of exceptional SPS-ConfigIndex can be included in Cell DTX configuration. gNB only wakes up to transmit low latency traffic in the SPS occasions indicated by the list during non-active duration of Cell DTX.

Proposal 3: A list of exceptional ConfiguredGrantConfigIndex can be included in Cell DRX configuration. gNB wakes up to receive low latency CG-PUSCH in the CG occasions indicated by the list during non-active duration of Cell DRX.

Proposal 4: A list of exceptional schedulingRequestID can be included in Cell DRX configuration. gNB wakes up to receive SR associated with low latency traffic in the SR occasions indicated by the list during non-active duration of Cell DRX.

For periodic/SP CSI-RS, basically they are used for T/F tracking, CSI computation, L1-RSRP/SINR measurement, RRM, RLM, tracking for fast SCell activation, etc.

For TRS, it is required that CONNECTED UE will always receive a TRS configuration as described in 38.213, “A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.” What's more, in Rel-17 WI of UE power saving enhancement, IDLE/INACTIVE UEs can also be configured to use TRS. And the WID objective has a NOTE that impact to IDLE/INACTIVE UEs should be avoided.

To not impact legacy UEs that do not support NES feature, it is proposed that TRS is still maintained during non-active duration of cell DTX.

Proposal 5: To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

For CSI-RS for CSI computation, L1-RSRP/SINR measurement, RRM and RLM, although not explicitly required as TRS, they do have impacts especially on UE measurement latency requirement. Currently, UE measurement requirements have all be made assuming that CSI-RS are still transmitted although with UE DRX configured. As can be seen from the RAN4 requirement (take the CSI-RS for RLM as an example, Table 8.1.3.2-1 in TS38.133), only the CSI-RS periodicity and UE DRX cycle is considered in the requirement design so far with a basic assumption that when configured, CSI-RS will always be transmitted and it is up to UE implementation to determine which CSI-RS occasion to collect the measurement samples on.

TS38.133 Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P) × TDRX | Ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

More specifically, for UE DRX cycles smaller than 320ms, the measurement is relaxed (1.5×Mout×P) due to reason that the CSI-RS transmission periodicity and the DRX cycle are not well aligned (as shown in the left figure of Fig.1). Ideally, the measurement interval would be a lowest common multiple (LCM) of TCSI-RS and TDRX, however, since the delay may be too large if LCM is used, a compromise of 1.5 times of the original defined value was determined. This compromise was made under the assumption that CSI-RS is always there even when UE DRX is configured. However, if it is agreed that CSI-RS during cell DTX non-active duration are not transmitted (as shown in the right figure of Fig.1), there is no way for UE to measure during the non-active duration to meet the requirement. This will lead to additional discussions in RAN4 and will increase UE measurement latency. This cannot be easily compromised to a larger number as in UE DRX case since UE can not find a proper CSI-RS to measure in any non-active duration.

Fig. 1 UE DRX/Cell DTX not aligned with CSI-RS

Similar for all CSI-RS for purposes other than tracking, whether they can be stopped during the non-active duration highly impacts the UE measurement latency requirement. Before we could make any further decisions on that, it is safer for RAN4 to study on the latency requirement and check on how much increase would be foreseen.

Observation 3: Allowing P/SP CSI-RS to be stopped during cell DTX/DRX non-active duration may significantly increase UE measurement latency.

Proposal 6: Send an LS to RAN4 to study on the how much measurement latency increase is foreseen for P/SP CSI-RS before determining the transmission of CSI-RS in cell DTX/DRX non-active duration.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: Because gNB has to wake up for preamble reception in all ROs in non-active duration of Cell DRX, marginal NES loss is expected if the occasions of exceptional CG/SR are configured close to ROs.

Observation 2: Because gNB has to wake up for SSB/SIB/paging in non-active duration of Cell DTX, marginal NES loss is expected if the exceptional SPS are configured close to occasions to transmit SSB/SIB/paging.

Observation 3: Allowing P/SP CSI-RS to be stopped during cell DTX/DRX non-active duration may significantly increase UE measurement latency.

Proposal 1: UE does not monitor PDCCH including UE-specific RNTI and Type-3 CSS in cell DTX non-active duration,

- For DG-PDSCH/PUSCH scheduled by PDCCH received during on duration, up to gNB scheduling.

- HARQ-ACK still allowed based on gNB scheduling for PDSCH scheduled by PDCCH in ON duration.

Proposal 2: A list of exceptional SPS-ConfigIndex can be included in Cell DTX configuration. gNB only wakes up to transmit low latency traffic in the SPS occasions indicated by the list during non-active duration of Cell DTX.

Proposal 3: A list of exceptional ConfiguredGrantConfigIndex can be included in Cell DRX configuration. gNB wakes up to receive low latency CG-PUSCH in the CG occasions indicated by the list during non-active duration of Cell DRX.

Proposal 4: A list of exceptional schedulingRequestID can be included in Cell DRX configuration. gNB wakes up to receive SR associated with low latency traffic in the SR occasions indicated by the list during non-active duration of Cell DRX.

Proposal 5: To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

Proposal 6: Send an LS to RAN4 to study on the how much measurement latency increase is foreseen for P/SP CSI-RS before determining the transmission of CSI-RS in cell DTX/DRX non-active duration.

Reference

- R2-2303369, Further discussion on Cell DTX / DRX, Apple, RAN2 #121b-e, Apr. 2023.