3GPP TSG RAN WG1 #112bis-e			R1-2303499

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.9.1

Source:	Apple

Title:	On Coverage Enhancement for NR NTN

Document for:	Discussion/Decision

# Introduction

The Release-18 NR NTN enhancement work item was approved in RAN #94e meeting [1] and updated in RAN #98e meeting [2].

In the NR NTN work item, one of the RAN1 objectives is coverage enhancement. In RAN1 #112 meeting, there are some progresses made for NR NTN uplink channel coverage enhancement [3], [4].

Observation

For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following:

- Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed.

  - FFS: whether/how to consider the initial timing error at the beginning

  - FFS: TA pre-compensation update is assumed

- Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed.

- FFS: impact of phase difference limit

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE.

- Option A: PRACH preamble and/or occasion

  - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report]

  - FFS: whether or not indication of repetition factor is assumed

  - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings

- Option B: Higher layer signaling in Msg3 PUSCH

  - FFS: which signaling is used

  - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2.

- Option C: Physical layer signaling in Msg3 PUSCH

  - FFS: which signaling is used, e.g. DMRS ports

Agreement

For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB.

- Alt 1: Field in DCI scheduling the Msg4 PDSCH

  - Alt 1-1: One or two bits of the existing field

    - Alt 1-1a: MCS field

    - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource)

    - Alt 1-1c: HARQ process number filed

    - Alt 1-1d: DAI field

    - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field

  - Alt 1-2: New field with one or two bits

- Alt 2: Field in DCI scheduling Msg3 PUSCH

  - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor

  - Note: it is assumed that there is impact on DCI design

- Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH

  - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc.

- Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Working assumption

For PUCCH repetition for Msg4 HARQ-ACK,

- A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB.

  - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold.

  - If the RSRP threshold is not configured, or if the configured RSRP threshold is X,

    - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK

  - FFS: value of X (the maximum configurable value of the RSRP threshold)

  - Down-select one from the following alternatives for the RSRP threshold.

    - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used.

    - Alt B: New RSRP threshold is introduced.

- Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

In this contribution, we provide our views on the enhancements of PUCCH for Msg4 HARQ-ACK and DMRS bundling for PUSCH for NR NTN.

# Discussion

## PUCCH repetition for Msg4 HARQ-ACK

### Triggering of PUCCH repetition request or capability report

In RAN1 #112 meeting, the triggering of Msg4 PUCCH repetition request or capability report was discussed. The working assumption supports both UE indication of its capability of Msg4 PUCCH repetition and UE indication of its capability as well as the need of Msg4 PUCCH repetition. These two different features are supported and selected via SIB configuration. Specifically, if RSRP threshold is not configured or a specific RSRP threshold is configured, then UE only indicates its capability of Msg4 PUCCH repetition. Otherwise, UE indicates its capability and the need of Msg4 PUCCH repetition.

Overall, the working assumption on the triggering condition of PUCCH repetition for Msg4 HARQ-ACK is agreeable.

Proposal 1: Confirm the working assumption on the triggering condition of PUCCH repetition request or capability report for Msg4 HARQ-ACK.

In the working assumption on the triggering condition of PUCCH repetition request or capability report for Msg4 HARQ-ACK, it is open on the relationship between the RSRP threshold for Msg4 PUCCH repetition and the RSRP threshold for Msg3 PUSCH repetition. In one alternative, the same RSRP threshold as for Rel-17 Msg3 PUSCH repetition is used. In another alternative, a new RSRP threshold is introduced.

In our view, the RSRP threshold to trigger Msg3 PUSCH repetition is highly related to the RSRP threshold to trigger Msg4 PUCCH repetition, since the similar channel condition is expected for Msg3 PUSCH transmission and Msg4 HARQ-ACK transmission. According to our simulation results [5], Msg4 HARQ-ACK has about 3 dB link budget more than that for Msg3 PUSCH. This implies the RSRP threshold for Msg4 PUCCH repetition can be about 3 dB less than the RSRP threshold for Msg3 PUSCH repetition.

Overall, the RSRP threshold for Msg4 PUCCH repetition is based on the RSRP threshold for Msg3 PUSCH repetition. Specifically, the RSRP threshold for Msg4 PUCCH repetition is set to be rsrp-ThresholdMsg3-r17 minus an offset, where the offset or differential RSRP is either pre-defined or configured via SIB.

Proposal 2: The RSRP threshold for Msg4 PUCCH repetition is based on the RSRP threshold for Msg3 PUSCH repetition (i.e., RSRP threshold for Msg4 PUCCH repetition is rsrp-ThresholdMsg3-r17 minus an offset).

### Signaling of PUCCH repetition request or capability report

The signaling of UE’s PUCCH repetition request or capability report was discussed in RAN1 #112 meeting [3]. Three options were identified, where Option A is based on PRACH preamble and/or occasion, Option B is based on higher layer signaling in Msg3 PUSCH, and Option C is based on physical layer signaling in Msg3 PUSCH.

In Rel-17 NR coverage enhancement, to request Msg3 PUSCH repetition, a UE uses separate PRACH preamble in a shared RACH occasion configured by the same PRACH configuration index with legacy UEs. Similarly, to request PUCCH repetition for Msg4 HARQ-ACK, a UE transmits a dedicated PRACH preamble on a shared RACH occasion or transmit PRACH on a dedicated RACH occasion. Hence, we prefer Option A.

In Option B, higher layer signaling in Msg3 PUSCH is used. There are two alternatives of higher layer signaling in the discussions in RAN1 #112 meeting. The first alternative is to use reserved LCID codepoints to indicate PUCCH repetition request or capability report. The LCID has 6 bits with values between 0 and 63. The LCID values of 37-42 and 47 are reserved (e.g., Table 6.2.1-2 of [6]). In other words, there are only 7 reserved LCID codepoints available. If LCID is to be used for PUCCH repetition request or capability report, then at least 4 codepoints will be used to additionally distinguish 2 CCCH sizes (48 bits or 64 bits) and RedCap/non-RedCap cases. This is not preferred.

The second alternative is to use reserved bits in MAC sub-header. The MAC sub-header has 8 bits, where 6 LSB carries LCID and 2 MSB are reserved. If one of the two reserved bits is used to indicate PUCCH repetition request or capability report for Msg4 HARQ-ACK, then it prevents the support of potential key features in the future. Hence, it is not preferred. Overall, we do not support changing high layer signaling in Msg3 PUSCH for indicating PUCCH repetition request or capability report for Msg4 HARQ-ACK.

In Option C, physical layer signaling in Msg3 PUSCH is used. This may include either a dedicated scrambling to Msg3 PUSCH or a dedicated DMRS port for Msg3 PUSCH. Either of these approaches increases network detection complexity and has significant specification impact. Hence, it should be avoided.

Proposal 3: PUCCH repetition request or capability report for Msg4 HARQ-ACK is indicated by a dedicated PRACH preamble and/or occasion.

Within Option A using dedicated PRACH preamble and/or occasion to deliver the PUCCH repetition request or capability report, it is open whether PRACH resource partitioning is needed for indication of PUCCH repetition request or capability report for Msg4 HARQ-ACK.

In order to indicate PUCCH repetition request or capability report for Msg4 HARQ-ACK, the PRACH resource partitioning is needed.

It was proposed that this indication is jointly reported with Msg3 PUSCH repetition indication. There are four cases: 1). Msg3 PUSCH repetition indication and Msg4 PUCCH repetition indication; 2). Msg3 PUSCH repetition indication but no Msg4 PUCCH repetition indication; 3). No Msg3 PUSCH repetition indication but Msg4 PUCCH repetition indication; 4). No Msg3 PUSCH repetition indication and no Msg4 PUCCH repetition indication. Considering both Msg3 PUSCH repetition and Msg4 PUCCH repetition are optional UE features, all the 4 cases are possible. Hence, the joint indication of Msg3 PUSCH repetition and Msg4 PUCCH repetition is unnecessary.

It is open whether or not indication of repetition factor is assumed. To avoid further partitioning of PRACH resources, UE’s indication of required Msg4 PUCCH repetition factor is not supported.

Proposal 4: PRACH resource partitioning is needed for indication of PUCCH repetition request or capability report for Msg4 HARQ-ACK. To avoid further partitioning of PRACH resources, UE’s indication of required Msg4 PUCCH repetition factor is not supported.

### Indication of PUCCH repetition factor

Once a gNB receives a UE’s indication of PUCCH repetition request or capability report for Msg4 HARQ-ACK, it decides whether to schedule PUCCH repetition for Msg4 HARQ-ACK, as well as how many PUCCH repetitions are applied. Subsequently, gNB needs to indicate the determined repetition factor to UE so that UE follows the scheduled number of Msg4 HARQ-ACK repetition.

In RAN1 #112 meeting, a list of options of gNB dynamic indication of repetition factor was identified. Among these options, we prefer Alt 4 since it does not result in any changes on the existing DCI 1_0 format.

In Rel-17 Msg3 PUSCH repetition, to indicate Msg3 PUSCH repetition factor, the 2 MSB of MCS field in RAR grant are used to select one repetition factor from SIB configured set with 4 candidate values. The same bits could be also applied to Msg4 PUCCH.

In the case where UE indicates both Msg3 PUSCH repetition and Msg4 PUCCH repetition, the 2 MSB of MCS field could be used to indicate one Msg3 PUSCH repetition factor. Simultaneously, the indicated Msg3 PUSCH repetition factor is implicitly mapped to Msg4 PUCCH repetition factor. The mapping rule can be either configured or pre-defined. For example, if the indicated Msg3 PUSCH repetition factor is 4, then the corresponding Msg4 PUCCH repetition factor is 2. This is because the link budget of Msg4 PUCCH is about 3 dB better than that of Msg3 PUSCH.

In the case where UE indicates Msg3 PUSCH repetition but does not indicate Msg4 PUCCH repetition, the 2 MSB of MCS field could be used to indicate one Msg3 PUSCH repetition factor from SIB configured set as in Rel-17.

In the case where UE indicates Msg4 PUCCH repetition but does not indicate Msg3 PUSCH repetition, the 2 MSB of MCS field could be used to indicate one Msg4 PUCCH repetition factor from SIB configured set.

Proposal 5: For PUCCH repetition for Msg4 HARQ-ACK, gNB indicates the PUCCH repetition factor via 2 MSB of MCS field in RAR grant.

## DMRS bundling for PUSCH

According to updated WID [2], RAN1 is to specify the necessary enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation).

The DMRS bundling for PUSCH was introduced in Rel-17 NR coverage enhancement, to enable the joint channel estimation at gNB side. To achieve this purpose, UE is expected to maintain power consistency and phase continuity among a time duration window (TDW). This TDW is upper bounded by the maximum duration during which UE is able to maintain power consistency and phase continuity. A UE should not perform time and frequency update during this TDW.

This mechanism is similar to uplink segmented transmission introduced in Rel-17 IoT NTN. In IoT NTN, a UE does not adjust its TA value and frequency offset value within an uplink segmented transmission duration. Hence, the PUSCH DMRS bundling enhancement could reuse IoT NTN PUSCH segmented transmission mechanism.

Proposal 6: The PUSCH DMRS bundling reuses IoT NTN PUSCH segmented transmission mechanism.

In IoT NTN, PUSCH segmented transmission duration is indicated by SIB or dedicated RRC signaling. The similar signaling scheme for PUSCH segmented transmission duration can be applied to NR NTN for the case of PUSCH DMRS bundling. The unit of PUSCH segmented transmission duration is slot.

Proposal 7: The PUSCH segmented transmission duration is configured by the network and indicated via SIB or dedicated RRC signaling, in the unit of slot.

If PUSCH segmented transmission duration is separately configured from the configuration of PUSCH DMRS bundling, then the nominal TDW size of PUSCH DMRS bundling (if configured) is upper bounded by the indicated PUSCH segmented transmission duration. This is because UE is not expected to perform time and frequency update during PUSCH segmented transmission duration.

Note that PUSCH segmented transmission duration is determined by gNB based on the current timing drift, derived from satellite ephemeris. The update of indicated PUSCH segmented transmission duration autonomously reconfigures nominal TDW size of PUSCH DMRS bundling for all UEs in coverage.

Proposal 8: The nominal TDW size of PUSCH DMRS bundling (if configured) is upper bounded by the indicated PUSCH segmented transmission duration.

- Update of PUSCH segmented transmission duration autonomously reconfigures nominal TDW size of PUSCH DMRS bundling for all UEs in coverage.

If the nominal TDW size of PUSCH DMRS bundling is not configured, then the actual TDW size is equal to the minimum of maximum duration reported by UE, PUSCH segmented transmission duration, PUSCH repetition duration and the frequency hopping interval for PUSCH DMRS bundling.

Proposal 9: If the nominal TDW size of PUSCH DMRS bundling is not configured, then the actual TDW size is equal to the minimum of maximum duration reported by UE, PUSCH segmented transmission duration, PUSCH repetition duration and the frequency hopping interval for PUSCH DMRS bundling.

# Conclusion

In this contribution, we provided our views on coverage enhancement for NR NTN. Our proposals and observations are as follows:

Proposal 1: Confirm the working assumption on the triggering condition of PUCCH repetition request or capability report for Msg4 HARQ-ACK.

Proposal 2: The RSRP threshold for Msg4 PUCCH repetition is based on the RSRP threshold for Msg3 PUSCH repetition (i.e., RSRP threshold for Msg4 PUCCH repetition is rsrp-ThresholdMsg3-r17 minus an offset).

Proposal 3: PUCCH repetition request or capability report for Msg4 HARQ-ACK is indicated by a dedicated PRACH preamble and/or occasion.

Proposal 4: PRACH resource partitioning is needed for indication of PUCCH repetition request or capability report for Msg4 HARQ-ACK. To avoid further partitioning of PRACH resources, UE’s indication of required Msg4 PUCCH repetition factor is not supported.

Proposal 5: For PUCCH repetition for Msg4 HARQ-ACK, gNB indicates the PUCCH repetition factor via 2 MSB of MCS field in RAR grant.

Proposal 6: The PUSCH DMRS bundling reuses IoT NTN PUSCH segmented transmission mechanism.

Proposal 7: The PUSCH segmented transmission duration is configured by the network and indicated via SIB or dedicated RRC signaling, in the unit of slot.

Proposal 8: The nominal TDW size of PUSCH DMRS bundling (if configured) is upper bounded by the indicated PUSCH segmented transmission duration.

- Update of PUSCH segmented transmission duration autonomously reconfigures nominal TDW size of PUSCH DMRS bundling for all UEs in coverage.

Proposal 9: If the nominal TDW size of PUSCH DMRS bundling is not configured, then the actual TDW size is equal to the minimum of maximum duration reported by UE, PUSCH segmented transmission duration, PUSCH repetition duration and the frequency hopping interval for PUSCH DMRS bundling.

# References

- RP-213690, “New WI: NR NTN enhancements,” Dec. 2021.

- RP-230809, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements,” Mar. 2023.

- Chairman’s Notes, 3GPP TSG RAN WG1 #112 Meeting, Mar. 2023.

- R1-2301838, “Summary #4 on 9.11.1 Coverage enhancement for NR NTN,” Mar. 2023.

- R1-2207353, “On coverage enhancement for NR NTN,” Aug. 2023.

- 3GPP TS 38.321, NR; Medium Access Control (MAC) protocol specification, v17.3.0, Jan. 2023.