3GPP TSG RAN WG1 #112bis-e			R1-2303510

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.12.3

Source:	Apple Inc.

Title:	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion/Decision

# Introduction

A new Rel-18 WI on further coverage enhancement [1] was approved in RAN#94e. one of the objectivce of this WI is to improve PRACH coverage.

- Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In the RAN1#112 meeting, the following agreements were made on PRACH coverage enhancements [2].

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In this contribution, we discuss the aspect of dynamic waveform switching and give our proposals.

# Discussion on dynamic waveform switching

According to current specification, the waveform switching between DFT-S-OFDM and CP-OFDM is configured by RRC parameter transformPrecoder in pusch-Config, or by msg3-transformPrecoder if transformPrecoder is not configured. As the DFT-S-OFDM waveform has a lower PAPR than CP-OFDM, it can improve the UE PA power efficiency, it’s beneficial to coverage limited UE. The semi-static reconfiguring of the waveform would limit UE to adapt the waveform flexibly. The main use case for dynamic waveform switching is for cell edge users, it’s less attractive to cell center users. Thus, the dynamic waveform switching function could be enabled/disabled by the gNB according to the network scheduling strategy.

Proposal 1: Dynamic switching between DFT-S-OFDM and CP-OFDM is UE specifically enabled/disabled by RRC signaling.

Regarding the assistance information for waveform switching, the Pcmax value is different for CP-OFDM waveform and DFT-S-OFDM waveform. If both Pcmax are reported, it could help gNB to determine the power headroom for both waveforms. According to RAN2 specification TS38.321, single entry PHR MAC CE includes the PH and Pcmax. The standard impact is similar to introducing target waveform Pcmax or PHR on top of the source waveform PHR report. But, reporting both PHR reports makes gNB implementation easier, gNB doesn’t need to record the PUSCH scheduling info and calculate the target waveform power headroom.

Proposal 2: Both PHRs for source waveform and target waveform are reported together as assistance information for gNB waveform determination.

For the power headroom calculation, if the target RB location and MCS are different from the actual scheduled PUSCH, gNB could not compare the power difference between the two waveforms directly, and the assistance info may not be enough for gNB to determine the waveform. In addition, how to determine the virtual RB location, RB number, MCS is not clear. It can be imagined the standard impacts are not negligible, but the benefit is limited. Therefore, RB number and location, MCS shall be the same for calculating PH for both source and target waveform.

Proposal 3: RB number, PR location, and MCS for target waveform PH calculation are derived from the actual scheduled PUSCH.

According to current specification, PHR report can be triggered periodically based on a periodic timer, i.e., phr-PeriodicTimer. The minimum value of this timer is 10 subframes. As the waveform switching could be indicated via DCI, it makes the waveform change so fast. The periodic PHR timer could not match with waveform switching. If the periodic PHR timer can be configured with smaller values, the provided PHR could be more meaningful for gNB determining the waveform.

Proposal 4: Considering a shorter periodic PHR timer for dynamic waveform switching.

The DCI size alignment was extensively discussed in the last RAN1 meeting. Four options are shown below.

RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Proposal Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

- Option1 per-format alignment

- If the CRC is scrambled by a CS-RNTI, UE needs to know the NDI field position in the DCI to identify the initial transmission or re-transmission for the configured grant. Most of the fields before the NDI field are waveform independent. For FDRA field, there is a different understanding of whether it is the same size for both waveforms or not, i.e., Alt. A and Alt. B, this impacts placing DWS field in the DCI. According to the current specification 38.212, the FDRA bit width for resource allocation type 0 and type 1 is different. In addition, the resource allocation can be type 0 or type 1 or dynamic switching for CG type 2. As the type 0 resource allocation is not applicable to DFT-S-OFDM waveform. Thus, UE needs to decode the DWS field first to know the waveform, then it could determine the size of FDRA field.

- Observation: the bit width of FDRA field could be different for CP-OFDM and DFT-D-OFDM waveforms.

- Option 2: per-field alignment

- Comparing with per-format alignment, Per-field alignment is simple, there is no restriction to place the new DWS field in the DCI.

- Option 3: Per-field alignment only for fields preceding NDI field

- This Option is the combination of Option 1 and Option 2. It’s hard to see any benefits than Option 1 and 2. It makes the UE implementation complicated.

- Option 4: Per-field alignment only for DCI addressed to CS-RNTI

- Option 4 applies different DCI alignment procedures for C-RNTI and CS-RNTI. Additional implementation efforts would require for UE.

One common issue for all the Options is to align the field size of CS-RNTI to those of the C-RNTI according to the existing alignment clause between CS-RNTI and C-RNTI defined in 38.212. In the legacy procedure, the waveform associated with C-RNTI is fixed, thus the bit width of each field is fixed as well. However, with dynamic waveform switching, it’s not clear to align to the field size of CP-OFDM or of DFT-S-OFDM. One way to solve the issue is to determine the field size of DCI addressed to C-RNTI first by comparing the field size between CP-OFDM and DFT-S-OFDM. Then, for DCI addressed to CS-RNTI, align field sizes to those of C-RNTI after waveform comparison.

Considering per-field alignment between two waveforms is needed for DCI addressed to C-RNTI during the size alignment procedure for DCI addressed by CS-RNTI, Option 2 is simple and attractive.

7.3.1.1.2 Format 0_1 [3] …… A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0 1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.

Proposal 5: The updated Option2 Per-filed alignment is adopted.

- Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM).

- Step 2: If DCI is addressed to CS-RNTI, align field sizes to the largest field size of two waveforms of C-RNTI according to the existing alignment clause between CS-RNTI and C-RNTI.

# Summary

In this contribution, we discuss the mechanism of supporting dynamic waveform switching between DFT-s-OFDM and CP-OFDM, and have the following proposals:

Proposal 1: Dynamic switching between DFT-S-OFDM and CP-OFDM is UE specifically enabled/disabled by RRC signaling.

Proposal 2: Both PHRs for source waveform and target waveform are reported together as assistance information for gNB waveform determination.

Proposal 3: RB number, PR location, and MCS for target waveform PH calculation are derived from the actual scheduled PUSCH.

Proposal 4: Considering a shorter periodic PHR timer for dynamic waveform switching.

Observation: the bit width of FDRA field could be different for CP-OFDM and DFT-D-OFDM waveforms.

Proposal 5: The updated Option2 Per-filed alignment is adopted.

- Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM).

- Step 2: If DCI is addressed to CS-RNTI, align field sizes to the largest field size of two waveforms of C-RNTI according to the existing alignment clause between CS-RNTI and C-RNTI.

# References

- RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom

- R1-2302256, “Report of RAN1#112 meeting”, ETSI MCC

- TS38.212 NR Multiplexing and channel coding, v17.3.0