3GPP TSG RAN WG1 Meeting #112bis-e                                                    R1-2303514

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.18

Source:	Apple

Title:	On remaining issues for Rel-18 multi-carrier enhancements

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on remaining issues for Rel-18 multi-carrier enhancements including:

- Multi-cell scheduling

- UL Tx switching

# Discussion on remaining issues for multi-cell scheduling

In RAN1#111, it is agreed that DMRS antenna port indication field can either be configured as Type 1A field or Type 2 field [1]. Furthermore, in RAN1#112, following agreement has been made [2]:

Agreement

For a set of cells configured for multi-cell scheduling using DCI format 0_X/1_X,

- the size of a Type-1A field in the DCI format 0_X/1_X is determined as maximum field size of active BWP among all cells within the set of cells.

- the size of a Type-1B field in the DCI format 0_X/1_X is equal to ceiling(log2(N)), where N is the number of rows in RRC-configured table with each row containing multiple indexes for all cells within the set of cells.

  - The Type-1B field indicates one row of the configured table

  - The Type-1B index for a cell points to a corresponding index in a RRC configured table applicable for DCI format 0_1/1_1 or MAC CE activated values.

- the size of a per cell Type-2 field in the DCI format 0_X/1_X is determined based on active BWP for each cell.

With Type 1A field type for DMRS antenna port indication, there is going be an issue in cases when the indicated index by single bitfield is not available in all of the DMRS tables corresponding to each of the co-scheduled cells. For example, if for the first co-scheduled cell, DMRS table for DMRS type 1 and max_length = 1 is configured, while for the second co-scheduled cell, DMRS table for DMRS type 1 and max_length = 2 is configured, then index from 0-15 can be used for first cell and index 0-31 can be used for second cell. However, since only single index is used for both cells, therefore, if any index greater than 15 is indicated, then it is not clear on what index to use for first cell. In our view, one possibility to solve this issue is to configure a default index for DMRS antenna port indication table when the indicated entry by DCI format 0_X/1_X is not valid for one or more of co-scheduled cells.

Proposal 1: For DMRS antenna port indication with format 0_X/1_X, when the field type is 1A and when the indicated index by single bitfield is not valid/available in the DMRS table for a co-scheduled cell, then a default index is applied for DMRS antenna port indication from one of the below options:

- Option 1: default index can be the lowest index value in the table for the co-scheduled cell

- Option 2: default index can be the higher valid index value in the table for the co-scheduled cell

# Discussion on remaining issues for UL Tx switching

For UL Tx switching, one remaining issue is related to the UL Tx switching case of  A(1Tx) + B(1Tx) -> C(1Tx) + D(1Tx) when it is triggered by two DCIs. It needs to be discussed under what conditions this can be considered as single switching instance or two switching instances, for example first DCI triggering A+B->C and the second DCI triggering A+B->D. In our view, this should be specified to avoid any ambiguity between the network and the UE because the switching gap durations might be different considering which of the two scenarios is assumed. In our view, the baseline condition should be that at least the UE is able perform the switching simultaneously in a single switching period location (based on the two triggers), i.e. no two different switching period location should be required by UE to perform switching, otherwise, UE determines these two triggers of UL Tx switching as two different UL Tx switching instances. One example scenario is illustrated in Figure 1.

Figure 1: Illustration of switching case A+B->C+D triggered by two DCIs

Proposal 2: For UL Tx switching case A(1Tx) + B(1Tx) ->  C(1Tx) + D(1Tx) triggered by two DCIs, the baseline requirements should be that the UE is able perform the switching simultaneously in a single switching period location (based on the two triggers), i.e. no two different switching period location should be required by UE to perform switching, otherwise, UE determines these two triggers of UL Tx switching as two different UL Tx switching instances

Furthermore, to ensure that the baseline condition is met, one possibility in terms of triggering timeline can be considered. The possibility could be specified in terms of when the second DCI triggering the UL Tx switching is received by the UE. Essentially, it should be at least received before T0 – max{Toffset1, Toffset2} to allow for UE to determine the switching gap duration. Otherwise, UE might already start the switching process for the first UL Tx switching trigger and therefore, the second trigger then needs to be considered as separate switching instance..

Proposal 3: For NR Rel-18 UL Tx switching, for the switching case A(1Tx) + B(1Tx) ->  C(1Tx) + D(1Tx) triggered by two DCIs, the end of second DCI triggering the UL Tx switching should be received by UE no later than T0 – max{Toffset1, Toffset2}

# Conclusion

In this contribution, we have discussed our views on remaining issues for multi-cell scheduling and UL Tx switching in Rel-18 and provided following observations/proposals:

Multi-cell scheduling

Proposal 1: For DMRS antenna port indication with format 0_X/1_X, when the field type is 1A and when the indicated index by single bitfield is not valid/available in the DMRS table for a co-scheduled cell, then a default index is applied for DMRS antenna port indication from one of the below options:

- Option 1: default index can be the lowest index value in the table for the co-scheduled cell

- Option 2: default index can be the higher valid index value in the table for the co-scheduled cell

UL Tx switching

Proposal 2: For UL Tx switching case A(1Tx) + B(1Tx) ->  C(1Tx) + D(1Tx) triggered by two DCIs, the baseline requirements should be that the UE is able perform the switching simultaneously in a single switching period location (based on the two triggers), i.e. no two different switching period location should be required by UE to perform switching, otherwise, UE determines these two triggers of UL Tx switching as two different UL Tx switching instances

Proposal 3: For NR Rel-18 UL Tx switching, for the switching case A(1Tx) + B(1Tx) ->  C(1Tx) + D(1Tx) triggered by two DCIs, the end of second DCI triggering the UL Tx switching should be received by UE no later than T0 – max{Toffset1, Toffset2}

# Reference

- 3GPP RAN1#111, “Chairman Notes”

- 3GPP RAN1#112, “Chairman Notes”