3GPP TSG RAN WG1 #112bis-e			R1-2303516

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.1.1

Source:	Google

Title:	Discussion on unified TCI framework extension for multi-TRP

Document for: 	Discussion

- Introduction

In this contribution, we discuss some issues related to unified TCI framework extension for multiple-TRP scenario. In Rel-18 MIMO WID [1], RAN Plenary has agreed to specify extension of Rel-17 unified TCI to indicate multiple DL and UL TCI states. The target use case is multi-TRP scenario. In previous RAN1 meetings [2] [3], RAN1 has made some agreements with respect to beam indication and beam association with channels/RSs, as shown in the followings. In subsequent sections, we discuss and provide our opinions on these related issues.

RAN1 #110b(e) Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework RAN1 #111 Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

- Discussion

In RAN1#112, we discussed on how to inform UE which indicated TCI state(s) is applied for AP CSI-RS for S-DCI based M-TRP. However, we cannot reach consensus in the end. From our perspective, RRC configuration is sufficient for this issue. We prefer not to introduce other rules on indicating which indicated unified TCI state(s) to apply for AP CSI-RS. Therefore, we propose the followings.

Proposal 1: On unified TCI framework extension for S-DCI based MTRP, for an AP CSI-RS, an RRC configuration is provided to inform that the UE should apply the first or the second indicated TCI state to the AP CSI-RS resource.

On beam indication for S-DCI based M-TRP PDSCH, RAN1 has had several debates on how to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception. In RAN1#111, we finally agreed that a new indicator field is introduced to inform UE which indicated TCI state(s) to be applied for PDSCH. From what we have agreed, network has the flexibility to decide whether to enable DCI signalling for dynamic indication for beam association. If the new indicator field is configured, it can indicate which indicated TCI state to apply for PDSCH reception.

RAN1 #112 Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

However, as quoted above, an open issue is that how to determine the default indicated TCI state when the new indicator field is absent. Another one related issue is how to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by a fallback DCI, i.e., DCI format 1_0. We observe that DCI format 1_0 is also a case that the new indicator field is not present. Hence, we should work on a unified solution for DCI format 1_1/1_2 with the new indicator field absent and for DCI format 1_0. Actually, in legacy, default beam determination for these two cases follows the same principle, that is, following scheduling CORESET beam. Hence, we suggest also following indicated TCI state for scheduling CORESET in Rel-18 unified TCI extension.

Proposal 2: Strive to have a unified solution for determining default indicated TCI state for DCI format 1_1/1_2 with the new indicator field absent and for DCI format 1_0.

Proposal 3: On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured without the new indicator field or DCI format 1_0,

- UE applies the same joint/DL TCI state(s) applied to the PDCCH with the DCI scheduling/activating PDSCH reception.

Back to the case where the new indicator field is configured, default beam may be also needed, i.e., when the scheduling offset of PDSCH is below threshold. As shown in below agreement, when the UE supports capability of two default beams, the UE would use both first and second indicated TCI states as default beam to receive PDSCH.

RAN1 #112 Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value

However, when the UE does not support capability of two default beams, how to determine default beam is unclear for now. In our views, we can keep the behaviour as simple as possible. Since only one default beam can be decided, we suggest UE just uses the first indicated TCI to buffer possible PDSCH below the threshold.

Proposal 4: UE uses the first indicated joint/DL TCI states to buffer the received signal before a threshold, if UE does not support the capability of two default beams for S-DCI based MTRP in FR2.

In previous RAN1 meetings, companies have discussed a lot on possible combinations of indicated unified TCI states. In addition, how to associate indicated unified TCI states with each channel and RS is also a hot issue on the table. In our views, before concluding these issues, there is one essential issue that RAN1 should first conclude. That is, what is the meaning that when only one unified TCI state is indicated in S-DCI mode?

In details, in S-DCI mode, if the beam indication DCI/MAC-CE indicates only one TCI state for a serving cell, which following case does it stand for?

- Case 1: It may be still M-TRP mode now in the serving cell; network just wants to update the common beam for one TRP

- Case 2: It is now S-TRP mode in the serving cell. No association is needed to indicate for each channel and RS

For Case 1, how to switch M-TRP and S-TRP mode, and whether to switch it dynamically can be further discussed. For Case 2, it may mean that switching between M-TRP and S-TRP is done dynamically by number of indicated TCI states. Also, network is not able to indicate one TCI just for updating one beam of a TRP when operating in M-TRP mode. Since this general issue would have an impact on future detailed design, we suggest that RAN1 should tackle this issue first.

In our views, Case 1 should be supported; otherwise, when NW operates M-TRP mode and would like to update one beam for just one TRP, NW needs to indicate two beams simultaneously. Such behavior is not quite efficient from signaling overhead perspective.

Proposal 5: RAN1 to discuss, in S-DCI mode, whether M-TRP mode is operated in a serving cell when only one unified TCI for a TRP is indicated for the serving cell.

Proposal 6: Support that NW can update beams for one TRP by just indicating unified TCIs for the TRP while still maintaining M-TRP mode.

In RAN1#111, RAN1 had an agreement on how to indicate unified TCI by DCI for S-DCI based M-TRP in the following.

RAN1 #111 Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

We recall that, in RAN1 #109(e), RAN1 has agreed to support that, for M-TRP S-DCI mode, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list. However, the details are still unclear. Considering impact brought by multiple TRP, current 3-bit TCI field (i.e., 8 TCI field codepoints) may not be enough to indicate all possible combinations. Not to mention more combinations would emerge from subsequent discussions when coming to separate TCI mode. Based on these factors, we suggest increasing the size of the TCI field. In such way, there are more TCI codepoints to indicate possible combinations. According to above discussion, we propose the following proposals.

Proposal 7: Increase the size of the TCI field for indicating unified TCI at least for S-DCI based MTRP.

Another one critical issue is how UE understands which TRP is associated with an indicated TCI. UE needs to understand this information for updating all channels/RSs, which are associated with the TRP and suitable for applying unified TCI.

Proposal 8: FFS on how to indicate UE that which TRP is associated with an indicated TCI.

- Conclusion

According to the above discussion(s), we have the following observation(s) and proposal(s).

Proposal 1: On unified TCI framework extension for S-DCI based MTRP, for an aperiodic CSI-RS, an RRC configuration is provided to inform that the UE should apply the first or the second indicated TCI state to the aperiodic CSI-RS resource.

Proposal 2: Strive to have a unified solution for determining default indicated TCI state for DCI format 1_1/1_2 with the new indicator field absent and for DCI format 1_0.

Proposal 3: On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured without the new indicator field or DCI format 1_0,

- UE applies the same joint/DL TCI state(s) applied to the PDCCH with the DCI scheduling/activating PDSCH reception.

Proposal 4: UE uses the first indicated joint/DL TCI states to buffer the received signal before a threshold, if UE does not support the capability of two default beams for S-DCI based MTRP in FR2.

Proposal 5: RAN1 to discuss, in S-DCI mode, whether M-TRP mode is operated in a serving cell when only one unified TCI for a TRP is indicated for the serving cell.

Proposal 6: Support that NW can update beams for one TRP by just indicating unified TCIs for the TRP while still maintaining M-TRP mode.

Proposal 7: Increase the size of the TCI field for indicating unified TCI at least for S-DCI based MTRP.

Proposal 8: FFS on how to indicate UE that which TRP is associated with an indicated TCI.

# Reference

- RP-213598 New WID: MIMO Evolution for Downlink and Uplink, Samsung

- Draft Report of 3GPP TSG RAN WG1 #110bis-e v0.1.0 (Online, 10th – 19th October 2022)

- Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0 (Toulouse, France, 14th – 18th November 2022)