3GPP TSG RAN WG1 #112bis-e			R1- 2303520

e-Meeting, April 17th – April 26th, 2023

Title:	DRAFT Reply LS on NR support for dedicated spectrum less than 5 MHz for FR1

Response to:	R1-2302269 (R4-2303646)

Release:	Release 18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	Nokia (RAN WG1)

To:	RAN WG4

Cc:	-

Contact Person:

#### Name:	Timo Lunttila

#### E-mail Address:	timo.lunttila@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thanks RAN4 for providing the reply LS in [1] with answers to RAN1’s questions. RAN1 has discussed the questions from RAN4 and would like to provide the following responses:

RAN4’s question #1: For 3 MHz channel bandwidth, it was observed that the choice of the PBCH puncturing will impact the choice of finer synchronization raster. Therefore, RAN4 would like to ask RAN1 how PBCH within SSB will be punctured?

RAN1’s reply #1: At RAN1#112, RAN1 reached the following Working Assumption:

Working Assumption For transmission bandwidth[s] of <5MHz, for PBCH, in the case[s] that available PRBs for PBCH transmission is less than 20PRB, - PBCH based on RB-level puncturing (i.e., PBCH encoding is based on 20PRB. The encoded bits and DMRS are mapped to 20PRBs based on legacy SSB structure, and those PRBs that fall outside of available PRBs for PBCH transmission are punctured) - Note: No other optimization is needed

RAN1 will continue discussion on the details of PBCH puncturing at RAN1#113.

RAN4’s question #2: Finally, RAN4 would like to ask RAN1 what is the number of RBs in the SSB with punctured PBCH.

RAN1’s reply #2: RAN1 have not yet made agreements related to the number of RBs in the SSB with punctured PBCH. However, at RAN#99, as indicated in [2], it was agreed that:

- For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4), PBCH transmission bandwidth is 12 PRBs

- RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands.

- For the 5MHz channel bandwidth PBCH transmission bandwidth is 20 PRBs

RAN1 will continue discussion on the details of number of RBs in the SSB with punctured PBCH at RAN1#113.

2. Actions:

To RAN4 group.

ACTION: 	RAN1 respectfully requests RAN4 to take the provided information into account.

3. Dates of the next TSG-RAN WG1 meetings:

TSG-RAN1 Meeting#113		 22nd – 26th May 2023		Incheon, Korea

References:

[1] R1-2302269, “Reply LS on NR support for dedicated spectrum less than 5 MHz for FR1”, RAN4

[2] RP-230780, “Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz”, RAN