3GPP TSG RAN WG1 #112bis-e	R1-2303531

e-Meeting, April 17th – 26th, 2023

Source:	Lenovo

Title:	Network energy saving techniques in spatial and power domains

Agenda Item:	9.7.1

Document for:	Discussion and Decision

# Introduction

In RAN1#112 [1], the following agreements were made for spatial and power domain techniques for network energy savings enhancements:

Agreement For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases - Type 1: all antenna elements associated to a logical antenna port is disabled/enabled - Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns” Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account. Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included Agreement For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline. Agreement Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

In this contribution document, we discuss our views on the potential NES-based techniques in both spatial domain and power domain. More specifically, we shed light on potential NES techniques corresponding to the WID, in light of the sub-techniques provided in TR 38.864 [2]. Furthermore, we discuss the potential specification impact of the corresponding NES techniques.

# Spatial-domain NES techniques

In a network operating with multiple antennas/antenna panels, it may be desirable to dynamically turn on/off a certain antenna group or antenna panel, e.g., between an active state and an inactive state, based on traffic conditions to reduce energy consumption. In order to support such features without significantly impacting the performance at the UE(s), e.g., with respect to throughput and/or latency, the network should be able to properly select a subset of antenna ports, antenna elements or antenna panels at a given time for DL transmission to reduce the network energy consumption whenever necessary. In the sequel, we provide an overview of some potential spatial-domain NES techniques.

## 2.1 CSI resource configuration enhancements

One way to achieve notable NES is to reduce the transmission and/or signaling overhead associated with CSI framework, both for CSI and BM purposes. For instance, a CSI configuration that corresponds to NES would be associated with one of the following characteristics

- Alt1. An NZP CSI-RS resource set for BM comprising a reduced number of active beams

- Alt2. A CMR with a reduced number of CSI-RS ports

- Alt3. A CMR configured with a reduced frequency density value

- Alt4. A P/SP CMR configured with a larger periodicity value

As mentioned above, Alt1 aims at reducing the number of beams and hence reducing the number of active CSI-RS resources associated with beam measurement and reporting. Alt2 reduces the number of antenna ports when NES is activated. Alt3 reduces the density of the CSI-RS symbols over RBs, whereas Alt4 increases the periodicity value, i.e., time period between two consecutive CSI-RS transmissions, for periodic and/or semi-persistent CSI-RS transmission.

Note that to enable energy savings at the network side, two modes may need to be supported for Rel-18 capable UEs, corresponding to whether the NES mode is activated or deactivated. By default, semi-static switching between the two modes is supported in legacy design via RRC re-configuration of a new CSI reporting configuration, and additionally, dynamic switching via DCI triggering of aperiodic CSI reporting configurations is also supported. Whether dynamic indication of the NES mode via DCI or MAC CE corresponding to P/SP CSI reporting should be further discussed.

Two CSI resource configurations, corresponding to whether the NES mode is activated or deactivated, are supported

Evaluate whether dynamic switching between two P/SP CSI reporting configurations corresponding to activated and deactivated NES modes is needed

## 2.2 Antenna adaptation for NES

In RAN1#112, two types of NES spatial domain adaptation have been discussed, as follows:

- Type1. Antenna port adaptation, i.e., turning on/off a subset of the antenna ports

- Type2. Antenna element per port adaptation, i.e., turning on/off a subset of antenna elements within an antenna port without modifying the activated antenna ports

Under Type1, it is assumed that the same set of antenna elements per antenna port are configured whether NES is in an activated or a deactivated mode, where only a subset of the antenna ports is activated when the NES mode is activated. Note that Type1 incurs specification impact corresponding to signaling the activated/deactivated subset of antenna ports corresponding to the NES mode being activated. Moreover, UE-assisted selection of antenna ports can help reduce the performance loss associated with the antenna sub-selection, where the performance loss reduction can be carried out via selecting a subset of the antenna ports with the best channel gain. In our opinion, two approaches for UE-based signaling of the selected antenna ports for NES can be supported, as follows

- Type1-A. Port-selection codebook-based antenna port indication. This approach utilizes the port-selection reporting mechanism supported in Type-II PS codebooks, e.g., Rel-17 FeType-II PS codebook to feed back indices of a subset of the antenna ports to be associated with the NES mode being activated.

- Type1-B. CRI-based antenna port indication. This approach exploits the CRI field of a CSI report to indicate an antenna port group ID from a set of pre-configured antenna port groups corresponding to the same CMR, and hence enables feeding back indices of a subset of the antenna ports to be associated with NES.

Further discussion on the alternatives above are needed. Hence, we have the following proposal

For Type1 spatial domain adaptation, evaluate the following sub-types for determining the selected antenna ports when the NES mode is activated

- Type1-A. Antenna port group indication via port-selection parameter in Type-II PS codebook type

- Type1-B. Antenna port group indication via CRI field, where different CRI codepoints correspond to different antenna port groups of the same CMR

On the other hand, Type2 has no straightforward specification impact since the antenna ports are unchanged, however, modifying the on/off state of antenna elements per antenna port based on whether NES mode is activated or deactivated may change the beam pattern and hence impact the QCL relationships between different UL/DL RSs, e.g., QCL relationship between CSI-RS resources with SSB or DMRS for PDSCH. Hence, whether antenna element adaptation within an antenna port impacts the accuracy of the QCL relationships needs to be evaluated.

For Type2 spatial domain adaptation, evaluate whether the antenna element per port adaptation impacts the accuracy of the QCL relationships between DL/UL RSs

## 2.3 CSI reporting configuration for NES

- In RAN1#112, two alternatives for CSI reporting configuration corresponding to NES were discussed, as follows:

- Alt1. Independent/separate CSI reporting settings where each CSI report configuration corresponds to one spatial domain adaptation pattern

- Alt2. One CSI reporting setting contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial domain adaptation pattern

- Clearly, Alt1 is a straightforward solution for adopting two spatial domain adaptation patterns corresponding to the whether the NES mode is activated or deactivated, whereas Alt2 requires modifying the current CSI reporting setting to allow configuring two spatial domain adaptation patterns. In our opinion, Alt2 is preferred since most CSI reporting setting parameters would be shared between the two candidate spatial domain adaptation patterns, e.g., report quantities, reporting bands and codebook configuration. Moreover, configuring a unified CSI reporting setting also facilitates supporting a solution in which the UE feeds back a single CSI report comprising CSI corresponding to a UE-selected spatial domain adaptation pattern, or a dynamically indicated spatial domain adaptation pattern from a set of higher-layer configured patterns.

Support a single CSI reporting setting corresponding to multiple higher-layer configured spatial domain adaptation patterns

- For a single CSI reporting setting, the specification impact can be in the form of indicating multiple CSI resource settings for channel measurement, corresponding to the different spatial domain adaptation patterns. Alternatively, one CSI resource setting can be supported, where the NZP CSI-RS resource set associated with channel measurement would comprise multiple NZP CSI-RS resources associated with the multiple spatial domain adaptation patterns.

For a CSI reporting setting corresponding to multiple higher-layer configured spatial domain adaptation patterns, support one of the following alternatives

- Alt1. Multiple CSI resource settings for channel measurement corresponding to the multiple spatial domain adaptation patterns

- Alt2. A single CSI resource setting for channel measurement associated with one NZP CSI-RS resource set, where the NZP CSI-RS resource set further comprises multiple NZP CSI-RS resources for channel measurement corresponding to the multiple spatial domain adaptation patterns

- Finally, in case the UE reports CSI corresponding to multiple spatial domain adaptation patterns, further discussion is needed on whether the CSI hypotheses corresponding to the multiple spatial domain adaptation patterns are reported over multiple CSI reports or within the same CSI report. We would like to note that a similar discussion has taken place in Rel-17 CSI enhancements for NCJT, where it was agreed that CSI corresponding to multiple (up to three) CSI hypotheses are reported in the same CSI report, and hence one CSI report may comprise multiple, CRI, RI, PMI, CQI and LI values. Considering that, we prefer supporting a single CSI report for reporting CSI corresponding to multiple spatial adaptation patterns, similar to Rel-17 NCJT enhancements.

A single CSI report is supported for reporting CSI corresponding to multiple spatial domain adaptation patterns

## 2.4 Panel adaptation for NES

- One other approach that can help reduce the network energy in multi-panel deployments is via switching on/off a subset of the panels of a TRP during DL transmission, e.g., over PDSCH. Under multi-panel transmission, dynamic switching to single- panel/TRP transmission is supported in legacy design.

Dynamic switching between single-panel operation and multi-panel operation supported in legacy design can be used for NES purposes

- Additionally, for Rel-17 HST-SFN scheme, dynamic switching to single-TRP transmission is supported as an optional UE feature.

Dynamic switching between single-panel operation and Rel-17 SFN operation supported in legacy design as an optional feature can be used for NES purposes

Also, more recently, a Rel-18 work item targeting CSI reporting enhancements under CJT [3] is under discussion, with NPanel value up to 4 panels. In RAN1#110bis-e [4], it was agreed to support UE-assisted panel selection, where the UE selects a subset N’ of the NPanel panels, where N'≤NPanel. In case the NES mode is activated, further restriction on the value of N’ can be supported, e.g., N'≤2, to ensure energy savings are achieved.

If the NES mode is activated for DL CJT with NPanel panels, only a subset N'<NPanel of the panels are activated.  FFS: the value of N’

For Rel-17 BM under multi-TRP/panel transmission, when the higher layer parameter groupBasedBeamReporting-r17 in CSI-ReportConfig reporting setting is configured, a UE reports a configured number of groups of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of two CSI Resource Sets for a report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE. However, a UE cannot report the best N beams for each antenna panel independently in one CSI report. For Rel-18 NR, it would be beneficial that a UE can report the best N number of beams independently for each panel, with or without group reporting in each report setting when multiple CSI resource sets corresponding to multiple antenna panels are configured in each reporting setting. This enhanced beam reporting allows the network to dynamically switch between single-panel transmission and multi- panel transmission and to properly select its active antenna panel(s) at a given time. It is desired that the enhanced beam reporting maintains same or similar configuration signaling overhead and measurement time compared to Rel-17 group based beam reporting.

Support enhanced beam reporting, which allows a UE to report the best N beams for each antenna panel independently in one CSI report for NES purposes

# Power-domain NES techniques

One important technique to reduce the network energy consumption is via DL transmission power adaptation. While DL power control/adaptation is assumed in many cases to be a network-implementation feature without notable specification impact, the network still needs to signal to the UE(s) the ratio of the PDSCH EPRE to the NZP CSI-RS EPRE, i.e., powerControlOffset, a legacy RRC parameter, when deriving the CSI feedback so as to enable the UE to acquire a more precise characterization of the CQI that is signaled as part of the CSI report. In this section, we discuss different approaches of signaling the power control offset to the UE. Moreover, we discuss UE-assisted techniques that enable the network to select a proper power control offset that achieves a target tradeoff between the downlink performance and the energy consumption of the network.

## 3.1 Indication of the power control offset

As discussed, if the NES mode is activated, the network needs to signal the power control offset value(s) for deriving the CSI feedback, so that the UE can calibrate the CQI intended for PDSCH based on the NZP CSI-RS received as part of the CSI framework. In legacy design, the power control offset is semi-statically configured as part of the NZP CSI-RS resource IE, with a single value selected from a set of integer values of unit step from the range -8,…,15 dB, as explained in Clause 5.2.3.3.1 of [5]. Note that faster indication of the power control offset corresponding to toggling between the NES mode being activated or deactivated may be needed based on the traffic demand, scheduling restrictions and/or emission regulations. Given that, the following alternatives for power control offset indication are provided

- Alt1. Configuring two power control offset values per CSI-RS resource with selection of the active power control offset based on the NES mode activation. Under this alternative, two power control offset values are configured in the NZP CSI-RS resource configuration corresponding to whether the NES mode is activated or deactivated, where the activated power control offset is identified via a dynamic indicator parameter that activates or deactivates the NES mode

- Alt2. Dynamic indication of the power control offset, based on value(s) of the legacy RRC-configured parameter powerControlOffset, e.g., via DCI indication. For instance, the power control offset can be indicated via a codepoint corresponding to the CSI request field in PUSCH-scheduling DCI, e.g., DCI Format 0_2

- Alt3. MAC-CE based indication of the power control offset, based on value(s) of the legacy RRC-configured parameter powerControlOffset

While the three alternatives above differ in the configuration, signaling and triggering details, they all enable faster switching/update of the power control offset corresponding to the ratio of the PDSCH EPRE to the NZP CSI-RS EPRE. Further discussion of these alternatives is needed for evaluation and potential down selection. We therefore have the following proposal

For power control offset indication for Rel-18 NES-capable UEs, evaluate the following alternatives for down selection:

- Alt1. Configuring two power control offset values per CSI-RS resource, along with dynamic indication of whether the NES mode is activated or deactivated

- Alt2. Dynamic indication of the power control offset, e.g., via DCI indication

- Alt3. MAC-CE based power control offset indication

## 3.2 UE-assisted power control offset selection

One other aspect that needs enhancement for specification of power-based NES techniques is supporting UE-assisted power control offset reporting to help the network to select a proper power control offset that achieves a target tradeoff between the downlink performance and the energy consumption of the network. For instance, the UE can aid the network select the best power control offset that achieves a metric defined by the network, e.g., SINR, or alternatively report multiple values of the metric corresponding to pre-configured power control offset values provided by the network, e.g., different CQI values for different power control offset values. The following alternatives provide different techniques for UE-assisted power control offset selection

- Alt1. The UE selects and reports a power control offset value as part of the CSI report based on a target CQI identified by the network corresponding to the NES mode being activated. Under this approach, the UE computes the lowest power control offset value that achieves a configured WB CQI target value, to help the network reduce the energy consumption conditioned on a quality-of-service constraint

- Alt2. Multiple CSIs assuming multiple hypothetical power control offset values are reported. The hypothetical power control offset values may include all or a subset of power control offsets configured in the CSI-RS resource IE or candidate power control offsets in DCI or MAC CE. The UE reports multiple CSIs within one or more CSI report, where each CSI comprises a distinct CQI value corresponding to a distinct power control offset value. Alternatively, the UE can report CQI values corresponding to a differential CQI value with respect to different power control offset values, e.g., associated with the NES mode being activated or deactivated, or associated with two candidate power control offset values indicated in DCI or MAC CE.

Based on the alternatives discussed above, we have the following proposal

Evaluate the following UE-assisted power control offset selection techniques for possible down selection for Rel-18 NES-capable UEs:

- Alt1. Reporting a power control offset value based on a target WB CQI configured by the network

- Alt2. Reporting two CQI values associated with two power control offset values corresponding to whether NES mode is deactivated or activated, respectively

  - FFS: Whether the second CQI value is a differential CQI value with respect to the first CQI value

- In general, when the UE is configured to report two CQI values associated with two power control offset values corresponding to whether NES mode is deactivated or activated, respectively, a CQI value associated with the second power control offset value (where NES mode is activated) cannot be larger than a corresponding CQI value associated with the first power control offset value (where NES mode is deactivated). Therefore for Alt2, the CSI feedback overhead corresponding to the second CQI value reported in the CSI report can be reduced when the CQI format is set to ‘subband’ via reporting one bit per subband differential CQI value, where the first of the two possible offset values is zero corresponding to no change in CQI value, and the second offset value is -1 corresponding to a one-level drop of the CQI value.

If the UE is configured to report two CQI values associated with two power control offset values corresponding to whether NES mode is deactivated or activated, respectively, the subband differential CQI values corresponding to the second power control offset are reported via one bit each

# Conclusion

This contribution addressed spatial-domain and power-domain based NES techniques. In light of the discussion in this technical document, we have the following proposals:

- Two CSI resource configurations, corresponding to whether the NES mode is activated or deactivated, are supported

- Evaluate whether dynamic switching between two P/SP CSI reporting configurations corresponding to activated and deactivated NES modes is needed

- For Type1 spatial domain adaptation, evaluate the following sub-types for determining the selected antenna ports when the NES mode is activated

- Type1-A. Antenna port group indication via port-selection parameter in Type-II PS codebook type

- Type1-B. Antenna port group indication via CRI field, where different CRI codepoints correspond to different antenna port groups of the same CMR

- For Type2 spatial domain adaptation, evaluate whether the antenna element per port adaptation impacts the accuracy of the QCL relationships between DL/UL RSs

- Support a single CSI reporting setting corresponding to multiple higher-layer configured spatial domain adaptation patterns

- For a CSI reporting setting corresponding to multiple higher-layer configured spatial domain adaptation patterns, support one of the following alternatives

- Alt1. Multiple CSI resource settings for channel measurement corresponding to the multiple spatial domain adaptation patterns

- Alt2. A single CSI resource setting for channel measurement associated with one NZP CSI-RS resource set, where the NZP CSI-RS resource set further comprises multiple NZP CSI-RS resources for channel measurement corresponding to the multiple spatial domain adaptation patterns

- A single CSI report is supported for reporting CSI corresponding to multiple spatial domain adaptation patterns

- Dynamic switching between single-panel operation and multi-panel operation supported in legacy design can be used for NES purposes

- Dynamic switching between single-panel operation and multi-panel operation supported in legacy design can be used for NES purposes

- If the NES mode is activated for DL CJT with NPanel panels, only a subset N'<NPanel of the panels are activated.  FFS: the value of N’

- Support enhanced beam reporting, which allows a UE to report the best N beams for each antenna panel independently in one CSI report for NES purposes

- For power control offset indication for Rel-18 NES-capable UEs, evaluate the following alternatives for down selection:

- Alt1. Configuring two power control offset values per CSI-RS resource, along with dynamic indication of whether the NES mode is activated or deactivated

- Alt2. Dynamic indication of the power control offset, e.g., via DCI indication

- Alt3. MAC-CE based power control offset indication

- Evaluate the following UE-assisted power control offset selection techniques for possible down selection for Rel-18 NES-capable UEs:

- Alt1. Reporting a power control offset value based on a target WB CQI configured by the network

- Alt2. Reporting two CQI values associated with two power control offset values corresponding to whether NES mode is deactivated or activated, respectively

  - FFS: Whether the second CQI value is a differential CQI value with respect to the first CQI value

- If the UE is configured to report two CQI values associated with two power control offset values corresponding to whether NES mode is deactivated or activated, respectively, the subband differential CQI values corresponding to the second power control offset are reported via one bit each

# References

| [1] | 3GPP TSG RAN WG1 #112, "Draft Meeting Report," Athens, Greece, Feb. 27 - Mar. 3, 2023. |
| --- | --- |
| [2] | 3GPP TR 38.864, "Study on network energy savings for NR," Dec. 2022. |
| [3] | RP-213598, Samsung, "New WID: MIMO Evlution for Downlink and Uplink," Online, Dec. 6-17, 2021. |
| [4] | 3GPP TSG RAN WG1 #110bis-e, "Draft Meeting Report," Online, Oct. 10-19, 2022. |
| [5] | 3GPP TS 38.214, "Physical layer procedures for data," Dec. 2022. |