3GPP TSG RAN WG1 #112b-e	R1-2303537

e-Meeting, Apr 17 – Apr 26, 2023

Agenda item:		9.13.1

Source:	Nordic Semiconductor ASA

Title:					On LP-WUS evaluation

Document for:		Discussion and Decision

# Introduction

In RAN#97, study item in RP-222644 has been approved, and in RAN1#110b-e RAN1#111 several agreements reached.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

# Discussion

## Use-cases

Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any.

Industrial sensors, controllers and actuators are mentioned as the IoT use-case. Sensing is MO data use-case, and reporting is typically event driven, there is no need for wake-up signal, per se. On the other hand, as outcome of sensing, actuators and controllers must be contacted to prevent severe damage to the production line or its components, e.g. gas valves to be shut, engine(s) stopped or spins reduced. We believe latency of seconds could be a typical use-case, where 10s could be the upper-bound, 100 of ms could be the lower bound and mainstream corresponding to iDRX cycle of 1-2s.

Proposal-1: For the latency of industrial IoT use-case, consider 100ms to 10s as design criteria.

## On LP-WUS coverage

Agreement - For LP-WUS coverage evaluation, the noise figure of LP-WUR is - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies - FFS: how to determine the NF option. - The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion. Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

Methodology for coverage enhancements has been agreed, what remains open is the coverage target. From Table 1 and Table 2 (considering WUS Required SNR to be 0), it may become obvious that LP-WUS may have hard time to match the coverage of PDCCH. Even if Noise figure would be the best possible (in the range of agreed values) the required SNR would need to be pushed down from 0dB to -7dB to match PDCCH coverage. This resulting in very low data rates. Therefore, to give room for worse noise figure, LP-WUS should be designed to target coverage not worse than that of PUSCH.

Proposal-2: Design of LP-WUR should target coverage not worse than that of PUSCH with target bitrate as specified in TS38.830.

Table 1 MIL RedCap 1Rx 700MHz

| 700MHz (15kHz SCS) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR CSS PDCCH | NR PUSCH | LP-WUS | LP-WUS |
| Carrier BW (MHz) | 20,00 | 20,00 | 4,00 | 4,00 |
| PSD (dBm/MHz) | 36,00 | - | 36,00 | 36,00 |
| Occupied BW (PRBs) | 48,00 | 4,00 | 11,00 | 11,00 |
| Occupied BW (MHz) | 8,64 | 0,72 | 1,98 | 1,98 |
| Tx Power in occupied BW(dBm) | 45,37 | 23,00 | 38,97 | 38,97 |
| Tx Array gain | 0,00 |  | 0,00 | 0,00 |
| Tx Antenna Gain (dB) | 8,00 | 0,00 | 8,00 | 8,00 |
| Tx EIRP (dBm) | 53,37 | 23,00 | 46,97 | 46,97 |
| Rx Antenna gain (dB) | 0,00 | 8,00 | 0,00 | 0,00 |
| Beamforming Rx gain (dB) | 3,00 | 9,00 | 0,00 | 0,00 |
| Thermal noise density (dBm/Hz) | -174,00 | -174,00 | -174,00 | -174,00 |
| Rx interference density (dBm/Hz) [37.910] | -169,30 | -165,70 | -169,30 | -169,30 |
| Rx Noise figure (dB) | 7,00 | 5,00 | 9,00 | 24,00 |
| Total Rx Noise + int density (dBm/Hz) | -164,99 | -164,03 | -163,63 | -149,95 |
| Effective noise power (dBm) | -95,62 | -105,46 | -100,66 | -86,98 |
| Required SNR* (dB) from [1] | -3,10 | -2,40 | 0,00 | 0,00 |
| Rx sensitivity (dBm) | -98,72 | -107,86 | -100,66 | -86,98 |
| Link budget (MIL) (dB) | 155,09 | 147,86 | 147,63 | 133,95 |

Table 2 MIL RedCap 1Rx 2.6GHz

| 2,6GHz (30kHz SCS) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR CSS PDCCH | NR PUSCH | LP-WUS | LP-WUS |
| Carrier BW (MHz) | 20,00 | 20,00 | 4,00 | 4,00 |
| PSD (dBm/MHz) | 33,00 | - | 33,00 | 33,00 |
| Occupied BW (PRBs) | 48,00 | 30,00 | 11,00 | 11,00 |
| Occupied BW (MHz) | 17,28 | 10,80 | 3,96 | 3,96 |
| Tx Power in occupied BW(dBm) | 45,38 | 23,00 | 38,98 | 38,98 |
| Tx Array gain | 0,00 |  | 0,00 | 0,00 |
| Tx Antenna Gain (dB) | 8,00 | 0,00 | 8,00 | 8,00 |
| Tx EIRP (dBm) | 53,38 | 23,00 | 46,98 | 46,98 |
| Rx Antenna gain (dB) | 0,00 | 8,00 | 0,00 | 0,00 |
| Beamforming Rx gain (dB) | 3,00 | 9,00 | 0,00 | 0,00 |
| Thermal noise density (dBm/Hz) | -174,00 | -174,00 | -174,00 | -174,00 |
| Rx interference density (dBm/Hz) [37.910] | -169,30 | -165,70 | -169,30 | -169,30 |
| Rx Noise figure (dB) | 7,00 | 5,00 | 9,00 | 24,00 |
| Total Rx Noise + int density (dBm/Hz) | -164,99 | -164,03 | -163,63 | -149,95 |
| Effective noise power (dBm) | -92,61 | -93,70 | -97,65 | -83,97 |
| Required SNR* (dB) from [1] | -3,00 | -10,50 | 0,00 | 0,00 |
| Rx sensitivity (dBm) | -95,61 | -104,20 | -97,65 | -83,97 |
| Link budget (MIL) (dB) | 151,99 | 144,20 | 144,63 | 130,95 |

## LP-WUS power saving

### Paging rate

Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note： - For i-DRX with i-DRX cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - L=4 (as agreed in RAN1#110bis)

As can be seen above, for e-DRX it was agreed to set UE paging probability higher for the first i-DRX cycle within PTW. The equation for the first i-DRX cycle within the PTW takes into account only the probability of K-L i-DRX cycles, the equation for the remaining i-DRX cycles is only used for L-1 i-DRX cycles. Therefore, in our opinion there is an error. For the first iDRX cycle p, the probability of data arrival should be 1 minus probability that no data arrived in K-L+1 cycles. This is illustrated in Figure 1.

Figure 1

Observation-1: For e-DRX, the per UE paging probability of the first i-DRX cycle contribution of DRX cycle is ignored in current methodology.

Proposal-3: For e-DRX, per UE paging probability should corrected to  RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF

### Power consumption evaluation

We evaluated power consumption for the following cases

- With RRM measurement (Table 5 and 6) and with no RRM measurement (Table 7).

- WUR wakes-up every iDRX cycle (1,28s) 500ms before PO, for 1ms with 5ms ramp-up time

- MR operates with 1.28s iDRX cycle with up to 1s latency (Table 1) or eDRX (20cycles) with up to 20s latency (Table 6).

- Table 3 and Table 4 summarizes the remaining assumptions.

Table 3 WUR prams

| Parameter | Value |
| --- | --- |
| WUR passive | 0,001 [units x ms] |
| WUR consumption | 4 [units x ms] |
| Preamble tracking cycle | 1280ms |
| Active time | 1ms |
| Transition time | 5ms |

Table 4 Main radio params

| Parameter | Value |
| --- | --- |
| iDRX | 1280ms |
| eDRX | Yes, 20 cycles + 4 PTW cycles |
| SSB periodicity | 20ms |
| Number of SSBs ahead PO | 6 |
| Wake up from ultra-deep sleep penalty | 15000 units |
| PEI | no |
| RRM | 8ms each eDRX |

Table 5 Average power consumption [unit x ms] for IoT use-case FAR=1%, N=10, compared to MR in iDRX, RRM measurements

| Re,REF | 1,00 % | 0,10 % | 0,01 % | 0,00 % |
| --- | --- | --- | --- | --- |
| Group paging (iDRX) | 15,52 % | 7,87 % | 0,97 % | 0,10 % |
| Re iDRX | 1,00 % | 0,10 % | 0,01 % | 0,00 % |
| FAR | 1% |  |  |  |
| MR | 4,341 |  |  |  |
| WUR | 0,012 |  |  |  |
| UDS Penalty | 11,719 |  |  |  |
| With MR+WUR | 2,855 | 1,659 | 0,580 | 0,444 |
| Relative cons. | 65,78 % | 38,21 % | 13,36 % | 10,23 % |

Table 6 Average power consumption [unit x ms] for IoT use-case FAR=1%, N=10, compared to MR in eDRX, RRM measurements

| Re,REF | 1,00 % | 0,10 % | 0,01 % | 0,00 % |
| --- | --- | --- | --- | --- |
| Group paging (iDRX) | 15,52 % | 7,87 % | 0,97 % | 0,10 % |
| Re iDRX | 1,00 % | 0,10 % | 0,01 % | 0,00 % |
| FAR | 1% |  |  |  |
| MR | 1,884 |  |  |  |
| WUR | 0,012 |  |  |  |
| UDS Penalty | 11,719 |  |  |  |
| With MR+WUR | 2,855 | 1,659 | 0,580 | 0,444 |
| Relative cons. | 151,53 % | 88,02 % | 30,77 % | 23,57 % |

Table 7 Average power consumption [unit x ms] for IoT use-case FAR=1%, N=10, compared to MR in iDRX, no RRM measurements

| Re,REF | 1,00 % | 0,10 % | 0,01 % | 0,00 % |
| --- | --- | --- | --- | --- |
| Group paging (iDRX) | 15,52 % | 7,87 % | 0,97 % | 0,10 % |
| Re iDRX | 1,00 % | 0,10 % | 0,01 % | 0,00 % |
| FAR | 1% |  |  |  |
| MR | 4,070 |  |  |  |
| WUR | 0,012 |  |  |  |
| UDS Penalty | 11,719 |  |  |  |
| With MR+WUR | 2,585 | 1,388 | 0,310 | 0,174 |
| Relative cons. | 63,51 % | 34,11 % | 7,60 % | 4,27 % |

From Table 5, Table 6 and Table 7 one can draw the following

Observation-2:

- When group paging rate is high, WUR is not bringing benefit compared to eDRX. However, max latency is decreased from 20 to 1 second.

- When latency of MR and MR+WUR is comparable, WUR reduces the power consumption from 65% to 10% depending on group paging rate.

- When RRM measurements are offloaded to WUR and assuming comparable latency between MR and MR+WUR, it is possible to reduce consumption of MR+WUR down to 5% of MR.

- When measurements are offloaded, FAR is reduced from 1%->0.1%, and with comparable latency, it is possible to reduce consumption of MR+WUR down to <1% of MR.

Few comments on the current evaluation methodology

- result-sheet could collect average energy consumption x ms rather than units across whole DRX or eDRX. This would allow better comparison between different scenarios.

- data transmission should be take into account, currently wake-up due to FAR or wake-up of other UE in the group and wake-up of UE itself is treated the same way. This is far from reality.

Observation-3: Power consumption evaluation methodology modelling could be further improved (see above comments).

# Conclusions

In this contribution we discussed issues related to LP-WUS evaluation and we had the following observations and proposals:

Proposal-1: For the latency of industrial IoT use-case, consider 100ms to 10s as design criteria.

Proposal-2: Design of LP-WUR should target coverage not worse than that of PUSCH with target bitrate as specified in TS38.830.

Observation-1: For e-DRX, the per UE paging probability of the first i-DRX cycle contribution of DRX cycle is ignored in current methodology.

Proposal-3: For e-DRX, per UE paging probability should corrected to RE = 1 – (1 – RE, REF )(K-L+1)Y/YREF

Observation-2:

- When group paging rate is high, WUR is not bringing benefit compared to eDRX. However, max latency is decreased from 20 to 1 second.

- When latency of MR and MR+WUR is comparable, WUR reduces the power consumption from 65% to 10% depending on group paging rate.

- When RRM measurements are offloaded to WUR and assuming comparable latency between MR and MR+WUR, it is possible to reduce consumption of MR+WUR down to 5% of MR.

- When measurements are offloaded, FAR is reduced from 1%->0.1%, and with comparable latency, it is possible to reduce consumption of MR+WUR down to <1% of MR.

Observation-3: Power consumption evaluation methodology modelling could be further improved (see above comments).

# References

- Saeedeh Moloudi et. al., “Coverage Evaluation for 5G Reduced Capability New Radio (NR-RedCap)”, IEEE