3GPP TSG RAN WG1 #112bis-e			R1-2303542

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.9.3

Source:	InterDigital, Inc.

Title:	Disabling of HARQ feedback in IoT-NTN

Document for:	Discussion and Decision

# Introduction

RAN1 has made the following agreements to narrow down the options for enabling/disabling of HARQ for eMTC and NB-IoT in RAN1 #112 [2]:

Agreement

For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity

- Alternative 1: applies to both semi-statically HARQ enabled and disabled processes

- Alternative 2: only applied to semi-statically HARQ disabled processes

- Alternative 3: only applied to semi-statically HARQ enabled processes

Agreement

Confirm the following working assumption with the following update:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

- Support Option 1 in case only per-HARQ process bitmap signaling is configured

- Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured

- Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes.

  - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI

  - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration.

RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”.

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Agreement

For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc

- Option 1: Indication by adding one field in DCI

- Option 2: Indication by reusing/reinterpreting existing field in DCI

In this contribution, we further discuss on the working assumption listed above for enabling/disabling DL HARQ feedback for NB-IoT NTN as well as for eMTC NTN.

# Discussion

## Remaining details of enabling/disabling HARQ feedback

In RAN1 #112, it has been agreed that three modes of operation for enabling/disabling HARQ feedback for NB-IoT NTN are supported, such as RRC configuration (i.e., Option 1 only), DCI direct indication (i.e., Option 3 only), and DCI indication to override RRC configuration (i.e., Option 1+ Option 3), where which mode of operation to use is based on configuration of Option 1 and/or Option 3.

There are still remaining details how to apply the indication from DCI-based overridden mechanism and indication method for DCI-based overridden/direct indication.

The following three alternatives have been discussed as case for which the DCI-based overridden indication applies:

- Alt-1: indication applies to both semi-statically HARQ enabled and disabled processes

- Alt-2: indication only applies to semi-statically HARQ disabled processes

- Alt-3: indication only applies to semi-statically HARQ enabled processes

The one of main benefits of having DCI-based overridden indication is scheduling flexibility at the eNB as it can change HARQ enabling/disabling status temporarily without RRC reconfiguration based on traffic characteristics. Also, it is beneficial for the UE with only CP solution as well in terms of flexibility since RRC reconfiguration is not supported for those type of UEs. However, Alt-1 seems to be equivalent to the DCI direction indication mode of operation (i.e., Option 3 only) assuming that DCI-based overridden indication exists in the DCI always. Therefore, considering Option 3 only is already a possible configuration, Alt-1 doesn’t seem to provide any additional benefit.

From DCI overhead perspective, Alt-2 may allow to reuse existing DCI field (e.g., NDI) which is not used when corresponding HARQ process is disabled, therefore lower DCI overhead without restricting any scheduling flexibility. On the other hand, Alt-1 and Alt-3 may require introducing a new DCI field or reuse existing field with potential scheduling restriction.

For down-selection of alternatives, the indication option has to be decided together as some of alternative has benefit only when a specific indication option is used. For example, if a new DCI field is introduced for DCI-based overridden/direct indication, Alt-1 seems to be the best choice although it is equivalent to the Option 3 only as it provides flexibility in a given DCI overhead. If existing field in DCI is reused, Alt-2 could be a preferable option as it can reuse an existing DCI field without losing any scheduling flexibility.

Observation 1: The benefits of each alternative for applicable cases of DCI-based overridden indication are tied with indication option. Therefore, when down-selection decision is made, the indication option should be decided together.

Observation 2: considering that Option 3 only is a possible configuration, Alt-1 doesn’t seem to provide any additional benefit compared to the Option 3 only. On the other hand, the Alt-2 is attractive if existing DCI field is reused as it doesn’t affect to scheduling flexibility.

Proposal 1: Down-selection of alternatives for applicable cases should be done together with the down-selection of options for DCI-based overridden/direct indication.

Proposal 2: Support Alt-2 with reusing existing DCI field (i.e., NDI) for DCI-based overridden/direct indication.

## Impacts of DCI-based indication to upper layers

One issue with a DCI-based override mechanism is an impact to upper layers handling of DRX active time. In Rel-17 NR NTN, a UE adapts DRX timers based on the configured HARQ characteristics of a HARQ process. DRX is adapted for both UL and DL to either adapt DRX active time account for additional propagation delay (e.g. if HARQ feedback is enabled) or to enable additional UE power saving (e.g. if HARQ feedback is disabled). Operation is adapted as follows:

- if downlinkHARQ-FeedbackDisabled is configured for this serving cell, upon DL reception:

  - if HARQ feedback is enabled for the HARQ process the UE will extend the length of the DL HARQ RTT Timer by the UE-gNB RTT (i.e. propagation delay).

  - If HARQ feedback is disabled for the HARQ process, the UE will not start drx-RetransmissionTimerDL to enable additional power saving.

  - If  downlinkHARQ-FeedbackDisabled is not configured, the UE will apply legacy behaviour (i.e. start drx-RetransmissionTimerDL after expiry of drx-HARQ-RTT-TimerDL).

If the HARQ behaviour indicated by RRC configuration is modified by a DCI indication, an indication will need to be sent to upper layers to ensure correct DRX operation.

Proposal 3: If Option 3 DCI-based indicates HARQ enable/disable (either directly or overriding an RRC configuration) an indication is sent to upper layers (e.g., to adjust DRX operation)

# Conclusion

In this contribution the following observations and proposals were made concerning enabling/disabling DL HARQ feedback in IoT NTN:

Observation 1: The benefits of each alternative for applicable cases of DCI-based overridden indication are tied with indication option. Therefore, when down-selection decision is made, the indication option should be decided together.

Observation 2: considering that Option 3 only is a possible configuration, Alt-1 doesn’t seem to provide any additional benefit compared to the Option 3 only. On the other hand, the Alt-2 is attractive if existing DCI field is reused as it doesn’t affect to scheduling flexibility.

Proposal 1: Down-selection of alternatives for applicable cases should be done together with the down-selection of options for DCI-based overridden/direct indication.

Proposal 2: Support Alt-2 with reusing existing DCI field (i.e., NDI) for DCI-based overridden/direct indication.

Proposal 3: If Option 3 DCI-based indicates HARQ enable/disable (either directly or overriding an RRC configuration) an indication is sent to upper layers (e.g., to adjust DRX operation)

# References

RP-221806 – WID on IoT NTN enhancements

RAN1 #112 Chairman’s note