3GPP TSG-RAN WG1 #112bis-e	R1-2303553

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.2

Source:	Ericsson

Title:	Positioning using NR carrier phase measurement

Document for:	Discussion, Decision

Introduction

The objective for NR carrier phase-based positioning as stated by the WID [5] is:

- Specify physical layer measurements and signaling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In this contribution we present proposals in line with this objective.

RAN1#112:

Agreement

NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency.

- FFS: the reference point of the RSCP

- FFS: whether/how the measurement timing is defined

- Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion

- Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP

NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency.

- FFS: the reference point of the UL RSCP

- FFS: whether/how the measurement timing is defined

- Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion

- Note: The support of MIMO SRS for positioning is transparent to UE

Measurement definition and reporting

## Carrier phase of the first path and additional paths

Regarding the question whether additional paths should be supported we note that no evidence has been presented that shows that the phase of additional paths can be useful for positioning. Consequently, we propose to only support the first path.

Only support carrier phase measurements of the first detected path.

## Reference point

It has been agreed to support carrier phase measurement reporting together with the legacy positioning methods and DL TDOA and UL TDOA are the main candidates. To enable joint reporting of carrier phase measurements with RSTD (for DL TDOA) resp. RTOA (for UL TDOA), the reference point should be defined in the same way as for these legacy measurements.

For DL carrier phase measurements, use the same reference point as DL-RSTD: For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE.

For UL carrier phase measurements, use the same reference point as UL-RTOA.

RAN1#112:

Agreement

For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP.

- FFS: whether/how to define per path RSCPD

- Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP

Agreement

To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced:

- DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP.

  - FFS: The detailed definition of the DL CP

- DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs

  - FFS: The detailed definition of the DL CPD

To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced:

- UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE.

  - FFS: The detailed definition of the UL CP

Absolute or relative phase measurements

For joint reporting, carrier phase measurements should harmonize with DL RSTD resp. UL RTOA measurements. In the downlink case this means to introduce a DL RSCPD (carrier phase difference) measurement similar to RSTD. Following Proposal 1 we think that it is enough to define the phase difference measurement for the first path (of both the reference signal and the measured signal).

In downlink, support RSCPD of the first path but not for additional paths.

For UL RTOA there is actually only one measurement, and the reference time is defined from the frame-structure. Therefore, we believe that a RSCP measurement is more aligned with UL RTOA.

In uplink, support RSCP of the first path but not for additional paths.

RAN1#112:

Agreement

To support NR carrier phase positioning, further consider the following options:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier

  - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

Reporting one or multiple measurements within a PFL

Reporting phase measurements of more than one frequency within a PFL has been discussed during several meetings. The motivation has been that it can be used with the virtual frequency method for integer ambiguity resolution.

In this section we show that reporting multiple phase measurements is not needed. We show that

- The phase ϕa at frequency fa within the PFL can be obtained from the phase of the center frequency ϕc and the time-of-arrival τ0 (or equivalently RSTD, RTOA, etc.)

- When ϕa is obtained from {ϕc, τ0} as we propose, then it is more accurate and less noisy than

  - when ϕa is a reported subcarrier measurement, or

  - when ϕa has been calculated from a segment of the bandwidth.

Consequently, the virtual frequency method does not require reporting of more than the phase of the center frequency and the time-of-arrival. For joint reporting of carrier phase measurements together with DL/UL TDOA, only ϕc needs to be added.

### Derivation: Phase as function of frequency and time-of-arrival

The carrier phase of the first path is a linear function of the carrier frequency, see for instance the measurement models presented by different companies [7][8][9][10][11] and more. Fundamentally, all the mentioned contributions present carrier phase measurement equations on the form

ϕ=- 2πτ0f+m                                                        (1)

where m collects different parameters that does not depend on the frequency (initial phase offset, integer ambiguity, measurement noise, etc.). Notice that τ0 include a clock-offset between the transmitter and the receiver. For the two frequencies fc and fa both inside the PFL, equation (1) gives that

ϕa=ϕc- 2πτ0Δf                                                           (2)

where Δf=fa-fc. This relation can be used (by LMF or the Target UE) to reconstruct the phase at any frequency fa from {ϕc, τ0}, which enables the application of the virtual frequency method for integer ambiguity. The same principle can be used for phase-difference of arrival measurements: From (2) we obtain

ϕa-ϕa*=ϕc-ϕc*- 2π(τ0-τ0*)Δf                                   (3)

where superscript * is used for the reference TRP.

### Evaluation: Phase as function of frequency and time-of-arrival

We simulate the transmission of a reference signal in an AWGN channel and evaluate the phase estimation accuracy of a receiver. Although the evaluations are performed with an AWGN channel, the results apply to any situation where the first path of the channel impulse response can be singled out because then the linear equation (1) hold. We compare the following three different methods:

Method 1, Subcarrier measurement: There is a subcarrier with frequency fa and ϕa,M1 is the measured phase of that subcarrier. Requires additional reporting.

Method 2, Segment of bandwidth: The bandwidth of the reference signal is split into two equally large segments and fa is the center frequency of the first segment. The phase estimate ϕa,M2 is obtained from all the subcarrier-measurements of this segment. Requires additional reporting.

Method 3, phase as function of frequency and time-of-arrival: The phase of the center frequency ϕc is reported and τ0 is estimated from the full bandwidth of the reference signal. Equation (2) is used to obtain the phase estimate ϕa,M3.

The simulation assumptions are summarized in Table 1. All methods are unbiased and their performances are evaluated from the empirical standard deviation over 1000 noise realizations, see Table 2. Method 3 performs best, then comes Method 2 and Method 1 is the worst. Additional figures and descriptions are presented in Appendix A.

Table 1: Simulation assumptions

| Channel model | Single path AWGN |
| --- | --- |
| Deployment | One transmitting cell and one receiving UE 100 meters apart. |
| Carrier frequency / BW / SCS /duplex mode | 3.5 GHz / 20 MHz / 30 kHz /TDD |
| Reference signal | Equivalent to: DL PRS, comb-2, repetition factor 1 |
| SNR | -3dB |
| Number of TRP/UE receive antennas | 1 tx and 1 rx cross polarization, isotropic |

Table 2: Phase estimation accuracy. The estimation accuracy is computed from 1000 noise realizations.

| Method 1 | std ϕa,M1=1.30 rad |
| --- | --- |
| Method 2 | std ϕa,M2=0.091 rad |
| Method 3 | std ϕa,M3=0.087 rad |

The phase ϕa at frequency fa within the PFL can be obtained from the phase of the center frequency ϕc and the time-of-arrival τ0 (or equivalently RSTD, RTOA, etc.)

When ϕa is obtained from {ϕc, τ0} then it is more accurate and less noisy than when ϕa is a reported subcarrier measurement, or when ϕa has been calculated from a segment of the bandwidth.

A phase measurement shall be defined and reported for the central carrier frequency.

Do not introduce reporting of phase measurements at more than one frequency within the PFL.

Measurement reporting

RAN1#112:

Agreement

For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF

- FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements

Agreement

Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Stand-alone carrier phase reporting or joint reporting with legacy positioning measurements

We propose that carrier phase measurements should be reported jointly with either DL-TDOA or UL-TDOA but not as a stand-alone procedure. This can be motivated from both an accuracy and a technical feasibility point of view:

- During the study item, most companies evaluated joint positioning using carrier phase measurements and some other positioning method. The integer ambiguity problem simplifies significantly if e.g. DL-TDOA can be employed to obtain a first, coarse, position estimate, to reduce the search space of a multi-hypothesis integer resolution algorithm.

- In other cases, companies simply limited the integer ambiguity problem to the true integer vector N plus some uncertainty margin. The technical feasibility of a stand-alone solution without prior information of the UE position has not been demonstrated during the SI.

Additionally, since we have agreed to define carrier phase measurements for the first path and since time-of-arrival estimation is needed to single out that path, there is little or no additional computations needed to obtain those as well. It makes no sense that a receiver computes time-of-arrival and carrier phase for a path and then only reports the carrier phase measurement.

Finally, the specification impact for joint reporting (with DL-TDOA, UL-TDOA) is minimal while a stand-alone solution would require a much larger specification effort that is not supported by the WID.

Add support to report DL PRS RSCPD for the first path with the DL-TDOA positioning method.

Add support to report UL SRS RSCP for the first path with the UL-TDOA positioning method.

Assistance information for double differentiation

## Downlink UE-based

The double-differentiation technique was evaluated during the SI, often successfully. For UE based carrier phase-based positioning, LMF needs to provide the target UE with assistance data to enable double-differentiation or phase calibration. We see two alternative formats of the assistance data:

- PRU measurements (of the carrier phase) for different network TRPs and the PRU position. With this format, the target UE can perform differentiation with respect to the PRU measurements to cancel out the Tx initial phase offset.

- Correction factors for the Tx initial phase offsets of TRPs. When the correction factors are applied by the target UE, all TRPs appear phase synchronized.

To better understand these two alternatives, consider the measurement model

λϕi k=- dik+λ Nik+λϕi0-ϕk0                                                   (1)

for transmitter i  and receiver k. Here dik,Nik, ϕi k are the distance between the transmitter and receiver, the integer number of wavelengths resp. the carrier phase measurement between them. The Rx initial phase offset is ϕk0 and the Tx initial phase offset is ϕi0. The Rx phase offset is cancelled out by computing the phase-difference-of-arrival at the target UE.

Alternative 1:

Consider Figure 1 including TRPs i,j, target UE k and PRU K. With Alternative 1 the assistance data to the target UE is

- The PRU measurements {ϕi K, ϕjK}

- The PRU position.

Given the exact PRU position and TRP positions, the target UE can compute the distances diK, djK as well as the integers NiK, NjK. The target UE computes the difference λϕi k- λϕi K to cancel out the Tx phase offset of TRP i (and similarly for TRP j),

λϕi k- λϕi K= -(dik-diK)+λ Nik-NiK+λϕi0-ϕk0 - ( ϕi0-ϕK0)       (2)

Notice that the PRU Rx phase offset, ϕK0, appears in the equation. This is not a problem since it will be cancelled in the second differentiation with respect to different TRPs (i.e. the phase-difference-of-arrival).

Figure 1: Double-differentiation example.

Alternative 2:

Reformulate eq. (2),

λ ϕi k-ϕi K+ diK/ λ- NiK = -dik+λNik+λϕi0-ϕk0 - ( ϕi0-ϕK0)   (3)

Define the correction factor for TRP i as

ϕi= ϕi K+ diK/ λ- NiK.

The correction factor can be computed by LMF for each TRP. With Alternative 2, the assistance data to the target UE is

- The phase correction factors ϕi, ϕj.

Since the PRU location does not need to be sent to the target UE, the signaling overhead of Alternative 2 is smaller.

To enable double-differentiation for UE-based, downlink CPP, LMF need to provide the UE with assistance data. The assistance data can be either
Alternative 1) PRU measurements and PRU position, or
Alternative 2) Phase correction factors.
The signaling overhead of Alt. 1 is larger than it is for Alt. 2.

One problem that need to be handled is the case where there are multiple nearby reference units, then which one(s) should be used? With Alternative 2, LMF can combine the measurements from multiple PRUs into one single set of compensation factors, for instance by averaging: Let nK be the number of available PRUs, then compute the compensation factors

ϕi= 1nKK=1,…,nKϕi K+ diK/ λ- NiK.

With Alternative 2, LMF can use measurements from multiple PRUs to compute phase correction factors.

If there are multiple reference units and their measurements are inconsistent, then LMF can select the most trustworthy or relevant measurements as a basis for the correction factors. Since both Observation 4 and Observation 5 speak in favor of Alternative 2, we propose:

LMF can provide a Target UE with phase correction factors to enable double differentiation.

Phase correction factors may have a limited validity time. The time-aspect was not sufficiently studied during the SI, but for GNSS RTK, real-time streaming of correction factors is needed to reach the best accuracy.

Phase corrections have a limited validity time. The time-variability was not studied during the Rel. 18 Positioning SI.

Consequently, we propose that it should be possible to provide updated correction factor with variable frequency. Whenever a new set of correction factors are available to LMF, then these can be sent to the UE.

It should be possible to provide a UE with updated correction factor with variable frequency.

Example: A simple algorithm to provide updated correction factors to the UE can be as follows:

- Initial state: The correction factors ϕ have been provided to the UE.

- for each time-instance t

- LMF obtains an updated set of correction factors, ϕ(t)

- if   ϕ(t)-ϕ >δ then ϕ≔ ϕ(t) and send the updated phase corrections ϕ to the UE. The parameter δ is a threshold.

To reduce the signaling overhead, LMF can skip sending an update if the new correction factors are only marginally different compared to the previous set of correction factors. There can be a threshold specified that defines when updated correction factors need to be sent to the UE.

Example:

The vector with correction factors ϕ(1) is valid during time-period [t0 t1], the correction factors ϕ(2) is valid during [t1 t2], etc. Several vectors ϕ(1),ϕ(2),… are reported to the UE in one single batch.

For applications that are less latency-critical another method to reduce the signaling overhead is to provide phase correction factors in batches where the absolute validity time of each set of correction factors is specified.

Investigate how phase correction factors can be provided in a resource efficient way.

Assistance data for error source mitigation

## Carrier phase measurement differentials and Rx/Tx branches

The notion of Rx- and Tx- timing error groups (TEGs) was introduced during 3GPP Release 17 to handle the fact that timing measurements performed with different antenna panels may be subject to different timing errors. We expect similar limitations for carrier phase measurements.

Carrier phase measurement differentials (single- or double-differentials) may not be useful on measurements performed with different Rx/Tx branches or Rx/Tx TEGs.

In Rel. 17 we added support to associate positioning transmissions and measurements to Rx- resp. Tx-TEGs and each Rx- and Tx-TEG is associated with a timing error margin. A straight-forward solution for carrier phase measurements is to replicate the framework for “Phase Error Groups”, PEGs. That is, to associate carrier phase transmissions and measurements with a PEG identity and define a phase error margin for each PEG.

The main direction for CPP positioning is however to perform carrier phase measurements together with e.g. DL RSTD measurements and report them together, meaning that the same antenna panel/elements are used for both measurements. It seems unnecessary to introduce a separate Rx-PEG identity for the same thing: there will be a one-to-one mapping between Rx-PEG and Rx-TEG. Same thing with Tx: since the same reference signals is used for both measurements there would be a one-to-one mapping between Tx-PEG and Tx-TEG.

As an alternative, we propose to reinterpret the definition of a “timing error group” so that it captures both timing and phase errors. Besides the timing-error margin, we propose to associate a phase-error margin to each Rx- and Tx-TEG.

There is no need to introduce “Phase error groups”, PEGs, since there would a one-to-one mapping between PEG and TEG.

Reinterpret the definition of a “timing error group” so that it captures both timing and phase errors.

Associate a phase-error margin to each Rx and Tx TEG.

RAN1#112:

Agreement

To support NR carrier phase positioning, further consider the following options:

- …

- Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF

- Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

Uncertainty range

Option 3 corresponds to a range uncertainty that the UE/TRP can report together with its measurements to LMF. Option 4 is instead assistance data that LMF can provide to the UE. For joint carrier phase reporting (with DL-TDOA resp. UL-TDOA) we already have mechanisms to report RSTD/RTOA uncertainty that can be straightforwardly translated into an uncertainty in number of wavelengths.

More specifically, consider Option 4 and the case of downlink phase-difference-of-arrival with one UE and k TRPs. Let TRP 1 be the reference TRP. Let N=[N1, N2, …Nk] be the expected integer wavelengths between the UE and the TRPs, and δN=[δN1, δN2, …δNk] the uncertainty. The uncertainty range is thus given by N±δN. With phase-difference-of-arrival it will always be the difference Ni-N1, i=2,…,k, appearing in the equations, that is the integer-difference with respect to the reference TRP. For the case that phase-difference-of-arrival is used, then it is more natural to report the integer-difference ambiguity range.

For the case that phase-difference-of-arrival is used, then it is more natural to report the integer-difference ambiguity range.

However, we can already obtain the integer-difference ambiguity from the Rel. 17 DL-TDOA assistance data: In LPP there is the assistance data from LMF to UE:

- NR-DL-PRS-AssistanceDataPerTRP-r16 including IEs

  - nr-DL-PRS-ExpectedRSTD-r16

  - nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

Together, these two IEs provide RSTD uncertainty range for a TRP. It is straight-forward to translate this into an interger difference ambiguity range (by multiplication by the speed of light and divition with the wavelength). In uplink we have similarly (in NRPPa) the assistance data, from LMF to NG-RAN node

- Search Window Information

  - Expected Propagation Delay

  - Delay Uncertainty

The integer ambiguity range in Option 4 can be deduced from the RSTD resp. RTOA uncertainty intervals standardized in Rel. 17.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The phase ϕa at frequency fa within the PFL can be obtained from the phase of the center frequency ϕc and the time-of-arrival τ0 (or equivalently RSTD, RTOA, etc.)

Observation 2	When ϕa is obtained from {ϕc, τ0} then it is more accurate and less noisy than when ϕa is a reported subcarrier measurement, or when ϕa has been calculated from a segment of the bandwidth.

Observation 3	To enable double-differentiation for UE-based, downlink CPP, LMF need to provide the UE with assistance data. The assistance data can be either Alternative 1) PRU measurements and PRU position, or Alternative 2) Phase correction factors. The signaling overhead of Alt. 1 is larger than it is for Alt. 2.

Observation 4	With Alternative 2, LMF can use measurements from multiple PRUs to compute phase correction factors.

Observation 5	Phase corrections have a limited validity time. The time-variability was not studied during the Rel. 18 Positioning SI.

Observation 6	Carrier phase measurement differentials (single- or double-differentials) may not be useful on measurements performed with different Rx/Tx branches or Rx/Tx TEGs.

Observation 7	There is no need to introduce “Phase error groups”, PEGs, since there would a one-to-one mapping between PEG and TEG.

Observation 8	For the case that phase-difference-of-arrival is used, then it is more natural to report the integer-difference ambiguity range.

Observation 9	The integer ambiguity range in Option 4 can be deduced from the RSTD resp. RTOA uncertainty intervals standardized in Rel. 17.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only support carrier phase measurements of the first detected path.

Proposal 2	For DL carrier phase measurements, use the same reference point as DL-RSTD: For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE.

Proposal 3	For UL carrier phase measurements, use the same reference point as UL-RTOA.

Proposal 4	In downlink, support RSCPD of the first path but not for additional paths.

Proposal 5	In uplink, support RSCP of the first path but not for additional paths.

Proposal 6	A phase measurement shall be defined and reported for the central carrier frequency.

Proposal 7	Do not introduce reporting of phase measurements at more than one frequency within the PFL.

Proposal 8	Add support to report DL PRS RSCPD for the first path with the DL-TDOA positioning method.

Proposal 9	Add support to report UL SRS RSCP for the first path with the UL-TDOA positioning method.

Proposal 10	LMF can provide a Target UE with phase correction factors to enable double differentiation.

Proposal 11	It should be possible to provide a UE with updated correction factor with variable frequency.

Proposal 12	Investigate how phase correction factors can be provided in a resource efficient way.

Proposal 13	Reinterpret the definition of a “timing error group” so that it captures both timing and phase errors.

Proposal 14	Associate a phase-error margin to each Rx and Tx TEG.

# References

3GPP RP-213561, New SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e, Dec. 6-17, 2021

3GPP TR 38.901 V17.0.0 (2022-03) Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)

Paul de Jonge and Christian Tiberius, The LAMBDA method for integer ambiguity estimation: implementation aspects. Publications of the Delft Geodetic Computing Centre, 1996.

Jay A. Farrel, Aided Navigation. Mc Graw Hill, 2006

3GPP RP-223549, New WID on Expanded and Improved NR Positioning, 3GPP TSG RAN Meeting #98-e, Dec. 12-16, 2022

3GPP TR 38.859 V18.0.0, Study on expanded and improved NR positioning (Release 18)

R1-2212515, Improved accuracy based on NR carrier phase measurements, Ericsson. 3GPP TSG-RAN WG1 #111

R1-2211312, Views on improved accuracy based on NR carrier phase measurements, Nokia. 3GPP TSG RAN WG1#111

R1-2212124, Phase Measurements in NR Positioning, Qualcomm Incorporated. 3GPP TSG RAN WG1#111

R1-2211205, Further discussion on improved accuracy based on NR carrier phase measurement, CATT, RAN WG1#111

R1-2205870, Evaluation and solutions for NR carrier phase positioning, Huawei, RAN WG1#110

Simulations: Phase measurement accuracy

By correlating the received signal with the known reference signal waveform, the receiver estimates the channel at each subcarrier. The phase at each subcarrier is plotted in Figure 2 for one noise realization. Notice that an individual subcarrier measurement can be very noisy, hence we expect that Method 1 will be quite inaccurate. The frequency fa and phase ϕa,M1 are highlighted in the figure.

Figure 2: The phases of individual subcarriers.

With Method 2, ϕa,M2 is estimated from the subcarrier measurements of the first half of the bandwidth, see Figure 2. More specifically, a linear function is fitted to the data points (wrapped to the interval -2π, 0 ) and ϕa,M2 is obtained from that.

Figure 3: The phase at fa estimated from a segment of the bandwidth.

With Method 3, ϕa,M3 is estimated from all the subcarrier measurements of the full reference signal bandwidth. A linear function parametrized in { ϕc, τ0} is fitted to the data and ϕa,M3 is obtained using eq (1).

Figure 4: Method 3, the phase at fa estimated from { ϕc, τ0}, using eq. (1).